3GPP TSG-RAN4 Meeting # 114	R4-2500638

# Athens, Greece, 17th – 21nd Feb, 2025

Title:	NTN and IoT-NTN HPUE co-existence study and calibration

Source:	Xiaomi

Agenda item:	7.10.2.1

Document for:	Discussion

- Introduction

In the RAN plenary meeting RAN#104, there was an agreement for a new Rel-19 RAN4 led WI Enhanced requirements and test methodology for NR and IoT NTN [1]. The baseline assumptions agreed in RAN4#112bis meeting have been captured in WF [2]. In this document, Xiaomi provides the simulation study results of the follow scenarios:

- NR-NTN PC1 Scenario 4

- NR-NTN PC1.5 Scenario 4

- NR-NTN PC2 Scenario 4

- IoT-NTN PC1 Scenario 4

- IoT-NTN PC2 Scenario 4

and the calibration results of 2GHz TN/NTN which caputured in the TR38.863[3].

- Cailibration

In order to make sure the correction of the simulation results, Xiaomi provides the coupling loss and SINR calibration results basic on the TR 38.863 Annex A of 2GHz TN which is missing in the calibration results. Calibration detail can be found in the Annex A of this document.

Observation 1: Xiaomi’s calibration results of TN scenarios align well with the TR 38.863 Annex A.

Proposal 1: Propose the meeting the approve the calibration results and Xiaomi’s simulation results of this WI.

- NR-NTN and IoT-NTN HPUE co-existence study

In this clause, Xiaomi provides the coexistence simulation results for NR-NTN and IoT-NTN high power UE.

3.1 Simulation results for NR-NTN HPUE

NR-NTN PC1 Scenario 4:

|  | Isolation | Wrap-around | Statistics | Deployment | NR BS Type | SAN Orbit | ACLR/ACIR | Requirement | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average | 1.5 | Y | TN cluster | Urban | non-AAS | LEO-1200 | ACIR | 18.5 |  |  |  |  | 7.54 | 5.33 | 3.9 | 2.8 |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 17.3 |  |  |  | 8.06 | 5.98 | 4.51 | 3.21 |  |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | GEO | ACIR | 18.8 |  |  |  | 10.42 | 7.61 | 5.58 | 4.12 | 2.94 |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | ACIR | 17.6 |  |  | 10.9 | 8.34 | 6.31 | 4.65 | 3.48 | 2.59 |
|  |  |  |  |  |  |  |  |  | 26 | 28 | 30 | 32 |  |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 29.3 | 7.27 | 5.8 | 4.57 | 3.57 |  |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | GEO | ACIR | 29.2 | 7.11 | 5.73 | 4.54 | 3.59 |  |  |  |  |
|  | Isolation | Wrap-around | Statistics | Deployment | NR BS Type | SAN Orbit | ACLR/ACIR | Requirement | 14 | 16 | 18 | 20 | 22 | 24 | 26 | 28 |
| 5%-tilt | 1.5 | Y | TN cluster | Urban | non-AAS | LEO-1200 | ACIR | 23.3 |  |  |  | 11.18 | 6.69 | 4.02 | 2.52 |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 22.7 |  |  |  |  | 8.78 | 5.67 | 3.59 | 2.29 |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | GEO | ACIR | 22.4 |  |  |  | 9.3 | 5.48 | 3.31 | 2.07 |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | ACIR | 25.1 |  |  |  |  | 9.76 | 6.3 | 3.98 | 2.52 |
|  |  |  |  |  |  |  |  |  | 34 | 36 | 38 | 40 |  |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 37.3 | 10.05 | 6.5 | 4.14 | 2.65 |  |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | GEO | ACIR | 37.2 | 9.94 | 6.31 | 4.07 | 2.6 |  |  |  |  |

Observation 1: For NR-NTN PC1 Scenario 4, if the isolation distance was considered in the study, ACIR requirement would be 25.1dB. Otherwise, the ACIR requirement would be 37.3dB.

NR-NTN PC1.5 Scenario 4:

|  | Isolation | Wrap-around | Statistics | Deployment | NR BS Type | SAN Orbit | ACLR/ACIR | Requirement | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average | 1.5 | Y | TN cluster | Urban | non-AAS | LEO-1200 | ACIR | 15.8 |  |  | 9.84 | 6.79 | 4.83 | 3.5 |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 15.3 |  |  | 8.08 | 6.07 | 4.46 | 3.26 |  |  |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | GEO | ACIR | 16.3 |  |  |  | 7.45 | 5.23 | 3.66 | 2.22 |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | ACIR | 16.0 |  |  | 8.59 | 6.6 | 4.96 | 3.72 |  |  |
|  |  |  |  |  |  |  |  |  | 26 | 28 | 30 | 32 | 34 | 36 | 38 |  |
|  | 0 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 32.5 | 10.31 | 8.33 | 6.68 | 5.26 | 4.11 | 3.26 | 2.5 |  |
|  | 0 | Y | TN cluster | Urban | AAS | GEO | ACIR | 32.4 |  |  | 6.61 | 5.21 | 4.07 | 3.12 |  |  |
|  | Isolation | Wrap-around | Statistics | Deployment | NR BS Type | SAN Orbit | ACLR/ACIR | Requirement | 12 | 14 | 16 | 18 | 20 | 22 | 24 | 26 |
| 5%-tilt | 1.5 | Y | TN cluster | Urban | non-AAS | LEO-1200 | ACIR | 20.2 |  |  | 14.99 | 8.22 | 5.25 | 3.23 |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 22.8 |  |  |  | 13.98 | 9.01 | 5.8 | 3.72 | 2.35 |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | GEO | ACIR | 20.5 |  |  |  | 9.22 | 5.5 | 3.5 | 2.19 |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | ACIR | 23.2 |  |  |  |  | 9.94 | 6.36 | 4.03 | 2.57 |
|  |  |  |  |  |  |  |  |  | 36 | 38 | 40 | 42 | 44 |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 40.4 | 13.03 | 8.39 | 5.43 | 3.41 | 2.1 |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | GEO | ACIR | 40.1 | 12.33 | 7.99 | 5.08 | 3.25 | 2.07 |  |  |  |

Observation 2: For NR-NTN PC1.5 Scenario 4, if the isolation distance was considered in the study, ACIR requirement would be 23.2dB. Otherwise, the ACIR requirement would be 40.4dB.

NR-NTN PC2 Scenario 4:

|  | Isolation | Wrap-around | Statistics | Deployment | NR BS Type | SAN Orbit | ACLR/ACIR | Requirement | 6 | 8 | 10 | 12 | 14 | 16 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average | 1.5 | Y | TN cluster | Urban | non-AAS | LEO-1200 | ACIR | 12.9 |  |  | 8.03 | 5.71 | 4.06 | 2.95 |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 12.6 |  |  | 7.15 | 5.44 | 3.99 | 2.88 |  |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | GEO | ACIR | 12.8 |  |  | 8.05 | 5.74 | 3.96 | 2.76 |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | ACIR | 14.3 |  |  |  | 6.96 | 5.18 | 3.84 | 2.77 |
|  |  |  |  |  |  |  |  |  | 32 | 34 | 36 | 38 |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 34.7 | 6.81 | 5.43 | 4.22 | 3.34 |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | GEO | ACIR | 34.3 | 6.47 | 5.13 | 4.09 | 3.26 |  |  |  |
|  | Isolation | Wrap-around | Statistics | Deployment | NR BS Type | SAN Orbit | ACLR/ACIR | Requirement | 10 | 12 | 14 | 16 | 18 | 20 | 22 |
| 5%-tilt | 1.5 | Y | TN cluster | Urban | non-AAS | LEO-1200 | ACIR | 16.8 |  |  | 9.65 | 5.85 | 3.69 | 2.35 |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 19.6 |  |  |  | 11.07 | 7.21 | 4.52 | 2.88 |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | GEO | ACIR | 17.5 |  |  | 11.52 | 7.16 | 4.26 | 2.59 |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | ACIR | 19.6 |  |  |  | 10.74 | 6.96 | 4.44 | 2.83 |
|  |  |  |  |  |  |  |  |  | 38 | 40 | 42 | 44 |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 42.6 | 13.66 | 8.78 | 5.56 | 3.54 |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | GEO | ACIR | 42.3 | 12.81 | 8.26 | 5.27 | 3.39 |  |  |  |

Observation 3: For NR-NTN PC2 Scenario 4, if the isolation distance was considered in the study, ACIR requirement would be 19.6dB. Otherwise, the ACIR requirement would be 42.6dB.

Observation 4: Isolation distance is necessary to protect the TN network from NR-NTN HPUE.

3.2 Simulation results for IoT-NTN HPUE

IoT-NTN PC1 Scenario 4:

|  | Isolation | Wrap-around | Statistics | Deployment | NR BS Type | SAN Orbit | SCS / UE# | ACLR/ACIR | Requirement | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 | 38 | 40 | 42 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average | 1.5 | Y | TN cluster | Urban | non-AAS | LEO-1200 | 15kHz / 9 | ACIR | 23.3 |  | 8.38 | 6.15 | 4.44 | 3.05 |  |  |  |  |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | LEO-1200 | 3.75kHz / 36 | ACIR | 31.5 |  |  |  |  | 12.58 | 9.1 | 6.45 | 4.57 | 3.25 |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | 15kHz / 9 | ACIR | 22.3 | 9.125 | 7.01 | 5.22 | 3.89 | 2.83 |  |  |  |  |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | 3.75kHz / 36 | ACIR | 30.9 |  |  |  |  |  | 7.82 | 5.7 | 4.06 | 2.89 |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | GEO | 15kHz / 9 | ACIR | 27.3 |  |  |  | 8.29 | 6.1 | 4.45 | 2.95 |  |  |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | GEO | 3.75kHz / 36 | ACIR | 40 |  |  |  |  |  |  |  |  |  | 9.91 | 7.13 | 4.89 | 3.5 |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | 15kHz / 9 | ACIR | 26.4 |  |  |  | 7.1 | 5.32 | 3.87 | 2.86 |  |  |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | 3.75kHz / 36 | ACIR | 39.7 |  |  |  |  |  |  |  |  | 11.7 | 8.81 | 6.48 | 4.72 | 3.35 |
|  | Isolation | Wrap-around | Statistics | Deployment | NR BS Type | SAN Orbit | SCS / UE# | ACLR/ACIR | Requirement | 24 | 26 | 28 | 30 | 32 | 34 | 36 | 38 | 40 | 42 | 44 | 46 | 48 |
| 5%-tilt | 1.5 | Y | TN cluster | Urban | non-AAS | LEO-1200 | 15kHz / 9 | ACIR | 27.8 | 12.45 | 7.62 | 4.77 | 3.06 |  |  |  |  |  |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | LEO-1200 | 3.75kHz / 36 | ACIR | 35.8 |  |  |  |  | 13.13 | 7.69 | 4.7 | 2.9 |  |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | 15kHz / 9 | ACIR | 29.5 |  | 10.7 | 6.98 | 4.37 | 2.76 |  |  |  |  |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | 3.75kHz / 36 | ACIR | 37.4 |  |  |  |  |  | 10.51 | 6.7 | 4.29 | 2.74 |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | GEO | 15kHz / 9 | ACIR | 31.4 |  |  |  | 6.97 | 4.23 | 2.68 | 1.67 |  |  |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | GEO | 3.75kHz / 36 | ACIR | 45.2 |  |  |  |  |  |  |  |  |  | 10.43 | 6.56 | 3.97 | 2.52 |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | 15kHz / 9 | ACIR | 33.6 |  |  |  | 11.03 | 6.99 | 4.43 | 2.79 |  |  |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | 3.75kHz / 36 | ACIR | 46.1 |  |  |  |  |  |  |  |  |  |  | 8.06 | 5.13 | 3.28 |

Observation 5: For IoT-NTN PC1 Scenario 4, if the 3.75kHz / 36 SCS / UE# assumption was considered in the study, ACIR requirement would be 46.1dB.

Observation 6: if the 15kHz / 9 SCS / UE# assumption was considered in the study, ACIR requirement would be 33.6dB.

IoT-NTN PC2 Scenario 4:

|  | Isolation | Wrap-around | Statistics | Deployment | NR BS Type | SAN Orbit | SCS / UE# | ACLR/ACIR | Requirement | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 | 38 | 40 | 42 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average | 1.5 | Y | TN cluster | Urban | non-AAS | LEO-1200 | 15kHz / 9 | ACIR | 23.2 |  | 8.38 | 5.98 | 4.29 | 2.91 |  |  |  |  |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | LEO-1200 | 3.75kHz / 36 | ACIR | 30.9 |  |  |  |  |  | 8.52 | 5.9 | 3.83 | 2.56 |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | 15kHz / 9 | ACIR | 22.5 | 9.12 | 7 | 5.35 | 3.9 |  |  |  |  |  |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | 3.75kHz / 36 | ACIR | 31.2 |  |  |  |  |  | 8.34 | 6.08 | 4.32 | 3.06 |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | GEO | 15kHz / 9 | ACIR | 27.4 |  |  |  | 8.46 | 6.07 | 4.53 | 3.19 | 2.35 |  |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | GEO | 3.75kHz / 36 | ACIR | 40.3 |  |  |  |  |  |  |  |  |  | 10.2 | 7.52 | 5.23 | 3.72 |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | 15kHz / 9 | ACIR | 26.1 |  |  |  | 6.75 | 5.06 | 3.74 | 2.66 |  |  |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | 3.75kHz / 36 | ACIR | 39.9 |  |  |  |  |  |  |  |  |  | 9.15 | 6.8 | 4.92 | 3.63 |
|  | Isolation | Wrap-around | Statistics | Deployment | NR BS Type | SAN Orbit | SCS / UE# | ACLR/ACIR | Requirement | 24 | 26 | 28 | 30 | 32 | 34 | 36 | 38 | 40 | 42 | 44 | 46 | 48 |
| 5%-tilt | 1.5 | Y | TN cluster | Urban | non-AAS | LEO-1200 | 15kHz / 9 | ACIR | 27.9 | 12.8 | 7.79 | 4.84 | 3.01 |  |  |  |  |  |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | 3.75kHz / 36 | ACIR | 35.1 |  |  |  |  | 10.57 | 6.26 | 3.96 | 2.5 |  |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | GEO | 15kHz / 9 | ACIR | 29.5 |  | 10.47 | 6.8 | 4.35 | 2.79 |  |  |  |  |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | 3.75kHz / 36 | ACIR | 37.6 |  |  |  |  |  | 11.19 | 7.12 | 4.53 | 2.88 |  |  |  |  |
|  | 0 | Y | TN cluster | Rural | non-AAS | LEO-1200 | 15kHz / 9 | ACIR | 31.3 |  |  | 11.89 | 6.85 | 4.08 | 2.55 |  |  |  |  |  |  |  |
|  | 0 | Y | TN cluster | Rural | AAS | LEO-1200 | 3.75kHz / 36 | ACIR | 45.0 |  |  |  |  |  |  |  |  |  | 9.89 | 6.08 | 3.86 | 2.26 |
|  | 0 | Y | TN cluster | Rural | non-AAS | GEO | 15kHz / 9 | ACIR | 33.4 |  |  |  | 10.35 | 6.69 | 4.3 | 2.69 |  |  |  |  |  |  |
|  | 0 | Y | TN cluster | Rural | AAS | GEO | 3.75kHz / 36 | ACIR | 45.9 |  |  |  |  |  |  |  |  |  | 12.45 | 7.91 | 4.92 | 3.1 |

Observation 7: For IoT-NTN PC2 Scenario 4, if the 3.75kHz / 36 SCS / UE# assumption was considered in the study, ACIR requirement would be 45.9dB.

Observation 8: if the 15kHz / 9 SCS / UE# assumption was considered in the study, ACIR requirement would be 33.4dB.

Proposal 1: Propose RAN4 consider the simulation results in this document during the discussion of the ACIR requirement.

- Conclusion

Observation 1: For NR-NTN PC1 Scenario 4, if the isolation distance was considered in the study, ACIR requirement would be 25.1dB. Otherwise, the ACIR requirement would be 37.3dB.

Observation 2: For NR-NTN PC1.5 Scenario 4, if the isolation distance was considered in the study, ACIR requirement would be 23.2dB. Otherwise, the ACIR requirement would be 40.4dB.

Observation 3: For NR-NTN PC2 Scenario 4, if the isolation distance was considered in the study, ACIR requirement would be 19.6dB. Otherwise, the ACIR requirement would be 42.6dB.

Observation 4: Isolation distance is necessary to protect the TN network from NR-NTN HPUE.

Observation 5: For IoT-NTN PC1 Scenario 4, if the 3.75kHz / 36 SCS / UE# assumption was considered in the study, ACIR requirement would be 46.1dB.

Observation 6: if the 15kHz / 9 SCS / UE# assumption was considered in the study, ACIR requirement would be 33.6dB.

Observation 7: For IoT-NTN PC2 Scenario 4, if the 3.75kHz / 36 SCS / UE# assumption was considered in the study, ACIR requirement would be 45.9dB.

Observation 8: if the 15kHz / 9 SCS / UE# assumption was considered in the study, ACIR requirement would be 33.4dB.

Proposal 1: Propose RAN4 consider the simulation results in this document during the discussion of the ACIR requirement.

- Reference

[1]	RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 chair (Huawei)

[2]	R4-2417122,”WF on co-existence study and HPUE for NR-NTN”, 3GPP RAN4#112bis, October 2024

[3]	TR38.863-i30

- Annex A: Calibration results of TN components

Calibration assumptions at 2GHz

Assumptions in Section 6.2 are adopted as the baseline for calibration. It should be noted there are different parts which are listed in Table B.1-1.

Table B.1-1: TN Assumptions for calibration

| Calibration assumptions |  |  |
| --- | --- | --- |
| TN AAS | Rural | Element gain: 7.1 dBi |
|  |  | 3dB: H 90 / V 54 |
|  |  | Front-back: 30 H/V |
|  |  | Array: 8x8 |
|  |  | Element spacing: H 0.5/V 0.9 |
|  |  | Conducted Tx: 25 dBm |
|  |  | Ohmic loss: 2 dB |
|  |  | Mechanical downtilt: 3 deg |
|  |  | Polarization gain 3 dB |
|  |  | #UE: 1 DL/ 3 UL |
|  |  | 100% Outdoor |
|  |  | ISD 7.5 KM |
|  | Urban | Element gain: 6.4 dBi |
|  |  | 3dB: H 90 / V 65 |
|  |  | Front-back: 30 H/V |
|  |  | Array: 8x8 |
|  |  | Element spacing: H 0.5/V 0.7 |
|  |  | Conducted Tx: 25 dBm |
|  |  | Ohmic loss: 2 dB |
|  |  | Mechanical downtilt: 10 deg |
|  |  | Polarization gain 3 dB |
|  |  | #UE: 1 DL/ 3 UL |
|  |  | 100% Outdoor |
|  |  | ISD 0.75 KM |
| TN non-AAS | Antenna gain | 17 dBi |
|  | Conducted Tx | 46 dBm |
|  | 3dB | R4-2108645 Section 2.4.2 |
|  | Front-back |  |
|  | Mechanical downtilt | Rural 3 / Urban 10 |

B.2	Calibration results at 2GHz

The calibration results include SINR and coupling loss distributions in DL, and SINR, coupling loss and transmit power distributions in UL. Both rural macro and urban macro propagation models defined in TR 38.901[10] are considered. NR BS with AAS and non-AAS antennas are calibrated separately and results are summarized in Table B.2-1, 2-2 and 2-3. NB-IoT scenarios are not calibrated.

NOTE:	The “-” means the data was not provided by its corresponding contributor.

Table B.2-1: Calibration summary for TN NR AAS DL cases

| Calibration metrics |  | DL Coupling Loss |  |  | DL Geometry SINR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CDF percentile |  | @5% | @50% | @95% | @5% | @50% | @95% |
| Urban Macro | Samsung | 60.05 | 88.58 | 98.98 | 8.47 | 24.01 | 45.77 |
|  | Huawei | 52.84 | 85.79 | 103.53 | 5.75 | 22.23 | 43.40 |
|  | Qualcomm | 57.89 | 87.11 | 100.61 | 5.08 | 19.51 | 43.65 |
|  | Nokia | 59.04 | 87.59 | 99.32 | 12.42 | 20.15 | 36.50 |
|  | Ericsson | - | - | - | 6.66 | 19.58 | 39.57 |
|  | ZTE | 58.76 | 86.90 | 99.74 | 5.63 | 20.56 | 45.14 |
|  | Variance | 6.42 | 0.83 | 2.69 | 6.35 | 2.62 | 10.70 |
|  | Mean | 57.72 | 87.19 | 100.44 | 7.33 | 21.01 | 42.34 |
|  | Xiaomi | 56.57 | 85.75 | 98.79 | 2.98 | 19.14 | 43.60 |
| Rural Macro | Samsung | 78.09 | 116.25 | 127.96 | 4.52 | 16.53 | 37.71 |
|  | Huawei | 77.53 | 114.99 | 128.13 | 3.02 | 15.15 | 31.35 |
|  | Qualcomm | 78.27 | 116.81 | 131.58 | 0.17 | 12.60 | 35.83 |
|  | Nokia | 78.55 | 116.18 | 127.72 | 5.60 | 14.93 | 28.23 |
|  | Ericsson | - | - | - | 10.03 | 20.17 | 37.72 |
|  | Variance | 0.14 | 0.44 | 2.51 | 10.51 | 6.21 | 14.21 |
|  | Mean | 78.11 | 116.06 | 128.85 | 4.67 | 15.87 | 34.17 |
|  | Xiaomi | 75.87 | 114.67 | 128.95 | 0.20 | 12.99 | 36.66 |

Table B.2-2: Calibration summary for TN NR non-AAS DL cases

| Calibration metrics |  | DL Coupling Loss |  |  | DL Geometry SINR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CDF percentile |  | @5% | @50% | @95% | @5% | @50% | @95% |
| Urban Macro | Samsung | 73.23 | 99.36 | 110.33 | -1.76 | 6.99 | 14.85 |
|  | Huawei | 70.73 | 99.50 | 113.81 | -5.02 | 5.47 | 14.44 |
|  | Qualcomm | 71.85 | 99.44 | 113.47 | -4.03 | 4.52 | 14.81 |
|  | ZTE | 71.00 | 99.00 | 112.09 | -2.89 | 5.24 | 14.99 |
|  | THALES | 71.84 | 100.22 | 112.26 | -3.03 | 4.25 | 14.40 |
|  | Variance | 0.95 | 0.04 | 1.88 | 1.50 | 0.81 | 0.04 |
|  | Mean | 71.73 | 99.50 | 112.39 | -3.35 | 5.29 | 14.70 |
|  | Xiaomi | 71.8 | 99.95 | 112.95 | -6.77 | 4.11 | 14.87 |
| Rural Macro | Samsung | 87.82 | 125.94 | 138.22 | -4.25 | 5.77 | 15.42 |
|  | Huawei | 93.87 | 126.75 | 141.60 | -8.93 | 1.10 | 15.33 |
|  | Qualcomm | 89.63 | 127.43 | 142.65 | -7.37 | 2.54 | 15.47 |
|  | THALES | 88.83 | 127.81 | 139.57 | -5.31 | 2.57 | 15.24 |
|  | Variance | 6.43 | 0.37 | 3.58 | 3.78 | 3.81 | 0.00 |
|  | Mean | 90.04 | 126.98 | 140.51 | -6.47 | 3.00 | 15.37 |
|  | Xiaomi | 90 | 125.60 | 139.94 | -8.58 | 3.15 | 15.59 |

Table B.2-3: Calibration summary for TN NR AAS UL cases

| Calibration metrics |  | UL Coupling Loss |  |  | UL Geometry SINR |  |  | UL UE Tx Power |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CDF percentile |  | @5% | @50% | @95% | @5% | @50% | @95% | @5% | @50% | @95% |
| Urban Macro | Samsung | 60.00 | 88.58 | 99.02 | 3.42 | 14.01 | 14.96 | -25.77 | 2.81 | 13.25 |
|  | Huawei | 52.84 | 85.79 | 103.53 | 7.53 | 13.89 | 14.89 | -31.46 | 4.14 | 16.51 |
|  | Qualcomm | 62.00 | 91.09 | 104.80 | 1.33 | 10.10 | 13.87 | -23.77 | 5.32 | 19.03 |
|  | Nokia | 59.04 | 87.59 | 99.32 | 10.98 | 14.55 | 15.10 | -27.23 | 1.32 | 13.05 |
|  | Ericsson | - | - | - | 6.83 | 12.94 | 15.08 | -28.52 | -1.70 | 13.98 |
|  | ZTE | 58.43 | 87.22 | 98.46 | 5.47 | 13.42 | 14.81 | -27.59 | 1.20 | 12.44 |
|  | Variance | 9.37 | 3.11 | 6.81 | 9.41 | 2.11 | 0.18 | 5.61 | 5.14 | 5.41 |
|  | Mean | 58.46 | 88.05 | 101.03 | 5.93 | 13.15 | 14.78 | -27.39 | 2.18 | 14.71 |
|  | Xiaomi | 58.21 | 86.32 | 99.02 | 3.38 | 13.5 | 14.75 |  |  |  |
| Rural Macro | Samsung | 78.00 | 116.25 | 127.95 | -4.86 | 7.03 | 14.96 | -7.77 | 23.00 | 23.00 |
|  | Huawei | 77.53 | 114.99 | 128.13 | -10.84 | 4.42 | 14.86 | -3.46 | 23.00 | 23.00 |
|  | Qualcomm | 82.05 | 120.47 | 135.46 | -13.41 | 2.07 | 14.69 | -3.72 | 23.00 | 23.00 |
|  | Nokia | 78.42 | 116.20 | 127.85 | -4.15 | 7.69 | 15.17 | -7.85 | 22.99 | 22.99 |
|  | Ericsson | - | - | - | 3.30 | 13.55 | 15.14 | -8.00 | 21.55 | 23.00 |
|  | Variance | 3.20 | 4.32 | 10.50 | 33.91 | 14.88 | 0.03 | 4.41 | 0.34 | 0.00 |
|  | Mean | 79.00 | 116.98 | 129.85 | -5.99 | 6.95 | 14.96 | -6.16 | 22.71 | 23.00 |
|  | Xiaomi | 78.41 | 115.29 | 130.04 | -6.47 | 7.8 | 14.77 |  |  |  |

Table B.2-4: Calibration summary for TN NR non-AAS UL cases

| Calibration metrics |  | UL Coupling Loss |  |  | UL Geometry SINR |  |  | UL UE Tx Power |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CDF percentile |  | @5% | @50% | @95% | @5% | @50% | @95% | @5% | @50% | @95% |
| Urban Macro | Samsung | 73.18 | 99.31 | 110.33 | -10.99 | 4.31 | 9.49 | -12.59 | 13.54 | 23.00 |
|  | Huawei | 70.73 | 99.50 | 113.81 | -6.46 | 3.08 | 8.88 | -10.26 | 18.51 | 23.00 |
|  | Qualcomm | 71.53 | 99.51 | 113.45 | -3.52 | 3.36 | 8.00 | -14.24 | 13.74 | 23.00 |
|  | ZTE | 70.11 | 98.57 | 111.77 | -1.39 | 3.87 | 8.01 | -16.23 | 12.57 | 23.00 |
|  | THALES | 71.84 | 100.22 | 112.26 | -0.01 | 4.13 | 6.80 | -14.05 | 14.90 | 22.76 |
|  | Variance | 1.32 | 0.15 | 1.94 | 12.98 | 0.22 | 0.39 | 4.79 | 5.32 | 0.00 |
|  | Mean | 71.48 | 99.42 | 112.32 | -4.48 | 3.75 | 8.24 | -13.47 | 14.65 | 22.95 |
|  | Xiaomi | 70.82 | 99.35 | 111.86 | -4.8 | 3.75 | 8.85 |  |  |  |
| Rural Macro | Samsung | 87.78 | 125.93 | 138.22 | -17.23 | -3.82 | 14.65 | 2.01 | 23.00 | 23.00 |
|  | Huawei | 93.87 | 126.75 | 141.60 | -24.83 | -9.72 | 14.75 | 12.88 | 23.00 | 23.00 |
|  | Qualcomm | 89.53 | 127.64 | 142.62 | -21.39 | -5.65 | 14.33 | 3.76 | 23.00 | 23.00 |
|  | THALES | 88.83 | 127.81 | 139.57 | -17.29 | -5.16 | 14.26 | 3.55 | 22.42 | 22.94 |
|  | Variance | 6.55 | 0.49 | 3.54 | 9.66 | 6.07 | 0.03 | 22.71 | 0.00 | 0.00 |
|  | Mean | 90.00 | 127.03 | 140.50 | -20.18 | -6.09 | 14.50 | 5.55 | 22.86 | 22.99 |
|  | Xiaomi | 89.44 | 126.62 | 141.08 | -19.2 | 4.05 | 14.21 |  |  |  |