3GPP TSG-RAN WG4 Meeting #114	                                                                                            R4-2500639

Athens, GR, 17 Feb –21 Feb, 2025

Source: 	Xiaomi

Title: 	Discussion on AIoT system parameters

Agenda item:	7.24.1

Document for:	Discussion

- Introduction

A new WI “Solutions for Ambient IoT (Internet of Things) in NR” [1] is approved in RAN#106. The scope of the WI is limited that only device 1 is studied. This contribution provides our views for AIoT system parameters based on the WI scope and the agreements in SI phase [2].

- Discussion

The R19 AIoT WI scope is limited that only D1T1-B with device 1 is considered. The SI phase agreements for system parameters corresponding to the scope is copied as below [3].

Agreement:

- Whether requirements potentially needed or not is summarized in the following table:

| System parameter for A-IoT |  | Potentially needed or not |  |
| --- | --- | --- | --- |
|  |  | A-IoT BS | A-IoT device |
| Operating band |  | YES |  |
| Channel bandwidth | Transmission bandwidth | YES the value needs be discussed in WI phase depending on other WG conclusion. |  |
|  | channel bandwidth | YES |  |
| Channel arrangement | Channel spacing | TBD in WI phase for standalone NA for in-band/Guard-band |  |
|  | Channel raster | IB/SA: YES GB:TBD | GB:NO IB/SA:TBD |
|  | Synchronization raster | NO for device 1/2a |  |

The details of each parameter are discussed in the following sections.

2.1 Operating band

In the WID, the scope declares that band n8 is the example band. So following the WID, band 8 can be defined as the operation band in R19, other bands can be considered after the WI is closed.

Proposal 1: Band 8 is defined as the operating band for ambient IoT in R19.

2.2 Terminology of bandwidth and guard band

The terminology of bandwidth may need some discussion because it seems RAN1 invented some new concepts which are different with what are used in NR RF specifications currently. Furthermore, RAN1 use different names for R2D and D2R bandwidth. The followings are copied from TR 38.769 to show the understandings/definitions in RAN1.

=====================================

6.1.1.4	R2D bandwidths

The study defines the following bandwidths for R2D:

-	Transmission bandwidth, Btx,R2D from a reader perspective: The frequency resources used for transmitting R2D. For an OFDM-based waveform with subcarrier spacing of 15 kHz, Btx,R2D ≤ twelve PRBs, and would be down-selected among:

-	Alt 1: Including 180 kHz, 360 kHz

-	Alt 2: Integer multiple(s) of 180 kHz

-	Alt 3: Integer multiple(s) of the subcarrier spacing

-	Occupied bandwidth, Bocc,R2D from a reader perspective: The frequency resources used for transmitting R2D, and potential guard band.

-	Bocc,R2D ≥ Btx,R2D.

==================================

6.1.2.4	D2R bandwidths

The following bandwidths for D2R are defined for the purpose of the study:

-	Transmission bandwidth, Btx,D2R: The frequency resources scheduled by a reader for a D2R transmission from one device.

-	Occupied bandwidth, Bocc,D2R: The transmission bandwidth plus the potential associated intra A-IoT guard-bands totalling Bguard,D2R. Note: this guard band is not for coexistence with NR/LTE.

-	Bocc,D2R ≥ Btx,D2R.

====================================

In the history RF standard discussion, occupied bandwidth originally comes from the regulation to measure the BW of the 99% transmitted power. It should be less than the (assigned) channel bandwidth. The current NR occupied BW concept is described in TS 38.101 as following,

Occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel.

The occupied BW may not include or include (part of) the guard band depending on BW of the transmitted 99% power occupies. However, the defined occupied bandwidth for R2D and D2R in TR 38.769 includes guard band. From this aspect, our understanding is that the occupied bandwidth for R2D and D2R defined in TR 38.769 can be understood as the reader channel bandwidth and device channel bandwidth corresponding to the concepts of NR BS CBW and UE CBW. Otherwise, the occupied BW name brings too much misleading compared with current NR RF specifications and regulations. So we have the following proposal,

Proposal 2: For Ambient IoT, RAN4 discuss the idea of replacing the occupied bandwidth name in TR 38.769 by channel bandwidth for both R2D and D2R.

-    BWChannel, R2D		RF bandwidth supporting a single ambient IoT channel with R2D transmission at the reader.

-    BWChannel, D2R		RF bandwidth supporting a single ambient IoT channel with D2R transmission at the device.

It should be noticed that the sets of BWChannel, R2D and BWChannel, D2R may be different depending on the ongoing discussion in RAN1. For reader side, the RF BW at reader side for D2R transmission can be wider that it can be equal to several channel BWs of BWChannel, R2D. And even for R2D, reader side seems can support to receive FDMed device R2D signals because frequency shift is supported for device 1. However, we think the proposed approach can still be a candidate for further discussion to be compatible as much as possible with the current RF terminology understanding and definition.

If the CBW understanding can be aligned, the transmission bandwidth and the guard band concepts are straightforward. They’re similar with the current terminology.

Proposal 3: The transmission bandwidth and guard band definition for R2D and D2R in TR 38.769 can be reused.

For the actual value of CBW, TBW and guard band, RAN4 need to wait for the conclusions from RAN1. Currently, three options are on the table for R2D BW. D2R BW also lacks of conclusions.

2.3 Channel raster and channel spacing

There’re some remaining TBDs for channel raster in the SI stage in [3]. One is for reader side in guard band, the other is the in-band and standalone mode for device side. In our understanding, for reader side, defining channel raster may be beneficial for the implementation that there’s some guidance if there’s no clear definition on the channel center frequencies. For device side, especially for device 1, it seems there’s no strong motivation to define channel raster. But we think channel raster serves as a system parameter for the ambient system not specific for reader or device. So we have the following proposal,

Proposal 4: Define channel raster for in-band, standalone and guard band as a system parameter in both reader and device RF specifications.

The detail design of channel raster can wait for the CBW conclusion in RAN1 or there can be another way that following NR channel raster. We think following NR channel raster is reasonable because the in-band channel raster can use the same channel raster with NR. Then it may not be necessary to use different channel raster for SA and GB scenarios. Technically sparse channel raster leads to small frequency resolution for LO, it may be beneficial to the reader implementation. If some justification is provided that low frequency resolution is needed for the cost consideration, we’re also ok to further discuss.

Proposal 5: Consider to reuse NR channel raster for ambient IoT system if there’s no implementation cost concern for reader implementation.

The detail for channel spacing needs to wait the conclusion in RAN1. Currently, the transmission BW and guard band of R2D/D2R is still under discussion. Channel spacing discussion cannot be started yet in RAN4.

Observation: The detail design of channel spacing needs to wait for RAN1 TBW and guard band conclusion.

- Conclusion

This contribution provides our consideration of AIoT system parameters. We have the following proposals and observations.

Proposal 1: Band 8 is defined as the operating band for ambient IoT in R19.

Proposal 2: For Ambient IoT, the occupied bandwidth name in TR 38.769 is replaced by channel bandwidth for both R2D and D2R.

-    BWChannel, R2D		RF bandwidth supporting a single ambient IoT channel with R2D transmission at the reader.

-    BWChannel, D2R		RF bandwidth supporting a single ambient IoT channel with D2R transmission at the device.

Proposal 3: The transmission bandwidth and guard band definition for R2D and D2R in TR 38.769 can be reused.

Proposal 4: Define channel raster for in-band, standalone and guard band as system parameters in both reader and device RF specifications.

Proposal 5: Consider to reuse NR channel raster for ambient IoT system if there’s no implementation cost concern for reader implementation.

Observation: The detail design of channel spacing needs to wait for RAN1 TBW and guard band conclusion.

- References

[1] RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN#106

[2] TR 38.769

[3] R4-2420470, “WF on RF requirements for A-IoT”, RAN4#113, Huawei

[4] TR 36.802

[5] TR 38.869