3GPP TSG-RAN WG4 Meeting # 114                                    R4-2500702

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.27.2.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Event triggered L1 measurement reporting

Document for:	Approval

Introduction

In Rel-18, LTM was introduced to reduce handover delay and interruption time compared to layer3 based mobility. As mentioned in WID [1], LTM as introduced in Rel-18 has a number of limitations compared to L3 mobility. In Rel-19, these limitations should be removed as much as possible.

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify RRM requirements related to the above objectives as necessary [RAN4]

In this contribution, we will propose our views on RRM requirement for event triggered L1 reporting.

Discussion

According to the WID, RAN4 should discuss the impact on RRM requirements depends on the RAN1/2 specification introduced for Event triggered L1 measurement reporting.

In the last meeting, we have reached consensus on Event-triggered L1 reporting delay requirement and accuracy requirement from high level and detail should be discussed in furture.

The RRM impact due to event triggered L1 measurement reporting could include the followings:

- Capabilities for Support of Event Triggering and Reporting Criteria

- whether and how to coordinate with R19 MIMO

- L1 filtering assumption in event evaluation

- Impact of UL resource waiting time

- Other potential impact of event-triggered L1 reporting

For the first bullet, some companies proposed that requirements for reporting criteria per measurement category needs to be updated to cover L1-RSRP measurement on neighbor cell in pervious meeting. In our understanding, it is necessary to introduce the requirements of Capabilities for Support of Event Triggering and Reporting Criteria specified in TS38.133, which is similar as legacy event triggered L3 measurement reporting. As for how to define relevant capabilities to support of event triggering L1 measurement reporting, we prefer to define a separate capability for the number of L1 measurement reports triggered by supported events to avoid affecting L3 measurement reports.

- Issue 2-7: whether to introduce/update requirements for reporting criteria per measurement category <Candidate options> - Option 1: Yes (CATT, MTK, ZTE, Apple, OPPO) - Option 2: RAN4 to postpone the detailed discussion on ‘Requirements for reporting criteria per measurement category’ until the RRM requirements on the remaining issues are sorted out (QC, Ericsson) - Option 3: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo) <Recommended WF> - Continue discussion. - Issue 2-8: whether to define a separate capability for supported number of events triggered L1 measurement reporting <Candidate options> - Option 1: Yes (MTK, Samsung, CMCC) - Option 2: RAN4 to postpone the detailed discussion on ‘Requirements for reporting criteria per measurement category’ until the RRM requirements on the remaining issues are sorted out (QC) - Option 2a: RAN4 to wait for RAN2 progress on the TTT handling before discussing the UE capability of number of events to support. (E///) - Option 3: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo) <Recommended WF> - Continue discussion.

Proposal 1: The requirements of “Capabilities for Support of Event Triggering and Reporting Criteria” specified in TS38.133 need to be updated to cover event triggered L1 measurement reporting. And RAN4 should define a separate capability for supported number of events triggered L1 measurement reporting.

For the second bullet, we should discuss whether and how to coordinate with R19 MIMO.

- Issue 2-10: whether and how to coordinate with R19 MIMO <Candidate options> - Option 1: (MTK) - Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold. - Define the capability for L1 events shared by mobility L1 events and MIMO L1 events. - Optoin 2: Define separate capabilities for MIMO and mobility. (Samsung, CMCC, Apple) - Option 3: Discuss the coordination between LTM and MIMO events at the end of the Wis. (Nokia) <Recommended WF> - Continue discussion.

We understand that this is done to reduce repetitive discussions on similar issues and avoid wasting resources. However, both the current topic and MIMO are independent topics of R19, and coordinate with R19 MIMO maybe affect overall progress of R19 Mobility. In order not to affect the discussion of the topic, we support option 2. and RAN4 should define separate capabilities for MIMO and mobility.

Proposal 2: Define a separate capability for supported number of events triggered L1 measurement reporting.

- Issue 2-11: L1 filtering assumption in event evaluation <Candidate options> - Option 1: For whether to consider the RRM impact due to L1 filtering for event triggered L1 report, RAN4 needs to wait for more RAN1 input. (CATT, ZTE, CTC, E///) - If L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report. - Otherwise, there is no impact on RAN4. - Option 2: In event triggered L1 measurement in mobility, do not consider L1 filter for requirements. (Samsung, Xiaomi, CMCC, Apple, HW) - Option 2a: RAN4 to define RRM requirements for the event triggered L1 report with the assumption of no specific L1 filtering. This decision can be revisited if other working groups overturn their decision on L1 filtering. (QC) <Recommended WF> - Continue discussion.

As to the filtering assumption in event evaluation, RAN2 assumes filtering of the L1 measure results is needed for LTM event and it’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. For event triggered reporting for L3 measurement, an additional delay can be expected in event triggered measurement reporting delay when L3 filtering is used. If L1 filtering is used, there maybe an impact on report delay requirement. Based on this, RAN4 should follow up on other details of the event evaluation and report triggering. Therefore, we should wait for RAN1/RAN2’s progress before defining requirements.

Based on RAN1’s progress, no L1 specified filtering for time and spatial domain is introduced. Therefore, there is no impact on event triggered L1 measurement in mobility.

Agreement The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH

Proposal 3: Based on RAN1’s progress, do not consider L1 filter for requirements in event triggered L1 measurement in mobility.

- Issue 2-15: Impact of UL resource waiting time. <Candidate options> - Option 1: For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting. (Xiaomi) - It means that if L1 reporting procedure starts, UE can continue to measure for L1, however, the following L1 result will not be evaluated for event anymore. - Option 2: for event triggered L1 measurement reporting, it is proposed that measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting. (CMCC, Apple, vivo) - Option 3: RAN4 to discuss components and criteria for defining TTime to first UL channel (E///) - Once the event is met, UE needs to send the indication on the SR or special SR to the NW to indicate to the NW that event is met. It can be denoted as TTime to first UL channel. <Recommended WF> - Continue discussion.

For the issue on impact of UL resource waiting time, when event is met, UE may send a SR or special SR to the NW if there is no existing UL grant. Meanwhile, if there is UL grant, UE will use this UL grant directly and no need for further SR transmission. For legacy event triggered L3 measurement reporting, the reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. In our understanding, it means that RRM requirements are defined assuming UL resource for the reporting is always available. Same as legacy, this approach can be used for L1 measurement reporting.

9.2.4.3 Event Triggered Reporting The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

Proposal 4: For event triggered L1 measurement reporting, it is proposed that measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting.

Conclusions

In this contribution, we put forward the following proposals on R19 L1/L2 triggered mobility.

Proposal 1: The requirements of “Capabilities for Support of Event Triggering and Reporting Criteria” specified in TS 38.133 need to be updated to cover event triggered L1 measurement reporting. And RAN4 should define a separate capability for supported number of events triggered L1 measurement reporting.

Proposal 2: Define a separate capability for supported number of events triggered L1 measurement reporting.

Proposal 3: Based on RAN1’s progress, do not consider L1 filter for requirements in event triggered L1 measurement in mobility.

Proposal 4: For event triggered L1 measurement reporting, it is proposed that measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting.

References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

- R4-2420101, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple