3GPP TSG-RAN4 Meeting #114	R4-2500728

Athens, GR, February 17 – 21, 2025

Agenda item:			7.1.1.2.1

Source:	vivo

Title:	Discussion on UE RF power enhancement for NR single carrier

Document for:	Discussion

- Introduction

In RAN#113 meeting, the WF[1] has been agreed, and the remaining issue is as follows:

### Sub-topic 1-1: Approaches to enable MPR reduction for both scenario 1 and scenario2 Scenarios discussed in previous RAN4 meetings: - Scenario 1-1: Scenario with no adjacent in-band/out-of-band co-existence issue (single operator) - Scenario 1-2: Scenario with no adjacent in-band/out-of-band co-existence issue (adjacent operators) - Scenario 2: Narrower UE channel BW within wider BS bandwidth #### Issue 1-1-1: Approaches of converting outer RB allocation to inner RB allocation Agreement in main session: - Extended CBW based approach - where the extended CBW or aggregated CBWs for the DL-only contiguous intra-band CA case is signalled from network to UE - Both scenario 1 and 2 should be covered - The network should ensure the co-existence be met when signaling the extended CBW. #### Issue 1-1-2: Approaches of calculating the new inner RB region - WF - FFS description of deriving the RB allocation region inside the UE CBW which could use inner RB MPR with information of the extended UE CBW - Candidate wordings, other options are not precluded - The intersection of the Inner region relative to the extended UE CBW and the Outer region relative to the configured UE CBW is where the inner MPR can be expected from UE - The intersection of the Inner MPR region of the extended UE CBW with the original MPR region based on the UE CBW - For enhanced MPR requirements, at least in the case where BW is extended by 0.5 * UE CBW on one or both sides of UE CBW, an RB allocation (LCRB) within the UE CBW is considered as an extended inner RB allocation if an RB allocation-extCBW (of extended CBW) fully covering the same frequency location is also determined as an inner RB allocation (according to existing 38.101-1 MPR formula with corresponding NRB of the extended CBW applied). - Further improve the wording (better with illustrated figures) to derive a generic solution - How UE get the extended RB number as well as the RB shift is one possible way, other methods are not precluded - FFS how to reflect the new inner region in the spec #### Issue 1-1-3: Width/RB numbers of extended CBW at each side of UE CBW - WF - Extension on each side of UE CBW could be different - FFS on extended RB numbers/channel bandwidth, which are up to X on each side of UE CBW, depends on the adjacent spectrum by operator in all possible scenarios - FFS whether extended RB number/channel bandwidth should be limited to max 50MHz at each side, which means the extended CBW is relevant to the configured UE CBW - Fixed extended RB number/channel bandwidth to X is not precluded. - Extension is not used for the specific UE UL transmission but can be used by other UEs UL transmissions. It is generally understood that going beyond X on each side does not further reduce the MPR. - FFS on X value ### Sub-topic 1-2: Applicable requirements #### Issue 1-2-1: Where to use extended IBE in the larger BS CBW - WF - IBE should be used between edges of UE CBW and extended UE CBW - FFS the application of IBE in guard band of UE CBW and/or extended UE CBW #### Issue 1-2-2: How to derive the extended IBE - WF - FFS the method to derive the extended IBE, which should cover both cases for partial RB allocation and full RB allocation in the UE CBW #### Issue 1-2-3: Boundary to apply ACLR and SEM - WF - ACLR and SEM should be applicable from the edge of extended UE CBW. - FFS the following alternatives - Alt 1: The OOBE requirements are based on UE CBW - Alt 2: The OOBE requirements are based on extended UE CBW #### Issue 1-2-4: Boundary to apply SE - WF - FFS with consideration of Issue 1-2-3 ### Sub-topic 1-3: Signaling and others #### Issue 1-3-2: Others - WF - In cases where additional emission should be met, the UE performs the max(MPRenh, MPR, A-MPR) function with: - MPRenh here means the approach converting outer allocation to inner allocation i.e. the method that is discussed in this WI - Companies are encouraged to propose further refinement

- Discussion

## 2.1 MPR requirements & Signaling mechanism

### Extension ratio, GB selection, conversion between extension CBW and extension RB number

In short, simply extending by 2*the original UE CBW (i.e., extend at 0.5 times the UE CBW each side) is sufficient to convert all outer allocations to inner without wasting more extended bandwidth. Therefore, the evaluation of MPR and OOBE requirements should be based on the extended UE CBW. And the extension could be asymmetrical, the extension condition of any side is the gap between the original UE CBW and the RX RF CBW is at least 2 times the UE CBW, so the symmetrical extension should be based on the premise that the gap is wide enough on both sides.

Proposal 1: For the extension ratio on one side, only 0.5* UE CBW is considered.

- Symmetrical extension is 0.5 * UE CBW on each side.

- Asymmetric extension is 0.5 * UE CBW on one side.

When the extension ratio is not fixed and the extended bandwidth does not exist in Table 5.3.2-1, then the UE does not have direct knowledge of the corresponding RB number. Therefore, it is unreasonable for the NW to only indicate the extended RB number or the extended bandwidth. The reasons are as follows:

- If the NW directly indicates to the UE the extended RB number instead of the extended bandwidth, then the UE still does not have direct knowledge of the specific OOBE evaluation frequency range, i.e., it still needs to perform a conversion of the extended RB number-extended bandwidth relationship.

- If the NW only indicates to the UE the extended bandwidth, the indication of the RB number in the extended part is not clear enough, i.e., the UE cannot directly know how many RBs have been extended on a certain side of the UE, then the IBE requirement for the extension part cannot apply because it is per-RB granularity.

Observation 1: When the extended bandwidth or the number of extended RBs does not exist in Table 5.3.2-1, the NW should not indicate only one of them because the UE does not have direct information about the other one.

To sum up, how to deal with the relationship between the extension RB and extension BW needs discussion, which is also related to the choice of the GB. Furthermore, the uncertainty of the relationship between the two extension information will also result in inconsistent definitions of the inner region.

Option1: If the NW only selects the BW present in Table 5.3.2-1 as the extension information, then the UE can directly learn the corresponding number of extension RBs and vice versa. But there are certain problems with such an extension. The guardband & RB number between different bandwidths is not proportionally extended. For example, the RB number for 20M is greater than twice the number of RB for 10M, so if the NW indicates that the extended bandwidth-expanded RB for one side is 10M-52RB, the total number of extended RBs obtained in this way is on the low side because the corresponding guardband on both sides of the extended part is also taken into account, as shown in Fig.1 , option1. Therefore, even though the extended CBW is twice the original CBW, the number of extended RBs is not double that of the original one. And the IBE inside the GB of the extension part near the original bandwidth side could not be evaluated, which is also not accurate enough.

Observation 2: Directly indicating the extended bandwidth in Table 5.3.2-1 and/or its corresponding RB number to the UE as the extension information is inaccurate because the GBs on both sides of the extended bandwidth are also taken into account.

Option 2: If only the GBs of the extended CBW are retained, which means that for the original CBW, the GBs on both sides need to be evaluated for radiation intensity by IBE, which is in a way to tighten the in-band requirement. From our perspective, it is not the preferred option either. In addition, considering the GB of the whole UE CBW will limit the possibility of extension, i.e. the extended CBW cannot exceed 100M because there is no corresponding GB in the spec.

Option 3: Retaining both the original CBW’s GB and the extension part’s GB is a better choice considering the evaluation of IBE. However, for the extension part, only one side of the GB can be retained, i.e., the side away from the edge of the original CBW, and the side close to the original UE still require confinement by the IBE. The ‘GB’ in the extension part does not directly correspond to the GB in the spec, it is actually a similar concept. In fact, the number of RBs allocated in the extension part is not always an integer with a high probability as the extension CBW on both sides is half the original CBW. At the same time, there are many ways to calculate the number of RBs in this part.

To simplify the calculation and definition of inner region, the extension RB number can be obtained by directly referring to the RB number of the original UE CBW. That is, the RB corresponding to the original bandwidth can be taken in half and assigned to both sides of the UE, while the extended bandwidth on both sides is also equal to half of the original UE CBW. After calculation, it can be easily concluded that in fact the extension RBs under this scheme also do not take up the entire extended bandwidth, i.e., it is also equivalent to retaining a portion of ‘GB’ and placing it on the outer side, whereas the IBE is evaluated per RB starting from the outside of the UE CBW.

Based on the relationship between the extension RB number and extension CBW, the principle of inner region’s  definition for extended CBW remains unchanged, i.e., LCRB ≤ ceil(1/2 * NRB_extended) , NRB_extended = NRB + extension RBs. NRB is the maximum transmission bandwidth configuration of the original CBW.

Observation 3: The number of extended RBs obtained from the extended bandwidth is not always an integer, so there will always be some ‘GBs’ in the extended portion.

Fig.1. 3 combinations of GB & extension RB

Proposal 2: The GB of the original CBW should be retained, and the ‘GB’ of the extension part should only be retained on the side that is far away from the original CBW.

- ‘GB’ of the extension part = [(0.5* BWChannel x 1000 (kHz) – ceil(NRB/2) x SCS x 12) -SCS/2], BWChannel and NRB correspond to the original UE CBW.

Proposal 3: The extended RBs (NRB_e) on each side should be equal to ceil(NRB/2), where NRB is the maximum transmission bandwidth configuration of the original UE CBW.

The correspondence between the original RB number and the extension RB number is shown in the following table.

Table 1: extension bandwidth configuration NRB_e

| SCS (kHz) |  | Maximum transmission bandwidth of original UE(MHz)/extension transmission bandwidth on one side(MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3/1.5 | 5/2.5 | 10/5 | 15/ 7.5 | 20/10 | 25/ 12.5 | 30/15 | 35/ 17.5 | 40/20 | 45/ 22.5 | 50/25 | 60/30 | 70/35 | 80/40 | 90/45 | 100/ 50 |
| 15 | NRB | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
|  | NRB_e = ceil(NRB/2) on one side | 8 | 13 | 26 | 40 | 53 | 67 | 80 | 94 | 108 | 121 | 135 | N/A | N/A | N/A | N/A | N/A |
| 30 | NRB | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
|  | NRB_e = ceil(NRB/2) on one side | N/A | 6 | 12 | 19 | 26 | 33 | 39 | 46 | 53 | 60 | 67 | 81 | 95 | 109 | 123 | 137 |
| 60 | NRB | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |
|  | NRB_e = ceil(NRB/2) on one side | N/A | N/A | 6 | 9 | 12 | 16 | 19 | 22 | 26 | 29 | 33 | 40 | 47 | 54 | 61 | 28 |

As shown in the ANNEX, there are some simulation results of MPR with different channel bandwidths, where the extension ratio is all set at 0.5 times the original CBW on both sides, resulting in an extended CBW of twice the original CBW. The extension RB number also follows the principle in Proposal 3. Considering some calibration error, it can be observed that for DFT-S-OFDM waveform with QPSK modulation, all outer configurations in PC3 cases could reduce to the MPR level of the inner configuration, provided that the evaluation methods for OOBE and IBE adhere to the rules in proposal 7~11.

### New inner region

When both sides are extended then all the outer allocations including full RB allocation could convert to inner and the MPR reduction could take effect no matter what the specific RB allocation is. However, for the asymmetric extension the RB region definition of the original UE CBW should change and the UE should judge its allocation according to the new definition. And based on the fixed extended ratio, the new RB region could be calculated directly but the UE’s RB allocation stays the same, so there is no need for the NW to indicate extension RB, or RB0shift & BS_NRB to the UE anymore.

Observation 4: When both sides are extended by 0.5* UE CBW, all the outer allocations including full RB allocation could convert to inner and the MPR could always refer to the inner value regardless the specific RB allocation.

Observation 5: Based on the fixed extended ratio, the new inner RB region of asymmetric extension could be calculated directly but the UE’s RB allocation stays the same, so there is no need for the NW to indicate extension RB, or RB0shift & BS_NRB to the UE anymore.

When only one side is extended, the inner region should be re-defined.

The following figure shows the range of RB allocation with maximum LCRB in the inner region. The original bandwidth of the UE is A and at maximum LCRB (0.5*NRB), the RB allocation range in the UE CBW should be 1/4A~3/4A. When the UE is extended by 0.5*UE CBW on the higher side, the bandwidth of the extended UE is 1.5A. At maximum LCRB (0.5*NRB_extended UE), the RB allocation range in the extended UE CBW should be 3/8A~9/8A. However, the RBs outside the original UE should not be scheduled so the furthest allocated RB should not exceed A.

According to this principle, assumes RBstart = X at maximum LCRB = A-X for the original UE with extension. For the lower side the parameter RBstart,Low should still follow the principle ≥ max(1, floor(LCRB /2)), and for RBstart,High there is no restriction other than ≤ NRB - LCRB. So A-X=2*X, deducing X= A/3. Therefore, LCRB for the UE with extension should be less than or equal to 2/3 * NRB.

Fig 2. The RB allocation range at maximum LCRB in the inner region, extension on the higher side

Similarly, when the extension is on the lower side, for the original UE with extension, the allowed RB allocation range in the new inner region is 0.5*A~X, X-0.5*A = 2* (1.5*A-X), deducing X= 7/6 *A. The LCRB for the UE with extension should also be less than or equal to 2/3 NRB, and for the higher side the parameter RBstart,High should still follow the principle ≤ NRB – RBStart,Low – LCRB, and for RBstart,Low there is no restriction.

Observation 6: For UEs that extend 0.5 times the original bandwidth on one side, LCRB should be less than or equal to 2/3 * NRB for the inner RB region.

Fig 3. The RB allocation range at maximum LCRB in the inner region, extension on the lower side

As the spec shows, the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

RBStart,Low  = max(1, floor(LCRB /2))

RBStart,High = NRB – RBStart,Low – LCRB

If only the higher side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be defined as:

RBStart,Low  ≤  RBStart  ≤  NRB – LCRB, and LCRB  ≤  ceil(2/3 *NRB)

Similarly, if only the lower side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be defined as:

RBStart  ≤  RBStart,High, and LCRB  ≤  ceil(2/3 *NRB)

Proposal 4:

- If both sides of the UE extend by 0.5 times the UE CBW, then the entire RB allocation inside the original UE CBW is inner region.

- If only the higher side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be re-defined as:

RBStart_Low  ≤  RBStart  ≤  NRB – LCRB, and LCRB  ≤  ceil(2/3 *NRB)

Fig 4. inner region of extended CBW, higher side extends by 0.5*UE CBW

- If only the lower side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be re-defined as:

RBStart  ≤  RBStart,High, and LCRB  ≤  ceil(2/3 *NRB)

Fig 5. inner region of extended CBW, lower side extends by 0.5*UE CBW

### Signaling mechanism

The new mechanism of the UE to reduce MPR should be discussed, and based on the fixed extension ratio, i.e., 0.5 * UE CBW on each side, the following one could be considered.

First of all, it is up to the NW to judge the deployment environment, when the gap on one side of the UE CBW is at least greater than 0.5* UE CBW, the NW should send the signaling (e.g., ExtendLowerSideRel19, or ExtendHigherSideRel19) to the UE to indicate extension on the corresponding side. Then the UE would judge its allocation type based on the new inner region defined in Proposal 4, and the corresponding MPR would be used.

If the gap on both sides of the UE is at least greater than 0.5* UE CBW, the NW should send two signaling (e.g., ExtendLowerSideRel19 + ExtendHigherSideRel19) or one combined signaling (e.g., ExtendBothSidesRel19) to the UE to indicate that the extension is on both sides and the MPR of inner region could be used no matter what the RB allocation type is.

Proposal 5: Based on the fixed extension ratio, i.e., 0.5* UE CBW on each side, the following signaling mechanism could be considered:

- It is up to the NW to judge the gap width on both sides of the UE.

- When the gap on one side is ≥ 0.5* UE CBW and the UE supports [MPR improvement with BW extension-r19], the NW sends the signaling (e.g., BWExtension -LowerSide-Rel19, or BWExtension-HigherSide-Rel19) to indicate the extension on the corresponding side.

When receiving the above extension signaling, the UE judges its allocation type based on the new RB region defined in Proposal 4.

- When the gap on both sides is ≥ 0.5* UE CBW and the UE supports [MPR improvement with BW extension-r19], the NW sends two signaling (e.g., BWExtension -LowerSide-Rel19 + BWExtension-HigherSide-Rel19) or one combined signaling (e.g., BWExtension -BothSides-Rel19) to indicate the extension on both sides.

When receiving the above extension signaling, the MPR of inner region could be used regardless of the RB allocation type.

The above extension signaling is rather stable as it depends on the original UE CBW and the gap width, and will not change with the RB allocation state, so the signaling overhead is very low. At the same time, as the extension ratio is fixed it should be stipulated in the spec, and there should be no other UE capability other than support for the BW extension.

Observation 7: The extension signaling overhead is very low as it only depends on the original UE CBW and the gap width and will not change with the RB allocation state.

Proposal 6: The extension ratio on both sides, 0.5* UE CBW should be stipulated in the spec, and there should be no other UE capability other than support for the BW extension.

## 2.2 Emission requirements

### Applying range of IBE

The general part of IBE in spec consists of four equations and the max of them is filtered out as the final requirement:

IBE_general=max(PRB - 30 dB, )

Therefore, there are two elements to consider: NRB and ΔRB. They are both related to UE CBW and corresponding RB allocations: NRB represents the maximum transmission bandwidth configuration for each UE channel bandwidth, ΔRB is the starting frequency offset between the allocated RB and the measured non-allocated RB.

According to [2], the NRB of the IBE requirement refers to the total number of RB in the uplink, which should be based on the actual UE bandwidth indicated by the NW, and should not change with the virtual extension of UE CBW edge. Similarly, the calculation of ΔRB should also be limited within the actual UE CBW.

The IBE requirement of the farthest non-allocated RB of the original UE CBW could be used as a reference to evaluate the radiation power level of the entire extended portion, and combined with the above analysis of the GB, the guardband of the original UE CBW should be excluded from the evaluation as usual.

Proposal 7: Extension of the IBE should not be used in the guard band of UE CBW or in the ‘GB’ of extension part, but should be applied from the edge of the original bandwidth and evaluated in the extension RBs on each side at a per-RB granularity.

Fig 6. IBE applying range after bandwidth extension

If calculating IBE based on the entire extended UE CBW, then it would limit the extended bandwidth to the maximum transmission bandwidth range in the spec, i.e., after extension the whole UE CBW should not be wider than 100M or else the mapping NRB could not be found and IBE could not be calculated and evaluated anymore.

Observation 8: Calculating IBE based on the extended UE CBW would limit the extended UE CBW inside the maximum transmission bandwidth range of the spec.

On the other hand, since IBE is tested for each RB separately and the test results are also compared at the RB granularity, using the IBE at the actual UE edge as a reference value for the entire extension does not increase the test burden, but rather reduces it, since it is no longer necessary to re-calculate the IBE requirement for each RB in the extension part.

Observation 9: Using IBE of the actual UE CBW edge as a reference value to evaluate the emission level of the entire extended portion will reduce the testing burden.

Proposal 8: For IBE of both original CBW and extended CBW, NRB should be based on the original UE CBW, ΔRB should also be limited within the original UE CBW.

Based on the assumption above, there is another situation that needs to be discussed: Full RB allocation cases. Since there are no remaining unallocated RBs, there is no IBE requirement in the full-RB allocation of the original UE CBW. However, for the extended part, ΔRB can be set to 0 to indicate that the frequency difference between the allocated RB and the edge of the UE is 0, excluding the range occupied by the guard band. At the same time, NRB is equal to LCRB in full RB allocation. Therefore, the IBE requirement could be simplified to the following formula and it is higher than the IBE of the first non-allocated RB of a partial RB allocation.

IBE_general = max⁡(PRB - 30 dB,

max⁡{-25, 20*log10EVM -3 +5LCRB,

-57dBm + 10log10(SCS/15kHz)- PRB})

Fig 7. IBE applying range after bandwidth extension (full RB allocation for the original UE CBW)

Observation 10: For UE CBW with full RB allocation, the IBE of the extended part could be simplified to:

IBE_general = max⁡(PRB - 30 dB,

max⁡{-25,  20*log10EVM -3 +5LCRB,

-57dBm + 10log10(SCS/15kHz)- PRB})

Proposal 9：For UE CBW with extension, the emission requirement of the extended part should change to IBE_extension, which is equal to that of the farthest non-allocated RB in the original UE CBW:

IBE_extension= max⁡(PRB - 30 dB,

max⁡{-25-10lg(NRB/LCRB),  20*log10EVM -3-5*∆RB_o-1LCRB,

-57dBm + 10log10(SCS/15kHz)- PRB})

- NRB represents the maximum transmission bandwidth configuration for each UE channel bandwidth before extending.

- LCRB is the RB length of the original transmission Bandwidth before extending.

- ∆RB_o is the frequency offset between the allocated RB and the measured farthest non-allocated RB inside the original UE CBW before extending.

- For UE CBW with full RB allocation, ∆RB_o=0, NRB= LCRB.

### Change of the spurious emission boundary

For UE with extension, the application range of SE should be altered with the edge of the UE CBW shifting and the change of ACLR and SEM application range. Otherwise, SE would become the limiting factor for MPR reduction, and the application range will partially overlap with SEM, and it would not adhere to the definition and motivation of SE evaluation in 3GPP: Spurious emissions are emissions which are caused by unwanted transmitter effects but exclude out of band emissions unless otherwise stated. In ITU regulation, the definition of SE is similar: Spurious emissions include harmonic emissions, parasitic emissions, intermodulation products and frequency conversion products but exclude out-of-band emissions.

Proposal 10: For UE with extension, SE should apply beyond (UE CBW+5MHz) from the edge of the extended bandwidth.

### The start point of ΔFOOB & integral region of OOBE

ACLR and SEM would be applicable from the edge of extended CBW, and the integral region and the boundary of OOBE should be based on UE CBW. From one side, SE needs to be evaluated from the edge of SEM and according to ITU-R regulations, the SE should apply from 250% of the center frequency of the “necessary bandwidth”. In TR 38.803, it points out that for the wider carriers and higher bands the “250% rule” could be replaced by the separation between 150% and 250% of the necessary bandwidth. In total, the acceptable separation ratio should not exceed 250%. If the integral region is based on extended UE CBW, the boundary of OOBE would not only be extended to farther than current 3GPP requirements but also disobey the ITU requirement.

For example, in Figure 1(a), the integral CBW of OOBE is equal to the UE CBW. The SE applies from the edge of SEM and the frequency spacing with center of UE CBW is 200% * UE CBW+5M, which meets with the requirement of ITU. However, in Figure 1(b), the integral CBW of OOBE is equal to the extended UE CBW. And the SE applies from 300% * UE CBW+5M, which is too much far away from the UE center especially for the wider UEs and the protection effect of UE co-existence would be weaken a lot.

Proposal 11: For UE CBW with extension, the integral region of OOBE should be based on original UE CBW.

(a)

(b)

Fig 8. Comparison of two integral bandwidths

## ANNEX: simulation results of MPR, extension ratio=0.5 on each side

Fig 9. simulation results of MPR, extension CBW=2*original CBW(DFT, PC3, QPSK, SCS=15K)

- Conclusion

Proposal 1: For the extension ratio on one side, only 0.5* UE CBW is considered.

- Symmetrical extension is 0.5 * UE CBW on each side.

- Asymmetric extension is 0.5 * UE CBW on one side.

Observation 1: When the extended bandwidth or the number of extended RBs does not exist in Table 5.3.2-1, the NW should not indicate only one of them because the UE does not have direct information about the other one.

Observation 2: Directly indicating the extended bandwidth in Table 5.3.2-1 and/or its corresponding RB number to the UE as the extension information is inaccurate because the GBs on both sides of the extended bandwidth are also taken into account.

Observation 3: The number of extended RBs obtained from the extended bandwidth is not always an integer, so there will always be some ‘GBs’ in the extended portion.

Proposal 2: The GB of the original CBW should be retained, and the ‘GB’ of the extension part should only be retained on the side that is far away from the original CBW.

- ‘GB’ of the extension part = [(0.5* BWChannel x 1000 (kHz) – ceil(NRB/2) x SCS x 12) -SCS/2], BWChannel and NRB correspond to the original UE CBW.

Proposal 3: The extended RBs (NRB_e) on each side should be equal to ceil(NRB/2), where NRB is the maximum transmission bandwidth configuration of the original UE CBW.

Observation 4: When both sides are extended by 0.5* UE CBW, all the outer allocations including full RB allocation could convert to inner and the MPR could always refer to the inner value regardless the specific RB allocation.

Observation 5: Based on the fixed extended ratio, the new inner RB region of asymmetric extension could be calculated directly but the UE’s RB allocation stays the same, so there is no need for the NW to indicate extension RB, or RB0shift & BS_NRB to the UE anymore.

Observation 6: For UEs that extend 0.5 times the original bandwidth on one side, LCRB should be less than or equal to 2/3 * NRB for the inner RB region.

Proposal 4:

- If both sides of the UE extend by 0.5 times the UE CBW, then the entire RB allocation inside the original UE CBW is inner region.

- If only the higher side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be re-defined as:

RBStart_Low  ≤  RBStart  ≤  NRB – LCRB, and LCRB  ≤  ceil(2/3 *NRB)

- If only the lower side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be re-defined as:

RBStart  ≤  RBStart,High, and LCRB  ≤  ceil(2/3 *NRB)

Proposal 5: Based on the fixed extension ratio, i.e., 0.5* UE CBW on each side, the following signaling mechanism could be considered:

- It is up to the NW to judge the gap width on both sides of the UE.

- When the gap on one side is ≥ 0.5* UE CBW and the UE supports [MPR improvement with BW extension-r19], the NW sends the signaling (e.g., BWExtension -LowerSide-Rel19, or BWExtension-HigherSide-Rel19) to indicate the extension on the corresponding side.

When receiving the above extension signaling, the UE judges its allocation type based on the new RB region defined in Proposal 4.

- When the gap on both sides is ≥ 0.5* UE CBW and the UE supports [MPR improvement with BW extension-r19], the NW sends two signaling (e.g., BWExtension -LowerSide-Rel19 + BWExtension-HigherSide-Rel19) or one combined signaling (e.g., BWExtension -BothSides-Rel19) to indicate the extension on both sides.

When receiving the above extension signaling, the MPR of inner region could be used regardless of the RB allocation type.

Observation 7: The extension signaling overhead is very low as it only depends on the original UE CBW and the gap width and will not change with the RB allocation state.

Proposal 6: The extension ratio on both sides, 0.5* UE CBW should be stipulated in the spec, and there should be no other UE capability other than support for the BW extension.

Proposal 7: Extension of the IBE should not be used in the guard band of UE CBW or in the ‘GB’ of extension part, but should be applied from the edge of the original bandwidth and evaluated in the extension RBs on each side at a per-RB granularity.

Observation 8: Calculating IBE based on the extended UE CBW would limit the extended UE CBW inside the maximum transmission bandwidth range of the spec.

Observation 9: Using IBE of the actual UE CBW edge as a reference value to evaluate the emission level of the entire extended portion will reduce the testing burden.

Proposal 8: For IBE of both original CBW and extended CBW, NRB should be based on the original UE CBW, ΔRB should also be limited within the original UE CBW.

Observation 10: For UE CBW with full RB allocation, the IBE of the extended part could be simplified to:

IBE_general = max⁡(PRB - 30 dB,

max⁡{-25,  20*log10EVM -3 +5LCRB,

-57dBm + 10log10(SCS/15kHz)- PRB})

Proposal 9：For UE CBW with extension, the emission requirement of the extended part should change to IBE_extension, which is equal to that of the farthest non-allocated RB in the original UE CBW:

IBE_extension= max⁡(PRB - 30 dB,

max⁡{-25-10lg(NRB/LCRB),  20*log10EVM -3-5*∆RB_o-1LCRB,

-57dBm + 10log10(SCS/15kHz)- PRB})

- NRB represents the maximum transmission bandwidth configuration for each UE channel bandwidth before extending.

- LCRB is the RB length of the original transmission Bandwidth before extending.

- ∆RB_o is the frequency offset between the allocated RB and the measured farthest non-allocated RB inside the original UE CBW before extending.

- For UE CBW with full RB allocation, ∆RB_o=0, NRB= LCRB.

Proposal 10: For UE with extension, SE should apply beyond (UE CBW+5MHz) from the edge of the extended bandwidth.

Proposal 11: For UE CBW with extension, the integral region of OOBE should be based on original UE CBW.

- References

- R4-2420527, WF on power domain enhancement, Huawei, HiSilicon, RAN4 #113.

- R4-080678, UE In-band Emission Requirements, Qualcomm Europe, RAN4#46-bis.