# 3GPP TSG-RAN WG4 Meeting #114	R4-2500737

# Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Discussion on the subband configurations and guardbands for gNB SBFD

Agenda Item:	7.23.1

Document for:	Approval

Introduction

In the last meeting, we further discussed system parameters for gNB SBFD. The WF for gNB SBFD was captured in [1]. In this contribution, we focus on subband configurations and guard band issues for gNB SBFD.

Discussion

In the last meeting, we discussed operating bands and channel bandwidth support for SBFD operation. We precluded those bands with small channel bandwidth, NR-U bands and FR2-2 bands for SBFD. The agreements can be found as follows:

Issue 1-1-3: Specification implementation of operating bands support for SBFD Agreement: - The following TP is adopted for clause 5.2 in TS 38.104 - Subband full duplex (SBFD) is designed to operate in NR TDD operating bands in FR1 which are defined in Table 5.2-1 except bands n34, n46, n51, n53, n54, n96, n101 and n102, and NR operating bands in FR2-1 which are defined in Table 5.2-2. - FFS a note in the band table 5.2-1 to indicate the NR TDD bands not applicable to SBFD operation.

From our perspective, it is beneficial to leave note in the specification to indicate which TDD bands not applicable to SBFD operation. It can be a guidance if gNB vendors want to implement SBFD on specific bands.

Proposal 1: Leave a note in the band table 5.2-1 to indicate the NR TDD bands not applicable to SBFD operation.

##### Issue 1-3-1: Clarification on declaration and flexibility Agreement: - The SBFD capable BS product shall not operate any SBFD configuration (DL/UL subband) other than those that have been declared - The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested. - FFS on how to capture this in the specification. - This restriction shall at least be applied to FR1 WA BS - FFS other BS classes - FFS the possibility of testing the worst-case scenario in each RF front end operational configuration from the supported SBFD configurations. - FFS how to capture this in the specifications - Issue 1-3-2: DL/UL subband configurations support for SBFD Agreement: - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. - FFS different sizes for specific UL subband for DUD and DU/UD. - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth. - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS

For clarification on declaration and flexibility, we introduced very strict principles for SBFD configurations. We suggest it only applied to FR1 WA BS, not for FR1 MR/LA BS and FR BS for SBFD operation. For FR1 WA BS, we see the difficulty to implement SBFD from the feasibility study, since it requires large RSIC to reduce the self-interference within the BS. There are many different kinds of isolation techniques used for RSIC to guaranteed the feasibility of FR1 WA SBFD operation. Restricting the subband configurations for FR1 WA BS may reduce the design difficulty for such SBFD capable BS product. However, for other types of BS, it is less challenging to support SBFD. Restricting SBFD configuration will not be too necessary.

Proposal 2: Only apply the restriction of SBFD configuration to FR1 WA BS, not for FR1 MR/LA and FR2-1 SBFD capable BS product.

- Issue 1-3-6: Other restriction on DL subband size Way Forward: - FFS the following restriction on DL subband size: - For SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size). - FR2 minimum CBW to support SBFD is 100MHz.

For other restriction on DL subband size, we understand the intention is to gurantee DL subband can cover at least one SSB. This comes from the sync raster design. However, we think this restriction is unnecessary. From our perspective, we think the channel bandwidth can cover at least one SSB is enough, we don’t need further restriction DL subband size to cover at least one SSB.

For FR2 minimum CBW to support SBFD, unless there is a demand from operator request, we prefer not to have further restriction on FR2 minimum CBW. FR2-2 was preclude for SBFD. Then all FR2-1 channel bandwidths can support SBFD.

Proposal 3: It is not necessary to introduce the following restriction on DL subband size:

o	For SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size).

o	FR2 minimum CBW to support SBFD is 100MHz.

##### Issue 1-4-1: BS RF Specification structure for SBFD requirements ##### Agreement: - Draft CR’s endorsed against 38.104 does not imply that a new specification could not be agreed.

For BS RF specification structure for SBFD, we prefer to create new sub-clauses in TS 38.104.

SBFD-capable base stations do not only work in SBFD symbols, but also in UL or DL symbols; When operating in SBFD symbols, potentially new BS RF requirements for SBFD-capable BS apply; when working in UL or DL symbols, legacy BS Tx/ Rx requirements apply. The new SBFD requirements and legacy BS RF requirements together constitute the full set of specifications for SBFD-capable BS. In our view, defining them in one specification will not cause a sense of fragmentation, rather than defining a new specification. Furthermore, it is easy to edit in the same protocol for maintenance and so on.

Proposal 4: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD is preferred.

Conclusion

This contribution discusses SBFD configurations and guardbands for gNB SBFD. The following proposals are made:

Proposal 1: Leave a note in the band table 5.2-1 to indicate the NR TDD bands not applicable to SBFD operation.

Proposal 2: Only apply the restriction of SBFD configuration to FR1 WA BS, not for FR1 MR/LA and FR2-1 SBFD capable BS product.

Proposal 3: It is not necessary to introduce the following restriction on DL subband size:

o	For SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size).

o	FR2 minimum CBW to support SBFD is 100MHz.

Proposal 4: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD is preferred.

- References

[1] R4-2413514, Way Forward for [112][307] NR_duplex_evo_General, Samsung, RAN4#112.

[2] R4-2419907, Way Forward for [113][306] NR_duplex_evo_General, Samsung, RAN4#113