# 3GPP TSG-RAN WG4 Meeting #114	R4-2500738

# Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Discussion on band and system parameters for IoT-NTN TDD mode

Agenda Item:	7.31.2

Document for:	Approval

Introduction

In RAN#105, a new WID [1] on introduction of IoT-NTN TDD mode was approved. RAN4 started the discussion on the general aspect and study of impact due to periodic pattern in the last meeting. The work plan [2] and WF [3] were generated on IoT-NTN TDD mode. In this meeting, we will provide our views on band and system parameters for IoT-NTN TDD mode.

Discussion

According to the WID [1], the objectives of RF aspects for IoT NTN TDD mode are captured as follows:

- Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

According to the Workplan [2], the work was arranged for the first meeting.

- Begin discussions starting from Nov’24 and target completion by the RAN#106 checkpoint. Start specifying a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL. This includes detailing: - Band numbering & band definition (including EARFCN, raster etc.), DL and UL channelization, and specifying a channel bandwidth of 200 kHz (180 kHz occupied bandwidth) - Start discussion on SAN & UE RF requirements that can be re-used from previously defined S/L band. Address RRM and RF core requirements associated with the new NB-IoT TDD NTN mode Discuss requirements stemming from introduction of TDD mode for NB-IoT NTN and potential impact of the periodic pattern to RRM: offset-subframes, DL-UL gap

For band definition, it was clear to define a new NB-IoT NTN band from 1616MHz – 1626.5MHz for DL and UL. For band numbering, band 249 was suggested to be used for IoT NTN TDD band 1616-1626.5 MHz. Since band n251 and n250 [4] were defined as combined L-band for NR NTN, band number 251 and 250 were reserved for LTE IoT NTN

### Issue 1-1: Operating Bands <Band number> - RAN4 to consider the following proposals for the requirement definition during the normative work item phase in future RAN4 meetings. - Assign “[249]” E-UTRA operating band number for the new TDD [1616-1626.5] MHz NTN NB-IoT frequency band introduction. E-UTRA operating bands for satellite access E-UTRA Operating Band; Uplink (UL) operating band BS receive UE transmit; Downlink (DL) operating band BS transmit UE receive; Duplex Mode. FUL_low – FUL_high; FDL_low – FDL_high. 256; 1980 MHz; –; 2010 MHz; 2170 MHz; –; 2200 MHz; FDD. 255; 1626.5 MHz; –; 1660.5 MHz; 1525 MHz; –; 1559 MHz; FDD. 254; 1610 MHz; -; 1626.5 MHz; 2483.5 MHz; -; 2500 MHz; FDD. 2532; 1668 MHz; -; 1675 MHz; 1518 MHz; -; 1525 MHz; FDD. 252; 2000 MHz; -; 2020 MHz; 2180 MHz; -; 2200 MHz; FDD. [249]; [1616] MHz; -; [1626.5] MHz; [1616] MHz; -; [1626.5] MHz; TDD. NOTE 1: Satellite bands are numbered in descending order from 256 NOTE 2: UE assigned to channels and allocated frequency resources in the lower portion of Band 253 may experience blocking or harmful interference from terrestrial networks in adjacent or nearby frequencies when operating in the proximity with terrestrial base stations..

bands. Band [249] was the first choice for the first IoT NTN TDD band. The WF was captured as follows.

From our perspective, we can also allocate a new range for band number of NTN TDD band, as shown in Figure 1. For now, LTE band number was from 1-106, and band 106 is an LTE band with maximum band number. LTE FDD and TDD bands are defined separately, along with separate EARFCN number range. The maximum band number for IoT NTN FDD band number was 256 and band number was defined in a descending way. If we choose [249] for the first IoT NTN TDD band, there is possibility that IoT-NTN FDD and TDD bands are defined in a mixed way with more NTN bands introduced in the future. Then, the EARFCN design for IoT NTN bands can be complex. Thus, we suggest to define NTN TDD band from [128] in an ascending way, which is separated from IoT NTN FDD bands. The starting number [128] can be further discussed. In this way, IoT TDD bands and FDD bands can use separate band ranges, it is also beneficial for channel raster definition for new added IoT NTN bands.

LTE band number

IoT NTN TDD band

IoT NTN FDD band

256

1

106

[128]

Figure 1. Suggested band number allocation for IoT NTN TDD

Proposal 1: Another alternative is to define sperate band number range for IoT TDD bands, starting from band [128] in an ascending way.

For channel raster design, there are two Options proposed in the last meeting. From our perspective, we see the logic in Option 1 and the numbers were calculated based on the rational in [5]. We prefer to adopt Option 1 for the raster design. Since we proposed a new band number range for IoT TDD bands, we also want the EARFCN numbers can be separate from IoT FDD band. We donot want the IoT TDD EARFCN in between FDD NTN NOffs-DL and NOffs-UL. We can go with a new separate EARFCN range below FDD NTN NOffs-DL starting from 196607.

- Use a channel raster of 100 kHz and a Step size of <1> - Both FDL_low and FUL_low equal 1616 MHz. - For the range of both NDL and NUL: - Option 1: Starts from 2^18-2^16-1=196607 (below FDD NTN NOffs-DL) or 2^18-2^14-1=245759 (in-between FDD NTN NOffs-DL and FDD NTN NOffs-UL) - Option 2: {198196 - 198300}

Proposal 2: Define the E-UTRA channel numbers for IoT NTN TDD bands starting from 196607.

Conclusion

This contribution gives our initial views on system parameters for FR1-NTN supporting less than 5MHz channel bandwidth. The following proposals are made:

Proposal 1: Another alternative is to define sperate band number range for IoT TDD bands, starting from band [128] in an ascending way.

Proposal 2: Define the E-UTRA channel numbers for IoT NTN TDD bands starting from 196607.

- References

[1] RP-242415, New WID on introduction of IoT-NTN TDD mode, RAN#105.

[2] R4-2419663, IoT_NTN_TDD WP, RAN4#113.

[3] R4-2419795, Way Forward for [113][317] IoT_NTN_TDD, Qualcomm, RAN4#113.

[4] R4-2418357, draftCR to TS38.108 Introduction of NR-NTN combined L-band, ZTE Corporation, Sanechips, RAN4#113.

[5] R4-2419720, Discussion on the First Introduction of an NTN NB-IoT TDD Frequency Band,Thales, RAN4#113.