3GPP TSG-RAN WG4 Meeting # 114 	R4-2500747

Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Discussion on the RF requirement for device 1

Agenda Item:	7.24.3

Document for:	Approval

- Introduction

RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

  - Use band n8 as an example band

In [1], the WID for R19 AIoT was approved:

In this contribution, we provide our views on the OTA test method.

- Discussion

  - Test setup

Compared to legacy NR, the main difference for AIoT device 1 test is the CW node. Since the Tag can only reflect power from the CW, it is necessary to discuss how to set the CW source. In [2], two types of test method can be used for NR, i.e., anechoic chamber and reverberation chamber. Considering the CW exist, the reverberation chamber may bring unexpected interference to the device which will distort the test results. In ETSI and GS1 RFID test document, the anechoic chamber is also used. To save time, we prefer only consider the anechoic chamber in this release.

Proposal 1: Only focus on the anechoic chamber method for device 1 test in R19.

In [3] and [4], the test setup for RFID is shown in below:

Figure I RFID test setup in ETSI [3]

Figure I RFID test setup in GS1 [4]

In ETSI, the measurement antenna is oriented to the direction of maximum D2R power, but there is no detailed description on how to decide this direction. In GS1, the tag shall be oriented on the mast such that its longest dimension is horizontal, and the Tx and Rx antennas are rotated such that their major axial ratio axes are horizontal. From antenna design perspective, the tag usually uses meander-line dipole which is simple and has compact size and the typical pattern is shown in Figure II

Figure II radiation pattern for meander-line dipole [5]

For a dipole antenna, the maximum antenna gain appears at the phi = 90° plane which is the vertical plane of the dipole antenna long dimension center. In this plane, the antenna gain is similar and both Tx and Rx test antenna should be located on it.

Observation 1: For a dipole antenna, the maximum antenna gain appears at the vertical plane of the dipole antenna long dimension center.

Proposal 2: Both the Tx/Rx test antenna should locate at the vertical plane of the tag’s long dimension center.

Considering supporting 3D scan will significantly increase the cost of test, which is not suitable for the low-cost device, it is suggested to discuss whether the 3D scan is necessary.

Proposal 3: It is suggested to discuss whether the 3D scan is necessary for device test and the feasibility to verify the device performance on single angle.

  - Tx requirement test

For the Tag emission requirement verification, one critical issue the influence of CW. Since the Tag emission rely on the incoming CW signal, which means both the CW and D2R are transmitted simultaneously, the measurement antenna will receive the power from both CW and D2R, so it is necessary to consider how to separate them and get the pure D2R power.

Observation 2: Since the Tag emission rely on the CW transmission, the power of CW and Tag will be mixed at measurement antenna.

There are two possible ways to solve this issue:

- Measure the power P1 (D2R+ CW) when device exist, then remove the device and measure the power P2 (CW only), the D2R = P1 – P2

- Implement RF shielding to provide enough Tx/Rx isolation. In [4], the minimum isolation is 50 dB.

Proposal 4: The following two method can be considered to eliminate the influence of CW source during the device emission test:

Approach 1: Measure the emission power with/without device separately, and calculate the D2R power

Approach 2: Implement RF shielding to provide enough Tx/Rx isolation

  - Rx requirement test

Since the Tag is new hardware type in RAN4, it is still not clear how to define the Rx requirement. For example, the performance metric of AIoT device reception, the definition of REFSENS, etc. The tag also needs feedback to the TE whether the reception is correct or not, and such procedure may rely on the RAN1 design. All these puzzles will make the Rx test method discussion inefficient.

Observation 3: The REFSENS definition for tag is not clear and the test procedure may rely on the detailed design in RAN1.

So, the test method for Rx can be started later at least until RAN4 has clear definition

- Conclusion

In this contribution, we share our initial views on AIoT device OTA test method.

Proposal 1: Only focus on the anechoic chamber method for device 1 test in R19.

Observation 1: For a dipole antenna, the maximum antenna gain appears at the vertical plane of the dipole antenna long dimension center.

Proposal 2: Both the Tx/Rx test antenna should locate at the vertical plane of the tag’s long dimension center.

Proposal 3: It is suggested to discuss whether the 3D scan is necessary for device test and the feasibility to verify the device performance on single angle.

Observation 2: Since the Tag emission rely on the CW transmission, the power of CW and Tag will be mixed at measurement antenna.

Proposal 4: The following two method can be considered to eliminate the influence of CW source during the device emission test:

Approach 1: Measure the emission power with/without device separately, and calculate the D2R power

Approach 2: Implement RF shielding to provide enough Tx/Rx isolation

Observation 3: The REFSENS definition for tag is not clear and the test procedure may rely on the detailed design in RAN1.

- References

- RP-XXXX WID

- 3GPP TR 38.870

- ETSI EN 302 208

- GS1 EPC global, Tag Performance Parameters and Test Methods Version 1.1.3

- Tang, Fangye et al. “Meander Dipole Antenna design for Passive UHF RFID Tags.” (2015).