3GPP TSG-RAN WG4 Meeting # 114	R4- 2500772

Atens, GR, 17th – 21st February, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements of on-demand SSB Scell operation for Rel-19 NES

Agenda item:	7.25.2.1

Document for:	Discussion

Introduction

In agreed WID for Rel-19 NES operation [1], following objective was captured for RAN4 RRM core requirements:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we provide our views on RRM core requirements for RRM requirements of on-demand SSB Scell operation for Rel-19 NES.

Discussion

General On-demand SSB requirements

Scenario for On-demand SSB requirements in Case 2

In RAN4#112bis, RAN4 discussed scenario of OD-SSB Case 2 regarding the SSB relation between OD-SSB and AO-SSB, and RAN4 assumes to follow “Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities” for further discussion. Meanwhile, RAN4 sent LS regarding SSB relation in on-demand SSB and SSB adaptation on Scell to RAN1.

Agreement: - In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement. - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies. - Send the above information to RAN1, further discuss the text including the questions to RAN1 in the LS. - RAN4 will inform RAN1 if any further agreement.

In the previous meeting, RAN1 agreed to send reply LS[4] as follows:

Obj 1 Reply to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?): - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. - The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB. - RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB. Reply to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?): - RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst. - RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB. Reply to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?): - The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB. Reply to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?): - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

According to the reply LS, following relation can be obtained

- The periodicity of OD-SSB is equal to or smaller than that of AO-SSB

- The frequency location of OD-SSB is the same as the frequency location of AO-SSB at least for the case where AO-SSB is not CD-SSB

- SS/PBCH blocks with the same SSB indexes for AO-SSB and OD-SSB are QCLed at least for the case when the centre frequency locations of AO-SSB and OD-SSB is same

If AO-SSB and OD-SSB have same SSB carrier frequency, same periodicity, same SSB index, but different offset, a UE can measure and combine the result of AO-SSB and OD-SSB as illustrated in Figure 1. It is beneficial for accelerating the measurement with effective periodicity, which can be half of those of AO-SSB or OD-SSB.

In that senese, we think the scenario with same SSB carrier frequency, same periodicity, same SSB index, but different offset needs to be considered in Case 2.

Figure 1 Combination of AO-SSB and OD-SSB in Case 2

Proposal 1: RAN4 to consider the scenario with same SSB carrier frequency, same periodicity, same SSB index, but different offset in Case 2

Proposal 2: RAN4 to support AO-SSB and OD-SSB combination at least for the limited scenario

L1 measurement for SCell in Case 1

In previous RAN4 meeting, L1 measurement for deactivated SCell was discussed. The issue in the summary[3] is as follows:

Issue 1-1-2: L1 measurement before SCell activation - Proposals - Option 1: Qualcomm, Ericsson, Apple - L1 measurements are not supported for deactivated SCell. - Option 2: Nokia - Wait RAN1’s further clarification on the applicable scenario. - Recommended WF - Further discussion. Issue 1-1-3: L1 measurement after SCell activation - Proposals - Option 1: CMCC - Unified design between L1 and L3 measurement is preferred, regarding whether OD-SSB or AO-SSB or both will be used for measurement, and when/how to perform the measurement. - Option 2: CATT, Ericsson, Xiaomi - RAN4 to define OD-SSB based L1 measurement requirements based on RAN1 progress. - Option 3: LG, Apple - RAN4 to update the L1 measurement requirement based on OD-SSB. - TSSB = the periodicity of the activated OD-SSB configured for L1-RSRP measurement. - Recommended WF - Further discussion.

It seems that L1 measurement scenario needs to be clarified. In our understanding, L1 related measurement can be happended during SCell activation and after SCell activation. During SCell activation, L1 measurement can be performed based on SSB or semi-persistenet or periodic CSI-RS for CSI reporting. After SCell activation, L1 related measurement includes L1-RSRP, L1-SINR, BFD, CBD purpose. Since we are focusing measurements using SSB, we think L1 measurement during and after SCell activation is valid scenario for OD-SSB.

Regarding L1 measurement in Case 1, after SCell activation, a UE can perform L1 measurement such as L1-RSRP measurement using OD-SSB. In the current specification, the periodicity of the SSB-Index in ssb-periodicityServingCell configured for L1-RSRP measurement is used for the requirements. One simple approach is to replace SSB periodicity to activated OD-SSB periodicity in Case 1 as shown in Table 1.

Table 1 Measurement period TL1-RSRP_Measurement_Period_SSB for FR1 (OD-SSB in Case1)

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = the periodicity of the SSB-Index in activated OD-SSB configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17 |  |

Proposal 3: RAN4 to consider L1 measurement during and after SCell activation as below

Measurement period TL1-RSRP_Measurement_Period_SSB for FR1 (OD-SSB in Case1)

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = the periodicity of the SSB-Index in activated OD-SSB configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17 |  |

Cell identification in Case 1

In Case1, always-on SSB (AO-SSB) is not expected in SCell. A UE can be indicated multiple OD-SSB activation/deactivation as illustrated in Figure 1. In Figure 1, the UE receives OD-SSB indication1 after activation time, which is deactivated after 4 SSB bursts. After some time after OD-SSB deactivation, the UE receives another OD-SSB indication2. In the legacy SSB measurement, the time between each SSB bursts are always same (e.g. period). However, in OD-SSB operation, it is not guaranteed the time between multiple OD-SSB activation/deactivation is always same (e.g. time between OD-SSB deactivation and OD-SSB indication2), which may lead additional efforts for UE to perform cell identification. For example, if the time is long enough to lose the sync, the UE may need to detect PSS/SSS and/or acquire the index of the SSB being measured. For another example, if the time is short enough to keep the sync, the UE can skip PSS/SSS detection and/or SSB index acquisition.

Observation 1: If the time between multiple OD-SSB activation/deactivation is short enough to keep the sync, PSS/SSS detection and/or SSB index acquisition can be skipped

From the observation, the benefit is clear when the time between multiple OD-SSB activation/deactivation is ‘short’. One way to define whether the time is short or not is to define a threshold in order to decide which requirements to apply. If the time is shorter than the threshold, the UE can skip PSS/SSS detection and/or SSB index acquisition, and perform measurements.

Proposal 4: RAN4 to specify cell identification requirements based on the time between multiple OD-SSB activation/deactivation

- If the time is shorter than a threshold, the UE can skip PSS/SSS detection and/or SSB index acquisition

Figure 2 OD-SSB transmission in Case1

Cell identification in Case 2

In Case 2, it can be assumed that a UE has already detected PSS/SSS and acquired the index of the SSB being measured using AO-SSB before OD-SSB activation indication. At least for Scenario 1 in Case 2, OD-SSB can be in-sync because of same time/frequency relation with AO-SSB. So, we think cell identification procedure can be skipped at least for Scenario 1 in Case 2.

Proposal 5: Skip cell identification at least for Scenario 1 in Case 2

Additional reqiurment for MAC-CE processing

When a UE receives OD-SSB activation indication, NW transmits actual SSB burst staring from Time instance A. In some case, Time instance A can be located within the SSB burst. Per RAN1 agreement, NW is allowed to transmit the first actually transmitted SSB index within the first possible OD-SSB burst as illustrated in Figure 3. For instance, NW transmites SSB bursts except SSB index 0. From the UE perspective, the UE is required to measure every index, however, some SSB indices are missing. After the first OD-SSB burst, if the UE wants to combine the SSB to fulfil the required accuracy, the number of combination between SSB indexes can be different. In order to avoid such problem, we suggest to start measurement at the next OD-SSB burst, if Time instance A is located within the first possible OD-SSB burst.

Figure 3 Time instance A within OD-SSB burst

Proposal 6: If Time instance A is located within the first possible OD-SSB burst, start measurement at the next OD-SSB burst

On-demand based deactivated SCell L3 requirements

In RAN4#112bis, RAN4 introduced the time window in which a UE is required to measure OD-SSB. The motivation of the time window is to reduce the UE power consumption, since NW may not guarantee OD-SSB deactivation. According to the agreement, the UE performs ‘fast L3 measurement’ during the time window, otherwise ‘slow L3 measurement’ is expected. In RAN4#113, slow L3 measurement is defined as using legacy deactivated SCell measurement, which means that the UE shall meet the requirements using measCycleSCell for measurement periodicity. The definition of fast measurement, and the configuration of the time window including the relationship with OD-SSB deactivation indication is still open issue. We would like to touch upon the open issue one by one.

Agreements(RAN4#112bis): Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. - After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 2-1: fallback legacy requirement - Option 2-2: stop measurement - Option 2-3: No requirement - Time window means the time duration starting from receiving OD-SSB activation indication - FFS the time window configuration or pre-defined - FFS the fast L3 measurement requirement - FFS the relation between time window and OD-SSB deactivation command in the time domain When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

The relation between time window and OD-SSB deactivation command in the time domain

In the previous meeting, RAN1 draw the agreement on supporting two options for deactivating OD-SSB transmission as follows:

Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer

In our understanding, above options can be organized into two categories. Option 1 is explicit deactivation indication, which implies OD-SSB is activated until explicit deactivation indication. Option 2 is implicit deactivation indication, means that the the number of OD-SSB to be transmitted is already configured/indicated by NW. When a UE is indicated either Option 1 or Option 2, it is clear that the UE is not required to measure the deactivated SCell. However, the exact time is not clear especially for Option 1.

For Option 1, one approach can be that the UE stops the measurement (fast or slow) at the time when the UE receives OD-SSB deactivation indication. However, the indication is containd via MAC-CE, therefore, it may requires some time for processing. Hence, we think it is reasonable that the UE stops the measurement (fast or slow) at the first slot after 3ms transmitting feedback on OD-SSB deactivation indication.

For Option 2, it is rather straight forward that the deactivation time is already configured/indicated. We think the UE stops the measurement(fast or slow) at the end of Nth OD-SSB burst.

Proposal 7: Consider the relation between OD-SSB deactivation indication and measurement as follows:

- In case of Option 1 (Explicit deactivation indication): UE is no required to measure (fast or slow) deactivated SCell at the first slot after 3ms transmitting feedback on OD-SSB deactivation indication

- In case of Option 2 (Configuration of N OD-SSB bursts to be transmitted): UE is not required to measure (fast or slow) deactivated SCell at the end of Nth OD-SSB burst

Figure 4 OD-SSB deactivation indication and measurement

Duration of the time window

In the pervious meeting, following agreement was captured in [2].

Agreement: The time window is based on the number of OD-SSB samples and UE provided one valid report. - The uncertainty time should also be considered, such as T_min, T_processing and T_uncertainy - UE should have enough number of OD-SSB samples to achieve the measurement requirement. - The number of OD-SSB is FFS. - FFS whether UE measurement reporting is needed

In the previous meeting, RAN4 defines the duration of the time window with two parts. The one is the uncertainty time (e.g. T_min, T_processing and T_uncertainy), and the other is actual measurement time, which consists of enough number of OD-SSB samples to achieve the measurement requirement

In our understanding for the uncertainty time, T_min includes MAC-CE processing time, and T_processing is additional processing time for UE, if required. T_uncertainty is the first available SSB that UE can receive.

For the actual measurement time, one principle is that the time window should have enough number of OD-SSB samples to achieve the measurement requirement. The issue is which requirements to follow. During the discussion, common understanding between companies is that longer time window is applied, if the cell is not identified, and shorter time window is applied for already cell identified. We think it is reasonable to apply the majority view. Hence, for the cell not identified, the number of OD-SSB samples of actual measurement time follows cell identification requirements (e.g. Tidentify_intra_without_index or Tidentify_intra_with_index), while for the cell already identified, the number of OD-SSB samples of actual measurement time follows measurement period requirements (e.g. TSSB_measurement_period_intra).

For the UE measurement reporting, we think it is reasonable. When NW indicates OD-SSB, usually the intention is to measure the quality of deactivated SCell. So, we think one valid report can be enough for fast mode. Furthermore, providing one valid report and fallback to slow mode can help the UE save power. One may think it can be proplematic if the UE reports poor condition, and fallback. In this case, we think adding side condition can resolve the issue. For example, if the reports does not fulfil the side condition, the UE continues performing measurements in ‘fast mode’ during the time window.

Figure 5 Duration of the time window

Proposal 8: Support duration of the time window as below:

- Duration of the time window = Uncertainty_time + Actual_measurement_time

  - Uncertainty_time = Tmin+Tprocessing (if required)+Tuncertainty

  - Actual_measurement_time

    - For the cell not identified: follow cell identification requirements

    - For the cell identified: follow measurement period requirements

Proposal 9: Support fallback to slow mode when UE provided one valid report under side condition

- If side condition is not fullfiled, the UE continues performing measurements in ‘fast’ mode during the time window

Relationship between the time windows

According to the above agreement, RAN4 defined the starting point of the time window at the receiving time of OD-SSB activation indication. In general, the UE measures activated OD-SSB in the time window, if there is single activated OD-SSB. However, if there are multiple activated OD-SSB, the measurement behaviour is not clear.

For example in Figure 6a, Time window1 starts from OD-SSB activation1 indication, and measures OD-SSB1. Meanwhile, Time window 2 starts from OD-SSB activation2 indication, and measures OD-SSB2. Note that OD-SSB1 is not deactivated during Time window2. From UE perspective, it is clear that OD-SSB1 is measured during Time window1, and OD-SSB2 is measured during Time window2.

However, if two time windows are overlapped in Figure 6b red circle, the measurement behaviour is not clear. In this case, the UE may need to follow different measurement behaviour, which may depend on the configuration of the second OD-SSB activation indication at the overlapping area in the time domain.

Simple approach to address the issue is that the UE prioritizes to measure the second OD-SSB activated by the second OD-SSB activation indication. For example in Figure 6b, the UE prioritizes Time window2 when the UE receives OD-SSB activation 2.

Proposal 10: RAN4 to specify the relationship between the time windows

- In case of overlapping of time windows, the UE measures the OD-SSB indicated by the latest OD-SSB activation indication

Figure 6 Multiple OD-SSB activation indication and the time window

Fast L3 measurement requirements

In the pervious meeting, following agreement was captured in [2].

Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement Agreement: For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF.

According to agreement, the UE shall apply OD-SSB periodicity within the time window for deactivated SCell measurement regardless of measScycleSCell and DRX cycle at least for CSSF=1. The remaining open issues are whether a lower bound is specifid or not, and the other cases of CSSF.

For a lower bound of OD-SSB periodicity, the main concern is that the indicated OD-SSB periodicity may be small (e.g. 5ms), so the power consumption of the UE can be increased. In our understanding, even smallest SSB periodicity value has been supported from Rel-15, so there is no feasibility issue. Furthermore, RAN4 already have achieved agreement on the duration of the time window, which limits the ‘fast’ measurement time. Hence, the benefit of the lower bound is not clear.

For the other case of CSSF, we think applying legacy rule is enough. For example, OD-SSB periodicity within the time window can be scaled by CSSF.

Proposal 11: Not to specify lower bound

Proposal 12: The legacy CSSF can be reused

Starting point of the time window

In the previous meeting, RAN4 introduced the time window starting from receiving OD-SSB activation indication. Based on the agreement, in our understanding, it seems that the starting point of the time window is the slot or the symbol of the PDSCH containing the MAC-CE indicating OD-SSB activation. We have a concern that there can be different understanding between NW and UE regarding the starting point of the time window. From NW perspective, staring of the time window is clear. However, from UE perspective, a UE can acknowledge the OD-SSB activation after decoding the PDSCH and proceeding MAC-CE process in the higher layer. Technically, UE acknowledges OD-SSB activation inciation after Tmin, which implies that the time window starts after Tmin. In order to address the issue, we would like to revise the agreement as follows:

Time window means the time duration starting from receiving OD-SSB activation indication→ Time window means the time duration starting from Tmin after receiving OD-SSB activation indication

Proposal 13: Revise the agreement regarding the time window as follows:

- Time window means the time duration starting from receiving OD-SSB activation indication→ Time window means the time duration starting from Tmin after receiving OD-SSB activation indication

On-demand based SCell activation

Following agreements have been achieved:

RAN4#112bis Issue 1-4-1: General Agreement: - RAN4 focuses on OD-SSB based SCell activation known and unknown requirement for both Case 1 and Case 2 and not specifically differentiate Scenario 2/2A scenarios. Issue 1-4-3: known cell condition design – principle Agreement: - The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command. - When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met. - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB - Other principles are not precluded. Issue 1-4-5: Which type of SSB is used during SCell activation Agreement: - In Case 1, UE is required to always use OD-SSB for Scell activation. - In Case 2, when both OD-SSB and always-on SSB are transmitted, - RAN4 assumes UE to use OD-SSB to perform SCell activation as a baseline - The agreement can be revisited if different with other WG RAN4#113 Issue 1-4-1: The known cell condition – measurement reporting Agreement: - In case 1, known/unknown condition should be defined based on only OD-SSB. - In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB. - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue. - Note 2: the agreement is valid at least for the scenario with the same frequency for always-on SSB and OD-SSB. - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure. Issue 1-4-5: SCell activation requirement – OD-SSB periodicity Agreement: - In Case 1, the SCell activation delay requirement is defined based on OD-SSB periodicity. - Existing SCell activation delay requirement shall apply based on OD-SSB periodicity. - In Case 2, RAN4 to define the SCell activation delay requirement based on OD-SSB periodicity if the time instance receiving OD-SSB activation indication is no later than the time instance when receiving the SCell activation command. Otherwise, RAN4 to define the requirement based on always-on SSB for SCell activation. - This agreement is valid under the assumption of the same spatial relation for OD-SSB and always-on SSB [with the same SSB index], and this can be revisited based on RAN1 LS reply. - Note: T_firstSSB_max for intra-band co-located scenario is discussed in another issue.

Known/unknown cell condition period

In the previous meeting, the principle of known cell and unkown cell using legacy principle has been agreed. In our understanding, as speficied in 38.133 - 8.3.2, Scell in FR1 is considered as ‘known’ when the UE has sent a valid measurement report for the SCell being activated, and the SSB measured remains detectable during the period max(5*measCycleSCell, 5*DRX cycles) before the reception of the SCell activation command. For SCell in FR2 is considered as ‘known’ when the UE has sent a valid L3-RSRP measurement report with SSB index, and SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation during the period equal to 4s or 3s before UE receives the last activation command, which depends on the UE power class. Otherwise, the SCell is considered to be unknown.

One issue to be resolved is whether to use legacy known/unknown cell condition. As defined in the legacy requirement, measCycleSCell is applied in the legacy known/unknown condition. The motivation of indicating OD-SSB activation is to perform measurements frequently. In order to align the motivation, we think the periodicity of OD-SSB can be used instead of measCycleSCell. Furthermore, it is already agreed that OD-SSB is used as baseline during SCell activation. To guaranteed synchronization and AGC, it is reasonable to use the period of OD-SSB for known/unknown condition.

Proposal 14: RAN4 to specify known/unknown cell condition period as below:

- At least for FR1: max(5*(period of OD-SSB), 5*DRX cycles)

Known/unknown cell condition in case of multiple OD-SSB activation indication

The other issue is that the OD-SSB used for known/unknown cell condition when there are multiple OD-SSB activation indication. As illustrated in Figure 6, multiple OD-SSB can be activated at SCell activation command. NW may activate multiple times to renew the time window. In this case, a UE may confuse which activated OD-SSB to be used for determining known/unknown cell condition. From NW perspective, better measurements report is valid for activating SCell. Therefore, the OD-SSB provided the better valid measurement report can be used for SCell activation.

Proposal 15: For the OD-SSB used for known/unknown cell condition when there are multiple OD-SSB activation indication, the OD-SSB provided better valid measurement report can be used for SCell activation

Figure 7 Known/unknown cell condition with multiple OD-SSB activation indication

Restriction during SCell activation

In general, during SCell activation, the UE measures SSB for synchronization, AGC, and time/frequency tracking. If the cell is known, the UE utilizes the information acquaired from measurements, and applies short activation delay. In that sense, if the cell is known, the same SSB should be applied for the OD-SSB used for fulfil known condition and the OD-SSB used for SCell activation. Also, we think OD-SSB should not be changed or deactivated during SCell activation.

Proposal 16: During the SCell activation following should be specified:

- If the cell is known, the same SSB should be applied for both known condition and SCell activation

- OD-SSB should not be changed or deactivated during SCell activation

Margin for applying OD-SSB activation indication

In the previous meeting, RAN4 acheived agreement whether to use OD-SSB for SCell activation. In Case 1, the SCell activation delay requirement is defined based on OD-SSB periodicity. In Case 2, SCell activation delay requirement based on OD-SSB periodicity if the time instance receiving OD-SSB activation indication is no later than the time instance when receiving the SCell activation command; otherwise, the requirement is based on AO-SSB for SCell activation.

Meanwhile, in RAN2#127bis following agreement has been captured

Agreements on OD-SSB SCell (RAN2#127bis)

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

As highlighted in the agreement, RAN2 agreed not to introduce new MAC-CE that combines SCell activation and OD-SSB indication. It means that it is not guaranteed that SCell activation command and OD-SSB activation indication are in the same DL-SCH or PDSCH.

In our view, problematic situation can be happended when NW cannot find resource to contain both SCell activation command and OD-SSB activation indication. Fortunately, if NW indicates OD-SSB activation indication before SCell activation command, the UE can apply requirements based on OD-SSB periodicity. However, if NW indicates OD-SSB activation indication after SCell activation command, the UE behaviour is unclear especially for Case 1. In other words, given the limited resource situation, NW is forced to indicate OD-SSB activation indication before SCell activation command, which can be strict limitation. To alleviate the burden, providing margin for applying OD-SSB can be considered. As illustrated in Figure 7, if NW indicates OD-SSB activation indication after SCell activation command within the margin interval, the UE can apply OD-SSB for SCell activation. From UE perspective, the UE expects to receive OD-SSB activation indication no later than margin interval after receiving SCell activation command.

Figure 8 Margin for applying OD-SSB activation indication

Proposal 17: In order to provide flexible scheduling, following can be specified at least for Case 1:

- UE expects to receive OD-SSB activation indication no later than margin interval after receiving SCell activation command

Additional measurement delay for SCell activation delay requirement

We would like to point out the issue for OD-SSB measurement during and after SCell activation. RAN4 agreed to reuse the legacy known cell condition principle. If we apply the legacy known condition principle, sending ‘valid measurement report or L3-RSRP measurement report with SSB index’ may depend on the OD-SSB indication timeline. For example in Figure 7, a UE received OD-SSB activation indication, and SCell activation command. If the UE have enough time to provide valid measurement report, UE may decide SCell as known. However, as shown in Figure 7, the UE cannot provide valid measurement report due to the lack of enough time. In this case, SCell can be remained unkown, and it may lead wasting measured OD-SSB.

In order to avoid wasting resources, additional measurement and reporting after SCell activation command can be provided in Figure 7. For example the UE measures OD-SSB using OD-SSB before SCell activation command and after SCell activation command, and provides valid measurement reports. The UE can apply shorter delay requirements precluding synchronization, AGC, or time/frequency tracking( the UE can also apply known condition requirements). We are not proposing to modify the known condition period, but having impact on SCell activation delay. In other words, additional measurement can be included in SCell activation delay requirements.

Figure 9 Additional measurement for SCell activation

Proposal 18: For SCell activation delay requirement, propose to add additional measurement delay

SCell activation delay requirement

In Rel-19 NES operation, targeted scenario is for both intra-/inter-band CA. Both SCell activation delay requirement for intra-/inter-band needs to be specified. In order to minimize the specification impact, basic principle needs to be followed for TSMTC_MAX, TFirstSSB,TFirstSSB_MAX, and Trs. It is straightforward that periodicity of SMTC of TSMTC_MAX and Trs can be replaced by OD-SSB periodicity in Case 1. For TFirstSSB,TFirstSSB_MAX, it can be the time to the end of the first complete OD-SSB burst, which is no earlier than Time_instance_A+Additional_delay indicated by OD-SSB of the SCell being activated, after slot n +(𝑇𝐻𝐴𝑅𝑄+3𝑚𝑠)/ 𝑁𝑅 𝑠𝑙𝑜𝑡 𝑙𝑒𝑛𝑔𝑡ℎ in Case 1. In Case 2, Scell can be activated based on either AO-SSB or OD-SSB. If AO-SSB is applied, legacy requirement can be used in Case 2. If OD-SSB is applied, same requirement as Case 1 can be used. Furthermore, if SCell activation based on the combination of AO-SSB and OD-SSB is supported, it needs to be specified.

Proposal 19: In order to minimize the specification impact, basic principle needs to be followed for TSMTC_MAX, TFirstSSB,TFirstSSB_MAX, and Trs as follows:

- In Case 1, periodicity of SMTC of TSMTC_MAX and Trs can be replaced by OD-SSB periodicity

- In Case 1, TFirstSSB,TFirstSSB_MAX can be the time to the end of the first complete OD-SSB burst, which is no earlier than Time_instance_A+Additional_delay indicated by OD-SSB of the SCell being activated, after slot n +(𝑇𝐻𝐴𝑅𝑄+3𝑚𝑠)/ 𝑁𝑅 𝑠𝑙𝑜𝑡 𝑙𝑒𝑛𝑔𝑡ℎ

- In Case 2, if AO-SSB is applied, legacy requirement can be used

- In Case 2, if OD-SSB is applied, same requirement as Case 1 can be used

- FSS: In Case 2, specify requirements of combination of AO-SSB and OD-SSB, if it is supported

Miscellaneous

RRC based signaling

RAN4 agreed to focus on at least for MAC-CE based single SCell activation case as staring point. Meanwhile, in RAN1#118, RAN1 confirms supporting both RRC based signaling and MAC-CE signaling as follows:

Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

Generally, for RRC based signlaing to indicate on-demand SSB transmission, the NW can provide SCell configuration and on-demand SSB configuration before SCell activation command. When the UE receives SCell activation command, UE can measure OD-SSB after decoding MAC-CE.

In our understanding, the difference between RRC based signlaing and MAC-CE based signaling is applicapable scenario and processing delay for receiving on-demand SSB indication. For applicapable scenario, we think RRC based signaling can only be applied for Scenario #2A. For the processing delay, we think additional RRC processing delay can be expected. Since it was agreed to introduce RRC based signlaing, we think RAN4 needs to specify RRC based signlaing procedure requirement in addition to MAC-CE signaling procedure requirement.

Proposal 20: Specify RRC based signaling procedure in addition to MAC-CE signaling procedure

Conclusion

In this contribution, we provide views on RRM requireiments for on-demand SSB SCell operation, and we propose

Proposal 1: RAN4 to consider the scenario with same SSB carrier frequency, same periodicity, same SSB index, but different offset in Case 2

Proposal 2: RAN4 to support AO-SSB and OD-SSB combination at least for the limited scenario

Proposal 3: RAN4 to consider L1 measurement during and after SCell activation as below

Measurement period TL1-RSRP_Measurement_Period_SSB for FR1 (OD-SSB in Case1)

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = the periodicity of the SSB-Index in activated OD-SSB configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17 |  |

Observation 1: If the time between multiple OD-SSB activation/deactivation is short enough to keep the sync, PSS/SSS detection and/or SSB index acquisition can be skipped

Proposal 4: RAN4 to specify cell identification requirements based on the time between multiple OD-SSB activation/deactivation

- If the time is shorter than a threshold, the UE can skip PSS/SSS detection and/or SSB index acquisition

Proposal 5: Skip cell identification at least for Scenario 1 in Case 2

Proposal 6: If Time instance A is located within the first possible OD-SSB burst, start measurement at the next OD-SSB burst

Proposal 7: Consider the relation between OD-SSB deactivation indication and measurement as follows:

- In case of Option 1 (Explicit deactivation indication): UE is no required to measure (fast or slow) deactivated SCell at the first slot after 3ms transmitting feedback on OD-SSB deactivation indication

- In case of Option 2 (Configuration of N OD-SSB bursts to be transmitted): UE is not required to measure (fast or slow) deactivated SCell at the end of Nth OD-SSB burst

Proposal 8: Support duration of the time window as below:

- Duration of the time window = Uncertainty_time + Actual_measurement_time

  - Uncertainty_time = Tmin+Tprocessing (if required)+Tuncertainty

  - Actual_measurement_time

    - For the cell not identified: follow cell identification requirements

    - For the cell identified: follow measurement period requirements

Proposal 9: Support fallback to slow mode when UE provided one valid report under side condition

- If side condition is not fullfiled, the UE continues performing measurements in ‘fast’ mode during the time window

Proposal 10: RAN4 to specify the relationship between the time windows

- In case of overlapping of time windows, the UE measures the OD-SSB indicated by the latest OD-SSB activation indication

Proposal 11: Not to specify lower bound Not to specify lower bound (1st)

Proposal 12: The legacy CSSF can be reused

Proposal 13: Revise the agreement regarding the time window as follows:

- Time window means the time duration starting from receiving OD-SSB activation indication→ Time window means the time duration starting from Tmin after receiving OD-SSB activation indication

Proposal 14: RAN4 to specify known/unknown cell condition period as below:

- At least for FR1: max(5*(period of OD-SSB), 5*DRX cycles)

Proposal 15: For the OD-SSB used for known/unknown cell condition when there are multiple OD-SSB activation indication, the OD-SSB provided better valid measurement report can be used for SCell activation

Proposal 16: During the SCell activation following should be specified:

- If the cell is known, the same SSB should be applied for both known condition and SCell activation

- OD-SSB should not be changed or deactivated during SCell activation

Proposal 17: In order to provide flexible scheduling, following can be specified at least for Case 1:

- UE expects to receive OD-SSB activation indication no later than margin interval after receiving SCell activation command

Proposal 18: For SCell activation delay requirement, propose to add additional measurement delay

Proposal 19: In order to minimize the specification impact, basic principle needs to be followed for TSMTC_MAX, TFirstSSB,TFirstSSB_MAX, and Trs as follows:

- In Case 1, periodicity of SMTC of TSMTC_MAX and Trs can be replaced by OD-SSB periodicity

- In Case 1, TFirstSSB,TFirstSSB_MAX can be the time to the end of the first complete OD-SSB burst, which is no earlier than Time_instance_A+Additional_delay indicated by OD-SSB of the SCell being activated, after slot n +(𝑇𝐻𝐴𝑅𝑄+3𝑚𝑠)/ 𝑁𝑅 𝑠𝑙𝑜𝑡 𝑙𝑒𝑛𝑔𝑡ℎ

- In Case 2, if AO-SSB is applied, legacy requirement can be used

- In Case 2, if OD-SSB is applied, same requirement as Case 1 can be used

- FSS: In Case 2, specify requirements of combination of AO-SSB and OD-SSB, if it is supported

Proposal 20: Specify RRC based signaling procedure in addition to MAC-CE signaling procedure

Reference

[1] RP-242354, “Revised WID: Enhancements of network energy savings for NR,” Ericsson

[2] R4-2420098, “WF on RRM requirements for Netw_Energy_NR_enh_Part1,” Ericsson

[3] R4-2418282, “Topic summary for [113][224] Netw_Energy_NR_enh_Part1,” Ericsson

[4] R1-2410917,” Reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell,” Ericsson