3GPP TSG RAN WG4 Meeting #114	R4-2500828

Athens, Greece, 17th – 21st February, 2025

Agenda item:			7.8.3

Source:	vivo

Title:	Discussion on RRM requirements for low band carrier aggregation via switching

Document for:	Discussion

# Introduction

In the RAN#106 meeting, a new WI [1] on low band carrier aggregation via switching was approved.

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

In this contribution, we provide our views on RRM requirements impact due to low band carrier aggregation with switching.

# Discussion

Carrier aggregation is the important feature to utilizing spectrum owned by operators. However, for some FDD band combinations, it is challenging to implement by the UE considering complexity, cost and feasibility of RF front architectures. To address operators’ concern, a new WID [1] was approved to specify switching scheme.

According to the WID [1], the following two cases are considered.

-	Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

-	Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

When UE works on FDD carrier 1, which should be PCell, the UE doesn’t need to receive on SDL carrier 2. If UE switches to SDL carrier 2 to received, then the UE doesn’t need to receive/transmit on FDD carrier 1. However, when UE switches between the two carriers, there would be switch period during which the UE cannot receive/transmit on either one of the carriers.

Based on the WID objective, only a semi-static switching pattern based on RRC configuration will be introduced. It is not clear for now how this semi-static switching pattern will be specified. We can discuss impact of triggering scheme later after there is conclusion in RAN1.

Similar to UL Tx switching, switching period for RF retuning is also needed for DL switching. The switching period will be decided in RF session. Additionally, from RRM perspective receive timing difference should also be considered when defining RRM requirements. Since both carriers are in a single TAG and co-located, there is no need to consider impact of TA.

Observation 1: Switch delay between two carriers at least includes RF switch period and receive timing difference.

From network deployment perspective, co-located and synchronized network deployment for both carriers is considered. There is no synchronization requirement for FDD carriers in existing RRM spec. In our view, the cell phase synchronization requirements for TDD can be reused in general. It seems no reason to relax or tight the synchronization requirements compared to TDD.

Proposal 1: Define synchronization requirements for FDD and synchronization accuracy is 3µs at BS antenna connectors or radiated interface boundaries.

For UL Tx switching, only DL interruption requirements are specified. However, the example band combination in this WI is CA_n5A-n29A. Only two carriers are considered. It is more appropriate to specify switch delay requirements for switching between two carriers during which there is no reception/transmission on both carriers.

Proposal 2: Define switch delay requirements for switching between two carriers and no reception/transmission is required during the period.

Proposal 3: The switch delay between switching two carriers at least include switch period for RF retuning and receive timing difference. Additionally delay due to triggering scheme can be considered later.

For the SDL carrier, an SCell would be configured. In general, all the existing requirements for SCell can be applied. However, since it is collocated and synchronized with PCell, some optimization may be made. For example, whether timing for the PCell can be reused, or whether the SCell should always be measured etc.

Proposal 4: RAN4 to discuss whether to enhance SCell related requirements for collocated and synchronized FDD deployment.

# Summary

In this contribution, we provided views on RRM requirements impact due to low band carrier aggregation with switching. Following proposals are present.

Proposal 1: Define synchronization requirements for FDD and synchronization accuracy is 3µs at BS antenna connectors or radiated interface boundaries.

Proposal 2: Define switch delay requirements for switching between two carriers and no reception/transmission is required during the period.

Proposal 3: The switch delay between switching two carriers at least include switch period for RF retuning and receive timing difference. Additionally delay due to triggering scheme can be considered later.

Proposal 4: RAN4 to discuss whether to enhance SCell related requirements for collocated and synchronized FDD deployment.

- References

- RP-243317 	New WID on low band carrier aggregation via switching