3GPP TSG-RAN WG4 Meeting #114     	      R4-2500844

# Athens, Greece, 17th – 21st February, 2025

Agenda Item:	7.18.2

Source: 	CMCC

Title: 			 Discussion on interference suppressing performance for 8Rx UE

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed test scope and test parameters about interference suppressing performance for 8Rx UE, the WF has been approved in [1].

In this contribution, we further discuss the remaining open test parameters.

- Discussion

  - General

Definition of MMSE-IRC receiver capability for 8Rx UE

- Proposals

  - Option 1:
Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE supporting SU-MIMO 8Rx receiver.

  - Option 2:
Update the existing Rel-18 UE capability definition as a maintenance part of the R18 FR1 RF enhancement WI.

  - Option 3: 
Introduce new Rel-19 UE capabilities for MU-MIMO 8Rx receiver. And clearly state that the UE supporting ‘MU-MIMO 8Rx receiver’ are also required to support ‘SU-MIMO 8Rx receiver’ capability.

  - Option 4:
Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following:
- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

Considering the baseline and simplified 8Rx MMSE-IRC receiver have the same implementation in both SU-MIMO and MU-MIMO scenario, we prefer to reuse the Rel-18 capabilities rather than defining new Rel-19 capabilities. To ensure accurate description of Rel-18 capabilities, the existing Rel-18 UE capability definition need to be updated like stated in Option 4, through the method proposed in Option 2.

Proposal 1: Update the existing Rel-18 UE capabilities definition as below in R18 FR1 RF enhancement WI maintenance:

- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

8Rx UE MMSE-IRC receiver coverage

- Proposals

  - Option 1:
RAN4 to define two sets of requirements (baseline 8Rx receiver and simplified 8Rx receiver).

  - Option 2: 
Set the criteria for defining two sets of requirements as: If the span after removing a reasonable number or outliers (at least 5 results should be left) is >2.5dB, define two sets of requirements. The evaluation of the span shall be done for each configuration agreed for requirement definition.

  - Option 3: 
Postpone defining criteria for decision of one or two sets of requirements until all test cases are finalized.

Based on our simulation results shown in R4-2500845, for ICI scenario, the span between baseline 8Rx receiver and simplified 8Rx receiver is about 0.09-5.52dB. For IUI scenario, the span between baseline 8Rx receiver and simplified 8Rx receiver is about 3.18-Inf dB.

| ICI scenario |  | SNR span between baseline and simplified 8Rx IRC receiver |  |
| --- | --- | --- | --- |
|  |  | HomNet | HetNet |
| FDD | 2T8R Rank 1 | 0.81 | 0.27 |
|  | 2T8R Rank 2 | 2.41 | 1.19 |
|  | 4T8R Rank 4 | 5.47 | 4.28 |
| TDD | 2T8R Rank 1 | 0.79 | 0.09 |
|  | 2T8R Rank 2 | 2.08 | 1.24 |
|  | 4T8R Rank 4 | 5.52 | 4.32 |

| IUI scenario |  | SNR span between baseline and simplified 8Rx IRC receiver |
| --- | --- | --- |
| FDD | 4T8R Rank 2+2 (MCS17) | 3.18 |
|  | 4T8R Rank 2+2（MCS19） | 3.41 |
|  | 8T8R Rank 4+4 | Inf |

To define two sets of requirements, there should be valid SNR requirements and obvious SNR span between baseline receiver and simplified receiver. In our simulation results, for ICI scenario, 4T8R Rank 4 meet such criteria, and for IUI scenario, Rank 2+2 meet the criteria. Therefore, we propose at least define two sets of requirements for the case of 4T8R Rank 4 for ICI scenario, and define two sets of requirements for the case of Rank 2+2 for IUI scenario.

- Proposal 2: For ICI scenario, at least define two sets of requirements for the case of 4T8R Rank 4.

- Proposal 3: For IUI scenario, define two sets of requirements for the case of Rank 2+2.

  - PDSCH requirements with inter cell interference

Number of interference cells

- Background:

  - We have an existing agreement from RAN#112bis, to reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx, with TBD for applicability rules to reduce UE efforts [R4-2416555]

- Proposals:

  - Option 1: Both 1 interference cell and 2 interference cells

RAN4 has agreed to introduce both HomNet and HetNet scenario in RAN4#112bis meeting, both scenarios are common and typical in real network. But some companies concern about test effort, we think the appliability rules from Rel-17 can be reused, that if UE supporting both duplex mode TDD and FDD, then only HomNet FDD test and HetNet TDD test will be applied.

Proposal 4: Confirm the previous agreement that cover both HomNet and HetNet scenarios.

Proposal 5: Introduce the applicability rule that if UE supporting both duplex mode TDD and FDD with 8Rx, then only HomNet FDD test and HetNet TDD test will be applied.

  - PDSCH demodulation requirements with MMSE-IRC receiver for intra-cell inter-user interference scenario

Rank and MCS

Agreement:

- Define requirements for:

  - Rank 2+2

  - FFS: MCS17/19

- TBD: Rank 4+4: MCS13

Based on our simulation results, for Rank 2+2, infinite gain can be observed for both MCS 17 and 19. Here, we prefer MCS 19 since larger span can be observed between baseline receiver and simplified receiver.

For Rank 4+4, only baseline receiver can achieve 70% THUP. Therefore, we propose to define requirements for Rank 4+4 for baseline receiver.

Proposal 6: Define requirements for Rank 2+2 MCS 19 for both baseline receiver and simplified receiver.

Proposal 7: Define requirements for Rank 4+4 MCS 13 for baseline receiver.

  - Applicability rule and release independent

Test applicability to skip 2/4Rx requirements

- Proposals

  - Option 1: 
Defining applicability rules to skip 2/4Rx requirements if 8Rx tests are passed.

    - Option 1a: 
For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

    - Option 1b: 
For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Option 1b follows the legacy applicability rule, which ensure UE pass the related test case with minimum test effort.

Proposal 8: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Release independent

- Proposals

  - Option 1: 
Discuss release independence requirements at a later stage in the WI.

    - Option 1a: 
RAN4 start discussing the issue of release independent when RAN4 agree on baseline assumptions, test parameters and test metrics, i.e., after RAN4#113.

  - Option 2: 
The new requirements for 8Rx should be release independent from Rel-18.

    - Option 2a: 
The requirement should be released independent from Rel-18 if no new signalling or UE capability introduced.

  - Option 3: 
The performance requirements for 8Rx MMSE-IRC should be release independent from Rel-17, but should be mandatory for Rel-18 and optional for Rel-17.

If no new signalling or UE capability introduced, the requirement should at least be released independent from Rel-18. We are open to further discuss whether it can be release independent from Rel-17.

Proposal 9: The requirement should at least be released independent from Rel-18 if no new signalling or UE capability introduced.

- Conclusion

In this contribution, we provides some test parameter and test rule configuration for UE 8Rx interference suppressing requirement. The proposals are:

Proposal 1: Update the existing Rel-18 UE capabilities definition as below in R18 FR1 RF enhancement WI maintenance:

- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Proposal 2: For ICI scenario, at least define two sets of requirements for the case of 4T8R Rank 4.

- Proposal 3: For IUI scenario, define two sets of requirements for the case of Rank 2+2.

Proposal 4: Confirm the previous agreement that cover both HomNet and HetNet scenarios.

Proposal 5: Introduce the applicability rule that if UE supporting both duplex mode TDD and FDD with 8Rx, then only HomNet FDD test and HetNet TDD test will be applied.

Proposal 6: Define requirements for Rank 2+2 MCS 19 for both baseline receiver and simplified receiver.

Proposal 7: Define requirements for Rank 4+4 MCS 13 for baseline receiver.

Proposal 8: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Proposal 9: The requirement should at least be released independent from Rel-18 if no new signalling or UE capability introduced.

- Reference

[1] R4-2419784, WF for [113][322] NR_demod_Ph5_Part2_UE, Moderator (Nokia)