3GPP TSG-RAN WG4 Meeting # 114		R4-2500850

# Athens, Greece, 17th – 21st February, 2025

Agenda Item:	7.26.4.1

Source: 	CMCC

Title: 			  Simulation assumption and results for LP-WUR

Document for:	Discussion

- Introduction

In RAN4#112bis meeting, RAN4 provided the updated RRM simulation assumption. The WF has been approved in [1].

In RAN1# meeting, some further agreements have been agreed, which will impact RRM simulation assumption.

In this contribution, we give our simulation results based on RAN4 simulation assumption and RAN1’s further agreement.

- Simulation result

Our simulation results are provided below by using the simulation assumption in Annex.1.

Table 1. AWGN, Es/IoT = -3, SNR of cell 2 is 6 dB lower compared with cell 2, 4 OFDM symbol, 30kHz SCS

|  | OOK-1 RSRP error | OOK-1 RSRQ error | OOK-4 M=2 RSRP error | OOK-4 M=2 RSRQ error | OOK-4 M=4 RSRP error | OOK-4 M=4 RSRQ error |
| --- | --- | --- | --- | --- | --- | --- |
| 3 sample | 0.32 | 0.22 | 0.29 | 0.2 | 0.53 | 0.42 |
| 4 sample | 0.27 | 0.19 | 0.24 | 0.16 | 0.41 | 0.32 |
| 5 sample | 0.24 | 0.17 | 0.21 | 0.14 | 0.3 | 0.23 |

Table 2. TDL-C 300ns, Es/IoT = -3, SNR of cell 2 is 6 dB lower compared with cell 2, 4 OFDM symbol, 30kHz SCS

|  | OOK-1 RSRP error | OOK-1 RSRQ error | OOK-4 M=2 RSRP error | OOK-4 M=2 RSRQ error | OOK-4 M=4 RSRP error | OOK-4 M=4 RSRQ error |
| --- | --- | --- | --- | --- | --- | --- |
| 3 sample | 2.76 | 0.93 | 2.97 | 1.13 | 3.02 | 1.16 |
| 4 sample | 2.43 | 0.81 | 2.51 | 0.91 | 2.74 | 1.15 |
| 5 sample | 2.15 | 0.72 | 2.39 | 0.81 | 2.61 | 0.81 |

Table 3. TDL-C 300ns, Es/IoT = -3, SNR of cell 2 is 6 dB lower compared with cell 2, 4 OFDM symbol, 15kHz SCS

|  | OOK-1 RSRP error | OOK-1 RSRQ error | OOK-4 M=2 RSRP error | OOK-4 M=2 RSRQ error | OOK-4 M=4 RSRP error | OOK-4 M=4 RSRQ error |
| --- | --- | --- | --- | --- | --- | --- |
| 3 sample | 3.34 | 1.16 | 3.52 | 1.32 | 3.91 | 1.82 |

Table 4. AWGN, Es/IoT = -3, SNR of cell 2 is 6 dB lower compared with cell 2, 8 OFDM symbol, 30kHz SCS

|  | OOK-1 RSRP error | OOK-1 RSRQ error | OOK-4 M=2 RSRP error | OOK-4 M=2 RSRQ error | OOK-4 M=4 RSRP error | OOK-4 M=4 RSRQ error |
| --- | --- | --- | --- | --- | --- | --- |
| 3 sample | 0.25 | 0.17 | 0.21 | 0.15 | 0.18 | 0.11 |
| 4 sample | 0.2 | 0.14 | 0.19 | 0.13 | 0.17 | 0.12 |
| 5 sample | 0.18 | 0.13 | 0.15 | 0.1 | 0.13 | 0.08 |

Table 5. TDL-C 300ns, Es/IoT = -3, SNR of cell 2 is 6 dB lower compared with cell 2, 8 OFDM symbol, 30kHz SCS

|  | OOK-1 RSRP error | OOK-1 RSRQ error | OOK-4 M=2 RSRP error | OOK-4 M=2 RSRQ error | OOK-4 M=4 RSRP error | OOK-4 M=4 RSRQ error |
| --- | --- | --- | --- | --- | --- | --- |
| 3 sample | 2.72 | 0.92 | 2.86 | 1.09 | 3.19 | 1.45 |
| 4 sample | 2.37 | 0.81 | 2.43 | 0.87 | 2.55 | 1.22 |
| 5 sample | 2.15 | 0.73 | 2.17 | 0.75 | 2.41 | 1.04 |

Table 6. TDL-C 300ns, Es/IoT = -3, SNR of cell 2 is 6 dB lower compared with cell 2, 8 OFDM symbol, 15kHz SCS

|  | OOK-1 RSRP error | OOK-1 RSRQ error | OOK-4 M=2 RSRP error | OOK-4 M=2 RSRQ error | OOK-4 M=4 RSRP error | OOK-4 M=4 RSRQ error |
| --- | --- | --- | --- | --- | --- | --- |
| 3 sample | 3.42 | 1.18 | 3.51 | 1.32 | 3.58 | 1.45 |

Table 7. AWGN, Es/IoT = -3, SNR of cell 2 is 9 dB lower compared with cell 2, 4 OFDM symbol, 30kHz SCS

|  | OOK-1 RSRP error | OOK-1 RSRQ error | OOK-4 M=2 RSRP error | OOK-4 M=2 RSRQ error | OOK-4 M=4 RSRP error | OOK-4 M=4 RSRQ error |
| --- | --- | --- | --- | --- | --- | --- |
| 3 sample | 0.25 | 0.15 | 0.31 | 0.21 | 0.29 | 0.18 |
| 4 sample | 0.23 | 0.14 | 0.28 | 0.2 | 0.22 | 0.13 |
| 5 sample | 0.21 | 0.12 | 0.21 | 0.13 | 0.19 | 0.11 |

Table 8. TDL-C 300ns, Es/IoT = -3, SNR of cell 2 is 9 dB lower compared with cell 2, 4 OFDM symbol, 30kHz SCS

|  | OOK-1 RSRP error | OOK-1 RSRQ error | OOK-4 M=2 RSRP error | OOK-4 M=2 RSRQ error | OOK-4 M=4 RSRP error | OOK-4 M=4 RSRQ error |
| --- | --- | --- | --- | --- | --- | --- |
| 3 sample | 2.76 | 0.98 | 3.02 | 1.22 | 3.34 | 1.73 |
| 4 sample | 2.37 | 0.83 | 2.72 | 1.12 | 2.93 | 1.43 |
| 5 sample | 2.18 | 0.76 | 2.21 | 0.78 | 2.51 | 1.12 |

Table 9. TDL-C 300ns, Es/IoT = -3, SNR of cell 2 is 9 dB lower compared with cell 2, 4 OFDM symbol, 15kHz SCS

|  | OOK-1 RSRP error | OOK-1 RSRQ error | OOK-4 M=2 RSRP error | OOK-4 M=2 RSRQ error | OOK-4 M=4 RSRP error | OOK-4 M=4 RSRQ error |
| --- | --- | --- | --- | --- | --- | --- |
| 3 sample | 3.41 | 1.23 | 3.54 | 1.35 | 3.81 | 1.74 |

Table 10. AWGN, Es/IoT = -3, SNR of cell 2 is 9 dB lower compared with cell 2, 8 OFDM symbol, 30kHz SCS

|  | OOK-1 RSRP error | OOK-1 RSRQ error | OOK-4 M=2 RSRP error | OOK-4 M=2 RSRQ error | OOK-4 M=4 RSRP error | OOK-4 M=4 RSRQ error |
| --- | --- | --- | --- | --- | --- | --- |
| 3 sample | 0.18 | 0.1 | 0.22 | 0.15 | 0.17 | 0.1 |
| 4 sample | 0.15 | 0.09 | 0.18 | 0.12 | 0.14 | 0.08 |
| 5 sample | 0.14 | 0.08 | 0.14 | 0.08 | 0.14 | 0.08 |

Table 11. TDL-C 300ns, Es/IoT = -3, SNR of cell 2 is 9 dB lower compared with cell 2, 8 OFDM symbol, 30kHz SCS

|  | OOK-1 RSRP error | OOK-1 RSRQ error | OOK-4 M=2 RSRP error | OOK-4 M=2 RSRQ error | OOK-4 M=4 RSRP error | OOK-4 M=4 RSRQ error |
| --- | --- | --- | --- | --- | --- | --- |
| 3 sample | 2.78 | 0.98 | 3.06 | 2.03 | 2.99 | 1.26 |
| 4 sample | 2.4 | 0.86 | 3 | 1.55 | 2.49 | 0.93 |
| 5 sample | 2.18 | 0.78 | 2.43 | 1.09 | 2.4 | 1.03 |

Table 12. TDL-C 300ns, Es/IoT = -3, SNR of cell 2 is 9 dB lower compared with cell 2, 8 OFDM symbol, 15kHz SCS

|  | OOK-1 RSRP error | OOK-1 RSRQ error | OOK-4 M=2 RSRP error | OOK-4 M=2 RSRQ error | OOK-4 M=4 RSRP error | OOK-4 M=4 RSRQ error |
| --- | --- | --- | --- | --- | --- | --- |
| 3 sample | 3.4 | 1.2 | 3.62 | 1.46 | 3.43 | 1.28 |

Observation 1: For OOK based LP-WUR, in order to fulfilled the accuracy requirement 3.5dB for LP-RSRP and 3.5dB for LP-RSRQ, 3 or 4 samples are needed.

Observation 2: Limited or no RRM accuracy gain is achieved by extending the LP-SS overhead from 4 OFDM symbol to 8 OFDM symbol.

Observation 3: When other simulation parameter keeps same, the RSRP and RSRQ error is larger in the SCS 15kHz case.

- Discussion

Issue 2-1-4: On LP-SS sequence for simulation purpose

Agreement from RAN1#119

For the length L of LP-SS binary sequence, limit the selection to the following:

- M=1, L= {4, 6, 8}

- M=2, L= {8, 12, 16}

- M=4, L= {16, 32}

Currently, we use following sequences for our simulation,

M=1, L=4, LP-SS sequence = [1 0 1 0]

M=1, L=8, LP-SS sequence = [1 0 1 0 1 0 1 0]

M=2, L=8, LP-SS sequence = [1 0 1 0 1 0 1 0]

M=2, L=16, LP-SS sequence = [1 0 1 0 0 1 0 1 0 1 0 1 0 1 0 1]

M=4, L=16, LP-SS sequence = [1 0 1 0 0 1 0 1 0 1 0 1 0 1 0 1]

M=4, L=32, LP-SS sequence = [1 0 0 1 1 0 1 0 0 1 1 0 0 1 0 1 0 1 1 0 0 1 1 0 1 0 0 1 1 0 0 1]

The LP-SS sequence length and binary sequence may be further updated based on future RAN1’s conclusion.

Proposal 1: The LP-SS sequence length and binary sequence may be further updated based on future RAN1’s conclusion.

Issue 2-1-5: On ideal RSRP/RSRQ in simulation

- Proposals

  - P1: Ideal RSRP is derived from the serving cell signal after the fading channel; Ideal RSRQ is derived based on ideal RSRP divides the calculated RSSI (Ericsson)

  - P2: Suggest to discuss and align the ideal RSRP calculation (vivo)

  - Method 1: The first one is the RSRP is pass the fading channel and with the perfect channel estimation and the real RSRP pass the same channel.

  - Method 2: The second one assumes the impact of the fading channel can be completely removed hence the ideal RSRP does not need pass the channel whereas the real RSRP will pass the fading channel.

Recommendations:

Suggest to align ideal RSRP/RSRQ calculation/simulation

Currently, we use the Method 2 in P2 to derive our simulation results. We think the Method 1 and Method 2 of P2 may have similar performance, which taking the channel estimation error into account.

For P1, although we understand the reason behind, but we concern about how to conduct the test with P1, could TE acquire the RSRP value after the fading channel?

Observation 4: How to test with the P1 is not clear, could TE acquire the RSRP value after the fading channel?

Proposal 2: Use Method 2 of P2 to derive the ideal RSRP value.

Issue 2-1-6: On SCS in simulation

Recommendations:

Check whether only using SCS = 30KHz in the simulation is agreeable

| Subcarrier spacing | 30KHz initially |
| --- | --- |

Based on our simulation results, when other simulation parameter keeps same, the RSRP and RSRQ error is larger in the SCS 15kHz case. Although the gap is not large, we are willing to also check the accuracy performance under SCS 15kHz scenario.

Proposal 3: Prefer to check the accuracy performance under SCS 15kHz scenario as well.

- Conclusion

Based on the discussion above, the following observations are concluded.

Observation 1: For OOK based LP-WUR, in order to fulfilled the accuracy requirement 3.5dB for LP-RSRP and 3.5dB for LP-RSRQ, 3 or 4 samples are needed.

Observation 2: Limited or no RRM accuracy gain is achieved by extending the LP-SS overhead from 4 OFDM symbol to 8 OFDM symbol.

Observation 3: When other simulation parameter keeps same, the RSRP and RSRQ error is larger in the SCS 15kHz case.

Proposal 1: The LP-SS sequence length and binary sequence may be further updated based on future RAN1’s conclusion.

Observation 4: How to test with the P1 is not clear, could TE acquire the RSRP value after the fading channel?

Proposal 2: Use Method 2 of P2 to derive the ideal RSRP value.

Proposal 3: Prefer to check the accuracy performance under SCS 15kHz scenario as well.

- Reference

- R4-2420100, WF on RRM requirements for NR_LPWUS, vivo

- R4-2418284,Topic summary for [113][226] NR_LPWUS, vivo

- Annex.A

Simulation assumption

Note: Highlight part is updated base on the WF of last meeting

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 2.6 GHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Interfering cell (Cell 2) is not known to UE |
| DRX | No applicable for LP-WUR |
| BS transmit antennas for LP-SS blocks | 1 Tx |
| UE receive antennas | 1 Rx |
| Data and control channel subcarrier spacing | Data, SSB and LP-SS have the same SCS |
| Subcarrier spacing | 30kHz |
| Measurement period (in number of measurement samples) | [3, 4, 5] |
| LP-SS/SSB measurement interval | LP-SS: 320 ms SSB: 320 ms |
| LP-SS BW | 132 subcarriers for SCS=30kHz for LP-SS initially |
| SSS | 30KHz for SSS |
| Actual LP-SS transmissions | always transmitted |
| Guard band | 1 RB on each side of LP-SS/LP-WUS signal |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS, NR-SSS (OFDM based LP-WUR) | - | To be indicated by companies | To be indicated by companies |
| LP-SS |  | OOK-1; or OOK-4 with M = [2,4] | when Cell 1 uses OOK-1; OOK-1 or NR signal is used for Cell 2 when Cell 1 uses OOK-4, OOK-4 with same M value as cell 1 or NR signal is used for Cell 2 |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency offset relative to UE frequency reference | Hz | OFDM based receiver [5] ppm OOK based receiver [10] ppm | N/A |
| Timing error |  | Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: T=5us for OOK-1 T=2us for OOK-4 with M=2 T=1us for OOK-4 with M=4 |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 3 ms |
| SNR | dB | SNR setting for serving and interference cell are derived based on agreement of Issue 2-1-1-1 |  |
|  |  | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-2.7] - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-2.4] | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-11.7] - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-8.4] |
|  |  | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
|  |  | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
| Ês/Iot | dB | -3; -0.5dB; 2dB | N/A |
| Propagation conditions | - | FR1: AWGN TDL-C 300ns |  |
| UE speed |  | 3 km/h |  |

Table 3: UE-specific parameters

| [Receiver Filter] | [3th/5th Order Butterworth with 3.96MHz bandwidth] |
| --- | --- |
| [Receiver ADC bit width] | [4 or 8-bitADC] |
| [Receiver Sampling Rate for LP-SS only] | [7.68MHz] |