3GPP TSG-RAN4 Meeting #114	R4-2500855

# Athens, Greece, 17th - 21st, February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2640 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38101-1 on ATG UE antenna connector clarification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-Core |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For ATG UE Tx requirements, current power control and transmit signal quality requirements for ATG do not properly clarify how to consider omni-directional antenna connectors or array boundary connectors. For ATG UE Rx requirements, the following agreement was reached at RAN4#113 meeting: - Rx antenna port requirements of the ATG UE need to be specified at the antenna connector(s) or TAB connectors. Based on the analysis provided above, the description of the ATG UE antenna ports needs to be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify ATG UE Rx requirements with omni-directional antenna and antenna array. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Core specification will remain unclear for ATG UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4J, 7.2J, 7.3J |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change for TS 38.101-1>>

6.4J	Transmit signal quality for ATG

6.4J.1	Frequency error for ATG

The ATG UE basic measurement interval of modulated carrier frequency is 1 UL slot. The ATG UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift. The mean value of basic measurements of ATG UE modulated carrier frequency at each transmit antenna connector or at each TAB connector shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency.

UE [shall] rely on the ATG BS location broadcasted by the SIB22 in TS 38.331 [7].

NOTE 1:	the ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by ATG BS and UL precompensated doppler frequency shift.

6.4J.2	Transmit modulation quality for ATG

The requirements for transmit modulation quality defined in clause 6.4.2 shall apply for ATG UE except for the phase continuity requirements for DMRS bundling in 6.4.2.5 in case of UE using a single omnidirectional antenna. In case of UE using multiple omnidirectional antennas, the requirements for transmit modulation quality in clause 6.4G.2 apply and in case of UE using antenna array, the requirements for transmit modulation quality in clause 6.4G.2 apply replacing each antenna connector by each transceiver array boundary connector. And the requirements for 256QAM modulation are only applicable to ATG UE indicating support of 256QAM.

<<Next of Change>>

7.2J	Diversity characteristics for ATG

The ATG UE is required to be equipped with a minimum of two Rx antenna connectors or two Rx TAB connectors in all operating bands. ATG UE is required optionally to be equipped with four Rx antenna connectors or four Rx TAB connectors.

<<Next of Change>>

7.3J	Reference sensitivity for ATG

7.3J.1	General

For ATG UE with multiple omni-directional antennas not indicating the capability antennaArrayType-r18, the reference sensitivity power level REFSENS is the minimum mean power per polarization at antenna connector, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

For ATG UE with antenna array indicating the capability antennaArrayType-r18,the reference sensitivity power level REFSENS is the minimum mean power per polarization at TAB antenna connector, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

7.3J.2	Reference sensitivity power level

For a ATG UE(s) equipped with two Rx antenna connectors or two Rx TAB connectors, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1) with parameters specified in Table 7.3.2-1a and Table 7.3.2-1b for the applicable operating bands.

For ATG UE(s) equipped with four Rx antenna connectors or four Rx TAB connectors, reference sensitivity for four Rx antenna connectors or four Rx TAB connectors shall be modified by the amount given in ΔRIB,4R in Table 7.3.2-2 for the applicable operating bands.

The reference sensitivity (REFSENS) requirement for a ATG UE shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3.

<<End of Change>>