3GPP TSG-RAN WG4 Meeting # 114                                                              R4-2500860

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.12.1.1

Source: 	CMCC

Title: 	Discussion on UE RF requirements for ATG with inter-band CA

Document for:	Discussion

# 1. Introduction

RAN#106 meeting approved revised WID on Enhancements for Air-to-ground network for NR in Rel-19 [1]. After discussions of several meetings, we have made good progress on most of the ATG inter-band CA requirement, and we will focus on the n39 UL for CA_n3A-n39A.

# 2. Discussion

## 2.1. n39 UL for CA_n3A-n39A

During last meeting, we have reached the agreement as below.

WF: - Further check these two sets of parameters - Option 1 Parameters; Values. Antenna Isolation; 20dB. The rejection for band n3 Rx filter at 1885~1915MHz; 50dB. n39 Tx filter rejection at 1805~1880MHz; 50dB. - Option 2 Parameters; Values. Antenna Isolation; 20dB. The rejection for band n3 Rx filter at 1885~1915MHz; 20dB. n39 Tx filter rejection at 1805~1880MHz; 40dB. - Consider the assumed parameters with separate antenna. Issue 3-3: MSD for CA_n3A-n39A with n39 UL Agreement: - Define the MSD value with 40dBm output power - FFS on how to scale MSD for other output power level. - Following the parameters in the table below. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n39; n3; 1900; 30; 15; 160 (RBstart=0); 1817.5; 25; [TBD]; ACLR2.

According to the current specifications, only UL for band n39 has been considered. Regarding the shared antenna architecture, companies have proposed different parameters. The main point of contention among these parameters lies in the performance of the n3 receive filter. Given the unique structure of airborne CPEs, we believe that the performance of this filter can be sufficiently enhanced. Therefore, we recommend selecting option 1 as the baseline parameter.

Proposal 1:  Set option 1 as the baseline parameters.

Regarding the issue of MSD, the previous conclusion was to define an MSD of 40dBm MOP. However, considering that the maximum output power range of ATG UE is 23dBm to 40dBm, an MSD of 23dBm should also be defined as a reference, in addition to the 40dBm MSD.

Proposal 2:  MSD with 23dBm output power should also be defined.

## 2.2. UE antenna type for inter-band CA

Issue 3-4-1: whether preclude Case 4. Issue 1-1-2: UE antenna type for inter-band CA Agreement: - Refine the agreement from last meeting as: - For intra-band contiguous CA, same antenna type should be applied on a band. - For inter-band CA, either different or same antenna type can be applied on different bands. - For inter-band CA, consider the following cases: - 1. ATG UE with omnidirectional antennas on the bands of PCC and SCC. - 2. ATG UE with an omnidirectional antenna on the band of PCC and an antenna array on the band of SCC. - 3. ATG UE with an antenna array on the band of PCC and an omnidirectional antenna on the band of SCC. - [4. ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam assumed.] - 5. ATG UE with an antenna array on the bands of PCC and SCC, and two simultaneous separate Rx beams assumed. Further check which understanding is correct: - Understanding #1: Case 4 is excluded in RF session. - Understanding #2: Case 4 is not explicitly discussed in RF session, since no case 4 specific RF requirement. Case 4 UE implementation is possible.. WF: - Further check whether preclude case 4.

In our view, the consensus reached during the RF session was to define the type of UE antenna and associated RF requirements. The five cases presented are distinct implementation proposals aimed at achieving the specified UE antenna configurations. Specifically, for Case 4, if two bands assume the same antenna type and the component CCs are in close proximity, it is possible to share a single antenna array. In this scenario, the implementation of Case 4 is not an issue. Therefore, we believe that the aforementioned cases should be considered, and these implementations are feasible.

Proposal 3: RF session doesn’t exclude the Case 4 based on the reached agreement.

# 3. Conclusions

In this contribution, ATG UE with inter band CA requirements are discussed with the following proposals:

Proposal 1:  Set option 1 as the baseline parameters.

Proposal 2:  MSD with 23dBm output power should also be defined.

Proposal 3: RF session doesn’t exclude the Case 4 based on the reached agreement.

# 4. References

[1] RP-242385, Revised WID on Enhancements for Air-to-ground network for NR