3GPP TSG-RAN WG4 Meeting # 114                                    R4-2500868

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.23.2.3

Source: 	CMCC

Title: 	Discussion on existing Rx requirements for SBFD

Document for:	Discussion

# 1. Introduction

In TR 38.858, the impacts on SBFD requirements have been analyzed. RAN4 has identified which legacy requirements are still applicable, which legacy requirements are not applicable and which new requirements are needed.

In last meeting, WF on SBFD BS requirement is approved with several agreements. [1]

In this contribution, we continue the RF requirements discussion based on the output of study item phase.

- Discussion

2.1 ACS requirements

Following capture the agreements from TR 38.858 for ACS requirements:

- ACS requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of ACS requirement: - - Conducted ACS: Take the existing wanted signal of ACS requirement by using the existing reference sensitivity level. - - OTA ACS: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power.

Last meeting, we have the following requirements.

Agreement: - FFS on the followings: - The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD.

Similar to ACLR requirements, adjacent-channel co-existence simulation only focuses on not less than 10% grid shift values, and 0% grid shift is avoided. Considering it is hard to define the co-location ACS requirement, it is better to prioritize discussing the scenarios under non-colocation case first.

The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked.

Proposal 1: Prioritize discussing the scenarios under non-colocation case first.

Proposal 2: The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked.

2.2 In-band blocking

Last meeting we have approved the WF below:

Agreement: - For FR2 local area SBFD BS, re-use existing FR2 local area BS in-band blocking power requirements. - For FR1 WA BS, FFS following options. - Option 1: a unified blocking requirement to consider both scenarios - Option 2: separate requirements are defined - One set of requirements for coexisting with legacy TDD system in adjacent channel - One set of requirements for coexisting with new SBFD system in adjacent channel - Option 3: In-band blocking requirement is defined by manufacture declared level of interference in the specified range, which comes from the system-level evaluation (from 100% grid-shifting to 10% grid-shifting). - For FR1 WA BS co-existence study, FFS what grid-shift value(s) should be considered. - Option 1: 10% - Option 2: 20% - Option 3: 50% - Option 4: 100% - Option 5: 5% - Option 6: 20% ~ 50% - For FR2-1 WA SBFD BS, FFS following two options. - Option 1: The current FR2-1 WA BS in-band blocking requirements can be re-used - Option 2: Define in-band blocking interferer signal power to -23 dBm.

For the first issue whether a unified in-band blocking requirement is defined, we think it is hard to define two set of requirements for coexisting with legacy TDD or SBFD system. Since it is also challenging to encapsulate these requirements within the specification.

Proposal 3: Set a unified blocking requirement to consider both scenarios.

Since the study item utilizes a 100% grid-shift as the simulation assumption, to align with the conclusions drawn from the study item, it is important to maintain consistency in simulation assumptions. Therefore, we recommend considering a 100% grid-shift for our simulations.

Proposal 4: 100% grid-shift should be considered.

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: Prioritize discussing the scenarios under non-colocation case first.

Proposal 2: The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked.

Proposal 3: Set a unified blocking requirement to consider both scenarios.

Proposal 4: 100% grid-shift should be considered.

# 4. Reference

[1] TR 38.858,

[2] R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF