3GPP TSG-RAN WG4 Meeting #114				                       R4-2500876

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.24.1

Source: 	CMCC

Title: 	Discussion on A-IoT System parameters

Document for:	Discussion

# 1. Introduction

In RAN plenary #106 meeting, a new WI [1] of A-IoT is approved with following RAN4 lead RF part objectives:

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

In this contribution, we discuss the system parameters for A-IOT.

- Discussion

The conclusion on system parameters in study item are captured in following table.

| System parameter for A-IoT |  | Potentially needed or not |  |  |
| --- | --- | --- | --- | --- |
|  |  | A-IoT BS | A-IoT device | Intermediate UE |
| Operating band |  | YES |  |  |
| Channel bandwidth | Transmission bandwidth | YES the value needs be discussed in WI phase depending on other WG conclusion. |  |  |
|  | channel bandwidth | YES |  |  |
| Channel arrangement | Channel spacing | TBD in WI phase for standalone NA for in-band/Guard-band |  |  |
|  | Channel raster | IB/SA:YES GB:TBD | GB:NO IB/SA:TBD | IB/SA:YES GB:TBD |
|  | Synchronization raster | NO for device 1/2a |  |  |

- Transmission bandwidth

According to the conclusion of SI, the minimum transmission bandwidth for R2D is 1PRB. R2D transmission bandwidth is related to the M value of OOK-4, since the max M value is 32, the max minimum R2D transmission bandwidth is 4PRB. It is suggested that R2D transmission bandwidth is integer multiple(s) of 1PRB (i.e. 180KHz). Hence, R2D transmission bandwidth can be defined as 1PRB, 2PRB, 3PRB, 4PRB, down selection and whether any guard band is required can be further considered.

Proposal 1: For R2D transmission bandwidth, define 1PRB, 2PRB, 3PRB, 4PRB, whether any guard band is required can be further considered.

D2R only supports DSB in Rel-19, we first need to discuss whether to define bandwidth based on 1SB or 2SB. The advantage to define bandwidth based on 1SB is that when considering small frequency shift, the definition of bandwidth can be unified. Otherwise, after small frequency shift, how to define the gap between 2SBs will be a problem, as illustrated in the following figure.

Except for the transmission bandwidth of 1SB, which is the baseband chip rate F, SFO impact needs to be considered for the channel bandwidth of 1SB. Consider the worst SFO 10%, the channel bandwidth of 1SB would become F*(1+10%).

Proposal 2: Define D2R transmission bandwidth of 1SB, define channel bandwidth of 1SB considering 10% SFO impact.

Value of D2R transmission bandwidth is related to the uplink data rate and corresponding chip rate, which can be very flexible. According to the SI conclusion, D2R transmission bandwidth of 2SB range is from 15KHz to 2400KHz, and it’s integer multiple(s) of 15KHz. We can consider to define transmission bandwidth with an equation, and define requirements based on some typical transmission bandwidth, e.g. 15KHz, 180KHz, 2400KHz.

Proposal 3: Define D2R transmission bandwidth and channel bandwidth with an equation, and further discuss whether to define corresponding requirements based on some typical transmission bandwidth. For example:

- D2R transmission bandwidth of 1SB=baseband chip rate

- D2R channel bandwidth of 1SB=baseband chip rate*1.1

According to the WID, small frequency shift is supported in Rel-19. As illustrated in the following figure, the frequency will be shifted by R*F, R is codeword repetition number and can be configured within a very large range, F is the transmission bandwidth of 1SB. Due to the shift, the 10% SFO will have impact on R*F. Hence, the channel bandwidth after small frequency shift will become F+20%*R*F. Considering small frequency shift was not analyzed in RAN4 in SI phase, RAN4 needs to discuss whether and how to define the channel bandwidth for device with small frequency shift.

Also considering FDMA operation among different devices, RAN4 needs to discuss whether define system bandwidth for R2D considering FDMA operation.

Proposal 4: RAN4 further discuss whether and how to consider small frequency shift and FDMA operation when define D2R bandwidth.

- Channel raster

For channel raster, in SI phase, it was agreed that for in-band and standalone spectrum operation mode, channel raster is potentially needed. Considering the R2D and D2R transmission are at least integer multiples if 15KHz, it is suggested to define channel raster as 15KHz. The channel raster is mainly from the reader side, for device side, there is no need to define channel raster.

Proposal 5: Define channel raster as 15KHz for reader.

# 3. Conclusions

In this contribution, system parameters are discussed with following proposals.

Proposal 1: For R2D transmission bandwidth, define 1PRB, 2PRB, 3PRB, 4PRB, whether any guard band is required can be further considered.

Proposal 2: Define D2R transmission bandwidth of 1SB, define channel bandwidth of 1SB considering 10% SFO impact.

Proposal 3: Define D2R transmission bandwidth and channel bandwidth with an equation, and further discuss whether to define corresponding requirements based on some typical transmission bandwidth. For example:

- D2R transmission bandwidth of 1SB=baseband chip rate

- D2R channel bandwidth of 1SB=baseband chip rate*1.1

Proposal 4: RAN4 further discuss whether and how to consider small frequency shift and FDMA operation when define D2R bandwidth.

Proposal 5: Define channel raster as 15KHz for reader.

# 4. Reference

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- R4-2420470, WF on RF requirements for A-IOT, Moderator (Huawei).

- R4-2417088, WF on RF requirement impact of A-IoT, Huawei.

- R4-2414305, WF on RF requirements for A-IoT, Huawei

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID, Specification for RFID Air Interface Protocol for Communications at 860 MHz - 960 MHz, Version 2.0.0 Ratified