3GPP TSG-RAN WG4 Meeting #114				                      R4-2500878

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.24.2.2

Source: 	CMCC

Title: 	Discussion on A-IoT device RF requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #106 meeting, a new WI[1] of A-IoT is approved with following RAN4 lead RF part objectives:

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

In this contribution, we discuss the RF requirements A-IOT devices.

- Discussion

2.1 Tx requirements

Agreement in SI phase:

- Whether requirements potentially needed or not is summarized in the following table:

| A-IoT device TX (D2R) RF Requirement |  |
| --- | --- |
|  |  | Device 1 |
| Transmit output power | Maximum output power /output power | NO for maximum output power YES for backscattering power or power backscattering loss |
| Output power dynamic | Transmit OFF power | No |
|  | Transmit ON/OFF time mask | NO, as RAN4#112 agreed |
|  | Minimum output power | N/A |
|  | Power control | NO, as RAN4#112 agreed |
| Transmit signal quality | Frequency error | NO, as RAN4#112 agreed |
|  | Modulation quality requirements | TBD |
|  | In band emissions | NO, as RAN4#112 agreed |
|  | Carrier leakage | NO, as RAN4#112 agreed |
| Output RF spectrum emissions | Occupied bandwidth | YES |
|  | SEM | TBD |
|  | ACLR | TBD |
|  | Spurious emissions | TBD |
| Unwanted emissions |  | TBD |
| Transmit intermodulation |  | NO |

For device 1, considering its low cost, RFID spec can be considered as baseline which only has output power (backscattering power or power backscattering loss) and unwanted emission requirements.

For output power, backscattering power can be defined for device 1. Due to only outside topology is considered in Rel-17, the location of CW node will also impact the backscattering power. One possible way is to define a typical condition for test, define corresponding backscattering power or we can consider device declaration its backscattering power.

Proposal 1: For output power, define backscattering power with a typical condition for test or consider device declaration its backscattering power.

Based on the co-existence evaluation, ACLR is not needed for device. For other emission requirements, legacy SEM and spurious emission can be used as starting point, some adjustment on the offsets need to be considered.

Proposal 2: for emission requirements, use legacy requirements as starting point and adjust the offsets.

2.1 Rx requirements

Agreement in SI phase:

- Whether requirements potentially needed or not is summarized in the following table:

| A-IoT device- RX Requirement |
| --- |
|  | Device 1 |
| Reference sensitivity | YES, as RAN4#112 agreed |
| Maximum input power | TBD |
| ACS | NO |
| ACSC | NO |
| In-band blocking | NO |
| Out-of-band blocking | NO |
| Receiver intermodulation | NO |
| Rx spurious emission | NO |
| Spurious response | NO, as RAN4#112 agreed |
| Interference rejection | TBD |

For device 1, considering its low cost, only define reference sensitivity for Rx requirements. In SI phase, -36dBm is used for co-existence evaluation. In our view, device sensitivity depends on the device capability. So the legacy approach to use NF+target SNR is not appropriate to define device reference sensitivity. Similar as backscattering power, device declaration on reference sensitivity can also be considered.

Proposal 3: REFSENSE requirement can be defined considering device capability or based on device declaration.

# 3. Conclusions

In this contribution, RF requirements for device 1 are discussed with following observations and proposals.

Proposal 1: For output power, define backscattering power with a typical condition for test or consider device declaration its backscattering power.

Proposal 2: for emission requirements, use legacy requirements as starting point and adjust the offsets.

Proposal 3: REFSENSE requirement can be defined considering device capability or based on device declaration.

# 4. Reference

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- R4-2420470, WF on RF requirements for A-IOT, Moderator (Huawei).