3GPP TSG-RAN WG4 Meeting # 114	                                        R4-2500892

Athens, GR, Feb. 17 – Feb. 21, 2025

Agenda Item: 5.23.1

Source: MediaTek Inc.

Title: Discussion on R18 mobility

Document for: Discussion

# Introduction

Although R18 mobility has been closed, there are some further optimization proposals which are still open. In addition, after further checking, we found some missed points in the past discussions. We will discuss the related issues in this contribution.

# Discussion

## 2.1 Cell switch delay requirements

### 2.1.1. Conditions of Tfirst-RS =0 in cell switch delay

In RAN4#112, it was agreed to extend the condition for Tfirst-RS = 0 “the time gap between TCI state activation and the cell switch command is not more than 160 ms” to “480ms” in FR1 for UE not supporting/configured with L1 measurement. It was also clarified that “This is not to extent 160ms to 480ms directly. Longer L3 measurement delay may be expected in the 480ms duration”, which means UE may perform several shots of T/F tracking during the 480ms. Following the timing requirements, UE is supposed to track the target cell at least every 160ms.

Agreement in RAN4#112 Issue 1-4-1-2: Conditions of Tfirst-RS =0 in cell switch delay <Agreement>: - For cell switch with L1 measurement: Extend the condition for Tfirst-RS = 0 “the time gap between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is not more than [160 ms]” from the agreed 160 ms, to TCI state activation delay stated in section 8.25.3 + 160 ms. - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if - the time gap between completion of TCI activation and cell switch command is larger than 480ms - Note: This is not to extent 160ms to 480ms directly. Longer L3 measurement delay may be expected in the 480ms duration. - Discuss the wording in CR. - FFS the following assumption - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if - the time gap between early RACH transmission and cell switch command is larger than [160ms] and the L3 measurement interval is larger than 160 ms. Agreement in RAN4#113 Issue 1-3-1-2: Conditions of Tfirst-RS =0 in cell switch delay < Agreement>: Conclude the issue in RAN4#114 - Option 1: - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if - For intra-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 480ms. - During the 480ms after the TCI state activation, longer L3 measurement delay may be expected. - For inter-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 160ms. - Option 2: - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if - For intra-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 480ms. - For inter-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 480ms with conditions - Supporters are encouraged to propose the conditions for discussion next meeting - During the 480ms after the TCI state activation, longer L3 measurement delay may be expected.

After further checking the agreement reached in RAN4#112, we found the agreement is not applicable to most of inter-frequency scenarios, where UE is supposed to measure and track the target cells within MG. The typical MGRP is 80ms. If NW activates the TCI states on more than two different frequency layers, it is not possible to track each cell every 160ms. We understand that the above agreement is workable for some inter-frequency cases, such as a shorter MGRP configured and cells on less frequency layers to track. But we don’t think it is good to go that far by adding complicated conditions. As this is actually some further optimization and the WI has been closed for over a year, we prefer not to do more further unnecessary optimization.

Proposal 1: Keep the condition for Tfirst-RS = 0 as “the time gap between TCI state activation and the cell switch command is not more than 160 ms” in FR1 for UE not supporting/configured with L1 measurement.

### 2.1.2. Conditions for early ASN.1 decoding and validity check

Issue 1-3-2-1: Which cell(s) TLTM-RRC-processing = 0 apply to when the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability? <Agreement>: Conclude the issue in RAN4#114 - Option 1 (HW) - When the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) or PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18. - The requirements are applicable to the case NW does not trigger TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion, and the wording can be updated. - Option 2 (Nokia, ZTE, E///) - When the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) or PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18. - Option 3 (Apple, HW) - The issue is closed

In previous meetings, RAN4 agreed that both early TCI state activation command and PDCCH-order can be used to trigger early RRC decoding and validity check. In RAN4#112, RAN4 further agreed that “If the total number of cells to be fast RRC decoded does not exceed UE capability maxNumberStoredConfigCells-r18 and the number of LTMCandidateConfigs fast decoded does not exceed maxNumberConfigs-r18, TLTM_RRC-processing =0 applies to the LTM candidates with early TCI activation or early PDCCH order RACH”.

Last meeting, there were extensive discussions on whether to further optimize the case that the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability. If to do further optimization, it is reasonable to perform early RRC decoding on LTM candidates with the most recently activated TCI states (if any) or PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18. From our perspective, we are open for the further optimization. There are arguments that the scenario is not typical that NW triggers TCI state activation or PDCCH order RACH on different LTM candidates. We are not sure about that. It is possible that channel condition changes after NW triggers TCI state activation or PDCCH order RACH on a LTM candidate. At this stage, we cannot tell whether the scenario is typical or not. But with the further optimization, UE and NW can have common understanding on which LTM candidate(s) to do early RRC decoding. It is beneficial for the whole feature.

Similar as the above issue, since the WI has been closed for over a year. If there is no consensus on further optimization, we prefer not to discuss this issue anymore.

Proposal 2: Open to have the following optimization. If there is no consensus in RAN4, prefer to close the issue.

When the configured candidate cells are more than number of candidates that UE supports early RRC decoding and validity check, prefer that TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) or PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18.

- The requirements are applicable to the case NW does not trigger TCI state activation on different candidate cells at the same occasion.

### 2.1.3 Clarification on the agreement for PL-RS

RAN4#112 reached the following agreement on PL-RS. There was proposal to further clarify the agreement.

RAN4#112 Issue 1-4-2-1: Conditions of no extra time for PL-RS measurement in cell switch delay <Agreement>: - No additional PL-RS measurement time is needed, provided L3-RSRP or L1-RSRP on the SSB associated with PL-RS has been measured/reported. - PL-RS is associated with TCI state indicated by LTM cell switch command in terms of QCL chain. - Further discuss the wording in the CR next meeting.

Let’s recall the discussion in RAN4#112. The root condition to skip PL-RS measurement during cell switch is that UE has already performed L3 or L1 measurement on the SSB associated with PL-RS recently. That’s why we had the first main bullet. During the discussion, we proposed to make it clear that UE L3 or L1 measurement on the on the SSB associated with PL-RS has been measured/reported in [1280ms]. But there were arguments that cell switch delay requirements are only applicable to the following cases:

- known cell & known TCI state in FR2,

- known cell & UE has reported L3-RSRP measurement result with SSB index associated to the target TCI state within 5 seconds before receiving the LTM cell switch command.

These conditions have guaranteed that UE has reported L1 or L3 measurement result in 1280ms or 5s. But actually the above conditions can only guarantee that UE has reported L1 or L3 measurement result on SSB index associated to the target TCI state in 1280ms or 5s, and SSB associated to the target TCI state may not be the same as the SSB associated with PL-RS. The sub-bullet is added to make sure “SSB associated to the target TCI state is the same as the SSB associated with PL-RS”.

Observation 1: The sub-bullet in the following agreement is to make sure “SSB associated to the target TCI state is the same as the SSB associated with PL-RS”.

<Agreement>: - No additional PL-RS measurement time is needed, provided L3-RSRP or L1-RSRP on the SSB associated with PL-RS has been measured/reported. - PL-RS is associated with TCI state indicated by LTM cell switch command in terms of QCL chain. Further discuss the wording in the CR next meeting.

### 2.1.4 Additional applicable condition

In the scenario of RACH-based cell switch, UE may receive the LTM CSC MAC-CE with both the TCI state for the target cell and the CFRA information for the random access procedure. In the current specification, there is no limitation on whether the indicated TCI state and the SSB indicated in the CFRA information should be identical in RAN1/2. In our understanding, the indicated TCI state is the TCI state that NW want UE to use during RACH procedure and after RACH procedure. We believe there is no motivation to configure a TCI state not associated with the SSB for CFRA in the LTM cell switch command.

As mentioned in 2.1.3, cell switch delay requirement are only applicable to the following cases:

- known cell & known TCI state in FR2,

- known cell & UE has reported L3-RSRP measurement result with SSB index associated to the target TCI state within 5 seconds before receiving the LTM cell switch command.

The above conditions actually guarantee that UE has reported L1 or L3 measurement result on SSB index associated to the target TCI state in 1280ms or 5s.

When RAN4 define the requirements, we think the mis-alignment case that the TCI state indicated in cell switch command is not associated with the SSB for CFRA is not considered. That’s why the legacy applicable conditions mainly focus on SSB index associated to the target TCI state. We prefer to make it clear in RAN4 spec.

Proposal 3: Make it clear: for CFRA cell switch, the requirements are only applicable to the case that the TCI state indicated in cell switch command is associated with the SSB for CFRA.

## 2.2 UE Capability

Issue 1-4-3: Clarification on LTM L1-RSRP related capabilities < Agreement > For RAN4 39-3-6: - ‘SSB resources of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and all SSBs from serving cells, including the serving cells that are not configured with LTM L1 measurements, i.e. SSB resources configured by CSI-MeasConfig are also counted.

Last meeting, we agreed the above further clarification on 39-3-6. It seems further clarification on the following features are still needed.

| Index | Feature group | Components |
| --- | --- | --- |
| 39-3-3 | Number of total cells to be measured | The max number of total cells of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. |
| 39-3-5 | Number of SSB resources for L1-RSRP measurement per frequency layer | - The max number of SSB resources UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps 2. The max number of SSB resources UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps |

In our understanding, the same logic of 39-3-6 also applies to 39-3-3 and 39-3-5. From the point of measurement, no matter the SSB to be measured is configured in CSI-ResourceConfig or LTM-CSI-ResourceConfig-r18 or both, as long as UE needs to measure the resource, it shares UE L1-RSRP measurement capability with other SSB resources.

Proposal 4: For RAN4 39-3-3, “The max number of total cells of serving cells and neighboring cells” includes the serving cells that are configured L1-RSRP measurement in CSI-ResourceConfig but not configured in LTM-CSI-ResourceConfig-r18.

Proposal 5: For the first component of RAN4 39-3-5, “The max number of SSB resources” includes the SSB resources that are configured L1-RSRP measurement in CSI-ResourceConfig but not configured in LTM-CSI-ResourceConfig-r18.

## 2.3 Test case for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 (A.6.3.2.4.2)

We found a missed point in the test case for PDCCH-order RACH to inter-frequency candidate cell in FR1. We will elaborate the issue at first, and then propose a solution.

In the test case, MG is configured for inter-frequency measurement. In the current version, MGRP is set as 80ms, MGL and gapOffset are missing. The default value in RAN5 is MGL=6ms, and gapOffset=39 as specified in 38.508-1 table 4.6.3-70. In the next, we will use these settings for discussion. Based on the Gap configuration, MG starts from subframe 9 (Subframe = gapOffset mod10  Subframe=9) of the frames with “SFN mod 8 = 3” (SFN mod (MGRP/10) = FLOOR (gapOffset/10)  SFN mod 8 = 3). According to the SSB configuration used, SSB is on the first one or two slots in even SFNs.

In the test, RACH configuration 6 is used. After checking table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211, it can be found that RACH occasions are in subframe 7 under FDD, subframe 2&7 under TDD.

For information: From 38.133 A.3.8.2.6 RACH configuration 6: Field; Value; Comment. prach-ConfigurationIndex; 102; 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6].. From TS 38.211 table 6.3.3.2-2 FDD: PRACH Configuration Index; Preamble format; nf mod x=y; Subframe number; Starting symbol; Number of PRACH slots within a subframe; , number of time-domain PRACH occasions within a PRACH slot; , PRACH duration. 102; A1; 1; 0; 7; 0; 2; 6; 2. From TS 38.211 table 6.3.3.2-3 TDD: PRACH Configuration Index; Preamble format; nf mod x=y; Subframe number; Starting symbol; Number of PRACH slots within a subframe; , number of time-domain PRACH occasions within a PRACH slot; , PRACH duration. 102; A2; 1; 0; 2,7; 0; 1; 3; 4.

In test 1, with early TCI state activation, PDCCH order RACH delay is NT,2+TBWPswitchDelay+∆Delay+Tswitch+TSSB+∆RF/BB preparation, i.e., N2 symbols + 0 + 0.5ms + 0 + 0 + {1ms,3ms,5ms,10ms} ≈ {2ms, 4ms, 6ms, 11ms}.

The interruption due to RF retuning before and after RACH transmission is calculated by ceil (Y/NR Slot length) +1 slots ({2, 2, 2, 3} slots with 15kHz corresponding to Y= {0.25,0.5,1,2}). There will be at most 3 slots interruption at 15kHz before and after RACH transmission.

Based on the configurations, it can be observed that:

- Under TDD, with 15kHz SCS, if PDCCH order is sent at slot#6 in the frame whose SFN mod 8 =3, UE may not have enough time to reschedule the up-coming MG and will fail to transmit RACH on slot#2 in the next frame. That’s because is slot#2 overlapped with MG as shown in Fig.1. In addition, RACH transmission on slot#7 will also fail because the RF retuning time before RACH overlaps with MG.

- Under FDD, with 15kHz, as shown in Fig.2, RACH transmission on slot#7 is also impacted by MG due to the retuning time is overlapped with MG. If PDCCH order is sent at slot#6,7,8 in the frame whose SFN mod 8 =3, UE may not have enough time to reschedule MG and fail to transmit RACH on slot#7 in the next frame.

Fig. 1

Fig.2

Observation 2: In the test case for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 (A.6.3.2.4.2), UE may fail to transmit RACH to the target cell on the first RACH occasion after PDCCH order RACH delay as the RACH occasion or the retuning time is overlapped by MG and UE has no sufficient time to reschedule MG.

Recalling the requirements defined in 38.213 cl.8.1, UE is not mandatory to transmit RACH on the first available RACH occasion after PDCCH order RACH delay which is similar as legacy PDCCH order RACH on serving cell. According to the spec, UE is allowed to transmit RACH on the next RACH occasion in the above mentioned scenarios. But in the test, UE are mandatory to transmit RACH on the first RACH occasion. The intension to ask UE to transmit on the first occasion is to save NW RACH resources and reach common understanding on the occasion that UE transmits RACH. In the field, we think NW can avoid the above cases through smart scheduling or expect UE to transmit RACH on the next RACH occasion.

38.213 cl.8.1 If a random access procedure is initiated by a PDCCH order, the UE, if requested by higher layers, transmits a PRACH in the selected PRACH occasion, as described in [11, TS 38.321], for which a time between the last symbol of the PDCCH order reception and the first symbol of the PRACH transmission is larger than or equal to NT,2+TBWPswitchDelay+∆Delay+Tswitch+TSSB+∆RF/BB preparation msec, ……

To avoid the case that the first RACH occasion or the retuning time is overlapped by MG and UE has no sufficient time to reschedule MG, it is necessary to add some limitation on the PDCCH order occasions. We propose to add a limitation that PDCCH order is not expected on the frame with SFN mod 8 =3 which is simple and can solve the issue.

Proposal 6: In the test case for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 (A.6.3.2.4.2), add a limitation that PDCCH order is not expected on the frame with SFN mod 8 =3.

## 2.4 Test cases for SSB based L1-RSRP measurement in FR1 – intra-f & inter-f without gap (A.6.6.26.1 & A.6.6.28.1.3)

In the test cases for SSB based L1-RSRP measurement in FR1, intra-f & inter-f without gap, report period is set as 80 slots, SSB period is 20ms and DRX is not used. According to core part requirements, measurement period = max max(TReport, ceil(M*P)*TSSB_NBC) = 80ms with 15kHz SCS and 40ms with 30kHz SCS.

Regarding the test requirements, as shown in Fig. 4, there may be some time gap between the report occasion and the time UE finishes the first round of measurement. Therefore, the test requirements should be measurement delay + report delay considering the worst case. But in the above two test cases, UE is required to report a new value no later than 20 ms plus 80 slots from the beginning of time period T2. It is not aligned with core part requirements.

Fig. 4

Proposal 7: In LTM L1-RSRP test cases in FR1 (A.6.6.26.1 & A.6.6.28.1.3), intra-f and inter-f without gap, the test requirements should be measurement delay + report delay considering the worst case.

## 2.5 LS from RAN1 on collision between SSB and RA occasion for LTM

In RAN1 LS R1-2410911/ R4-2500008, they identified possible collision between valid RA occasion (RO) and SSB associated with different cells for unpaired spectrum and ask RAN4 on whether/how to handle the collision between an SSB and a valid RO. As far as we know, their question is for PDCCH order RACH.

In our understanding, if the valid RO collided with SSB, RO shall be prioritized if possible. We also think it is reasonable to prioritize RACH. UE is supposed to measure SSB periodically but PDCCH order RACH on neighbor cell is one shot transmission. RO and SSB may be always collided as both RO and SSB are periodic. If SSB based measurement is prioritized, UE may have no chance to transmit RACH. As mentioned in 2.3, in the test, UE are mandatory to transmit RACH on the first RACH occasion. The intension to ask UE to transmit on the first occasion is to save NW RACH resources and reach common understanding on the occasion that UE transmits RACH. This is also evidence of RACH prioritized.

But as discussed in 2.3, if the first RACH occasion or the retuning time is overlapped by MG and UE has no sufficient time to reschedule MG, UE may fail to reschedule MG due to it takes some time to do that. We think it is fine to keep the core requirements in RAN1 and RAN4 as it is since UE is not mandatory to transmit RACH on the first available RACH occasion after PDCCH order RACH delay according to the requirements defined in 38.213 cl.8.1. It is also ok for us to make it clear in spec.

Proposal 8: For PDCCH order RACH, RACH transmission is prioritized if valid RO collides with SSB in LTM.

Observation 3: If the valid RACH occasion is collided with MG and UE has no sufficient time to reschedule MG, UE is allowed to postpone RACH transmission.

# Summary

In this paper, we provide some views on the open issues of R18 L1/L2 triggered mobility. We have the following proposals:

Proposal 1: Keep the condition for Tfirst-RS = 0 as “the time gap between TCI state activation and the cell switch command is not more than 160 ms” in FR1 for UE not supporting/configured with L1 measurement.

Proposal 2: Open to have the following optimization. If there is no consensus in RAN4, prefer to close the issue.

When the configured candidate cells are more than number of candidates that UE supports early RRC decoding and validity check, prefer that TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) or PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18.

- The requirements are applicable to the case NW does not trigger TCI state activation on different candidate cells at the same occasion.

Observation 2: The sub-bullet in the following agreement is to make sure “SSB associated to the target TCI state is the same as the SSB associated with PL-RS”.

<Agreement>: - No additional PL-RS measurement time is needed, provided L3-RSRP or L1-RSRP on the SSB associated with PL-RS has been measured/reported. - PL-RS is associated with TCI state indicated by LTM cell switch command in terms of QCL chain. Further discuss the wording in the CR next meeting.

Proposal 3: Make it clear: for CFRA cell switch, the requirements are only applicable to the case that the TCI state indicated in cell switch command is associated with the SSB for CFRA.

Proposal 4: For RAN4 39-3-3, “The max number of total cells of serving cells and neighboring cells” includes the serving cells that are configured L1-RSRP measurement in CSI-ResourceConfig but not configured in LTM-CSI-ResourceConfig-r18.

Proposal 5: For the first component of RAN4 39-3-5, “The max number of SSB resources” includes the SSB resources that are configured L1-RSRP measurement in CSI-ResourceConfig but not configured in LTM-CSI-ResourceConfig-r18.

Observation 1: In the test case for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 (A.6.3.2.4.2), UE may fail to transmit RACH to the target cell on the first RACH occasion after PDCCH order RACH delay as the RACH occasion or the retuning time is overlapped by MG and UE has no sufficient time to reschedule MG.

Proposal 6: In the test case for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 (A.6.3.2.4.2), add a limitation that PDCCH order is not expected on the frame with SFN mod 8 =3.

Proposal 7: In LTM L1-RSRP test cases in FR1 (A.6.6.26.1 & A.6.6.28.1.3), intra-f and inter-f without gap, the test requirements should be measurement delay + report delay considering the worst case.

Proposal 8: For PDCCH order RACH, RACH transmission is prioritized if valid RO collides with SSB in LTM.

Observation 3: If the valid RACH occasion is collided with MG and UE has no sufficient time to reschedule MG, UE is allowed to postpone RACH transmission.

# Annex

3GPP TSG-RAN WG4 Meeting #114						                  R5-25xxxxx

Athens, GR, 17th – 21st Feburary, 2025

Title:	Reply LS to RAN1 on collision between SSB and RA occasion for LTM

Response to:	R1-2410911/R4-2500008

Release:	Release 18

Work Item:	NR_Mob_enh2-Core

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

#### Name:	Ada Wang

E-mail Address:	< ada.wang@mediatek.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN4 thanks RAN1 for the LS on collision between SSB and RA occasion for LTM. RAN4 discussed the question from RAN1 and reached the following consensus:

- For PDCCH order RACH, RACH transmission is prioritized if valid RO collides with SSB in LTM.

- If the valid RACH occasion is collided with MG and UE has no sufficient time to reschedule MG, UE is allowed to postpone RACH transmission.

2. Actions:

To RAN1.

ACTION: 	RAN4 kindly asks RAN1 to take above information into consideration.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #114bis		              7 - 1 April 2025				China

TSG RAN WG4 Meeting #115		              19 - 23 May 2025 			Malta