3GPP TSG-RAN4 Meeting #114	R4-2500901

# Ahtens, Greece, 17th Feb 2025 – 21st Feb 2025

Source:	LG Electronics

Title:	Discussion for LP-WUR REFSENS requirement

Agenda item:	7.26.2.2

Document for:	Discussion

Introduction

In the RAN4#113,  the WF on LP-WUS was approved. In this discussion, we share our point of view for the LP-WUS requirement based on the approved WF[1].

Discussion

FR-1 LP WUS/WUR

REFSENS requirements

Table 1 WF on LP-WUS REFSENS requirements in the RAN4#113 [1]

Issue 2-2-1: General views on REFSENS for LP-WUS Agreements: - Discuss REFSENS for the two types of receivers respectively, and then check if one single value or two values are specified pending on the final REFSENS values. Issue 2-2-3: companies input on SNR, NF and IM value for FR1 OOK-based LR - Proposals - Proposal 1: check the following input from companies and converge the range for OOK-based LR. (Moderator) FR1 OOK-based receiver; Huawei; Sony; Spread trum; Apple; vivo; QC; Samsung; LGE; CMCC; ZTE; Nokia; E//. IM(dB); 2.5; 2.5; 2.5; >=2.5; 2.5; 2.5; 3; 2.5; 2.5; 2.5; 1; 2.5. NF(dB) (assume MR=9dB); 17; 10; 17; 16; 15; 11.5; 15; 16; 12; 14; 12; 12. SNR(dB); -2; -7~-2; -3; -2; -1 for sequence length of 18; -2; -2.5; -2; -1.5~ -5. Note: the SNR value may be updated based on new simulation results. RAN1 waveform design is not finalized as of this WF. Agreements: - Use 2.5dB IM as a starting point, encourage companies to report the difference aspects due to OOK waveform receiver. The corresponding band should also be reported. - The total value of NF+IM can be considered. - On how to handle the NF values: - Option 1: For NF whether average value can be adopted. - Option 2: two sets NFs for different LP-WUR types with different power consumption target - Option 3: also consider coverage - Capture the table in WF as background information Issue 2-2-4: companies input on SNR, NF and IM value for FR1 OFDM-based LR - Proposals - Proposal 1: check the following input from companies and converge the range for OFDM-based LR. (Moderator) FR1 OFDM-based receiver; Huawei; Sony; Spread trum; Apple; vivo; QC; Samsung; LGE; CMCC; ZTE; Nokia; E//. IM(dB); 2.5; 2.5; 2.5; >=2.5; 2.5; 2.5; 2.5; 2.5; 1; 2.5. NF(dB)(assume MR=9dB); 11; 10; 11; 16; 11; 12; 9; 11; 12; 12~13. SNR(dB); -2; -7~-2; -3; -2 or -5; -4.7; -5; -3.2. Note: the SNR value may be updated based on new simulation results. RAN1 waveform design is not finalized as of this WF. Agreements: - Further discuss next meeting

Based on the simulation results submitted in RAN4#112bis[3], OOK-4 has an SNR that is 4 dB greater than OOK-1, which corresponds to approximately -7 dB based on a 1% MDR. However, when considering Carrier Frequency Offset (CFO), OOK-1 is unaffected by CFO, while OOK-4 is influenced by it. In scenarios with high CFO, a larger SNR can be defined. For instance, considering a CFO of 10 ppm results in an SNR of -2 dB, and with a CFO of 20 ppm, the SNR is 8 dB, indicating a substantial impact based on the magnitude of CFO. Therefore, depending on the size of the considered CFO, the SNR can range from -7 dB to 8 dB, and it is necessary to discuss which CFO value should be used as a reference for definition.

Observation 1: The MDR performance of OOK-4 degrades significantly when the CFO value more than 10 PPM

Proposal 1: It is necessary to consider the SNR value based on OOK signal from -7 to 8, taking into account the CFO effect.

During the last meeting, we discussed how to define the Noise Figure (NF), and there are three options as outlined below

On how to handle the NF values:

    - Option 1: For NF whether average value can be adopted.

    - Option 2: two sets NFs for different LP-WUR types with different power consumption target

    - Option 3: also consider coverage

Our understanding is that from the perspective of terminal implementation, power consumption has the greatest impact on Noise Figure (NF). If a receiver has low power consumption, there is a trade-off that results in a higher NF. Therefore, option 2 seems to be a valid approach.

Proposal 2: Support option 2, taking into account the trade off relation between power consumption and NF for LP-WUR

LLS simulation with phase noise

Table 2 WF on LP-WUS REFSENS requirements in the RAN4#113 [2]

Issue 2-4-1: Phase noise for simulation assumption Agreements: - Companies can use different phase noise assumptions, the phase noise assumption should be reported Issue 2-4-2: General ACS requirements value and guard RB Agreements: - For ACS evaluation, companies are encouraged to provide the simulation results by assuming - Baseline: the same interference experienced by LP-WUR and main radio. - Optional: the different interference level for LP-WUR and main radio Issue 2-4-4: Test parameters for LP-WUR ACS case WF: - RAN4 further discuss detailed test parameters table and how to modify spec.

Table 3 Simulation assumption

| Parameters | Assumption |
| --- | --- |
| Waveform | OOK-1 |
| Carrier Frequency | 2.6GHz (FR1) |
| Number symbols | 1 |
| Time error | 0 us |
| CFO | 0 ppm |
| SCS | 30kHz |
| UE Channel BW | 20 MHz (51 RBs) |
| WUS RBs | 11 RBs (~3.96 MHz) |
| Filter | Butterworth (5 Order) |
| ADC | 4 bits |
| Channel propagation | AWGN |
| Antenna configuration | 1x1 Low |
| Phase noise model | 802.11ba/n |
| Interference Power boosting | 7.5-35.5 dB |
| Guard RB | 0-4 RBs |

Figure 1 The signal distribution for the scenario

Figure 2 Phase noise profile for LP-WUS simulation

Figure 3 LLS simulation results for OOK-1

Observation 2: SNR degradation due to PN on OOK-1

- No-ACS interferer case: 3 dB for 1% MDR

- Interference case

  - For GB 0 RB : 5 dB for 1% MDR

  - For GB 1-4 RBs : 2.5 dB for 1% MDR

Figure 4 LLS simulation results for OOK-1 with phase noise

Observation 3: The simulation results for OOK-4 ,considering phase noise, showed that even without ACS interference, it was not possible to achieve a 1% MDR. This indicates that when accounting for phase noise, the performance degradation is more significantly affected by phase noise than by ACS performance degradation.

Proposal 3: Compensation for phase noise for LP-WUS is necessary considering the practical phase noise.

Conclusion

Observation 1: The MDR performance of OOK-4 degrades significantly when the CFO value more than 10 PPM

Observation 2: SNR degradation due to PN on OOK-1

- No-ACS interferer case: 3 dB for 1% MDR

- Interference case

  - For GB 0 RB : 5 dB for 1% MDR

  - For GB 1-4 RBs : 2.5 dB for 1% MDR

Observation 3: The simulation results for OOK-4 ,considering phase noise, showed that even without ACS interference, it was not possible to achieve a 1% MDR. This indicates that when accounting for phase noise, the performance degradation is more significantly affected by phase noise than by ACS performance degradation.

Proposal 1: It is necessary to consider the SNR value based on OOK signal from -7 to 8, taking into account the CFO effect.

Proposal 2: Support option 2, taking into account the trade off relation between power consumption and NF for LP-WUR

Proposal 3: Compensation for phase noise for LP-WUS is necessary considering the practical phase noise.

- References

# [1] R4-2420394, WF on UE RF requirements for LP-WUS (vivo)