# 3GPP TSG-RAN WG4 Meeting #114                               R4-2500941

# Athens, Greece, 17th-21th Feb, 2025

# Title: 	Discussion the methods for reducing the number of UE Rx chain

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.6.2

# Document for:	Approval

# 1 Introduction

In the last meeting, companies continue discussing the methods of reducing the number of UE Rx chain[1], and there are some agreements captured in the approved WF [2]. Meanwhile, there still some open issues which was not discussed due to lack of time.

In this contribution, we give some views on some left open issues on the methods for reducing the number of Rx chain for fragment carriers.

# 2	Discussion

For the baseband assumption, we think different FFT size implementation for intra-band DL NC CA is a modern design, no matter whether or not fragment carriers feature is supported. Nevertheless, it still possible to use same FFT size implementation when the same numerology are used for each carrier for feasibility purpose, and also the baseband assumption seems out of the scope for fragment carrier discussion due to only RF assumption, i.e. shared Rx RF architecture, is mentioned in the SID, and it will impact the RF requirements using shared Rx RF architecture for intra-band DL NC CA.

Given that, RAN4 agreed to keep the implementation feasibility for the baseband, where the agreements are:

- Issue 2-1: Discussion on baseband assumptions Agreement: - Consider only same SCS for both CCs for one Rx RF chain architecture in this SI - 15KHz SCS is used as the starting point Single FFT or separate FFT can be left to UE implementation, and however the condition of single FFT compared to separate FFT can be captured in the TR.

The triggering condition to enable “one RF Rx chain” or “shared RF Rx chain” was also discussed, the WF is:

- Issue 2-5: Triggering condition to enable ”One RF Rx chain” RAN4 to identify the mechanisms that surround the UE implementation, i.e. whether one Rx RF chain operation can be enabled - Further study whether the absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled - Other determining factors are not precluded to be studied Study feasibility to measure total aggregated power of all interferences in the gap between CC1 and CC2

In our understanding, the purpose for the condition is to make NW configure/schedule UE band configuration correctly, which is when UE reports the capability of shared Rx RF chain to the NW after initial access, NW would configure/schedule the band configuration for the UE whose report the capability. However, when there are some conditions such as the power measured in the gap is larger which makes the DL CC performance poorer comparing with traditional way or single carrier due to poor filter attenuation in the gap, the benefit of shared Rx RF chain would loss. In this case, NW should disable the shared RF Rx chain feature to maintain the DL performance.

For the triggering condition, it is straightforward to use the absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC as the trigger condition. In addition to that, we think the frequency span should be also considered as one of the condition since although 100MHz frequency span is for the SID, up to 200MHz for one single Rx RF chain can be supported in current design, and also 100MHz frequency span can be seen as the precondition for the shared RF Rx chain. For those UE who can support >100MHz frequency range but report the capability of shared Rx RF chain, NW should disable this capability.

Proposal 1. Both frequency span≤100MHz and the absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled.

In addition to the above condition, how UE reports the shared Rx RF chain to inform the network of appropriate CA configuration it can support is also discussed in last meeting, but no consensus were achieved. The WF is:

- Issue 2-4: Signaling related WF: Companies are encouraged to provide analysis with pros and cons on preferred signalling solutions so that they can be discussed in next meeting.

There are four alternations mentioned in the contribution [3][4], which are:

- Alt. 1: Legacy band combination reporting scheme + Rx sharing capability indication

- Alt. 2: Band combination capability update with the supported number of CCs

- Alt. 3: New band combination definition e.g. CA_F_n25(2A) + new set of signaling

- Alt. 4: Introduce undefined NR CA bandwidth class ‘F’.

In our understanding, the inter-band DL NC CA band configurations supported by a UE capable of shared Rx RF chain are essentially traditional CA band configuration, where the existing Tx requirement shall be applied. In other words, there are no impacts on Tx requirements. The only impacts are the Rx requirements.

For Alt. 4, if new bandwidth class ‘F’ is introduced, it can be seen as brand new CA configuration, which needs more information like supported channel bandwidths and BCS, and also it should describe that the relationship between the traditional CA band configuration and new CA band configuration, even it needs basket WID procedure, which would make things a bit more complicated. We think the band combination notation should not be changed.

For Alt. 3, comparing with Alt.1, it seems the new band combination definition is no needed since there is a new set of signaling, and it may also encounter the similar situation as Alt.4.

For Alt. 2, we are not sure the impacts on RAN2 since it seems the legacy band combination reporting scheme to be updated. Moreover, it seems it doesnot make sense to report the supported number of CCs since the legacy band combination reporting scheme are based on the band combination defined in RAN4 specification.

For Alt. 1, we think it is straightforward and simple, and also the impacts on RAN2 is minimum since it is normal procedure similar as other new feature introduce in later releases.

Proposal 2. We prefer to adopt legacy band combination reporting scheme + Rx sharing capability indication for fragment carrier feature reporting.

For the Rx sharing capability indication, there may exist several approaches, including implicit and explicit indications, such as shared Rx RF capability IE, frequency separation IE, or other IEs, and it seems it belongs to RAN2’s action, and it would better to make decision in the WI phase.

Proposal 3: Implicit and explicit indications for the Rx sharing capability indication should be considered, more details can be discussed in the WI phase.

Another thing is fallback solution, which is when a UE report the shared Rx RF chain related capability, but the condition to enable this capability is not met, then what is the UE behaviour. It seems there are two options:

1. Fallback to legacy CA behaviour

2. Fallback to non-CA behaviour, i.e. single carrier

For the first one, as pointed out by some companies in last meeting, it seems it would cause UE’s complexity since legacy Rx RF architecture (i.e. partial shared) and one Rx RF architecture should be supported for UE, and also switching between these two architectures are needed to be considered which would cause like-Rx switching related discussion. Meanwhile, the condition for the switching between the architectures may need to be discussed. Therefore, we prefer to fallback to non-CA behaviour for the same RF architecture, it is simple and compliance to the legacy band combination fallback behaviour.

Proposal 4: We prefer to fallback to non-CA behaviour for the band combination supported by a UE enable of full shared Rx RF chain.

# 3 Conclusion

In this contribution, we give some discussions on some left open issue on the methods for reducing the number of Rx chain for fragment carriers:

Proposal 1. Both frequency span≤100MHz and the absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled.

Proposal 2. We prefer to adopt legacy band combination reporting scheme + Rx sharing capability indication for fragment carrier feature reporting.

Proposal 3: Implicit and explicit indications for the Rx sharing capability indication should be considered, more details can be discussed in the WI phase.

Proposal 4: We prefer to fallback to non-CA behaviour for the band combination supported by a UE enable of full shared Rx RF chain.

# 4	Reference

[1] R4-2418146, Topic summary for [113][124] FS_NR_DL_Frag_Carrier, Mediatek

[2] R4-2420519, WF on fragmented DL study, Mediatek

[3] R4-2418531, On methods for reducing the number of UE Rx chains for fragmented carriers, Apple

[4] R4-2419454, Discussion on Fragmented Carrier reduction of Rx RF chains, Nokia