3GPP TSG-RAN WG4 Meeting # 114	R4-2500983

# Athens, GR,  17th - 21st February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.108 | CR | 0103 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_LSband-Core)CR for TS 38.108, Correction on sync raster for band n254 for NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_LSband-Core |  |  |  |  |  | Date: |  |  | 2025-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The sync raster for band n254 for 30kHz SS Block SCS is incorrect, it should be 6220 – <1> – 6238 instead of 6218 – <1> – 6241. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the sync raster for band n254 for 30kHz SS Block SCS from 6218 – <1> – 6241 to 6220 – <1> – 6238 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The sync raster for band n254 for 30kHz SS Block SCS would be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.101-5 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

5.4.3.3	Synchronization raster entries for each operating band

The synchronization raster for each band is given in table 5.4.3.3-1 and table 5.4.3.3-2. The distance between applicable GSCN entries is given by the <Step size> indicated in table 5.4.3.3-1 for FR1-NTN and table 5.4.3.3-2 for FR2-NTN.

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30kHz | Case C | 6220 – <1> – 6238 |
| NOTE: SS Block pattern is defined in clause 4.1 in TS 38.213 [7]. |  |  |  |

Table 5.4.3.3-2: Applicable SS raster entries per operating band (FR2-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n511 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n510 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| NOTE: SS Block pattern is defined in section 4.1 in TS 38.213 [7]. |  |  |  |

<End of Change 1>