3GPP TSG-RAN WG4 Meeting #114                                                                                                      R4-2500994

Athens, GR, 17th –21st February, 2025

Source: 	CATT

Title: 	Discussion on UE RF requirments for RedCap PC2 FDD

Agenda item:	7.7.2

Document for:	Approval

- Introduction

In RAN#106 meeting, the revised WID on NR power class 2 RedCap (Reduced Capability) UE in FR1 was approved [1] with the updated objectives captured as below:

The objectives of the core part are as follows: - Specify the PC2 RF requirements for RedCap UE on both TDD and FDD bands, based on 1Tx architecture with 26 dBm PA. - The PC2 RF requirements include: UE maximum output power, Tx power tolerance, MPR, A-MPR and RSD requirements if needed. - For TDD bands, PC2 RF requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way. - Note: Not differentiate the applicable form factors for TDD bands in the specification. - For FDD bands, specify RF requirements for PC2 RedCap UE - Band specific requirements are specified for the example bands including n3 and n1 when band agnostic way is not applicable (Example bands are selected by considering different sensitivity degradation causes) - Note: Wearable device supporting HPUE is not in the scope for FDD bands in this work item Note 1: Only the operating bands for which PC2 on non-RedCap UE has been introduced can be considered for RedCap UE. Note 2: PC2 for Rel-17 Redcap and Rel-18 eRedcap are considered for this WI.

As above, the updated objective in revised WID [1] is to specify RF requirements for PC2 RedCap UEs for FDD bands. In RAN4#113 meeting, a draft CR [2] for RedCap PC2 UEs for TDD bands was endorsed. So, in this contribution, we provide our views on UE RF requirements for RedCap PC2 UEs for FDD bands.

- Discussion

As mentioned in revised WID [1], for 1Tx architecture with 26 dBm PA, the PC2 RF requirements include: UE maximum output power, Tx power tolerance, MPR, A-MPR and RSD requirements if needed. The UE maximum output power, Tx power tolerance, MPR, A-MPR are related to Transmitter power for (e)RedCap in sub-clause 6.2I of TS 38101-1. And RSD requirements are related to the Reference sensitivity for (e)RedCap in sub-clause 7.3I of TS 38.101-1. So, we discuss the UE RF requirements for RedCap PC2 UEs for FDD bands in term of Transmitter power for (e)RedCap and Reference sensitivity for (e)RedCap.

Transmitter power for (e)RedCap for FDD bands

In endorsed draft CR [2] for RedCap PC2 UEs for TDD bands, adding RedCap supporting PC2 as optional feature on TDD bands in the band agnostic way is copied as below:

------------------------------------------------------------- START CHANGE ------------------------------------------------------ ## 6.2I Transmitter power for RedCap ### 6.2I.1 Maximum output power for RedCap For Redcap UE, the requirements for power class 3 specified in clause 6.2.1 apply. For (e)Redcap UE supporting PC2, the requirements specified for PC2 in clause 6.2.1 [for TDD bands] apply. ------------------------------------------------------------- END OF CHANGES ------------------------------------------------------

Endorsed draft CR indicates that maximum output power (Tx power tolerance) specified for normal UE (non-RedCap UE) for PC2 in clause 6.2.1 for TDD bands is applicable for (e)RedCap UE supporting PC2 for TDD bands. From above, only Maximum output power for RedCap is defined, the MPR, A-MPR for RedCap are not independently defined in separate sub-clause. From the perspective of specification, the MPR, A-MPR for normal UE for PC2 for TDD bands defined in clause 6.2.2 and 6.2.3 respectively are also applicable for (e)RedCap UE supporting PC2. So, it is not necessary to define separate sub-clause for MPR, A-MPR for (e)RedCap.

When it comes to (e)RedCap PC2 for FDD bands, it is sufficient that the maximum output power (Tx power tolerance), MPR, and A-MPR specified for non-RedCap UE PC2 for FDD bands can be reused for RedCap UE PC2 for FDD bands, since the same RF hardware for 1Tx architecture for non-RedCap UE PC2 for FDD bands can be reused for RedCap UE. So it is reasonable to remove [for TDD bands] in endorsed draft CR [2] for RedCap PC2 UEs for TDD bands to support RedCap UE PC2 for FDD bands.

Proposal 1: Remove the “[for TDD bands]” in endorsed draft CR [2] for RedCap PC2 UEs for TDD bands to support RedCap UE PC2 for FDD bands.

Reference sensitivity for (e)RedCap for FDD bands

For existing RedCap PC3 UE for FDD bands, the reference sensitivity for both HD-FDD and FD-FDD are defined. It is reasonable that both HD-FDD and FD-FDD should be supported for RedCap PC2 UEs for FDD bands.

Proposal 2: Both HD-FDD and FD-FDD should be supported for RedCap PC2 UEs for FDD bands.

For non-RedCap UE, the reference sensitivity for PC2 and PC3 for TDD bands is same, so for RedCap PC2 UEs for TDD bands, the reference sensitivity specified for RedCap PC 3 in sub-clause 7.3I is applicable for RedCap PC2 for TDD bands, so there is not any modifications related to reference sensitivity for RedCap PC2 for TDD bands in the endorsed draft CR [2] for RedCap PC2 UEs for TDD bands.

Since HD-FDD is more like TDD, and reference sensitivity for HD-FDD is independent on Reference Sensitivity Degradation (RSD).  So the existing reference sensitivity for RedCap PC3 for HD-FDD in sub-clause 7.3I of TS 38.101-1 can be applicable for RedCap PC2 for HD-FDD.

Proposal 3: The existing reference sensitivity for RedCap PC3 for HD-FDD in sub-clause 7.3I of TS 38.101-1 can be applicable for RedCap PC2 for HD-FDD.

The reference sensitivity specified for RedCap PC 3 in sub-clause 7.3I of TS 38.101- 1is copied as below:

For a RedCap UE equipped with 2 Rx antenna ports, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1a and Table 7.3.2-1b for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a RedCap UE equipped with 2 Rx antenna ports shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3 and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth. For a RedCap UE equipped with 1 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔR1R in Table 7.3I.2-1 for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a RedCap UE equipped with 1 Rx antenna ports shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3 and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth. Table 7.3I.2-1: Single antenna port reference sensitivity allowance ΔR1R Operating band; Channel bandwidth (MHz); ΔR1R (dB). TDD band; 5, 10, 15, 20; 2.5. FDD band; 5; 2.5. FDD band; 10, 15, 20; 3.0.

As above, for RedCap PC3 UE for FDD bands, the reference sensitivity for 1 Rx antenna port is specified as reference sensitivity for 2Rx antenna ports modified by ΔR1R in Table 7.3I.2-1. According to the RSD for non-RedCap UE, the RSD requirement for RedCap PC2 UE for FDD bands is required. For a RedCap PC2 UE equipped with 2 Rx antenna ports for FD-FDD bands, the RSD requirement defined for non-RedCap UE not supporting Tx Diversity for 5, 10, 15, and 20MHz CBW is applicable.

Proposal 4: For a RedCap PC2 UE equipped with 2 Rx antenna ports for FD-FDD bands, the RSD requirement defined for non-RedCap UE not supporting Tx Diversity for 5, 10, 15, and 20MHz CBW is applicable.

For a RedCap PC2 UE equipped with 1 Rx antenna ports for FD-FDD bands, both the ΔR1R and RSD requirement are needed to be considered. We think that ΔR1R for RedCap PC3 UE equipped with 1 Rx antenna ports for FDD bands and the RSD requirement defined for non-RedCap UE not supporting Tx Diversity for 5, 10, 15, and 20MHz CBW can be considered as starting point.

Proposal 5: For a RedCap PC2 UE equipped with 1 Rx antenna ports for FD-FDD bands, ΔR1R for RedCap PC3 UE equipped with 1 Rx antenna ports for FDD bands and the RSD requirement defined for non-RedCap UE not supporting Tx Diversity for 5, 10, 15, and 20MHz CBW can be considered as starting point.

- Conclusion

This contribution provides analysis on UE RF requirements for RedCap PC2 UEs for FDD bands, and the following proposal is concluded as follows:

Proposal 1: Remove the “[for TDD bands]” in endorsed draft CR [2] for RedCap PC2 UEs for TDD bands to support RedCap UE PC2 for FDD bands.

Proposal 2: Both HD-FDD and FD-FDD should be supported for RedCap PC2 UEs for FDD bands.

Proposal 3: The existing reference sensitivity for RedCap PC3 for HD-FDD in sub-clause 7.3I of TS 38.101-1 can be applicable for RedCap PC2 for HD-FDD.

Proposal 4: For a RedCap PC2 UE equipped with 2 Rx antenna ports for FD-FDD bands, the RSD requirement defined for non-RedCap UE not supporting Tx Diversity for 5, 10, 15, and 20MHz CBW is applicable.

Proposal 5: For a RedCap PC2 UE equipped with 1 Rx antenna ports for FD-FDD bands, ΔR1R for RedCap PC3 UE equipped with 1 Rx antenna ports for FDD bands and the RSD requirement defined for non-RedCap UE not supporting Tx Diversity for 5, 10, 15, and 20MHz CBW can be considered as starting point.

- References

- RP‑243267, Revised WID on NR power class 2 RedCap (Reduced Capability) UE in FR1, China Telecom, RAN#106

- R4-2420357, (NR_PC2_RedCap_UE) CR to 38.101-1: PC2 Redcap support for TDD bands, Ericsson, RAN4#113