3GPP TSG-RAN WG4 Meeting #114	R4-2501022

# Athens , GR, 17th – 21st Feb, 2025

Title: 	Discussion on conditional Intra-CU LTM for Rel-19 mobility enhancements

Source: 	CATT

Agenda item:	7.27.2.3

Document for:	Discussion

- Introduction

In the last meeting, the WF on RRM requirements for NR_Mob_Ph4_Part2 was approved and some agreements were reached [1], but there still many issues left [2]. This document will further discuss the remaining RRM issues about Conditional LTM/Subsequent Conditional LTM, and give our understanding and proposals.

- Discussion

Sub-topic 1-1: Conditional LTM

- Measurement requirements on conditional LTM

In the last meeting, whether L3 measurement based conditional LTM in FR1 can be supported was discussed in RAN4, and the previous agreements are copied as follows [1]:

RAN4#112-bis: < Agreement > - On whether L3 measurement based conditional LTM in FR1 can be supported - RAN4 can further discuss this issue in FR1 and make recommendation to RAN2 if RAN4 consensus can be reached. - As generic procedure for the objective led by RAN2, if any conclusion will be made in RAN2, RAN4 will follow the RAN2 conclusion unless any technical issue is identified. RAN4#113: < Way forward > FFS the following proposal: - Support L3 measurement based conditional LTM for FR1 from RAN4 perspective. - Note: QC need further check. Continue to discuss in the next meeting.

In RAN2 #128 meeting, RAN2 further discussed whether L3 measurement based conditional LTM can be supported as R18 LTM. RAN2 has agreed to support L3 measurement based conditional LTM and also sent the LS including its agreement to RAN4, and the related agreement is copied as follows:

Agreements on C-LTM - The triggering condition of conditional LTM can be based on L3 measurement.

Observation 1: RAN2 has agreed to support L3 measurement based conditional LTM.

Since in RAN4#112-bis meeting, RAN4 has already agreed that as generic procedure for the objective led by RAN2, if any conclusion will be made in RAN2, RAN4 will follow the RAN2 conclusion unless any technical issue is identified. Therefore, we think that RAN4 need to follow RAN2’s agreement to support L3 measurement based conditional LTM for FR1.

Proposal 1: RAN4 to follow RAN2’s agreement to support L3 measurement based conditional LTM for FR1.

Sub-topic 1-2: (Subsequent)Conditional delay

- The starting point of conditional LTM delay

In the last meeting, the candidate options of the starting point of conditional LTM delay are copied as follows [1]:

< Way forward > FFS the following proposals: - Proposal 1: The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM. - Proposal 2: The starting point is the time when UE transmits an RRCReconfigurationComplete message that is for confirmation of the LTM configuration. - Proposal 3: Similar as event-triggered L1 reporting. The CondEvent is met over the air. - Proposal 4: Wait for more RAN2 agreements.

In our view, the difference between proposal 1 and proposal 2 lies in whether RAN4 defines the delay requirements for conditional LTM and subsequent conditional LTM together. The proponent of proposal 2 clarified that it considers both conditional LTM and subsequent conditional LTM. In our view, if there is still no conclusion on whether to define the delay requirements for conditional LTM and subsequent conditional LTM together at this stage, we should prioritize discussing the delay requirements for conditional LTM here. Therefore, we support proposal 1 to reuse the legacy definition.

Proposal 2: Before RAN4 reaches a conclusion on whether to define the delay requirements for conditional LTM and subsequent conditional LTM together, RAN4 should prioritize discussing the delay requirements for conditional LTM here.

Proposal 3: For the starting point of conditional LTM delay, the legacy definition in R18 LTM can be reused.

- The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM.

The starting point of Subsequent conditional LTM delay

In the last meeting, the Way forward for the starting point of Subsequent conditional LTM delay is copied as below [1]:

< Way forward > FFS the following proposals: - Proposal 1: The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM. - Proposal 2: Wait for RAN2 further progress.

In our view, the similar definition of starting point for subsequent CPAC can be reused for subsequent conditional LTM delay, so the starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM.

Proposal 4: The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM.

- The components of (Subsequent)conditional LTM delay

In the last meeting, the components of conditional LTM delay was still FFS and the Way Forward is copied as follows [3]:

< Way forward > FFS the following proposals: - Proposal 1: DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process. - Proposal 1a: TEarly_process is the time for early candidate TCI State activation/deactivation and early TA acquisition. The time for early TA acquisition could be as the time for UE based TA measurement or PDCCH ordered early TA acquisition further. - Proposal 2: DCLTM = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution. - Proposal 3: DCLTM = TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution. - Proposal 4: DC-LTM = TRRC + TEvent_DU + Tmeasure + TCHO_execution + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. - Proposal 4a: TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU which are defined in R18 LTM cell switch delay can be reused. - Proposal 5: The delay shall include the measurements time (Tmeasure) and the conditional execution preparation time (TCLTM_execution) and interruption time (Tinterrupt) at least. Others are waiting for further progress in other groups. - Proposal 6: TEvent_DU and Tmeasure like components are likely needed, exact components are needed to wait for more RAN2 agreements. - Proposal 7: Wait for RAN2 progress.

In detail, the legacy L3 CHO delay (DCHO) was defined as follows in TS 38.133 [3]:

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

Compared to L3 HO delay requirements, the CHO delay requirement takes into account the delay uncertainty, the measurements time and the conditional execution preparation time in addition to the RRC procedure delay and the interruption time. Therefore, for Rel-19 conditional LTM, we believe that the legacy DCHO can be a baseline, so the conditional LTM delay can be:

DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution.

For the interruption time in conditional LTM Cell Switch delay, the interruption time defined in Rel-18 LTM Cell Switch delay can be a baseline, and RAN4 can further discuss it according to more progress from RAN1/RAN2.

Proposal 5: The legacy requirments for L3 CHO can be a baseline, that is, DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution.

Proposal 6: RAN4 need to wait for RAN1/RAN2 more progress and further study the LTM related features based on the above requirments.

For subsequent conditional LTM delay, it should be discussed based on conditional LTM delay, we believe that at least TRRC will not be considered in subsequent conditional LTM delay.

Proposal 7: At least TRRC will not be considered in subsequent conditional LTM delay.

- Tmeasure

In the last meeting, the Way forward on the components of Tmeasure is copied as below [1]:

< Way forward > FFS the following proposals: - Proposal 1: Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts. - Proposal 1a: - If SSB based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of SSB based L1-RSRP measurement as introduced in R18. - If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of CSI-RS based L1-RSRP measurement which will be introduced in R19. - Proposal 1b: If L3 measurement is used in the event, Tmeasure is same as one measurement period of legacy L3 measurement. - Proposal 1c: FFS on L3 measurement period depending on the conclusion of whether L3 based measurement is considered in R19 C-LTM. - Proposal 2: Wait for more RAN2 progress.

Firstly, the general description in proposal 1 is ok for us, and it is the same with the existing definition. However, the detailed measurement period need to be clarified, and it is related to whether L3 based measurement is considered. Therefore, this issue can further wait for corresponding conclusion. Since RAN2 has agreed to support L3 based measurement, if RAN4 also agree to define the related requirement, then Tmeasure can refer to proposal 1a.

Proposal 8: The following general description in proposal 1 is ok for us:

- Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts.

Observation 2: The detailed measurement period is related to whether L3 based measurement is considered.

Proposal 9: If RAN4 also agree to define the related requirement for L3 measurement based conditional LTM, Tmeasure can refer to proposal 1a.

- TCLTM_execution / TLTM_execution

The Way forward on the components of TCLTM_execution / TLTM_execution in the last meeting is copied as below [1]:

< Way forward > FFS the following proposal: - Proposal 1: TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

We are fine with the description in proposal 1 to reuse the legacy definition in existing CHO delay requirements [3].

Proposal 10: TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

- Tinterrupt

In the last meeting, the Way forward on the components of Tinterrupt is copied as below [1]:

< Way forward > FFS the following proposal: - Proposal 1: Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. - TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. - Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. - TRS-proc is the time for SSB processing. - TLTM-IU is the interruption uncertainty during LTM cell switch.

According to existing Rel-18 LTM Cell Switch delay requirements, the interruption time include the following components [2]:

TLTM-interrupt  = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms,

Since TLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command, so conditional LTM Cell Switch delay will not consider this component. For other components defined in the interruption time for Rel-18 LTM Cell Switch delay (TLTM-processing, Tfirst-RS, TRS-proc and TLTM-IU), the existing requirements can be used as baseline [3].

Proposal 11: For Tinterrupt in Rel-19 conditional LTM Cell Switch delay, excluding TLTM-RRC-processing, other components defined in the interruption time for Rel-18 LTM Cell Switch delay can be a baseline, including TLTM-processing, Tfirst-RS, TRS-proc and TLTM-IU.

Sub-topic 1-3: Early UL sync

- UE based TA measurement

In the last meeting, RAN4 further discussed the early UL sync related issues and the Way Forward and agreement are copied as follows [1]:

For UE based TA measurement: < Way forward > FFS the following proposals: - Define RRM requirements for UE based TA measurement. (CTC, CMCC, vivo, QC, CATT) - Do not define RRM requirements for UE based TA measurement. (Apple, MTK) For PDCCH ordered RACH: < Agreement > - PDCCH ordered RACH is supported for conditional intra-CU LTM from RAN4 perspective. - Note: If no new PDCCH ordered RACH procedure will be introduced in RAN2, the related R18 RAN4 requirements are applicable.

In RAN2 #127bis meeting, the relevant agreements reached are as follows:

Agreements on C-LTM - UE based TA measurement mechanism is supported for conditional intra-CU LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM.

According to the above RAN2’s agreements, both UE based TA measurement mechanism and PDCCH ordered early TA acquisition are supported for conditional intra-CU LTM.

Observation 3: Both UE based TA measurement mechanism and PDCCH ordered early TA acquisition are supported for conditional intra-CU LTM in RAN2.

Although RAN2 also supported UE based TA measurement mechanism in R18 LTM, and RAN4 did not define the related requirements for it. However, compared with R18 LTM, we believe that the UE based TA measurement mechanism in CLTM is more suitable for UE, because UE can start measuring at any time and have more autonomy. When the condition of switching are met and the UE measures the TA in advance, the UE can use the corresponding TA when accessing, and the network can save resources without transmitting PDCCH order.

Therefore, we believe that RAN4 need to define RRM requirements for UE based TA measurement for conditional intra-CU LTM.

Observation 4: Compared with R18 LTM, the UE based TA measurement mechanism in CLTM is more suitable for UE, because UE can start measuring at any time and have more autonomy.

Proposal 12: RAN4 need to define RRM requirements for UE based TA measurement for conditional intra-CU LTM.

- Conclusion

This document discussed preliminary the issues for conditional Intra-CU LTM and presented following proposals and observations:

Observation 1: RAN2 has agreed to support L3 measurement based conditional LTM.

Proposal 1: RAN4 to follow RAN2’s agreement to support L3 measurement based conditional LTM for FR1.

Proposal 2: Before RAN4 reaches a conclusion on whether to define the delay requirements for conditional LTM and subsequent conditional LTM together, RAN4 should prioritize discussing the delay requirements for conditional LTM here.

Proposal 3: For the starting point of conditional LTM delay, the legacy definition in R18 LTM can be reused.

- The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM.

Proposal 4: The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM.

Proposal 5: The legacy requirments for L3 CHO can be a baseline, that is, DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution.

Proposal 6: RAN4 need to wait for RAN1/RAN2 more progress and further study the LTM related features based on the above requirments.

Proposal 7: At least TRRC will not be considered in subsequent conditional LTM delay.

Proposal 8: The following general description in proposal 1 is ok for us:

- Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts.

Observation 2: The detailed measurement period is related to whether L3 based measurement is considered.

Proposal 9: If RAN4 also agree to define the related requirement for L3 measurement based conditional LTM, Tmeasure can refer to proposal 1a.

Proposal 10: TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

Proposal 11: For Tinterrupt in Rel-19 conditional LTM Cell Switch delay, excluding TLTM-RRC-processing, other components defined in the interruption time for Rel-18 LTM Cell Switch delay can be a baseline, including TLTM-processing, Tfirst-RS, TRS-proc and TLTM-IU.

Observation 3: Both UE based TA measurement mechanism and PDCCH ordered early TA acquisition are supported for conditional intra-CU LTM in RAN2.

Observation 4: Compared with R18 LTM, the UE based TA measurement mechanism in CLTM is more suitable for UE, because UE can start measuring at any time and have more autonomy.

Proposal 12: RAN4 need to define RRM requirements for UE based TA measurement for conditional intra-CU LTM.

- Reference

[1]     R4-2420102, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom

[2]     R4-2418286, Topic summary for [113][228] NR_Mob_Ph4_Part2, China Telecom

[3]     3GPP TS 38.133, NR Requirements for support of radio resource management