3GPP TSG-RAN WG4 Meeting #114	R4-2501024

# Athens , GR, 17th – 21st Feb, 2025

Title: 	Discussion on other RRM requirements for Rel-19 NTN phase3

Source: 	CATT

Agenda item:	7.29.4.2

Document for:	Discussion

- Introduction

In the last meeting, the WF on RRM requirements for NR_NTN_Ph3_Part2 was approved [1]. This document will discuss other RRM requirements including SSB periodicity enhancement for NR NTN phase 3 and give our understanding and proposals.

- Discussion

Downlink coverage enhancements

- SSB periodicity enhancement

In the last meeting, according to RAN1’s agreements on RAN4 SSB periodicity, RAN4 firstly focuced on discussing RRM impact on SSB periodicity up to 160ms and the agreements are copied as follows [1]:

Agreement (online): For SSB periodicity up to 160ms: - No further discussion on the impact on initial access from the perspective of RRM requirement - Discuss whether to Modify the default values for SSB and SMTC periodicities for the following if the UE is not provided with SMTC configuration or measurement object on the corresponding frequency by NW - RRC Re-establishment: TSMTC,i assumption - RRC connection release with redirection to NR: Trs assumption - Handover, Conditional handover, RACH-less HO and satellite switch with re-sync: Trs assumption - Further discuss the details on how to update the requirement. - Further discuss the following for SSB periodicity > 20ms: - Cell Reselection: for the SSB-periodicity of 160ms, whether the values for Tmeasure_NR_xxx_enh need to be updated for DRX Cycles equal to 0.32 s and other DRX cycles - Paging interruption in RRC Idle/Inactive: whether/how to adjust the requirements when SSB periods =160 ms in NGSO scenarios - RRC re-establishment: RRC re-establishment requirements for Serving cell SSB Ês/Iot < -8dB - Handover, Conditional handover, RACH-less HO and satellite switch with re-sync - Further discuss whether and how to update the requirement

According to the above, RAN4 just agreed that there will have no further discussion on the impact on initial access from the perspective of RRM requirement. However, the specific RRM impact including whether and how to modify the default values for SSB and SMTC periodicities for the requirements listed in second main bullet and other requirements listed in second main bullet were not be discussed. We will further give our views.

In existing RRM requirements in TS 38.133, some requirements are related to the default values (20ms) for SSB and SMTC periodicities if the UE is not provided with SMTC configuration or measurement object on the corresponding frequency by NW, such as the following [2]:

- RRC Re-establishment: TSMTC,i assumption

For RRC Re-establishment for Satellite Access, the definition of TSMTC,i is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms.

- RRC connection release with redirection to NR: Trs assumption

For RRC connection release with redirection to NR, if the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection then: the requirement in this clause is applied with Trs = 20 ms if the SSB transmission periodicity is not larger than 20 ms; otherwise, there is no requirement if the SSB transmission periodicity is larger than 20ms.

Therefore, the above parameter may require assuming a default value according to SSB transmission periodicity in certain situations. If the default value of SSB periodicity can be up to 160ms, we think that the relevant requirements need to be revised.

Proposal 1: If the UE is not provided with SMTC configuration or measurement object on the corresponding frequency by NW, RAN4 need to consider the RRM impact on the following requirements:

- RRC Re-establishment: TSMTC,i assumption

- RRC connection release with redirection to NR: Trs assumption

In addition, RAN4 should focus on discussing how to modify relevant requirements. Obviously, directly modifying the corresponding parameters to 160ms and defining the relevant requirements for 160ms are two different methods, and the key issue is whether the default SSB transmission periodicity of 20ms will still be used in the R19 NR NTN network.

In our view, 160ms is just an additional default value for SSB transmission periodicity for Rel-19 NR NTN network, which means the existing 20ms value still needs to be considered in Rel-19 NR NTN network. So, we think that it is better for RAN4 to define the relevant requirements for default SSB transmission periodicity of 160ms instead of directly replacing the original parameters, otherwise, according to the following observation from RAN1 #119 meeting, some legacy UEs (i.e. Rel-17 and Rel-18 UEs) will not be able to initially access R19 NTN network.

Observation Backward compatibility for legacy UEs (i.e. Rel-17 and Rel-18 UEs) assuming a default SSB periodicity of 20ms is not guaranteed when SS/PBCH blocks periodicity is larger than 20ms within the cell used for initial frequency scan.

Observation 1: For how to modify the related requirements, the key issue is whether the default SSB transmission periodicity of 20ms will still be used in the R19 NR NTN network.

Observation 2: 160ms is just an additional default value for SSB transmission periodicity for Rel-19 NR NTN network, which means the existing 20ms value still needs to be considered in Rel-19 NR NTN network.

Proposal 2: For how to modify requirements, it is better for RAN4 to additionally define relevant requirements for default SSB transmission periodicity of 160ms instead of directly replacing the original parameters.

In addition, for default SSB periodicity extension, we also support RAN4 to define new RRC re-establishment requirements for the case with serving cell SSB Ês/Iot (dB) < -8. The time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell are defined in the following table:

Table 6.2C.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); Kmulti_SMTC * MAX (800 ms, 10 x TSMTC). < -8; FR1; N/A; k * 800 ms Note1, 3. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: Kmulti_SMTC is defined in clause 9.2C.5.1. Note 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbor cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt..

According to the above, Note 1 in the Table is also related to SSB periodicity of 20ms. In our view, RAN4 need to define new RRC re-establishment requirements for the case with serving cell SSB Ês/Iot (dB) < -8 for SSB periodicity of 160ms and Note 1 should be further discussed.

Proposal 3: RAN4 need to define new RRC re-establishment requirements for the case with serving cell SSB Ês/Iot (dB) < -8 for SSB periodicity of 160ms.

- Conclusion

This document discussed the issues about SSB periodicity enhancement for NR NTN Phase 3 and the following observations and proposals are presented:

Proposal 1: If the UE is not provided with SMTC configuration or measurement object on the corresponding frequency by NW, RAN4 need to consider the RRM impact on the following requirements.

- RRC Re-establishment: TSMTC,i assumption

- RRC connection release with redirection to NR: Trs assumption

Observation 1: For how to modify the related requirements, the key issue is whether the default SSB transmission periodicity of 20ms will still be used in the R19 NR NTN network.

Observation 2: 160ms is just an additional default value for SSB transmission periodicity for Rel-19 NR NTN network, which means the existing 20ms value still needs to be considered in Rel-19 NR NTN network.

Proposal 2: For how to modify requirements, it is better for RAN4 to additionally define relevant requirements for default SSB transmission periodicity of 160ms instead of directly replacing the original parameters.

Proposal 3: RAN4 need to define new RRC re-establishment requirements for the case with serving cell SSB Ês/Iot (dB) < -8 for SSB periodicity of 160ms.

- Reference

- R4-2420105, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)

- 3GPP TS 38.133, NR Requirements for support of radio resource management