3GPP RAN WG4 Meeting #114	R4-2501036

Athens, GR, Feburary 17th – Feburary 21st, 2025

Title: 	Discussion on RRM core requirements for BM

Source: 	CATT

Agenda item:	7.19.3

Document for:	Discussion

Introduction

In RAN4#113 meeting, RAN4 discussed on Rel-19 AI/ML for NR air interface and agreements are captured in [1]. In this contribution, we will continue to discuss the open issues summarized in [2].

Discussion

UE Rx beam knowledge

In last meeting, RAN4 discussed the issue that whether UE knows the Rx beam for the predicted best DL Tx beam and the way forward is reproduced:

RAN4#113 way forward: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions

As we discussed in our paper submitted in last meeting, the prediction is performed based on measurement results of SetB beams. How SetB beams are measured by UE should be known to UE itself, i.e., UE should know the Rx beam that it uses to receive those SetB beams. However, based on RAN4 discussions, we found that the slow progress is caused by the involvement of UE Rx beam sweeping. We will briefly analyse the case when UE Rx beam sweeping is considered in the following contents.

In general, there are two patterns for UE Rx beams: 1) UE receives SetB beams with a fixed Rx beam; 2) UE sweeps its Rx beams. For pattern 1, it is clear that the fixed Rx beam should be used for receiving the predicted DL Tx beam. For pattern 2, though there is not a straightforward way to decide which Rx beam should be used for the predicted DL Tx beam, UE can choose an Rx beam according to the spatial relationship between the predicted DL Tx beam and SetB beams, which depends on UE implementation. Otherwise, the ‘predicted best DL Tx beam’ is meaningless if UE does not which Rx beam should be used to receive it.

In addition, the reliability of choosing UE Rx beam also depends on the training datasets. For examples, if model is trained with a dataset containing RSRP results among all Tx beam and Rx beam pairs, then UE can directly know which Rx beam is the most appropriate beam to be used for the predicted beam. Therefor RAN4 should assume that the UE Rx beam is known for the predicted beam.

But, according to the latest WID [3], this use case is to predict the DL Tx beam prediction:

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

Hence, in order to make test simple, it would be better for UE to fix its Rx beam when measuring SetB beams, in which case UE Rx beam will be known to UE itself for a predicted best DL Tx beam.

Proposal 1: For DL Tx prediction, UE Rx beam should be known to UE itself.

TCI state knowledge

There were some proposals discussing known/unknown TCI state for predicted beams in last meeting but this issue was not discussed. Actually the discussion on this issue can base on the existing known conditions with some modifications to include the AI/ML based inference. The proposed changes are highlighted in red below:

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state or AI/ML based inference to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state, or the RS resources for AI/ML based inference is the RS in SetB or in target TCI state or QCLed to the target TCI state. - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement or inference - The UE has sent at least 1 L1-RSRP report for the target TCI state or completes AI/ML based inference before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3dB Otherwise, the TCI state is unknown.

Proposal 2: Known conditions for TCI state for AI/ML BM can reuse the legacy conditions and modifications are adopted in red:

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state or AI/ML based inference to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state, or the RS resources for AI/ML based inference is the RS in SetB or in target TCI state or QCLed to the target TCI state. - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement or inference - The UE has sent at least 1 L1-RSRP report for the target TCI state or completes AI/ML based inference before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3dB Otherwise, the TCI state is unknown.

Impacts on RRM requirements

In RAN4#112 meeting, a table including potential requirement impacts was provided:

| UE- sided model | Data collection | FFS new clause for data collection for UE-sided model training | FFS RAN4 impact from data collection for UE-sided model training (depending on RAN1/2 input). |
| --- | --- | --- | --- |
|  | Additional assistance information | Added as new side condition: consistency/association shall be guaranteed for inference and perf. monitoring requirement testing | For the testing of UE-sided model inference and monitoring (if any), the consistency of NW-side additional condition across training and inference shall be guaranteed as side condition. |
|  | Inference | New clause for UE-side AI/ML model inference | RAN4 shall specify RRM requirement for UE-side AI/ML model inference: to guarantee (1) accuracy of inference results (2) the delay of reporting inference results (containing the inference latency). |
|  | Perf. monitoring | FFS new clause is needed for Type 1 - Option 1 New clause for Type 1 - Option 2 No RAN4 impact expected for Type 2 | For different types of performance monitoring for UE-sided model, the necessity of RAN4 requirement is provided as: - Type 1, Option 1 (NW-side performance monitoring): The necessity of RAN4 requirement on data collection for monitoring is not significant, because it is similar to data collection for other purposes. - Type 1, Option 2 (UE-assisted performance monitoring): RAN4 requirement on data collection for monitoring can be specified to test the accuracy of performance metrics calculated by UE. - Type 2 (UE-side performance monitoring): No RAN4 requirement is needed because no UE feedback will be performed. |
|  | LCM | New clause for RAN4 RRM requirement for “Network decision, network-initiated” LCM | For UE-sided model LCM, the necessity of RAN4 requirement: - “Network decision, network-initiated” LCM: RAN4 requirement is needed. - FFS the detailed RAN4 requirement (depending on RAN1/2 input). |

Measurement and inference delay requirements

For prediction delay requirements, an agreement was achieved in RAN4#113 as below:

Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS

There are two sub-use cases that are defined in TR 38.843 [4] including spatial-frequency domain prediction and time- spatial-frequency domain prediction. The details are reproduced below:

The following are selected as representative sub-use cases: - BM-Case1: Spatial-domain Downlink beam prediction for Set A of beams based on measurement results of Set B of beams - Consider: Alt. 1): AI/ML model training and inference at NW side. Alt. 2): AI/ML model training and inference at UE side. - Consider: Alt. i): Set A and Set B are different (Set B is NOT a subset of Set A). Alt. ii): Set B is a subset of Set A. Note: Set A is for DL beam prediction. The codebook construction of Set A and Set B can be clarified by companies. - AI/ML model input consider: Alt 1): Only L1-RSRP measurement based on Set B; Alt.2): L1-RSRP measurement based on Set B and assistance information; Alt. 3): CIR based on Set B; Alt. 4): L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID. - BM-Case2: Temporal Downlink beam prediction for Set A of beams based on the historic measurement results of Set B of beams - Consider: Alt. 1): AI/ML model training and inference at NW side. Alt. 2): AI/ML model training and inference at UE side. - Consider: Alt. i): Set A and Set B are different (Set B is NOT a subset of Set A). Alt. ii): Set B is a subset of Set A (Set A and Set B are not the same). Alt. iii): Set A and Set B are the same. - AI/ML model input consider: measurement results of K (K≥1) latest measurement instances with the following alternatives: Alt. 1): Only L1-RSRP measurement based on Set B; Alt 2): L1-RSRP measurement based on Set B and assistance information; Alt. 3): L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID. - F predictions for F future time instances can be obtained based on the output of AI/ML model, where each prediction is for each time instance. At least F=1. Set B is a set of beams whose measurements are taken as inputs of the AI/ML model.

According to the highlighted contents, BM-Case1 can be regarded as a special case of BM-Case2 in terms of delay requirements, which means it is possible to define unified requirements for both BM sub-use cases. Hence, RAN4 can prioritize defining inference delay requirements for BM-Case2 and modifications can be adopted for BM-Case1.

Proposal 3: RAN4 defines unified inference delay requirements for both BM-Case1 and Case2 and discussion is based on BM-Case2.

As other delay requirements defined in TS 38.133, some key parameters that impact the delay requirements should be considered, such as periodicity of measurement resources, periodicity of SMTC/MG and duration of DRX cycles, etc. In additional, the F and K, which are for BM-case2, also affect the inference delay requirements. The details are FFS after RAN1/2’s design on inference is completed.

Proposal 4: Inference delay requirements are defined by considering at least the follow parameters:

- Periodicity of measurement resources.

- Periodicity of SMTC/MG.

- Duration of DRX cycles.

- F and K (For BM-Case 2).

Details are FFS and RAN4 can wait for more RAN1/2 inputs.

Performance monitoring delay requirements

Considering that AI/ML models may be deployed in different entities, i.e., UE side or gNB side, and the controllability may belong to an entity that is different from the one where the AI/ML models are deployed. Therefore different performance monitoring procedures should be considered. The possible scenarios are already captured in TR 38.843 [4]. And for BM-Case1 and BM-Case2 with a NW-side AI/ML model, no AI/ML-specific impacts are identified for now.

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, two types of performance monitoring mechanisms are defined and are generally shown in Figure 2.

Figure 1 Two types of performance monitoring mechanisms for BM-Cases with a UE-side AI/ML model

For Type1 network-based performance monitoring, UE needs to measure the configured measurement resources and report either measurement results or performance monitoring metric/event and implements LCM procedures indicated by gNB. Therefore the time duration between the reception of the configuration/signalling for performance monitoring and UE reporting can be defined as performance monitoring delay.

For Type2 UE-based performance monitoring, indication/request/report from UE to gNB for performance monitoring and reporting may not be needed for all scenarios which are not clear yet. Hence, more discussions are required for Type2 performance monitoring to identify the necessity of defining performance monitoring delay requirements.

Proposal 5: For BM cases with NW-side AI/ML model, no AI/ML-specific RRM requirements are needed.

Proposal 6: For Type1 network-based performance monitoring, the time duration between the reception of the configuration/signalling for performance monitoring and UE reporting can be defined as performance monitoring delay.

Proposal 7: For Type2 UE-based performance monitoring, RAN4 to further discuss the necessity of performance monitoring delay requirements.

Conclusions

This paper discussed of RRM core requirement issues related to AI/ML for beam management, and following proposals are provided:

Proposal 1: For DL Tx prediction, UE Rx beam should be known to UE itself.

Proposal 2: Known conditions for TCI state for AI/ML BM can reuse the legacy conditions and modifications are adopted in red:

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state or AI/ML based inference to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state, or the RS resources for AI/ML based inference is the RS in SetB or in target TCI state or QCLed to the target TCI state. - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement or inference - The UE has sent at least 1 L1-RSRP report for the target TCI state or completes AI/ML based inference before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3dB Otherwise, the TCI state is unknown.

Proposal 3: RAN4 defines unified inference delay requirements for both BM-Case1 and Case2 and discussion is based on BM-Case2.

Proposal 4: Inference delay requirements are defined by considering at least the follow parameters:

- Periodicity of measurement resources.

- Periodicity of SMTC/MG.

- Duration of DRX cycles.

- F and K (For BM-Case 2).

Details are FFS and RAN4 can wait for more RAN1/2 inputs.

Proposal 5: For BM cases with NW-side AI/ML model, no AI/ML-specific RRM requirements are needed.

Proposal 6: For Type1 network-based performance monitoring, the time duration between the reception of the configuration/signalling for performance monitoring and UE reporting can be defined as performance monitoring delay.

Proposal 7: For Type2 UE-based performance monitoring, RAN4 to further discuss the necessity of performance monitoring delay requirements.

References

[1] R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated, RAN4#113.

[2] R4-2418151, Topic summary for [113][129] NR_AI/ML_air, Qualcomm Incorporated, RAN4#113.

[3] RP-243244, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN#106.

[4] TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, V18.0.0.