3GPP RAN WG4 Meeting #114	R4-2501043

Athens, GR, February 17th – February 21st, 2025

Title: 	Discussion on impacts of SBFD on RRM requirements

Source: 	CATT

Agenda item:	7.23.3.2

Document for:	Discussion

Introduction

In RAN4#113 meeting, RAN4 continued discussing the RRM requirements of R19 SBFD feature. Some agreements were achieved and captured in the way forward [1]. In this contribution, we will further discuss the impacts of SBFD on RRM requirements.

Discussion

Requirements for CSI-RS-based L1 measurements

Requirements for CSI-RS based L1 measurements were discussed in last meeting in terms of three aspects:

- CSI-RS colliding with UL transmission

- CSI-RS puncturing in Frequency domain in SBFD symbols

- CSI-RS distributed over SBFD and non-SBFD symbols

Regarding the first and third bullets, a LS [2] was sent to RAN1 for clarification on whether related RAN1 agreements apply for L1 measurements (L1-RSRP/L1-SINR and RLM/BFD/CBD measurements) and RAN4 is still waiting for the reply LS. Therefore we will focus on the second aspect. The related RAN4 agreements are copied below:

- RAN4#113 Agreements (from online session) - For CSI-RS puncturing in frequency domain in SBFD symbols, - For CSI-RS on SBFD symbols, SBFD-aware UE is required to measure only CSI-RS frequency resources within DL usable PRBs. - For CSI-RS on SBFD symbols, existing accuracy requirements for CSI-RS based measurements apply if the number of consecutive CSI-RS PRBs (for the DU configuration) is no less than the existing threshold. - Note: in existing requirements, CSI-RS based RLM/BFD/CBD requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 24 RBs. CSI-RS based L1-RSRP/L1-SINR requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 48 RBs. - FFS for the DUD configuration. - For CSI-RS on SBFD symbols, if the CSI-RS RB bandwidth is at least 48 PRBs but it is split across the two subbands, FFS whether existing accuracy requirements for CSI-RS based measurements can apply and related additional side conditions. - For CSI-RS on SBFD symbols, no accuracy requirements apply if the usable number of CSI-RS PRBs after applying the puncturing operation is below the existing threshold. - FFS the measurement behavior for DUD configuration. - Above bullets apply to L1-RSRP/SINR measurement and also RLM/BFD/CBD measurement.

As highlighted above, there are open issues that need further discussions. First is the UE measurement behaviour for DUD configuration, and second is whether the existing accuracy requirements for CSI-RS based measurements can apply when the CSI-RS RB bandwidth is at least 48 PRBs but it is split across the two subbands.

In our understanding, when two discontinuous DL subbands are configured and one CSI-RS resource is mapped to these two DL subbands, UE shall measure both DL subbands at one time instance and then derive a single wideband result, i.e., these two DL subbands are measured in parallel. But during the discussion in last meeting, another proposal is that two DL subbands are measured sequentially. However, this measurement mechanism will waste half of the CSI-RS bandwidth from the network perspective, and it is not applicable for aperiodic CSI-RS resources.

Proposal 1: When one CSI-RS resource is mapped to two DL subbands, UE shall measure two DL subbands at the same time instance.

For the second issue, after UE finishes measurements on both DL subbands, it needs to derive a single wideband result. However, there will be some performance degradation when UE combines the results over two discontinuous subbands. Considering the existing CSI-RS based measurement accuracy requirements apply when the BW of CSI-RS is not smaller than 48 consecutive PRBs, it is not appropriate to directly reuse the existing accuracy requirements for the above scenario. A margin could be added to the exiting accuracy requirements to mitigate the performance degradation due to measurement combination over two DL subbands. The value of the margin can be further discussed.

Proposal 2: When the CSI-RS bandwidth is at least 48 PRBs but it is split across the two subbands, the exiting accuracy requirements apply with an additional margin. The value of margin can be further discussed.

Requirements for CSI-RS-based L3 measurements

For CSI-RS-based neighbor cell measurements, it is clearer to be discussed in terms of intra-frequency measurements and inter-frequency measurements with two R18 prerequisites, i.e., MG is not needed for intra-frequency measurements and MG is required for inter-frequency measurements.

For measurements with MG, since communication transmissions are not allowed during MG occasions, UE follows this rule and perform measurements during MG occasions.

For measurements without MG, considering RAN1/2 barely achieved agreements related to RRM measurements so far, RAN4 can use the existing measurements as a starting point and further discusses the details pending on RAN1/2 discussion.

Proposal 3: For CSI-RS based L3 inter-frequency measurements, the exiting measurement requirements can be reused.

Proposal 4: For CSI-RS based L3 intra-frequency measurements, use the exiting measurement requirements as a starting point and details can be further discussed pending on RAN1/2 discussion.

Conclusions

This contribution discussed the impacts of SBFD on RRM requirements and some proposals are provided:

Proposal 1: When one CSI-RS resource is mapped to two DL subbands, UE shall measure two DL subbands at the same time instance.

Proposal 2: When the CSI-RS bandwidth is at least 48 PRBs but it is split across the two subbands, the exiting accuracy requirements apply with an additional margin. The value of margin can be further discussed.

Proposal 3: For CSI-RS based L3 inter-frequency measurements, the exiting measurement requirements can be reused.

Proposal 4: For CSI-RS based L3 intra-frequency measurements, use the exiting measurement requirements as a starting point and details can be further discussed pending on RAN1/2 discussion.

References

[1] R4-2420097, WF on RRM requirements for NR_duplex_evo, Huawei, RAN4#113.

[2] R4-2420165, LS on CSI-RS measurement with SBFD operation, MediaTek, RAN4#113.