3GPP TSG-RAN WG4 Meeting #114				                                               R4-2501045

Athens, GR, February 17th – February 21st, 2025

Title:	Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Source:	CATT

Agenda Item:	7.17.2.2

Document for:	Discussion

1 Introduction

In last RAN4 meeting, the enhancement for FR2-1 SSB based measurement delay reduction by optimizing CSSF outside gap in CA/DC was discussed and the conclusions were captured in the approved WF [1].In this paper, we will continue the discussion on open issues of FR2-1 SSB based measurement delay reduction by optimizing CSSF outside gap in CA/DC [2] and give our views.

2 Discussion

Regarding the scenarios/measurement procedure to apply CSSF outside gap optimization, the issue was discussed without agreement achieved in RAN4#113 meeting. The candidate options are reproduced below:

- Option 1 (CATT, Apple, HW, Ericsson): The enhanced CSSF apply to all the 3 aspects and do not define the priority. - Option 2 (Xiaomi, MTK, ZTE): the following aspect to use L3 measurement delay reduction by optimizing CSSF shall be prioritized: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Option 2a (ZTE): To be more concentrated, prioritize the intra-frequency measurement without gap could make the discussion more efficient. But when re-design the CSSF factor, both inter-frequency and inter-RAT measurement without gap should also be considered. - Option 3 (Nokia, Samsung): To start from SSB-based intra-frequency and inter-frequency measurements without MG (aspect 1 and 2) to simplify the CSSF optimization discussion.

In our understanding, all the three aspects should be considered to apply CSSF optimization and no need to further discuss priority, since all the intra-frequency, inter-frequency and inter-RAT measurement requirements refer to the same section and use the same CSSF definition. Once the solutions are decided, the enhanced CSSF can be applied to all the three types of measurements. But some argued discussing all three aspects will lead to an inefficient discussion. In order to make some progresses in this meeting, we are okay to start the discussion from SSB-based intra-frequency and inter-frequency measurements without MG (aspect 1 and 2). After solutions are determined and impacts on CSSF calculation are concluded, the corresponding spec changes can be extended to the inter-RAT SSB measurement without MG.

Proposal 1: RAN4 to start the discussion from SSB-based intra-frequency and inter-frequency measurements without MG and extend the spec impacts to inter-RAT SSB measurement without MG.

In RAN4#112bis meeting, different solutions were raised to enhance CSSF outside gap, including adjusting searcher allocation, reducing unnecessary measurement and allowing additional searcher assumption and others, which can be summarized as the following:

Issue 2-5: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC Candidate options for further discussion: - Option 1: UE can measure one serving CC per band if multiple serving CCs are in the same band. For the CCs to be measurement, further discuss option 1A and option 1B - Option 1A: Reuse the legacy principle for neighboring cell measurement. - Option 1B: Based on network indication - The other CCs (not measured based on network indication) need not to be measured and reported. Network assume the same measurement results on the measured CCs and unmeasured CCs in the same band. - Option 2: UE can change the searcher occupancy ratio of measurement among the measured CCs - Option 2A: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions For 3 searchers: In addition to option 1 and 2 - Option 3: consider to use 3 searchers for CSSF reduction. - Depending on issue 2-4.

After the discussion in last meeting, three options were further paraphrased to align understandings. For solution 1, a new option was provided by merging all sub-options during the meeting:

- Option 2: UE only needs to measure one serving CC per band if multiple serving CCs are in the same band - Network indicates which of the MOs to be measured. The UE is required to measure the indicated CC, i.e. MO, and network can trigger measurement reporting on any of the MOs as in legacy - In case of no NW indication, UE is allowed to determine the CC in the band to be measured, - Reuse the legacy principle for neighbour cell measurement - Measure PCC if PCC in band or - Measure PSCC if PSCC in band or - Measure SCC with neighbor cell MO or - UE implementation

In our understanding, this merged Option 2 aligns with most of companies’ proposals and we are also okay with this new option.

Proposal 2: For solution 1, when NW indication exists, UE follows NW indication to measure one serving CC per band if multiple serving CCs are in the same band and. Otherwise, UE determines the CC in the band to be measured by reusing the legacy principle for neighbour cell measurement.

For solution 2, a new option was also given based on all sub-options:

- Option 2: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions. - NW indicates the ratio between PCC, PSCC and SCC. - Measurement ratio is configurable by NW, the MeasGapSharingScheme in MG can be a start point. - Details is FFS

In general we are okay with this merged option 2 that NW can indicate the searcher occupancy ratio of PCC, PSCC and SCC to UE via signallings. However, this configuration of searcher occupancy ratio, in our opinion, should not be a mandatory implementation for network. Therefore RAN4 needs to clarify the UE behaviour when NW does not indicate this ratio. From our perspective, UE shall follow the legacy searcher sharing rules if measurement ratio is not indicated by NW.

Proposal 3: NW indicates the ratio between PCC, PSCC and SCC. If no NW indication, UE shall follow the legacy searcher sharing rules.

For solution 3 which is based on 3 searchers assumption, it depends on existing UE capability, and no additional UE implementation complexity is required. But according to the discussion so far, it may not apply to all the cases and some conditions (e.g., both FR1 and FR2 measurement objects are configured) are needed. In addition, when UE indicates to NW that it supports 3 searchers, NW can indicate the searcher occupancy ratio by reusing the mechanism of solution 2.

Proposal 4: For the solution based on 3 searchers assumption, define the applicable scenarios (e.g., all the measurements are outside gap and both FR1 and FR2 measurement objects are configured). NW can reuse the mechanism of solution 2 to optionally indicate the searcher occupancy ratio.

For the demand on feature capability, it depends on the specified solutions, e.g., new capability is not needed for option 3 but is needed for other solutions. So we think it can be discussed at the end of core part when the solutions are decided.

Proposal 5: RAN4 to introduce a new individual capability for CSSF reduction in R19 and delay the capability detail discussion to the end of the core part.

3 Conclusion

In this paper, we have some discussions on FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, and the following proposals are given:

Proposal 1: RAN4 to start the discussion from SSB-based intra-frequency and inter-frequency measurements without MG and extend the spec impacts to inter-RAT SSB measurement without MG.

Proposal 2: For solution 1, when NW indication exists, UE follows NW indication to measure one serving CC per band if multiple serving CCs are in the same band and. Otherwise, UE determines the CC in the band to be measured by reusing the legacy principle for neighbour cell measurement.

Proposal 3: NW indicates the ratio between PCC, PSCC and SCC. If no NW indication, UE shall follow the legacy searcher sharing rules.

Proposal 4: For the solution based on 3 searchers assumption, define the applicable scenarios (e.g., all the measurements are outside gap and both FR1 and FR2 measurement objects are configured). NW can reuse the mechanism of solution 2 to optionally indicate the searcher occupancy ratio.

Proposal 5: RAN4 to introduce a new individual capability for CSSF reduction in R19 and delay the capability detail discussion to the end of the core part.

4 References

- R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN4#113.

- R4-2418276, Topic summary for [113][218] NR_RRM_Ph5_Part1, Apple, RAN4#113.