3GPP TSG RAN WG4 Meeting #114		R4-2501074

Athens, Greece, Feb 17th – 21st , 2025

Title: 	Further discussion on Testability for UE RF requirements for LP-WUS

Agenda Item:	7.26.2.4

Source: 	CATT

Document for:	Discussion

# 1 Introduction

Regarding testability for UE RF requirements for LP-WUS, RAN4#113 has made some progress [1]:

Issue 4-1-1: alignment of common understanding on LP-WUS requirements and corresponding testing Agreements: - The same LP-WUS RF requirements for all three RRC states are assumed. But only one RRC state needs to be tested. Issue 4-1-2: General framework of interface between the MR and the LR to verify RF performance Agreements: - Group discuss and conclude the general framework for LP-WUS performance verification, consider the following as starting point: - Option 1: Counter the detection rate without waking up the MR would be enough to verify LP-WUS RF performance. - FFS if the MR should only be woken up when data transfer the system simulator is needed. - Other options are not precluded.

One of the key areas of focus in the next step will be establishing a general framework for the interface between MR and LR, in order to verify RF performance. This includes determining when and under what conditions the MR will be awakened and how the detection rate is adjusted accordingly. In this contribution, we propose that RAN4 agree to test only in the RRC_CONNECTED state and minimize the MR wakeup time during the test.

# 2 Discussion

As agreed in the previous meeting, only one RRC state needs to be tested. Of all three RRC states, there are several benefits if the UE is tested in the RRC_CONNECTED state:

- A dedicated test mode for the UE is unnecessary: test parameters and configurations to UE under test could utilize ordinary message delivering procedure.

- Avoid waking up MR frequently, thus reducing the required testing time: Statistics such as MDR and FAR can be stored during the test and sent via MR to TE by waking up MR at the end of test.

- No need for auxiliary tools

Therefore, we propose to test LP-WUS performance solely in the RRC_CONNECTED state.

Proposal 1: LP-WUS is tested only in RRC_CONNECTED state.

Actually, after the test configuration is completed, and LR is ready for receiving LP-WUS, NR MR can be switched to sleep mode during the test while statistics such as MDR and FAR etc could be countered and buffered. At the end of the test, NR MR can be wakened up and utilized to send packed test statistics to TE.

Proposal 2: MR could be switched to sleep mode and wakened up to send the test statistics to TE at the end of the test.

# 3 Conclusion

In this contribution, we have the following proposals for testability for UE RF requirements for LP-WUS:

Proposal 1: LP-WUS is tested only in RRC_CONNECTED state.

Proposal 2: MR could be switched to sleep mode and wakened up to send the test statistics to TE at the end of the test.

# Reference

[1] R4-2420528, “WF on UE RF requirements for LP-WUS”, vivo