3GPP TSG-RAN WG4 Meeting#114		R4-2501080

Athens, GR, Feb 17th -21st, 2025

Agenda item:	7.29.2.1

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on NTN (e)Redcap UE RF requirements

Document for:	Discussion

Introduction

In RAN4#113 meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follows.

Discussion

PC3 output power for (e)Redcap

Agreement: - FFS whether and how a potential new scheme to increase output power is addressed in specifications - Alt 1:Further confirm whether rel-18 power boost is already applicable for NR NTN including (e)RedCap - If confirmed, no further work is needed. - Alt 2: Power increase by network controlled optional new signalling (e.g capability) - Alt 3: Change in power tolerances - E.g. Upper tolerance exception to UE in specific regions via table note. - FFS whether solution is common for HD-FDD and FD-FDD - If no solution agreed, PC3 tolerance will remain unchanged to +/- 2 dB.

In the previous meeting, the nominal Tx power of PC3 HD-FDD (e) Redcap UE was agreed to be 23dBm. However, whether and how to increase output power still needs to be further discussed. In our understanding, according to practical experience, regarding to the NR HD-FDD Redcap UE and FD-FDD Redcap UE, they share a set of hardware, which means that this UE can support HD-FDD Redcap and FD-FDD Redcap UE. Similar to NR Redcap UE, the NTN Redcap UE may support HD-FDD Recap and FD-FDD Redcap UE, hence, it maybe shouldn’t increase PC3 output power for HD-FDD (e) Redcap UE.

Proposal 1: PC3 power tolerance remains +/- 2dB.

HD-FDD 2Rx sensitivity

According to the agreement, we can start the discussions on the RF requirements for NTN Redcap with HD-FDD. The one important of RF requirements is REFSNES.As we all know, the REFSENS requirements for NR FDD and NR NTN band are band dependent.

We can follow R17 Redcap REFSENS, adopt the same scale for NTN Redcap REFNSENS requirements.

Table1: HD-FDD REFSENS tightening from FD-FDD for NR

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
| n254 | 30 |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
| NOTE: The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |

According to the table1, and Table 7.3.2-1, it can be found that n256 for 2RX HD-FDD Redcap REFSENS could be tightening 0.5 dB compared with 2RX FD-FDD REFSENS, n255 for 2RX HD-FDD Redcap REFSENS could be tightening 0 dB compared with 2RX FD-FDD REFSENS. N254 for 2RX HD-FDD Redcap REFSENS could be tightened by 0.5dB. Based on the amylase above, it is proposed in Table 1: 2RX HD-FDD Redcap REFSENS requirements.

Table 1: NR NTN 2RX HD-FDD Redcap REFSENS requirements

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 | -100.0 | -96.8 | -95.0 |  |  |  |  |  |  |  |
| n254 | 30 |  | -97.1 | -95.1 |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 |  |  |  |  |  |  |  |
| NOTE: The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2: Adopting the table 1 for 2RX NR NTN HD-FDD Redcap REFSENS requirements.

Conclusions

In this paper, we provide our views on on NTN (e)Redcap UE RF requirements, we have made the following

proposals.

Proposal 1: PC3 power tolerance remains +/- 2dB.

Proposal 2: Adopting the table 1 for 2RX NR NTN HD-FDD Redcap REFSENS requirements.

Table 1: NR NTN 2RX HD-FDD Redcap REFSENS requirements

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 | -100.0 | -96.8 | -95.0 |  |  |  |  |  |  |  |
| n254 | 30 |  | -97.1 | -95.1 |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 |  |  |  |  |  |  |  |
| NOTE: The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |

- Reference

- R4-2419897, WF on NR_NTN_Ph3_UERF, Qualcomm INC.TSG-RAN4#113 meeting