3GPP TSG-RAN WG4 Meeting #114 	   R4-2501112

# Athens, Greece, 17th – 21st , Feburary 2025

Agenda Item:	7.17.3

Source: 	Ericsson

Title:	On Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

# Introduction

From RAN #103 meeting the WID for RRM enhancement phase 5 is listed in [1]. The RAN4#113 WF is listed in [2].  In this paper we provide our view for the remaining open issues.

Background: WID objective #2

- Fast SCell activation for UE supporting Rel-18 EMR

- Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting

- Apply fast scell activation in FR1 and FR2-1

- Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

# Discussion

Based on the latest agreement for the open issues from RAN4 113 meeting:

Issue 1-2-4: UE capability

Agreement: • Discuss the UE capability by the end of core part.

The purpose of the Rel-19 RRM enhancement for expedited SCell activation, particularly for UEs supporting Rel-18 eEMR, is to minimize SCell activation delays by utilizing the advanced capabilities introduced in Rel-18.

The endorsed draft CR R4-2420132 has documented the scenarios that have been agreed upon for specification in relation to delay reduction. These scenarios are defined based on the UE capabilities established in Rel-18 eEMR and Rel-16 EMR, which serve as the foundation for outlining the SCell activation requirements in Rel-19.

- UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or, - UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or, - UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode,

The first bullet refers to the Rel-18 UE capability for measurement reporting based on the eEMR configuration. The second bullet highlights the Rel-18 UE capability for supporting cell reselection measurement reporting. The third bullet pertains to the Rel-16 UE capability for measurement reporting based on the EMR measurement configuration.

The scenarios and requirement applicability for the Rel-19 enhancements are well-defined and straightforward. Therefore, we do not see any need to introduce new capabilities that could unnecessarily complicate the requirement specification.

- Proposal 1: RAN4 shall refrain from introducing any new UE capability for Rel-19 fast scell activation for UE supporting Rel-18 eEMR.

# Summary

The following have been proposed in the current contribution:

	Proposal 1: RAN4 shall refrain from introducing any new UE capability for Rel-19 fast scell activation for UE supporting Rel-18 eEMR.

# References

- RP-240830 “New WID: WI resulting from discussion on RRM enhancements”, VIVO, March 2024

- R4-2420094 “RRM pha5 part II WF”, CATT, November 2024