3GPP TSG RAN WG4 Meeting #114		 R4-2501121

Athens, GR, 17th – 21nd February, 2025

Agenda Item:	7.8.2

Source: 	MediaTek Inc.

Title: 	Discussion on low-band CA switching

Document for:	Approval

1. Introduction

The low band carrier aggregation has been continued operators interest. The benefit on low band is mainly on the propagation distance thus the coverage is always better than higher frequencies. It is always challenge to implement low-band CA on handheld UE especially when there are some combinations has almost no frequency gap between UL transmission and DL reception channels. Considering to better utilize the low band fragmented spectrums owned by operator, a new WI was approved in RANP#106 last December[1] to introduce low-band CA with switching. The contribution provide initial discussion on the low-band CA with the new carrier switching feature.

2. Discussion

Considering the propagation distance and the benefit for mobile communication coverage, the low band CA is always operator’s interest. As stated in [2], it is quite challenge for OEM to implement low-low band CA due to the frequency spanning and the front-end component capability when SDL band n29 involves low bands carrier aggregation from single antenna design perspective. But it would be cost up and addition volume in the form factor if implement the carrier aggregation with multiple low-band antennas. The approved WI[1] introduces low band switching to make the low-band CA such as CA_n5-n29 become feasible to ease UE antenna design challenge that brings more flexibility on the low band spectrum utilization.

In our another contribution[3] for the WI scope clarification, we provide some clarification and they are copied below:

Observation 1: For the combos that are existed in the spec, no additional RF requirements would be needed to be specified except switching delay and time mask for carrier switching WI

Proposal 1: WI scope clarification: The WI only targets those completed combos in the existing R-19 or earlier release specs.

Proposal 2: It is expected no additional UE RF requirements based on the WI scope clarification except switching delay and the time mask

With above clarification, if agreed, we can further discuss how to specify switching delay and time mask for carrier switching. First of all, we would assume the reference RF front-end architecture would consider sharing low-band antenna (referring to [2]) as illustrated below as baseline:

Figure 1. RF antenna and front-end architecture assumption

From the reference architecture, we can see it is possible for band n5 and n29 sharing same Rx path in the RF transceiver. The possible configuration changes when band switching for example, n5 switch to n29, happens are listed here:

- RF front-end path change from n5 switched to n29

- Transceiver configuration change from n5 to n29

- Digital baseband and modem channel configuration change from n5 to n29

Since only a semi-static switching is applied, in UE RF switching delay, we may only consider hardware configuration change settling time. In the past, RAN4 had been discussed on the similar case such as Tx switching. The main portion of switching time is the synthesizer settling time when configured change from one band to the other band which is usually around 100us. Plus the lead time of configuring transceiver including synthesizer changing and the lead time of MIPI control bus configuring the RF front-end change, we would propose 140us for the switching delay. To consider different UE implementation, longer switching time may be needed. We would not preclude longer switching time. As for the MRTD and MTTD between the two bands and Timing Advance (TA) of the FDD uplink carrier, they shall be considered in addition to the switching time.

Proposal 1: We propose 140us for DL switching. Other switching time (for example, 35us or 210us reference from Tx switching) are not precluded and can be further discussed in RAN4. As for the MRTD and MTTD between the two bands and Timing Advance (TA) of the FDD uplink carrier, they shall be considered additionally on top of the switching time

The proposed time mask options are illustrated in the figures below in which TA MTTD and MRTD are included in the switching period:

- Option 1: Switching period is located in FDD carrier

Figure 2. Switching time mask for switching period is located in FDD band

- Option 2: Switching period is located in SDL carrier

Figure 3. Switching time mask for switching period is located in SDL band

- Option 3: Switching period always locate at the beginning of “switch-to” carrier

Figure 4. Switching time mask for switching period is located at the beginning of “switch-to” carrier

- Option 4: Switching period always locate at the end of “switch-from” carrier

Figure 5. Switching time mask for switching period is located at the end of “switch-from” carrier

To avoid impact on the PDCCH, we propose option 4, switching period always locate at the end of “switch-from” carrier. Furthermore, from UE control implementation complexity and in the WI scope stated switching pattern shall be based on RRC configuration, we propose the switching pattern is slot-based

Proposal 2: We propose switching period always locate at the end of “switch-from” carrier to avoid impact on the PDCCH. Furthermore, from UE control implementation complexity and in the WI scope stated switching pattern shall be based on RRC configuration, we propose the switching pattern is slot-based switching

3. Conclusion

Proposal 1: We propose 140us for DL switching. Other switching time (for example, 35us or 210us reference from Tx switching) are not precluded and can be further discussed in RAN4. As for the MRTD and MTTD between the two bands and Timing Advance (TA) of the FDD uplink carrier, they shall be considered additionally on top of the switching time

Proposal 2: We propose switching period always locate at the end of “switch-from” carrier to avoid impact on the PDCCH. Furthermore, from UE control implementation complexity and in the WI scope stated switching pattern shall be based on RRC configuration, we propose the switching pattern is slot-based switching

Reference:

- RP-243315 New WID on low band carrier aggregation via switching, RANP#106

- RP-241537 Rx switched nX-n29 low-low carrier aggregation, RANP#104

- R4-2501119 Discussion on scope clarification for low-band CA switching, RAN4#114