# 3GPP TSG-RAN WG4 Meeting #114	R4-2501126

Athens, GR, 17th – 21st February, 2025

Title: 	On RRM requirements of on-demand SSB SCell operation

Source: 	OPPO

Agenda item:	7.25.2.1

Document for:	Discussion

Introduction

In last meeting, a WF was approved for OD-SSB based requirements [1].

In this contribution, we will provide discussion on RRM requirements for On-demand SSB SCell operation.

Discussion

RAN4 has agreed to define requirements for OD-SSB based deactivated Scell measurement and Scell activation. In this paper, we mainly focus on the discussion on OD-SSB based deactivated SCell L3 measurement (Case 1). Most proposals about the fast mode windows below can also apply for case 2.

Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement Agreement: For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF.

About CSSF, whether CSSF outside gap or CSSF within gap applies depends on at least the conditions of gap needed or not, i.e., to be measured OD-SSB is contained in active BWP in frequency domain, and the number of SCells configured with SSB and/or CSI-RS based L3 measurement. However, it is not related to the periodicity of OD-SSB, SMTC or measurement  cycle of SCell in time domain.

For the case of CSSF = 1, which means only single SCell is configured with only SSB based L3 measurement, RAN4 agreed to use OD-SSB periodicity as measurement periodicity. For other cases of CSSF≠1, we think the same logic can be applied. Following the legacy time periods for PSS/SSS detection and time index detection for the deactivated Scell, the measCycleSCell can be replaced by OD-SSB periodicity, but the other impact on Kp and CSSF can be further discussed.

Proposal 1: Following the legacy timer periods for PSS/SSS detection and time index detection for the deactivated Scell, the measCycleSCell can be replaced by OD-SSB periodicity. The other possible impact on Kp and CSSF can be further discussed.

Issue 1-2-2: OD-SSB based deactivated SCell measurement – CSSF in fast mode measurement - Option 1: The same CSSF as legacy - Option 2: - OD-SSB based L3 measurement is prioritized over legacy L3 measurement, if cell identification is needed for OD-SSB-based L3 measurements. - RRM requirements on PSCC and other SCCs are not applicable during the time-window. - Legacy L3 measurement on other SCC are not impacted, if OD-SSB based cell identification is not needed. Other option is not precluded.

For issue 1-2-2, we do NOT support option 2 because it has impact on legacy L1 and L3 measurement. It needs to consider the overlapping between OD-SSB and L3 SMTC/GAP (e.g., in figure 1 sharing factor Kp for legacy L3 SSB), and additionally the overlapping between OD-SSB and L1 SSB occasions which are transmitted outside the gap and SMTC(e.g., in figure 2 sharing factor P for L1 RLM-RS).

Figure 1: sharing factor Kp for legacy L3 SSB based measurement

Figure 2: sharing factor P for L1 RLM-RS based measurement

If cell identification is needed for OD-SSB based L3 measurement, the L3 measurements on PSCC and other SCCs are assumed down-prioritized during the time-window, where OD-SSB based L3 measurement is prioritized. In this case, the sharing factor of Kp for legacy L3 measurement needs to be updated as well, which could have unexpected impact on legacy UE behaviour.

Thus, we support to reuse the legacy CSSF to OD-SSB based deactivated SCell measurement, considering the worst case of fully overlapping between OD-SSB and other legacy SMTC or gap occasions. In this case, the CSSF applies for all OD-SSB occasions regardless of the prioritization due to such overlapping.

Proposal 2: Support to reuse the legacy CSSF to OD-SSB based deactivated SCell measurement, and NOT support the prioritization between OD-SSB and legacy SSB for L3 measurement.

Issue 1-2-3: OD-SSB based deactivated SCell measurement – Procedure Figure 1. OD-SSB based deactivated SCell measurement – Procedure Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation? Agreement: - After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective. - The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism Sub-issue 2: What’s the expected minimum interval(T2, stage 4) between multiple times OD-SSB activation/deactivation(reconfiguration)? Sub-issue 3&4: How to define the time window and whether the time window is configured or pre-defined? Agreement: The time window is based on the number of OD-SSB samples and UE provided one valid report. - The uncertainty time should also be considered, such as T_min, T_processing and T_uncertainy - UE should have enough number of OD-SSB samples to achieve the measurement requirement. - The number of OD-SSB is FFS. - FFS whether UE measurement reporting is needed Sub-issue 5: What’s the UE behaviour when OD-SSB further activation/reconfiguration after T2? FFS.

In last meeting, we agreed the fast measurement window which is triggered by OD-SSB activation, limited based on the number of OD-SSB samples and/or reporting, and followed by slow measurement window to meet the legacy deactivated SCell measurement. OD-SSB is not allowed to be deactivated during the fast mode window in which UE performs the OD-SSB based fast L3 measurement for deactivated SCell, to ensure that UE can have enough number of OD-SSB samples to achieve the measurement requirement during one shot window.

Observation 1: OD-SSB is not allowed to be deactivated during the fast mode window in which UE performs the OD-SSB based fast L3 measurement for deactivated SCell.

From UE side, dynamic OD-SSB on/off transmission may not bring too much network energy saving gain but cause more UE power consumption by frequent RF and baseband turning on/off. If network received the valid report based on measurement in this window, it can decide whether to activate the Scell immediately or deactivate the OD-SSB to save energy. Otherwise, if SCell activation command is coming much later than OD-SSB activation, it does not make too much sense for both network energy saving and UE power saving.

Observation 2: The fast mode window(s) apply before the SCell activation. The network is expected to avoid too long time gap between OD-SSD activation command and SCell activation command.

In case of the time gap between OD-SSB activation command and SCell activation command is too large, e.g., no less than 10s (much larger than measurement period of deactivated SCell), then UE is allowed to perform measurements on deactivated SCell with fulfilling legacy requirements, or stop measurements (e.g., the max gap can be max(5s, 5*DRX cycle)), because UE and network may not believe this deactivated SCell is ready to be activated (e.g., due to bad radio link quality or other implementation issues). From RAN4 requirements perspective, it should restrict the time gap between OD-SSD activation command and SCell activation command to avoid too long waiting time and too much UE power consumption on deactivated SCell measurement.

From our side, we prefer UE to stop the measurement until the next shot of fast mode window if it would happen, which can be configured with the periodic interval or predefined with the minimum interval.

Figure 2. OD-SSB based fast mode measurement windows.

The max gap can come from the reporting validity time in current known SCell definition, e.g., max(5s, 5*DRX cycle). In this case, the Scell can be assumed as known cell if it was activated. Otherwise the Scell can be taken as unknown once entering the activation procedure, which can simplify the RRM requirements as well.

Proposal 3: If SCell activation command is received much later than the maximum gap (e.g., max(5s, 5*DRX cycle)), UE is allowed to stop measurements on deactivated SCell until the next shot of fast mode window.

The interval between each two windows can be configured by RRC together with OD-SSB related parameters, or can be predefined with the minimum value based on reporting validity time for known SCell definition. The T2 in the figure is to restrict the minimum interval between multiple times OD-SSB activation/deactivation(reconfiguration). In some specific case, it can equal to the expected max gap between OD-SSD activation command and SCell activation command.

Figure 3. the expected time gap between OD-SSD activation command and SCell activation command.

If the interval between multiple fast mode windows is predefined, the minimum value can be based on reporting validity time for known SCell definition. We propose:

Option 1: the minimum interval between multiple fast mode windows can follow the validity time window for known SCell definition, e.g., max(5s, 5*DRX cycle).

If the interval between multiple fast mode windows is configured by network, it can be contained in RRC together with OD-SSB related parameters. We propose:

Option 2: it is RRC configuration, which includes at least the length and periodicity of the time windows.

As a supplement of option 2, the starting point of the first windows at UE side is decided by the T_min, T_processing and T_uncertain of first valid OD-SSB reception after the OD-SSB triggering indication. For the other fast window(s), their start points can be decided by the starting point of first window and the periodicity of windows from RRC configuration. In the other words, the next shot of fast mode window may not rely on the OD-SSB activation/deactivation/reconfiguration, but it relies on the OD-SSB transmission and the interval between each two windows.

In addition, we prefer option 2 as below that the fast mode time window(s) is(are) also configured by network because OD-SSB transmission or reconfiguration are also controlled by network. They can be well organized including the starting point, the length and the periodicity of the windows if multiple times of fast mode windows are triggered before the SCell activation command.

Proposal 4: If the interval between multiple fast mode windows is predefined, we propose:

- Option 1: the minimum interval between multiple fast mode windows can follow the validity time window for known SCell definition, e.g., max(5s, 5*DRX cycle).

Proposal 5: If the interval between multiple fast mode windows is configured by network, we propose:

- Option 2: it is RRC configuration, which includes at least the length and periodicity of the time windows.

Proposal 6: If the interval between multiple fast mode windows is configured by network,

- the starting of the first fast window is decided by first valid OD-SSB reception after the OD-SSB triggering indication, and

- the next shot(s) of fast mode window may not rely on the OD-SSB activation/deactivation/reconfiguration but rely on the OD-SSB transmission and the periodic interval between each two windows.

During Scell activation, the similar logic of R18 faster Scell activation can be reused so that OD-SSB periodicity can be used to replace current Trs or Tfirst_SMTC.

After SCell activation, the L3 measurement on normal activated SCell should be unchanged due to UE power saving. Any OD-SSB should be directly taken as deactivated from UE side. For both Case 1 and case 2, we only expect to enhance requirements for SCell activation related procedure/purpose, but not L3 activated or deactivated SCell measurement themselves.

Proposal 7: During Scell activation, OD-SSB periodicity can be used to replace current Trs or Tfirst_SMTC for both case 1 and case 2.

Conclusions

In this paper, we mainly focus on the discussion on OD-SSB based deactivated SCell L3 measurement (Case 1). Most proposals about the fast mode windows below can also apply for case 2.

Proposal 1: Following the legacy timer periods for PSS/SSS detection and time index detection for the deactivated Scell, the measCycleSCell can be replaced by OD-SSB periodicity. The other possible impact on Kp and CSSF can be further discussed.

Proposal 2: Support to reuse the legacy CSSF to OD-SSB based deactivated SCell measurement, and NOT support the prioritization between OD-SSB and legacy SSB for L3 measurement.

Observation 1: OD-SSB is not allowed to be deactivated during the fast mode window in which UE performs the OD-SSB based fast L3 measurement for deactivated SCell.

Observation 2: The fast mode window(s) apply before the SCell activation. The network is expected to avoid too long time gap between OD-SSD activation command and SCell activation command.

Proposal 3: If SCell activation command is received much later than the maximum gap (e.g., max(5s, 5*DRX cycle)), UE is allowed to stop measurements on deactivated SCell until the next shot of fast mode window.

Proposal 4: If the interval between multiple fast mode windows is predefined, we propose:

- Option 1: the minimum interval between multiple fast mode windows can follow the validity time window for known SCell definition, e.g., max(5s, 5*DRX cycle).

Proposal 5: If the interval between multiple fast mode windows is configured by network, we propose:

- Option 2: it is RRC configuration, which includes at least the length and periodicity of the time windows.

Proposal 6: If the interval between multiple fast mode windows is configured by network,

- the starting of the first fast window is decided by first valid OD-SSB reception after the OD-SSB triggering indication, and

- the next shot(s) of fast mode window may not rely on the OD-SSB activation/deactivation/reconfiguration but rely on the OD-SSB transmission and the periodic interval between each two windows.

Proposal 7: During Scell activation, OD-SSB periodicity can be used to replace current Trs or Tfirst_SMTC for both case 1 and case 2.

Reference

- R4-2420098	WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson