3GPP TSG-RAN WG4 Meeting #114	                             R4-2501136

Athens, Greece, 17th – 21st February, 2025

Title: 	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Source: 	OPPO

Agenda item:	7.17.2.1

Document for:	Approval

Introduction

In last meeting, FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor was discussed and the agreements were captured in the approved WF[1].

In this contribution, we will provide our views on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor.

Discussion

In our view, two issues should be discussed for UE power consumption due to FBS with multi-Rx operation: how to define entering and exiting rules of FBS, and how UE can meet the conditions and corresponding requirements.

In last meeting, RAN4 agreed to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement, but not to support UE mobility status based triggering condition in R19.

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF (including activation condition and de-activation conditions): Agreement: - RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement. Agreement: - Not support UE mobility status based triggering condition in Rel-19.

For exiting condition, it was discussed in last meeting with the following options but did not reach any consensus.

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” : For conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”: - Option 2 (LGE, OPPO, Ericsson, MTK, CTC): UE indication to the NW when UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time). - Option 2a (LGE, OPPO, NTT DCM(scenario 1/2/3/4))Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point. - Option 3 (HW, Samsung, Ericsson, MTK, LGE - for scenario 5/7): Apart from the threshold conditions, UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results, since NW already has the candidate target cells for potential HO or SCell modification. - Option 7 (Samsung, Ericsson): Introduce validity timer configuration with certain ranges via dedicated RRC signaling - Option 8 (CMCC, QC, NTT DCM, CATT, ZTE): quitting condition is not needed.

Considering the UE power consumption is an important issue for Multi-Rx operation for L3 measurement, we prefer to have this quit condition. As discussed in the last meeting, if UE stays in Multi-Rx mode for a long time but not meets the handover condition, the power consumption would be high and may cause overheating issue. There are three types of conditions discussed for quitting Multi-Rx based L3 measurement including UAI indication, event trigger and timer based. We understand all the three approaches can achieve the goal but using UE indication is the simplest one and can be aligned with Rel-18 Multi-Rx. For the detailed signaling, we think either the legacy signalling like the overheating indication, or new UAI indication of UE preference is feasible. We can leave it to RAN2 decision.

Proposal 1: UE indication to the NW can be used to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction. The detailed signaling can leave to RAN2 decision.

Issue 1-1-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC Agreement: - For scenario 1/2/3/4 - additional processing time can be added after the last SMTC periodicity for intra/inter-frequency L3 measurement. - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms - where TSSB_processing = 2 ms Agreement: - For scenario 7 - to consider an additional processing delay (2ms) after the last SMTC periodicity [Wayforward] - For scenario 5 - FFS: - Option 1: to consider an additional processing delay (2ms) in the handover delay equation - Option 2: In the handover delay equation, SSB processing time (2ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - Option 3: no need to add additional processing time in the delay requirement.

For SSB processing delay for multiple beams received in a SMTC, it was agreed to add 2ms after the last SMTC periodicity for scenario 1/2/3/4/7 but was FFS for scenario 5.

For Scenario 5, we think it is similar as other scenarios and also prefer to consider the extended SSB processing delay. The solution proposed in option 2 can be used, by scaling the SSB processing time by the number of RX beams that UE employs to simultaneously receive signals from multiple directions, which is defined as M.

Proposal 2: For scenario 5, the additional SSB post-processing delay is M*Tmargin, where Tmargin can be up to 2ms.

Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF Agreement: For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability: - FFS: details of the capability Issue 1-2-2: Delay requirements of L3 measurement delay reduction by optimizing Rx BSF Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2418276.

For the solutions to specify Multi-Rx based L3 measurement delay, it was agreed that the Rx BSF can be reduced according to the UE capability. Then for the agreed scenarios, the measurement requirements can be updated by replacing beam sweeping factor (i.e., 8) with UE capability.

Proposal 3: For scenario1/2/3/4/5/7, the measurement requirements can be updated by replacing existing beam sweeping factor (i.e., 8) with UE capability.

Issue 1-2-3: FFS: requirement at transition between the normal L3 measurement and FBS L3 measurement - Option 1 (CATT, Apple): - Do not define the requirement at transition between the normal L3 measurement and FBS L3 measurement. - Option 2 (Ericsson): Regarding the transition time, - For the case from non-FBS to FBS, RAN4 specify a delay from the time instant when the condition is met. - FBS activation delay shall at the least includes below items. - When the UE receives a RRC message implying FBS the UE shall be ready to start the L3 measurements with FBS operation within DFBS seconds from the end of the last TTI containing the RRC command. DFBS = TRRC + TEvent_FBS + Tmeasure + Tprocessing_FBS - Where: - TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2]. - TEvent_FBS is the delay uncertainty which is the time from when the UE successfully decodes the RRC command until a condition exists at the measurement reference point which will trigger the FBS. - Tprocessing_FBS is for UE processing and preparing FBS operation before UE applying FBS. - For the case from FBS to non-FBS, the necessity of a delay during transition is unclear since the fallback solution isn’t fixed. - Proposal (ZTE): - The requirement at transition between the normal L3 measurement and FBS L3 measurement should be identified. There are two options: - Option 1: Reuse similar solution as the requirements at transition between measurement within gap and measurement outside gap, i.e. the longer delay is the period when transition happens. - Option 2: Once the transition happens, apply the requirements after the transition, no matter it is the normal L3 measurement or the FBS L3 measurement.

One additional issue raised in previous meeting was requirement at transition between the normal L3 measurement and FBS L3 measurement, but there are two different aspects in the above options. One is the applied measurement requirements when transition happens and the other one is the activation delay for entering Multi-Rx mode.

For the first one, there are such requirements defined for the transition between measurements with gaps and without gaps in the existing specification, and the longer delay is used. But we see there may be some differences between the Multi-Rx operation and legacy gap impact. Based on the discussions on trigger condition, whether UE is performing the measurements with or without Multi-Rx operation depends on UE current status, and network is not aware of it and never know whether UE has such transition. So we think it is meaningless to define the transition requirements.

For the second one, i.e., activation delay, we think it is not needed since it is UE implementation.

Proposal 4: Do not define the requirement at transition between the normal L3 measurement and FBS L3 measurement.

Proposal 5: Do not define activation delay when UE transition from non-FBS to FBS.

Conclusions

In this paper, we provided our views on FR2-1 SSB based L3 measurement delay reduction for optimizing BSF. And the following proposals are provided.

Proposal 1: UE indication to the NW can be used to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction. The detailed signaling can leave to RAN2 decision.

Proposal 2: For scenario 5, the additional SSB post-processing delay is M*Tmargin, where Tmargin can be up to 2ms.

Proposal 3: For scenario1/2/3/4/5/7, the measurement requirements can be updated by replacing existing beam sweeping factor (i.e., 8) with UE capability.

Proposal 4: Do not define the requirement at transition between the normal L3 measurement and FBS L3 measurement.

Proposal 5: Do not define activation delay when UE transition from non-FBS to FBS.

Reference

- R4-2420093 WF on RRM requirements for NR_RRM_Ph5_Part1, Apple