3GPP TSG-RAN WG4 Meeting #114				                             R4-2501153

Athens, GR, 17th – 21st February, 2025

Source:	Sony

Title:	AMPR value for NB-IoT in IoT NTN band b254

Agenda item:	5.2

Document for:	Approval

# Background

In RAN4#109, how to handle the ETSI emission requirement in EN 301 441 specifications [1] for IoT NTN in L+S (b254) was further discussed [2]. It has been agreed to capture the emission limit from EN 301 441 as NS_04N and NS_05N respectively for NB-IoT but the A-MPR values are remained TBD.

In this contribution, we share our further analysis on the required AMPR/guardRB that is required for NTN NB-IoT to meet the ETSI emission mask in band B254.

# NS_04N and NS_05N and the Unwanted emission limits in ETSI EN 301 411

In RAN4#109, the within the band emission limit from EN 301 441 as NS_04N and NS_05N, where NS_04N covers the out of the band emission limit and also the within the band emission limit for the frequency range of 1 610  MHz to 1 618.25 MHz, while NS_05N covers the within the band emission limit for the frequency range of 1 618,25 MHz to 1 626,5 MHz.

The “within the band unwanted emission limits” and the “outside band emission unwanted emission limits” in the EN 301 411 specification vs. 3GPP SEM for NB-IoT have been plotted in Fig. 1 and Fig. 2, respectively. The results are scaled to dBm/30kHz. It can be observed that significant discrepancies between ETSI and 3GPP emission requirements exist, and the ETSI requirement is much more stringent than 3GPP requirements at frequency offsets < ±100 kHz.

Fig. 1. The comparison of ETSI maximum unwanted emissions within the band vs. 3GPP SEM for NB-IoT.

Meanwhile, it can also observed that the within the band emission in EN 301 441 represent the worst case senarior for both NS_04N and NS_05N, and therefore, it is proposed to derive the required AMPR and guard RB based on the within the band emission limits.

# A-MPR for NS_05N and NS_04N

## NS_05N

As presented above, the emission requirement in ETSI is extremely challenging (or, if possible, at all) for NB-IoT devices to meet unless sufficiently large A-MPR or guard band/RBs can be allocated.

Meanwhile, to provide a more practical analysis, we have measured PSD of a commercial NB-IoT device with 1/3/6/12 Tones with specified MPR for NTN NB-IoT (TS 36.102), and compared them with the additional emission limit within the band 1618 to 1626.5 MHz as captured in NS_05N. The measurement plots are omitted here, but the corresponding A-MPR values that are needed with about 1 dB implementation margin is presented in Table 1.

In general, it can be concluded that the scenario is more challenging for a smaller number of tone configurations due to the higher PSD and smaller MPRs, especially when they are placed on the edge tone positions. Therefore, it is suggested to define the A-MPR value according to the tone position and tone numbering (similar to MPR) to ensure a sufficient implementation margin while limiting the A-MPR to be applied only in a few scenarios.

Table I A-MPR for NS_05N

| Modulation | QPSK |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for single tone allocation | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| A-MPR | ≤ 11.5 dB |  | TBD |  |  |  |  |  |  |  | ≤ 11. 5dB |  |
| Tone positions for 3 Tones allocation | 0-2 |  |  |  | 3-5 and 6-8 |  |  |  | 9-11 |  |  |  |
| A-MPR | ≤ 9 dB |  |  |  | TBD |  |  |  | ≤ 9 dB |  |  |  |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR | ≤ 6.5 dB |  |  |  |  |  | ≤ 6.5 dB |  |  |  |  |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR | ≤ 3.5 dB |  |  |  |  |  |  |  |  |  |  |  |

## NS_04N

The measurements are carried out for NS_04N as well, For NS_04N, as the within the band emission limit is more relaxed comparing to NS_05N, lower A-MPR values are proposed as in Tab. 2:

Table II A-MPR for NS_04N

| Modulation | QPSK |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for single tone allocation | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| A-MPR | ≤9.5 dB |  | TBD |  |  |  |  |  |  |  | ≤ 9.5 dB |  |
| Tone positions for 3 Tones allocation | 0-2 |  |  |  | 3-5 and 6-8 |  |  |  | 9-11 |  |  |  |
| A-MPR | ≤ 7 dB |  |  |  | TBD |  |  |  | ≤ 7 dB |  |  |  |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR | ≤ 4.5 dB |  |  |  |  |  | ≤ 4.5 dB |  |  |  |  |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR | ≤ 1.5 dB |  |  |  |  |  |  |  |  |  |  |  |

Proposal 1: It is proposed to adopt the A-MPR value in Tab. I and Tab. II for NS_04N and NS_05N if RAN4 decides to specify the A-MPR value at this moment.

Still, it is worth mentioning that the proposed A-MPR values are based on actual devices but not based on the general emission mask in 3GPP. The proposed A-MPR is not sufficient to close the gap between on the general emission mask in 3GPP and the emission limit in ETSI EN 301 441, and it is still possible for a device that is compliant with the 3GPP emission mask requires even larger A-MPR.

Observation 1: The proposed A-MPR is in Tab. I and II are not sufficient to close the gap between the 3GPP general emission mask for NTN NB-IoT and ETSI emission limits, and it is still possible for a device that is compliant with a 3GPP emission mask to require an even larger A-MPR.

# Measurement bandwidth of NS_04N and NS_05N

In addtion to the AMPR value, another issue that is asccoiated with NS_04N and NS_05N is the measurement bandwidth (MBW). Currently, 30 kHz MBW is specified for NS_04N and NS_05N follows the ETSI EN 301 441. However, based on our measurement,  it has been observed that significant measurement error may appears with 30 kHz MBW. For the general 3GPP emission mask of NTN NB-IoT, a smaller measurement BW is allowed to improve the accuracy of measurement.

In addtion, the EN 301 441 also allows smaller measurement BW (3 kHz) for within the band emission limit.

Observation 2: Smaller MBW (smaller than 30 kHz) is allowed for 3GPP general emission limit of NTN NB-IoT and ETSI EN 301 441 to improve the measurement accuracy.

Therefore, for the sake of imrpvoing measurement accuracy while keep alignment with EN 301 441, the following note is suggested to be added at least for in-band requirement of NS_04N and NS_05N:

Note. The measurement bandwidth used may be 3 kHz if the spectrum emission limit are reduced correspondingly.

For the out of band requirement in NS_04N, ETSI does not explicity allow smaller MBW. However, for the sake of improving the measurement accuracy of 3GPP specification, it is proposed that 3GPP should also allow smaller MBW for the out of band requirement in NS_04N.

Proposal 2:  adding the note “The measurement bandwidth used may be 3 kHz if the spectrum emission limit are reduced correspondingly.” to both the in-band requirement and out-of-band requirements in NS_04N and NS_05N.

# Conclusion

In this contribution, we make the following observations and conclusions:

Observation 1: The proposed A-MPR is in Tab. I and II are not sufficient to close the gap between the 3GPP general emission mask for NTN NB-IoT and ETSI emission limits, and it is still possible for a device that is compliant with a 3GPP emission mask to require an even larger A-MPR.

Observation 2: Smaller MBW (smaller than 30 kHz) is allowed for 3GPP general emission limit of NTN NB-IoT and ETSI EN 301 441 to improve the measurement accuracy.

Proposal 1: It is proposed to adopt the A-MPR value in Tab. I and Tab. II for NS_04N and NS_05N if RAN4 decides to specify the A-MPR value at this moment.

Table I A-MPR for NS_05N

| Modulation | QPSK |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for single tone allocation | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| A-MPR | ≤ 11.5 dB |  | TBD |  |  |  |  |  |  |  | ≤ 11. 5dB |  |
| Tone positions for 3 Tones allocation | 0-2 |  |  |  | 3-5 and 6-8 |  |  |  | 9-11 |  |  |  |
| A-MPR | ≤ 9 dB |  |  |  | TBD |  |  |  | ≤ 9 dB |  |  |  |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR | ≤ 6.5 dB |  |  |  |  |  | ≤ 6.5 dB |  |  |  |  |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR | ≤ 3.5 dB |  |  |  |  |  |  |  |  |  |  |  |

Table II A-MPR for NS_04N

| Modulation | QPSK |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for single tone allocation | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| A-MPR | ≤9.5 dB |  | TBD |  |  |  |  |  |  |  | ≤ 9.5 dB |  |
| Tone positions for 3 Tones allocation | 0-2 |  |  |  | 3-5 and 6-8 |  |  |  | 9-11 |  |  |  |
| A-MPR | ≤ 7 dB |  |  |  | TBD |  |  |  | ≤ 7 dB |  |  |  |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR | ≤ 4.5 dB |  |  |  |  |  | ≤ 4.5 dB |  |  |  |  |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR | ≤ 1.5 dB |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2:  adding the note “The measurement bandwidth used may be 3 kHz if the spectrum emission limit are reduced correspondingly.” to both the in-band requirement and out-of-band requirements in NS_04N and NS_05N.

# References

- ETSI EN 301 441 V2.1.1 (2016-06): “Satellite Earth Stations and Systems (SES); Harmonised Standard for Mobile Earth Stations (MES), including handheld earth stations, for Satellite Personal Communications Networks (S-PCN) operating in the 1,6 GHz/2,4 GHz frequency band under the Mobile Satellite Service (MSS) covering the essential requirements of article 3.2 of the Directive 2014/53/EU”

- R4-2321817	WF on IoT_NTN_FDD_LS_band	MediaTek