3GPP TSG-RAN WG4 Meeting #114	                                                                                            R4-2501161

Athens, GR, 17 Feb –21 Feb, 2025

Source: 	Xiaomi

Title: 	Discussion on PC1.5 PCMAX for intra-band NC CA with dual PA

Agenda item:	7.1.1.1.2

Document for:	Discussion

- Introduction

In last RAN4 meeting, how to define Pcmax for PC1.5 intra-band NC CA with dual PA was discussed. The agreement is that PCMAX,C is upper bounded by 26dBm. How to implement the specification is not concluded. We provide our views on this in this contribution.

- Discussion

For the agreement for PCMAX,C, our understanding is that no new item is needed in the specification. The agreement is that upper bound of PCMAX,C is 26 dBm which is 3 dB less than the Ppowerclass,CA, so the spec can just clarify that PCMAX,C ≤ Ppowerclass,CA -3 which is similar with what was proposed in proposal 4 in the WF. The proposal 4 needs some improvement that the upper bound of 26 dBm is on top of the PCMAX,C defined in 6.2.4. The following is a candidate implementation.

===================================

6.2A.4.1.2	Configured transmitted power for Intra-band non-contiguous CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in subclause 6.2.4. For power class 1.5 UE when the signalling is indicated for dualPA-Architecture IE, PCMAX,c is no greater than PPowerClass,CA-3.

The configured maximum output power PCMAX,c on serving cell c shall be set as specified in subclause 6.2.4, but with MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR as determined by subclause 6.2A.2 and 6.2A.3, respectively. For PH reporting the following exception applies: if the UE is configured with multiple uplink serving cells, the powerPCMAX,c  used for the purpose of PH reporting on first serving cell c = c1 does not consider for computation of the PH report transmissions on a second serving cell c2 as exempted  in subclause 7.7.1 in [8]. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR.

===================================

For the Pcmax part, we didn’t see the necessity to change PPowerClass,CA, because it’s the UE capability of the maximum output power. For PC1.5 UE, it’s 29 dBm, there’s no the other understanding. But to continue the discussion, for the following part in the specification, the MPRc and A-MPRc may need some discussion. If MPR is not the same for each CC, some clarification may be needed.

====================================

For uplink intra-band non-contiguous carrier aggregation when same slot pattern is used in all aggregated serving cells,

PCMAX_L  = MIN{10 log10 ∑ pEMAX,c  - TC , PEMAX,CA,(PPowerClass,CA– ΔPPowerClass,CA) – MAX(MAX(MPRc, A-MPRc) + ΔTIB,c + TC + DTRxSRS, P-MPR ) }

PCMAX_H  = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA ,PPowerClass,CA– ΔPPowerClass,CA)}

=====================================

Further more, the power control of intra-band NC CA with dual PA is very similar with interband UL CA. So our preliminary consideration is to mostly follow inter-band UL CA approach. Details can be further discussed when more evaluation for MPR is provided.

Observation 1: No necessity to modify PPowerClass,CA for PC1.5 intra-band NC CA with dual PA.

Observation 2: The power control of intra-band NC CA with dual PA is very similar with interband UL CA. The Pcmax spec can consider to follow inter-band UL CA approach.

- Conclusion

This contribution provides our view on PC1.5 PCMAX for intra-band NC CA with dual PA. We provide the following candidate to implement the agreement that PCMAX,C is upper bounded by 26dBm.

===================================

6.2A.4.1.2	Configured transmitted power for Intra-band non-contiguous CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in subclause 6.2.4. For power class 1.5 UE when the signalling is indicated for dualPA-Architecture IE, PCMAX,c is no greater than PPowerClass,CA-3.

The configured maximum output power PCMAX,c on serving cell c shall be set as specified in subclause 6.2.4, but with MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR as determined by subclause 6.2A.2 and 6.2A.3, respectively. For PH reporting the following exception applies: if the UE is configured with multiple uplink serving cells, the powerPCMAX,c  used for the purpose of PH reporting on first serving cell c = c1 does not consider for computation of the PH report transmissions on a second serving cell c2 as exempted  in subclause 7.7.1 in [8]. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR.

===================================

For Pcmax, we provide some preliminary analysis that we have the following observations,

Observation 1: No necessity to modify PPowerClass,CA for PC1.5 intra-band NC CA with dual PA.

Observation 2: The power control of intra-band NC CA with dual PA is very similar with interband UL CA. The Pcmax spec can consider to follow inter-band UL CA approach.

- References

[1] R4-2420329, “WF on HPUE for CA in TN”, RAN4#113, Samsung