# 3GPP TSG-RAN WG4 Meeting #114                                R4-2501175

# Athens, Greece, 17th – 21st February, 2025

Source:	ZTE Corporation

Title:	Discussion on Ku bands for NR NTN

Agenda item:	7.11.6

Document for:	Approval

Introduction

- At RAN #106 meeting the revised WI “Introduction of Ku bands for NR NTN” [1] was approved.  In the WID, Ku bands have been introduced for NR NTN with the mobile VSAT UE. In #114 meeting we will specify the scenarios and general view on the topics.

Discussion

  - 2.1 NR-NTN deployment on Ku bands

In current WID description, RAN4’s work could be seen as below:

The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate Tx/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services. - Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements - Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only. - RAN#107 checkpoint: Depending on the feedback from RAN1 and RAN2 on the LS in R4-2419902, further WID updates may be needed to address feedback on specifying bands for both FR1-NTN numerology and FR2-NTN numerology or for other issues such as support for a single SCS per frequency band.

2.1.1 General aspects

- In R17, NTN RRM requirements are defined based on band n255 and n256, both in FR1.

Table 1: NTN satellite bands in FR1

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| NOTE: NTN satellite bands are numbered in descending order from n256. |  |  |  |

- In R18, RAN4 has already studied the NTN RRM requirements based on band512, n511 and n510, both in FR2.

Table 2: NTN satellite bands in FR2

| NTN operating band | UL Earth-to-Space | DL Space-to-Earth |
| --- | --- | --- |
| n5121 | 27.5 - 30.0 GHz | 17.3 - 20.2 GHz |
| n5112 | 28.35 - 30.0 GHz | 17.3 - 20.2 GHz |
| n5103 | 27.5 - 28.35 GHz | 17.3 - 20.2 GHz |

- In R19, Ku band has introduced in RAN meeting for NR NTN, and there are two kinds of Ku band in WID, the following could be seen:

Table 3: Ku Band Downlink and Uplink Spectrum for ITU Regions 1 and 3

| Ku band for GSO and Non GSO Space Segment | Downlink (Space to Earth) | 10.70 - 12.75 GHz |
| --- | --- | --- |
|  | Uplink (Earth to Space) | 12.75-13.25 GHz & 13.75-14.5 GHz |

Table 4: Ku Band Downlink and Uplink Spectrum for ITU Region 2

| Ku band for GSO and Non GSO Space Segment | Downlink (Space to Earth) | 10.70 - 12.70 GHz |
| --- | --- | --- |
|  | Uplink (Earth to Space) | 12.70-13.25 GHz & 13.75-14.5 GHz |

- Based on the description of the WID, we know that RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only. RAN4 shall not exclude any of them.

- In 38.101-5 there are 5 types of VSAT UE:

Table 5: The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 52 | Mobile VSAT communicating with GSO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. |  |  |

- However, only mobile VSAT UE in GSO and NGSO shall be considered and fixed VSAT UE is not in the scope of this topic. To my understanding, the following four cases will be introduced:

- Case 1: mobile VSAT UE served by GSO in FR1

- Case 2: mobile VSAT UE served by NGSO in FR1

- Case 3: mobile VSAT UE served by GSO in FR2

- Case 4: mobile VSAT UE served by NGSO in FR2

- Another point is SCS, RAN4 considers SCS=15kHz and 30kHz in R17 and SCS=60kHz and 120kHz in FR2. For Ku band, RAN4 shall only consider SCS=30kHz in FR1 and 120kHz in FR2 especially in defining timing requirements.

Proposal 1: FR1 and FR2 shall be separately considered for Ku band in NR NTN.

Proposal 2: Only mobile VSAT UE shall be studied and fixed VSAT UE is precluded.

Proposal 3: For SCS, RAN4 shall only consider SCS=30kHz in FR1 and 120kHz in FR2.

- Proposal 4: There are four cases shall be considered:

- Case 1: mobile VSAT UE served by GSO in FR1

- Case 2: mobile VSAT UE served by NGSO in FR1

- Case 3: mobile VSAT UE served by GSO in FR2

- Case 4: mobile VSAT UE served by NGSO in FR2

2.1.2 RRM impact

- First of all, the potential RRM requirements for Ku band could be studied as below:

Table 6 Summary of RRM requirements which are possibly impacted by NTN operation in the NTN-Ku bands

| Requirements |  |
| --- | --- |
| Cell reselection | UE measurement capability |
|  | Measurements on intra/inter-frequency NR cells |
|  | ﻿Maximum interruption in paging reception |
|  | ﻿Measurements of intra/inter-frequency NR cells for UE configured with relaxed measurement criterion |
| Handover | Handover |
|  | NR conditional handover |
|  | ﻿RRC Connection Mobility Control: RRC re-establishment |
| Timing | UE transmit timing |
|  | UE timer accuracy |
|  | Timing advance |
| Signaling | RLM |
|  | BWP switch |
|  | TCI state switch |
| Measurement procedure | UE Measurement capability |
|  | Intra/inter-frequency measurements |
|  | L1-RSRP measurements |

- To my understanding, R17 NR NTN related requirements shall be reused or as baseline when we define the FR1-NTN requirements for Ku bands. On the contrary, R18 NR NTN related requirements shall be reused or as baseline when we define the FR2-NTN requirements for Ku bands including timing requirements served by NGSO.

- Proposal 5: The potential RRM requirements could be seen as below:

| Requirements |  |
| --- | --- |
| Cell reselection | UE measurement capability |
|  | Measurements on intra/inter-frequency NR cells |
|  | ﻿Maximum interruption in paging reception |
|  | ﻿Measurements of intra/inter-frequency NR cells for UE configured with relaxed measurement criterion |
| Handover | Handover |
|  | NR conditional handover |
|  | ﻿RRC Connection Mobility Control: RRC re-establishment |
| Timing | UE transmit timing |
|  | UE timer accuracy |
|  | Timing advance |
| Signaling | RLM |
|  | BWP switch |
|  | TCI state switch |
| Measurement procedure | UE Measurement capability |
|  | Intra/inter-frequency measurements |
|  | L1-RSRP measurements |

- Proposal 6: R17 NR NTN related requirements shall be reused or as baseline when we define the FR1-NTN requirements for Ku bands. On the contrary, R18 NR NTN related requirements shall be reused or as baseline when we define the FR2-NTN requirements for Ku bands.

Conclusion

In this paper we provided our views on RRM requirements on R19 NR NTN enhancement.

Proposal 1: FR1 and FR2 shall be both considered for Ku band in NR NTN.

Proposal 2: Only mobile VSAT UE shall be studied and fixed VSAT UE is precluded.

Proposal 3: For SCS, RAN4 shall only consider SCS=30kHz in FR1 and 120kHz in FR2.

- Proposal 4: There are four cases shall be considered:

- Case 1: mobile VSAT UE served by GSO in FR1

- Case 2: mobile VSAT UE served by NGSO in FR1

- Case 3: mobile VSAT UE served by GSO in FR2

- Case 4: mobile VSAT UE served by NGSO in FR2

- Proposal 5: The potential RRM requirements could be seen as below:

| Requirements |  |
| --- | --- |
| Cell reselection | UE measurement capability |
|  | Measurements on intra/inter-frequency NR cells |
|  | ﻿Maximum interruption in paging reception |
|  | ﻿Measurements of intra/inter-frequency NR cells for UE configured with relaxed measurement criterion |
| Handover | Handover |
|  | NR conditional handover |
|  | ﻿RRC Connection Mobility Control: RRC re-establishment |
| Timing | UE transmit timing |
|  | UE timer accuracy |
|  | Timing advance |
| Signaling | RLM |
|  | BWP switch |
|  | TCI state switch |
| Measurement procedure | UE Measurement capability |
|  | Intra/inter-frequency measurements |
|  | L1-RSRP measurements |

- Proposal 6: R17 NR NTN related requirements shall be reused or as baseline when we define the FR1-NTN requirements for Ku bands. On the contrary, R18 NR NTN related requirements shall be reused or as baseline when we define the FR2-NTN requirements for Ku bands.

- References

[1] RP-243314, Revised WID: Introduction of Ku bands for NR NTN.