3GPP TSG-RAN WG4 Meeting #114	R4-2501220

Athens, GR, 17th – 21st February, 2025

Agenda item:  7.25.2.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 NES OD-SSB

Document for:	Discussion

# Introduction

For Rel-19 NES, RAN4 discuss the RRM impacts of OD-SSB with issues and agreement captured in [1]. In this paper, we provide our views on remaining issues with further progress in other WGs considered.

# Discussion

Scenarios

In Oct RAN4 meeting, RAN4 send a LS [2] to RAN1 for clarification on time/frequency/spatial domain relation between OD-SSB and AO-SSB for case#2. RAN1 discussed the question asked in the LS and provide answers as follows:

Reply to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?): - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. - The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB. - RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB. Reply to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?): - RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst. - RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB. Reply to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?): - The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB. Reply to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?): - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

Besides the LS reply, RAN1 also achieved following agreements in last meeting:

Agreement - For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2. - For Case #1 (i.e., No always-on SSB on the cell), - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), down-select one of the following alternatives. - Alt A: Same as for Case #1 - Alt B: Based on a single parameter which is to indicate the time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) Agreement Down-select at least one of the following alternatives. - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - Alt 2: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt 3: Do not support the case where always-on SSB is CD-SSB on a synchronization raster. Down-select at least one of the following alternatives. - Alt A: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB can be same or different from the frequency location of always-on SSB, subject to its configuration. - Alt B: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster.

Based on above RAN1 progress, we can have following observations for further scenario clarification.

Observation 1: For time domain, for case#1, UE can determine the SFN and half frame of OD-SSB burst by SFN offset and half frame index; for case#2, it is FFS whether the same method as case#1 can apply or based on one single time offset between AO-SSB and OD-SSB.

Observation 2: For frequency domain, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB and FFS when the AO-SSB is CD-SSB.

Observation 3: For QCL relation, SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are QCLed. FFS whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

In following part of the contribution, above observations will be considered.

## OD-SSB processing time.

In last RAN4 meeting, RAN4 discussed and confirmed RAN1 working assumptions on OD-SSB processing timeline with following agreements:

Agreement: - RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1 from network transmission perspective. - Note 1: This does not imply UE shall be able to process/prepare the OD-SSB from time instance A from RAN4 requirement perspective. - Meanwhile, RAN4 is discussing the additional UE processing/preparation time for OD-SSB reception for RAN4 requirement.

From network point of views, what really matters is the time that UE is ready to processing OD-SSB. Otherwise, some of the SSB will be wasted, and there could be mismatch between UE and gNB. For additional processing time, we are not fully convinced on the necessary of having them. As pointed out at very beginning, in legacy SCell activation requirements, it is possible that the first SSB comes right after Tharq+3ms, and there is no additional processing time allowed. As explained by companies, one possible case is that UE may need to re-schedule the searcher in deactivated SCell measurement. Thus, one compromised solution is to only consider the additional processing time for deactivated SCell measurement.

Proposal 1: No additional UE processing/preparation for OD-SSB for SCell activation (i.e. Scenario 2A).

Proposal 2: RAN4 to discuss the value of additional UE processing/preparation for OD-SSB only for deactivated SCell L3 measurement (i.e. Scenario 2).

## Deactivated SCell L3 measurement (case 1)

For deactivated SCell, we start from case#1, and some issues are common for both case#1 and case#2. RAN4 achieved following agreements in last RAN4 meeting:

Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement Agreement: For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF. Issue 1-2-3: OD-SSB based deactivated SCell measurement – Procedure Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation? Agreement: - After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective. - The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism Sub-issue 2: What’s the expected minimum interval(T2, stage 4) between multiple times OD-SSB activation/deactivation(reconfiguration)? Sub-issue 3&4: How to define the time window and whether the time window is configured or pre-defined? Agreement: The time window is based on the number of OD-SSB samples and UE provided one valid report. - The uncertainty time should also be considered, such as T_min, T_processing and T_uncertainy - UE should have enough number of OD-SSB samples to achieve the measurement requirement. - The number of OD-SSB is FFS. - FFS whether UE measurement reporting is needed Sub-issue 5: What’s the UE behaviour when OD-SSB further activation/reconfiguration after T2? FFS.

For measurement period in fast mode, it is agreed that UE shall perform measurement based on OD-SSB periodicity regardless of measCycleSCell and DRX cycle. There is two FFS points. One is whether to have a lower bound. Companies commented that lower bound is included in legacy requirements. We can understand the motivation for UE power saving. However, the situation here is not exactly the same. Firstly, legacy L3 measurement is periodically happening. For instance, the lower bound is 600/200ms for FR1 L3 intra-frequency measurement. Without the lower bound, if the SMTC is configured as 20ms, it means the measurement period is 100ms. However, back to OD-SSB scenario, UE is only required to perform fast measurement when OD-SSB is triggered. It is not realistic that gNB trigger OD-SSB every 100 ms. If a lower bound is defined, it is meaningless to configure an SSB with shorter periodicity. For instance, if the lower bound is defined as 400ms for FR1, it means there is no difference from gNB point of view to trigger a SSB with periodicity smaller than 80ms.

Observation 4: Fast measurement only happens every time OD-SSB is triggered, and it is not that frequent compared with legacy lower bound of measurement period.

Based on above analysis, there is no need to define lower bound for fast mode measurement.

Proposal 3: No need to define lower bound for fast mode measurement.

Alternatively, for power saving purpose, RAN4 shall define RAN4 shall define minimum interval between fast mode measurement for deactivated SCell measurement, which is T2 in the figure.

Proposal 4: RAN4 shall define minimum interval between fast mode measurement for deactivated SCell measurement, which can take exiting measurement period when MeasCycle =160 ms as reference. UE is not required to perform “fast measurement” more frequently than the minimum interval.

Another FFS point is the case when CSSF is not equal to 1. One straightforward approach is to scale the measurement delay by CSSF in the fast measurement mode. However, for instance, if CSSF is equal to 2, it means the measurement delay is doubled and to guarantee a valid measurement, the numbers of OD-SSB burst that gNB needs to transmit is doubled compared with what is really needed for measurement, which goes against with the NW energy saving purpose.

Observation 5: If the measurement delay is scaled by CSSF, it means the number of OD-SSB burst that gNB needs to transmit is multiplied, which is against the power saving purpose.

Thus, we think RAN4 shall discussed whether to prioritize the measurement for deactivated SCell, for which the OD-SSB is triggered.

Proposal 5: RAN4 to discussed whether to prioritize the measurement for deactivated SCell, for which the OD-SSB is triggered.

Another issue is about the OD-SSB deactivated method. Companies commented to restrict the scenario that once OD-SSB is triggered, it can only be deactivated until SCell deactivated. The argument is that one of the options for deactivation is still under discussed. The agreements in RAN1#118b and 119 are shown as follows. The above statement only valid provide that ONLY option 4 is supported by RAN1. However, based on latest agreements in RAN1#119, explicit deactivation indication and indication of number of N OD-SSB burst is supported by RAN1. Option 4 is FFS for additional consideration.

Agreement 118b For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options Agreement 119 For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer

Observation 5: For OD-SSB deactivation, explicit deactivation indication and indication of number of N OD-SSB burst are supported by RAN1.

Besides, the restriction to one-time OD-SSB activation will severely limit the usage and energy saving gain of these feature.

Proposal 6: Per RAN1 agreements on OD-SSB deactivation, no needs to restrict the activation and deactivation of OD-SSB in RAN4, which will severely limit the usage and energy saving gain of the feature.

Thus, based on above analysis, we would like to explicitly clarify that following agreements applies for each time OD-SSB is activated.

Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1) Agreements: Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. - After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 2-1: fallback legacy requirement - Option 2-2: stop measurement - Option 2-3: No requirement - Time window means the time duration starting from receiving OD-SSB activation indication - FFS the time window configuration or pre-defined - FFS the fast L3 measurement requirement - FFS the relation between time window and OD-SSB deactivation command in the time domain When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell. Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement Agreement: For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF.

Proposal 7: The above agreements for OD-SSB based deactivated SCell measurement applies for each time OD-SSB is activated.

The another issue the how many OD-SSB burst is needed to define the fast measurement window. In other words, gNB needs know how many OD-SSB needs to be transmitted. As proposed by other companies, UE can trigger the measurement report if there is valid measurement report and UE can stop fast measurement. We think it is beneficial to have UE measurement report when OD-SSB is triggered for following reasons:

- gNB can deactivate OD-SSB bursts earlier if gNB receives measurement report from UE for NW energy saving.

- gNB can get fresh measurement results when OD-SSB is triggered.

The default value of numbers of OD-SSB burst for fast measurement is the number of samples as required in current requirements. For further NW energy saving gains, we think it is beneficial to indicate number of needed OD-SSB bursts thus gNB can trigger OD-SSB “On-demand” instead of the number of OD-SSB as required for minimum requirements.

Proposal 8: The default value of numbers of OD-SSB burst for fast measurement is the number of samples as required in current requirements. In addition, UE to indicate number of OD-SSB bursts needed as UE capability.

Proposal 9: RAN4 send LS to RAN2 to introduce L3 report procedure when UE obtain valid measurement results on OD-SSB after OD-SSB is triggered. UE can stop fast measurement when UE provide valid report of deactivated SCell

Another issue which has never been touched is about neighbor measurement. The discussion so far is only for deactivated SCell measurement. However, based on current agreements, it results in UE have different behavior for serving cell measurement and neighbor cell measurement in terms of measurement period/pattern for same frequency layer. If UE perform measurement of neighbor cell as legacy and UE perform measurement on deactivated SCell based on OD-SSB, it will anyway cause additional power consumption compared since there is additional measurement occasions.

Observation 6: Per current agreement on deactivated SCell measurement on OD-SSSB, UE may have different measurement behavior for deactivated SCell and neighbor cell on the same frequency in terms of period/pattern, and it will cause additional UE power consumptions.

Based on the observation 6, RAN4 to discuss whether and how to perform neighbor cell measurement when deactivated SCell configured with OD-SSB.

Proposal 10: RAN4 to discuss whether and how to perform neighbor cell measurement when deactivated SCell configured with OD-SSB.

## Deactivated SCell L3 measurement (case 2)

For deactivated SCell L3 measurement in case 2, RAN4 achieved following agreements in last meeting:

Issue 1-3-1: Scenario 1 - OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered - Option 1: UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 1-1: The measurement periodicity is OD-SSB periodicity - Option 1-2: The measurement periodicity is based on combining OD-SSB and AO-SSB - Option 2: UE always follow legacy requirements based on measCycleSCell - Option 3: The same approach in Case 1 can be reused if the serving cell is unknown. UE follows legacy requirements based on measCycleSCell if the serving cell is already known.

From our understanding, the only difference between case 1 and case 2 is that UE can only rely on AO-SSB, and the measurement behavior can be exactly the same as legacy case. However, if the legacy requirements apply, there is no need for gNB to trigger OD-SSB.

Observation 7: If UE always follow legacy requirements based on measCycleSCell for case 2, it is meaningless for gNB to trigger OD-SSB.

Proposal 11: For case 2, RAN4 specify same UE measurement behavior/requirements as case 1. FFS whether to combine OD-SSB and AO-SSB, which is a common issue for case 2.

## OD-SSB based SCell activation

For OD-SSB based SCell activation, RAN4 achieved following agreements in last meeting.

Issue 1-4-1: The known cell condition – measurement reporting Agreement: - In case 1, known/unknown condition should be defined based on only OD-SSB. - In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB. - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue. - Note 2: the agreement is valid at least for the scenario with the same frequency for always-on SSB and OD-SSB. - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure. Issue 1-4-5: SCell activation requirement – OD-SSB periodicity Agreement: - In Case 1, the SCell activation delay requirement is defined based on OD-SSB periodicity. - Existing SCell activation delay requirement shall apply based on OD-SSB periodicity. - In Case 2, RAN4 to define the SCell activation delay requirement based on OD-SSB periodicity if the time instance receiving OD-SSB activation indication is no later than the time instance when receiving the SCell activation command. Otherwise, RAN4 to define the requirement based on always-on SSB for SCell activation. - This agreement is valid under the assumption of the same spatial relation for OD-SSB and always-on SSB [with the same SSB index], and this can be revisited based on RAN1 LS reply. - Note: T_firstSSB_max for intra-band co-located scenario is discussed in another issue.

For OD-SSB based SCell activation, the status is pretty much stable. One of the issues is whether to combine OD-SSB and AO-SSB for case 2. As I mentioned before, this is a common issue for case#2. Per RAN1 LS reply and relevant agreements, as least for the case when OD-SSB and AO-SSB have the same frequency location and the SS/PBCH blocks with the same SSB indexes for AO-SSB and OD-SSB are quasi co-located.

Proposal 12: For case 2, as least for the case when OD-SSB and AO-SSB have the same frequency location and the SS/PBCH blocks with the same SSB indexes for AO-SSB and OD-SSB are quasi co-located, UE shall combine OD-SSB and AO-SSB for scenario 2 and 2A, and SSB measured for known cell condition and the SSB used in SCell activation procedure can be different (i.e. AO-SSB and OD-SSB). FFS other case per further RAN1 agreements.

Another issue the applicable requirements of SCell activation with following options. From our understanding, the impact of OD-SSB for SCell activation is quite common regardless of SCell activation case. We see no technical reason to artificially preclude any one of them.

Issue 1-5-1: The type of OD-SSB based SCell activation FFS: RAN4 to define OD-SSB based SCell activation delay requirements for the following one or more scenarios. - Option 1: Direct SCell activation at SCell addition - Option 2: Multiple SCells activation - Option 3: Deactivated PUCCH SCell activation - Option 4: Rel-17 Fast SCell Activation - Option 5: Rel-18 SCell Activation with the L3 reporting during activation

Proposal 13: Since the impact of OD-SSB for SCell activation requirements are quite common, the OD-SSB based SCell activation delay requirements can apply to all options (1-5). And the detailed spec updating can be discussed in CR.

# Conclusions

Observation 1: For time domain, for case#1, UE can determine the SFN and half frame of OD-SSB burst by SFN offset and half frame index; for case#2, it is FFS whether the same method as case#1 can apply or based on one single time offset between AO-SSB and OD-SSB.

Observation 2: For frequency domain, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB and FFS when the AO-SSB is CD-SSB.

Observation 3: For QCL relation, SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are QCLed. FFS whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

Proposal 1: No additional UE processing/preparation for OD-SSB for SCell activation (i.e. Scenario 2A).

Proposal 2: RAN4 to discuss the value of additional UE processing/preparation for OD-SSB only for deactivated SCell L3 measurement (i.e. Scenario 2).

Observation 4: Fast measurement only happens every time OD-SSB is triggered, and it is not that frequent compared with legacy lower bound of measurement period.

Proposal 3: No need to define lower bound for fast mode measurement.

Proposal 4: RAN4 shall define minimum interval between fast mode measurement for deactivated SCell measurement, which can take exiting measurement period when MeasCycle =160 ms as reference. UE is not required to perform “fast measurement” more frequently than the minimum interval.

Observation 5: If the measurement delay is scaled by CSSF, it means the number of OD-SSB burst that gNB needs to transmit is multiplied, which is against the power saving purpose.

Proposal 5: RAN4 to discussed whether to prioritize the measurement for deactivated SCell, for which the OD-SSB is triggered.

Observation 5: For OD-SSB deactivation, explicit deactivation indication and indication of number of N OD-SSB burst are supported by RAN1.

Proposal 6: Per RAN1 agreements on OD-SSB deactivation, no needs to restrict the activation and deactivation of OD-SSB in RAN4, which will severely limit the usage and energy saving gain of the feature.

Proposal 7: The agreements for OD-SSB based deactivated SCell measurement applies for each time OD-SSB is activated.

Proposal 8: The default value of numbers of OD-SSB burst for fast measurement is the number of samples as required in current requirements. In addition, UE to indicate number of OD-SSB bursts needed as UE capability.

Proposal 9: RAN4 send LS to RAN2 to introduce L3 report procedure when UE obtain valid measurement results on OD-SSB after OD-SSB is triggered. UE can stop fast measurement when UE provide valid report of deactivated SCell

Observation 6: Per current agreement on deactivated SCell measurement on OD-SSB, UE may have different measurement behavior for deactivated SCell and neighbor cell on the same frequency in terms of period/pattern, and it will cause additional UE power consumptions.

Proposal 10: RAN4 to discuss whether and how to perform neighbor cell measurement when deactivated SCell configured with OD-SSB.

Observation 7: If UE always follow legacy requirements based on measCycleSCell for case 2, it is meaningless for gNB to trigger OD-SSB.

Proposal 11: For case 2, RAN4 specify same UE measurement behavior/requirements as case 1. FFS whether to combine OD-SSB and AO-SSB, which is a common issue for case 2.

Proposal 12: For case 2, as least for the case when OD-SSB and AO-SSB have the same frequency location and the SS/PBCH blocks with the same SSB indexes for AO-SSB and OD-SSB are quasi co-located, UE shall combine OD-SSB and AO-SSB for scenario 2 and 2A, and SSB measured for known cell condition and the SSB used in SCell activation procedure can be different (i.e. AO-SSB and OD-SSB). FFS other case per further RAN1 agreements.

Proposal 13: Since the impact of OD-SSB for SCell activation requirements are quite common, the OD-SSB based SCell activation delay requirements can apply to all options (1-5). And the detailed spec updating can be discussed in CR.

# References

[1] R4-2420098 WF on RRM requirements for Netw_Energy_NR_enh_Part1

[2] R4-2416913 LS on SSB relation in OD-SSB and SSB adaptation on Scell

[3] R1-2410917 Reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell

[3] R1-2407565 LS on timeline for On-demand SSB operation on SCell