3GPP TSG-RAN WG4 Meeting # 114	R4-2501222

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.22.3.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 MIMO on UE initiated BM

Document for:	Discussion

# Introduction

The Rel-19 MIMO WI is approved in [1] and further updated in [2], RAN4 has initiated the discussion in previous meeting. In this contribution, we further provide our views on the objective on UE initiated BM.

# Discussion

Based on the discussion in last meeting, the agreements reached are summarized as follows [3]:

Issue 1-1: How to define Event triggered measurement reporting delay? < Agreement> - Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air. - Ending point is the point when the UE transmits a first PUCCH triggered by the event. Issue 1-2: How to define measurement period (evaluation time / measurement time) in Event triggered measurement reporting delay? < Agreement> Agreement: For specifying the RAN4 delay requirement for event 2 - Measure both the current beam and new beam after the starting point, FFS the details. - Note: this does not imply to define the delay requirement based on the sum of the current beam measurement period and new beam measurement period, which is pending on further RAN1 discussion. - Start the RAN4 RRM discussion for FR2. - Send LS to RAN1 to ask whether FR1 is covered as well. FFS the exact wording in the LS. Issue 1-3: Whether RAN4 to specify requirements for case 1 or cover both case1 and case2? Case 1: no time window based Case 2: with time window based, M is configurable < Agreement> Agreement: It is RAN4 common understanding that no additional filtering applies for case 2 compared to case 1, if RAN4 will define requirement for case 2.

Firstly, based on latest agreements in last RAN1 agreements, following events are supported by RAN1.

Agreement On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed: On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. - Q is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2.

It could be observed that for Event 2 and Event 7, there are event triggered compared by comparation of current beam and new beam. For Event 1, it is the comparation between current beam and a certain threshold.

Observation 1: Based on RAN1 agreements, Event 1/2/7 are supported, where

- Event 2 and Event 7 are event triggered compared by comparation of current beam and new beam

- Event 1 are triggered by comparation between current beam and a certain threshold.

For event 2, based on the discussion in last meeting, it is agreed the starting point is the event is met of the air, and both current beam and new beam shall be considered. Regarding how to define the measurement/evaluation period, there was discussion with following examples in last meeting. We start for the simplified the scenario whether the periodicity of current and new beam are the same.

Fig. 1 When to trigger the Event

As shown in above fig1, the conditions are met over the air at point A. Some companies commented that only new beam measurement needs to be considered. However, the event conditions could cause by:

1. Bad quality of current beam

2. Good quality of new beam

3. Change of both current beam and current beam, which resulting in offset better of new beam

If only considering the measurement of new beam, it seems it required UE to trigger event at NB2. However, UE may only aware of the condition is fulfilled caused by poor condition of current beam at CB2.

Observation 2: The condition of event 2/7 triggering shall consider all following cases:

1. Bad quality of current beam

2. Good quality of new beam

3. Change of both current beam and current beam, which resulting in offset better of new beam

Proposal 1: The measurement requirements are defined assume that UE use the latest results from both new beam and current beam after the condition is met over the air.

Note: Simplified solution can be considered in test case that the condition of only one of beams (i.e. new beam and current beam) is changed.

Another issue whether to define time-window based Event-2 requirements. From our understanding, there is not much RAN4 efforts on considering this. From UE measurement behaviour/requirements, there is no difference compared with no-time window based. The only additional part is how to determine number of event instance. However, the impact on RAN4 is how to define the evaluation behaviour/requirements, which is also common for non-time window based.

Observation 3: For time window-based Event triggering, there is no difference compared with non-time-window based triggering, in terms of measurement behaviour/requirements and evaluation.

Besides, we think time window-based triggering is also important for this feature, different from L3 event triggering with filtering, L1-RSRP could be impacted by instance channel fading, which may lead to wrong event triggering, and it may mislead NW’s following action (e.g. wrong TCI state switching).

Observation 4: Time window-based Event triggering is important for this feature, which may help mitigate the wrong event triggering cause by instant channel fading.

Proposal 2: RAN4 define requirements for both Time window-based Event triggering and no-time-window-based event triggering.

RAN4 received a LS from RAN1 about the periodicity of new beam and current beam [4]. It is related whether to consider different periodicity for current beams and new beam and how to define the evaluation window for event-2 instance.

Firstly, regarding the whether to restrict that the periodicity of current beam and new beam has to be the same, we see no reason to do so. The discussion in RAN1 is about the how to define the evaluation periodicity if the periodicity is different, it makes no senses to restrict NW configurations on RS because there are different options to define evaluation period.

Proposal 3: From RAN4 perspective, the periodicity of current beam and new beam(s) can be the same or different which is up to network configurations.

Regarding the evaluation periodicity to determine Event-2 instance, as also analysed in last RAN4 meeting in our contribution. If the evaluation periodicity is determined based on the periodicity of one of the RS, for instance, the periodicity of the new beam which is 20 ms. Then UE shall evaluate whether the event is triggered every 20 ms, in T2/T3/T4, actually there is no new results from current beam. Is UE still required to count number of instances? If UE obtains a low result of current beam in T1 due to channel fading, and if UE takes this result to evaluate the event in T1/2/3/4, it is possible that UE mistakenly counts 4 instances and triggering the report to NW, which will lead to wrong NW configuration (e.g. switch the TCI though the current beam is still good.)

Fig. 2 Event instance wrongly counted

Observation 5: Defining evaluation period based on periodicity of only one of the RS may cause instance wrongly counted (out-dated result of one beam will be used for evaluation for multiple times).

As analysed above, L1-RSRP measurement is easily be impacted by channel instant fading. To define minimum requirements, the evaluation period shall be defined as the largest periodicity of the current beam and new beam RS.

Proposal 4: The evaluation period shall be defined as the largest periodicity of the current beam and new beam RS.

We draft a LS reply about proposal 3 and proposal 4 as attached in this contribution.

Another issue is about CSI-RS based L1-RSRP for inter-Cell BM, the status is summarized as follows:

Issue 1-7: CSI-RS based L1-RSRP for inter-cell BM <Way forward > - Option 1: (Qualcomm, Nokia, Samsung, MediaTek) - RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case. - Option 2: (CMCC) - For reporting delay for CSI-RS based UE-initiated/event-driven beam reporting, it is proposed to define L1 measurement reporting delay requirements, and FFS whether legacy CSI-RS based L1 measurement delay requirements can be reused. - Option 3: (Apple, Huawei) - For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation. - Option 4: (ZTE) - RAN4 needs to identify whether additional clarifications on the following points are necessary: - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC; - The prerequisite on L3 measurement on NSC. - RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements. - Option 5: (Ericsson) - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2 - RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements

From our understanding, the inter-cell/intra-cell is the definition only differentiated w.r.t SSB since UE needs detect SSB with same SSB index from different TRP/Cell with different PCI. However, for CSI-RS there is no such differentiations. Different CSI-RS resources from different TRP/Cells can QCLed to SSB with different PCI.

Observation 6: From UE perspective, for CSI-RS based BM, there is no differentiation w.r.t. intra-cell or inter-cell.

Proposal 5: CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

Another issue is about whether there is impacts on known conditions for TCI state switching. Take known conditions as example, which is shown as follows, the known conditions are determined by whether there is L1-RSRP reporting before TCI state switching. For UEIBM, the reporting content is also L1-RSRP. Thus, the existing known conditions can already cover the case where the L1-RSRP is triggered by UEIBM.

### 8.15.2 Known conditions for downlink TCI state The downlink TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target downlink TCI state to the completion of active downlink TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target downlink TCI state or QCLed to the target downlink TCI state - Downlink TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target downlink TCI state before the downlink TCI state switch command - The target downlink TCI state remains detectable during the downlink TCI state switching period - The SSB associated with the downlink TCI state remain detectable during the downlink TCI switching period - SNR of the downlink TCI state ≥ -3dB - The SSB can be associated with either the serving cell PCI or a PCI different from serving cell PCI. Otherwise, the downlink TCI state is unknown.

Proposal 7: The existing known conditions for TCI state switching can cover the case that L1-RSRP reporting triggered by UEIBM.

Another issue is about whether to consider the UE capability defined in LTE with following proposals.

Issue 1-12: Compatibility of capabilities with LTM <Way forward > - Option 1: (Nokia) - RAN4 to define UE capabilities in the context of UE initiated beam reporting towards the later part of the work item. - Option 2: (Samsung, Apple) - Do not combine capabilities across different L1 measurements. - Option 3: (Ericsson) - RAN4 to wait for further progress in RAN1/2 for the UE capability discussions and compatibility between MIMO and mobility. - Option 4: (MediaTek) - Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold. - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

We are not in favour of combining two WI to define a single UE capability. In one hand, different group of peoples are involved in different WI discussion, and the technical details are not fully clear to all. In the other hand, it may cause difficulties when implement the feature from both UE side and NW side since NW may need to jointly consider two features when only configure one of them.

Observation 7: Do not combine UE capabilities across different WID, and postpone the discussion in RAN4 with more RAN1/2 progress.

# Conclusions

Observation 1: Based on RAN1 agreements, Event 1/2/7 are supported, where

- Event 2 and Event 7 are event triggered compared by comparation of current beam and new beam

- Event 1 are triggered by comparation between current beam and a certain threshold.

Observation 2: The condition of event 2/7 triggering shall consider all following cases:

1. Bad quality of current beam

2. Good quality of new beam

3. Change of both current beam and current beam, which resulting in offset better of new beam

Proposal 1: The measurement requirements are defined assume that UE use the latest results from both new beam and current beam after the condition is met over the air.

Note: Simplified solution can be considered in test case that the condition of only one of beams (i.e. new beam and current beam) is changed.

Observation 3: For time window-based Event triggering, there is no difference compared with non-time-window based triggering, in terms of measurement behaviour/requirements and evaluation.

Observation 4: Time window-based Event triggering is important for this feature, which may help mitigate the wrong event triggering cause by instant channel fading.

Proposal 2: RAN4 define requirements for both Time window-based Event triggering and no-time-window-based event triggering.

Proposal 3: From RAN4 perspective, the periodicity of current beam and new beam(s) can be the same or different which is up to network configurations.

Observation 5: Defining evaluation period based on periodicity of only one of the RS may cause instance wrongly counted (out-dated result of one beam will be used for evaluation for multiple times).

Proposal 4: The evaluation period shall be defined as the largest periodicity of the current beam and new beam RS.

Observation 6: From UE perspective, for CSI-RS based BM, there is no differentiation w.r.t. intra-cell or inter-cell.

Proposal 5: CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

Proposal 7: The existing known conditions for TCI state switching can cover the case that L1-RSRP reporting triggered by UEIBM.

Observation 7: Do not combine UE capabilities across different WID, and postpone the discussion in RAN4 with more RAN1/2 progress.

# References

[1] RP-234007 New WID: NR MIMO Phase 5

[2] RP-242394 WID revision: NR MIMO Phase 5

[3] R4-2420096 WF on RRM requirements for NR_MIMO_Ph5_Part1

[4] R1-2410914 LS to RAN4 on event-instance evaluation periodicity for UE-initiated/event-driven beam management

# Annex

3GPP TSG RAN WG4 Meeting #114                                 R4-25xxxxx

Athens, GR 17th – 21nd Feburary 2025

Title:	Reply LS on event-instance evaluation periodicity for UE-initiated/event-driven beam management

Response to:	R1-2410914

Release:	Rel-19

Work Item:	NR_MIMO_Ph5-Core

Source:	RAN4

To:	RAN1

Cc:

Contact person:	Zhongyi Shen, shenzhongyi3@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN4 would like to thank RAN1 for the LS on event-instance evaluation periodicity for UE-initiated/event-driven beam and would like to provide the following RAN4 agreements in RAN4#114.

From RAN4 perspective, the periodicity of current beam and new beam(s) can be the same or different which is up to network configurations.

The evaluation periodicity is defined as the largest periodicity of the current beam and new beam RS.

# 2	Actions

To RAN1:

ACTION: 	RAN4 respectfully asks RAN1 to take the above into account in their future work.

# 3	Dates of next TSG-RAN WG1 meetings

TSG-RAN WG4 Meeting #114bis	7th – 11th April 2025				TBD China

TSG-RAN WG1 Meeting #115	19th – 23th May 2025			Malta MT