3GPP TSG-RAN WG4 Meeting # 114	R4-2501235

Athens, Greece, February 17 – 21, 2025

Title: 	Discussion on testability and interoperability issues in AIML mobility

Source: 	Huawei, HiSilicon

Agenda item:	7.21.3

Document for:	Discussion

# Introduction

According to RAN4 #113 WF [1], we discuss the remaining issues of testability and interoperability in AIML mobility.

# 2. Testing goal

Two study goals are defined in RAN2 for verifying the gain introduced by AI. One is to reduce the measurement overhead, another is to enhance handover performance. From RAN4 perspective, both study goals are related to measurement prediction accuracy in RRM measurement prediction and in indirect HO failure/RLF/measurement event prediction.

General aspects in RAN2 AI mobility #126: - Conclusion 1: 1st study goal of evaluation is to reduce measurement overhead - Conclusion 2: 2nd study goal of evaluation is to enhance handover performance

In RAN4 #113 meeting, the testing goal is agreed to verify whether the minimum performance of AI/ML functionality/feature can be achieved. This means that RAN4 will define requirement with the granularity of AI/ML functionality/feature rather than AI/ML model.

RAN4 #113 WF [1]: Issue 3-2-1: Testing goal Agreement: As baseline, the testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved.

Regarding to the principle of defining the minimum performance in the testing goal needs further discussion. Two options are identified as below.

- Option 1: To verify whether the difference between the predicted measurement and the ideal measurement is within the existing relative measurement accuracy margin requirement.

- Option 2: To verify whether the difference between the predicted measurement and the ideal measurement is within X dB, where X is larger than legacy relative measurement accuracy margin requirement, under the side condition of MRRT and other parameters.

  - FFS: The value of MRRT.

  - FFS: Other parameters and the values, e.g., the prediction window.

Proposal 1: For testing goal, the principle of defining the minimum performance can be discussed based on the following two options:

- Option 1: To verify whether the difference between the predicted measurement and the ideal measurement is within the existing relative measurement accuracy requirement.

- Option 2: To verify whether the difference between the predicted measurement and the ideal measurement is within X dB, where X is larger than legacy relative measurement accuracy margin requirement, under the side condition of MRRT and other parameters.

  - FFS: The value of MRRT.

  - FFS: Other parameters and the values, e.g., the prediction window.

With option 1, RAN4 can ensure that the accuracy of the predicted measurement will not be worse than that of legacy measurement. Therefore, no handover performance degradation compared to non-AI will be introduced. However, it seems restrictive to force prediction method to achieve the same accuracy as measurement method. In addition, there is lack of evidence that handover performance is bound to degrade with slight measurement accuracy loss. For example, proactive prediction of RRM measurement variation trend will help NW to make handover decision earlier before performance starts to degrade, the gain of which cannot be achieved by using passive measurement method. In this case, as long as the RRM measurement variation trend is accurate enough, there may be some room to relax the requirement of prediction accuracy compared to measurement accuracy.

With option 2, whether adopting the predicted measurement will have an impact on handover performance may need to be evaluated. If there is no handover performance degradation or the degradation is acceptable, the benefit of AI can still be verified in terms of reducing the cost of RRM measurement.

It is also observed that in some cases, for example, in the inter frequency prediction, the measurement reduction helps reduce the measurement gap. If the measurement gap can be reduced, more resources for data transmission can be obtained. In these cases, additional benefit of AI is seen in terms of improving the user experience of services like XR in mobility.

Observation 1: AI based prediction may help reduce the power consuming due to the measurement reduction.

Observation 2: AI based prediction may help reduce the delay of data transmission and improve the throughput due to the reduction of measurement gap.

Proposal 2: When defining measurement prediction requirements, the gain derived from power consumption reduction and measurement gap reduction shall also be taken into account.

# 3. Testability of sub-use case

For cell level measurement prediction model, RAN2 considers the following cases:

•	sub-use case 1: To predict beam level results, then generate cell level results based on the predicted beam results;

•	sub-use case 2: To directly predict cell level results based on cell level results.

•	sub-use case 3: To directly predict cell level results based on beam level results.

For L3 filtered beam level measurement prediction model, RAN2 considers the following cases:

•	sub-use case 4: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results.

•	sub-use case 5: To directly predict L3 filtered beam level results based on the L3 beam level measurement results.

•	sub-use case 6: To directly predict L3 filtered beam level results based on the L1 beam level measurement results.

However, L3 filtered beam level prediction sub-use cases are lower priority according to the TR 38.744 [2]. Therefore, RAN4 can prioritize the discussion on the testability of Case 1, Case 2 and Case 3.

Observation 2: Both cell level prediction and L3 filtered beam level prediction sub-use cases are identified in RAN2, and L3 filtered beam level prediction sub-use cases are lower priority.

Proposal 3: RAN4 to discussion the testability of Case 1, Case 2 and Case 3.

In the following, we will discuss the testability in these three cases.

## Testability of sub-use case 1 and 3

It is observed that the testability issues in Case 1 and Case 3 is similar in that of AI enabled BM in [2], since the model input is the beam-level measurement. According to the latest progress in [3], the testability is still under discussion. The open issues are shown as follows.

RAN4 #112 WF [3]: Issue 2-3: Channel models Agreement: Further discuss and down-select the following options considering TE feasibility - Option 2: TDL - Option 5: simplified CDL - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed Issue 2-4: Test setup needs - Issues listed below to be further investigated in order to determine the capabilities of the test equipment: - set B Tx beams; set A Tx beams - number of beams transmitted simultaneously - overall number of beams transmitted during the test - AoA - AoD - UE rotation during the test Other aspects can also be considered

In Case 1, Case 3 and Case 4-6, the numbers of beams transmitted simultaneously and the overall number of beams transmitted during the test both need to check the capability of the test equipment. In order to avoid duplicate discussion, we proposed to postpone the discussion to wait the progress of the testability study in AI-BM.

Proposal 4: Postpone the testability discussion of Case 1 (i.e., to predict beam level results, then generate cell level results based on the predicted beam results), Case 3 (i.e., to directly predict cell level results based on beam level results) to wait the progress of the testability study in AI-BM.

## Testability of sub-use case 2

In Case 2, since there is no beam-level measurement related to the model input and the model output, the test setup can reuse legacy if the number of cells is smaller than 2.

Proposal 5: RAN4 to consider whether to reuse the test setup in legacy for Case 2 (i.e., to directly predict cell level results based on cell level results).

# 3. Conclusion

The contribution provides analysis on RRM measurement prediction in AI/ML mobility. The following proposals are proposed:

Proposal 1: For testing goal, the principle of defining the minimum performance can be discussed based on the following two options:

- Option 1: To verify whether the difference between the predicted measurement and the ideal measurement is within the existing relative measurement accuracy requirement.

- Option 2: To verify whether the difference between the predicted measurement and the ideal measurement is within X dB, where X is larger than legacy relative measurement accuracy margin requirement, under the side condition of MRRT and other parameters.

  - FFS: The value of MRRT.

  - FFS: Other parameters and the values, e.g., the prediction window.

Observation 1: AI based prediction may help reduce the power consuming due to the measurement reduction.

Observation 2: AI based prediction may help reduce the delay of data transmission and improve the throughput due to the reduction of measurement gap.

Proposal 2: When defining measurement prediction requirements, the gain derived from power consumption reduction and measurement gap reduction shall also be taken into account.

Observation 2: Both cell level prediction and L3 filtered beam level prediction sub-use cases are identified in RAN2, and L3 filtered beam level prediction sub-use cases are lower priority.

Proposal 3: RAN4 to discussion the testability of Case 1, Case 2 and Case 3.

Proposal 4: Postpone the testability discussion of Case 1 (i.e., to predict beam level results, then generate cell level results based on the predicted beam results), Case 3 (i.e., to directly predict cell level results based on beam level results) to wait the progress of the testability study in AI-BM.

Proposal 5: RAN4 to consider whether to reuse the test setup in legacy for Case 2 (i.e., to directly predict cell level results based on cell level results).

# 4. Reference

- R4-2420095, “WF on AI/ML Mobility”, 3GPP RAN4#113, 18th – 22th November, 2024.