3GPP TSG-RAN WG4 Meeting # 114	R4-2501236

Athens, GR, 17th – 21st February, 2025

Title: 	Discussion on low band carrier aggregation via switching

Source: 	Huawei, HiSilicon

Agenda item:	7.8.3

Document for:	Discussion

Introduction

WI on low band carrier aggregation via switching is approved in [1].

- 4.1 Objective of SI or Core part WI or Testing part WI Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

The contribution provides preliminary analysis on low band carrier aggregation via switching from RRM impact perspective.

Discussion

Operators and interested companies have provided plenty information of the background, motivation and possible solutions of low band CA [2]. The agreed WI aims to specify requirements with the example band combination CA_n5A-n29A where n29 is SDL band, and n5 is FDD band.  The two carriers are collocated and synchronized in a single TAG.

Figure 1. Example scenario (referenced from [RP-242532])

If a single antenna is implemented to aggregate the two bands, it is challenged for simultaneous Tx/Rx between the PCell and the SCell. Therefore switching between two carriers shall be considered. In our understanding, in this scenario, SCell is in activated state, and a semi-static switching pattern has been configured by RRC configuration. UE is allowed to switch between case 1 and case 2.

-	Case 1: Tx/Rx on FDD carrier and no Rx on SDL carrier

-	Case 2: Rx on SDL carrier and no Tx/Rx on FDD carrier

From RRM requirements perspective, it somewhat resembles with the feature of TX switching between two carriers in previous release. DL interruption requirements shall be specified. Regarding the specific DL interruption length value, more conclusion of switching period from RF are needed.

Proposal 1: DL interruption requirements due to low band CA via switching are to be specified in RRM. Detailed interruption length can wait for more progress in RF.

Conclusions

This contribution provides preliminary analysis on low band carrier aggregation via switching from RRM impact perspective. The following proposal is provided:

Proposal 1: DL interruption requirements due to low band CA via switching are to be specified in RRM. Detailed interruption length can wait for more progress in RF.

Reference

- RP-243317, New WID on low band carrier aggregation via switching

- RP-242532, Motivation for a new work item on low band carrier aggregation via switching in Rel-19