3GPP TSG-RAN WG4 Meeting #114	R4-2501249

Athens, Greece, 17 – 21 February, 2025

Title: 	Discussion on RLM/BFD test cases in R18 less than 5MHz

Source: 	Huawei, HiSilicon

Agenda item:	5.8.3

Document for:	Discussion

Introduction

RRM test cases for R18 less than 5MHz are discussed in RAN4#113. One outstanding remaining issue is the SNR levels for RLM and BFD TCs.

In this paper, we will provide our views on the SNR levels for RLM and BFD TCs for less than 5MHz.

Discussion

In our view, the SNR levels for RLM and BFD TCs should be derived with following steps.

- Step 1: companies simulate PDCCH decoding performance with the hypothetical PDCCH parameters and provide the SNR points for the 10% and 2% BLER

- Step 2: summarize the simulation results to get Qout and Qin levels

- Step 3: derive the SNR levels by adding margin on top of Qout and Qin levels

Step 1 has been ongoing and several companies have already submitted simulation results. Our results can be found in [1]. Step 2 will be conducted by rapporteur (Nokia) following the conclusion from last meeting. We need to discuss what margins to apply in Step 3.

RLM

Based on the R15 discussion to develop RLM/BFD test cases, the SNR levels are derived as:

- SNR2 = Qout + margin_2

- SNR3 = Qout – margin_3

- SNR4 = Qin – margin_4

- SNR5 = Qin + margin_5

- SNR1 = SNR5

This is illustrated e.g. in Figure A.6.5.1.2.1-1 which is copied below.

Figure A.6.5.1.2.1-1: SNR variation for in-sync testing in 38.133

We do not find explicit agreement on the values for Qout, Qin or margins used in R15 test cases. There is a summary of simulation results in [2] for RLM. Based on the summary, we would like to roughly take Qout as -10 dB and Qin as -1.5 dB. Comparing the SNR levels in the latest test case, we have

- margin_2 = 3 dB

- margin_3 = 5 dB

- margin_4 = 3 dB

- margin_5 = 2.5 dB

The next question is whether to re-use the margins from normal BW TC to less than 5MHz TC. If Qout/Qin levels are similar to R15, e.g. in 15 RB case, the margins should be re-used. For 12 RB case, the Qout/Qin levels are supposed to be higher than that in R15 due to smaller aggregation level, and technically margins can be reduced because the measurement is taken at a higher SNR.

However, deriving a new set of margins requires another simulation campaign to check the measurement accuracy at the new Qout/Qin levels. We do not prefer this approach considering that the WI has been closed for quite some time and there are only 2 test cases concerned, so we support to re-use.

Proposal 1: For RLM tests, re-use the R15 margin values to derive SNR levels in test cases.

- margin_2 = 3 dB

- margin_3 = 5 dB

- margin_4 = 3 dB

- margin_5 = 2.5 dB

BFD

For BFD, Qout would be roughly -6dB since the difference in PDDCH parameter compared to RLM is the 4dB power boosting. Comparing the SNR levels in the latest test case, we have

- margin_2 = 3 dB

- margin_3 = 6 dB

It is noted that in BFD tests, only SNR levels for q0 are dependent on PDCCH performance as shown in e.g. Figure A.6.5.5.1.1-1 of 38.133 which is copied below.

Figure A.6.5.5.1.1-1: SNR variation for SSB-based beam failure detection and link recovery testing in non-DRX mode in 38.133

q1 is used for L1-RSRP measurement, and its SNR levels can re-use those in the latest test case. Again, we suggest to re-use the margin for less than 5MHz test cases.

Proposal 2a: For BFD tests, re-use the R15 margin values to derive SNR levels for q0 in test cases.

- margin_2 = 3 dB

- margin_3 = 6 dB

Proposal 2b: For BFD tests, re-use the R15 SNR and RSRP levels for q1.

Conclusions

In this paper we provided provide our views on SNR levels for RLM and BFD TCs for less than 5MHz.

Proposal 1: For RLM tests, re-use the R15 margin values to derive SNR levels in test cases.

- margin_2 = 3 dB

- margin_3 = 5 dB

- margin_4 = 3 dB

- margin_5 = 2.5 dB

Proposal 2a: For BFD tests, re-use the R15 margin values to derive SNR levels for q0 in test cases.

- margin_2 = 3 dB

- margin_3 = 6 dB

Proposal 2b: For BFD tests, re-use the R15 SNR and RSRP levels for q1.

Reference

- R4-2412639, Discussion on remaining issues in R18 less than 5MHz, Huawei, HiSilicon

- R4-1814872, Summary of PDCCH simulation results for RLM, MediaTek inc.