3GPP TSG-RAN WG4 Meeting #114	R4-2501270

Athens, Greece, 17 – 21 February, 2025

Title: 	Discussion on other RRM requirements for Rel-19 NTN

Source: 	Huawei, HiSilicon

Agenda item:	7.29.4.2

Document for:	Discussion

Introduction

RRM requirements for other requirements in NTN are discussed in RAN4#113, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to further discuss the following issues for DL coverage enhancement.

- SSB periodicity extension

- Beam hopping

In this paper, we will provide our views on RRM requirements for DL coverage enhancement in NTN.

Discussion

SSB periodicity extension

One issue discussed last meeting is whether to update the default SMTC periodicity in RRC Re-establishment, RRC connection release with redirection to NR and HO (including normal HO, CHO, RACH-less HO and satellite switch with re-sync). The concerned parts for each requirement are copied below.

HO (taking normal HO as example)

- Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

We do not support to update the default SMTC periodicity in HO requirements, for the following reasons.

- The default assumption of SSB period for HO does not have to be aligned with the default assumption of SSB period for initial access. They are already not aligned in current specs, and we do not see strong need to update the default assumption of SSB period for HO because of additional default assumption of SSB period for initial access.

- Changing the default assumption of SSB period for HO to other values than 5ms means UE has to do blind search of target cell SSB. This has not been considered in any of the existing HO requirements, even SSB period can be already configured as 160ms today.

- In HO procedure, source cell has the information of the target cell, so providing SMTC of the target cell should not be a problem and it is corner case that UE is not provided with any information of SMTC of the target cell in HO.

Proposal 1: Do not update the default SMTC periodicity assumption in HO requirements.

RRC re-establishment

TSMTC: It is the periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell. TSMTC,i: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms. Table 6.2C.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); Kmulti_SMTC * MAX (800 ms, 10 x TSMTC). < -8; FR1; N/A; k * 800 ms Note1, 3. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: Kmulti_SMTC is defined in clause 9.2C.5.1. Note 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbor cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt..

We support to update the default SSB period for RRC re-establishment.

- When Serving cell SSB Ês/Iot < -8dB (the cyan part), UE is supposed to do blind search of target cell, which is similar to initial access. Clearly, it is not reasonable to use 20ms as the assumption when additional assumption of SSB period for initial access is introduced.

- When Serving cell SSB Ês/Iot ≥-8dB (the yellow part), UE is supposed to do search target cell based on SMTC provided by the source cell if configured. When source cell does not provide SMTC information, UE can only do blind search. For the same reason as above, this assumption should be updated as well.

Proposal 2: Update the default SMTC periodicity assumption in RRC re-establishment requirements.

RRC connection release with re-direction

- Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the redirection command, otherwise Trs is the SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing configured for the RRC connection release with redirection. If the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection then: - the requirement in this clause is applied with Trs = 20 ms if the SSB transmission periodicity is not larger than 20 ms; otherwise, - there is no requirement if the SSB transmission periodicity is larger than 20ms.

We do not support to update the default SSB period for RRC re-direction.

When UE is not provided with SMTC of the target frequency for re-direction, it is not reasonable for UE to meet the requirements with Trs = 20 ms even the SSB transmission periodicity is not larger than 20 ms. The reason is that without SMTC, UE has to do blind search on the target frequency, while the requirements with Trs are derived based on the assumption that UE searches based on SMTC.

In our view, it is better to leave no requirement for this case. The alternatives to define requirements are 1) assume SSB periodicity 5ms, or 2) derive the delay based on blind search. Either of the alternative would results in new requirements for UE at re-direction. Also, the target frequency for re-direction is provided by the source cell, so we believe it would be corner case where source cell does not provide SMTC of the target frequency. It is not preferred to define new requirements for a corner case.

Of course, we understand the issue with the yellow texts is not specific to NTN or to additional default SSB period assumption for initial access. We have provided a maintenance CR to correct it in R15.

Proposal 3: Do not update the default SMTC periodicity assumption in RRC release with re-direction requirements.

In last meeting, we proposed to discuss the RRC re-establishment requirements for Serving cell SSB Ês/Iot < -8dB. As mentioned above, existing requirements are for blind search with 20ms SSB periodicity, and RAN4 need to define requirements for other default SSB periodicities. The rationale behind the existing values are provided in [2], and the related part are copied below.

The simulation results have shown that when SMTC offset is known then the UE can detect the target cell within 3 attempts at SCH Es/Iot ≥ -6 dB [3]. But without using the timing information, the NR cell detection will be similar for inter-RAT SFTD measurement, and would require 5 SMTC periods for each attempt to capture PSS/SSS [1-2]. We assume that the UE after losing the serving cell detects the target NR cell assuming 20 ms SMTC periodicity. The cell search shall be done once every 20 ms by assuming 5 ms radio time. Then one additional SMTC is needed between each attempt to take into account the fact that due to resource restrictions (cell search is done over 5ms radio time/20ms) there is restriction on how the offset can be changed. Therefore assuming 3 attempts to detect the PSS/SSS of an unknown intra-frequency NR cell, total number SMTC will be: 3*5 SMTC periods + 2 SMTC periods = 17 SMTC periods. Then 5 additional SMTC periods are needed for SS-RSRP measurement leading to total cell search time of up to 22 SMTC periods for FR1. For FR2, the intra-frequency cell search time needs to be scaled (multiplied) with a scaling factor (K3). …… - Proposal # 3: When the UE has lost serving cell timing (i.e. SCH Ês/Iot < X) the UE shall be able to identify a known intra-frequency FR1 cell within MAX (800 ms, [22] x TSMTC) and an unknown intra-frequency FR2 cell within MAX (1000 ms, (K3 x [22]) x TSMTC)) for RRC re-establishment to that cell. - Proposal # 4: When the UE has lost serving cell timing (i.e. SCH Ês/Iot < X) the UE shall be able to identify a known inter-frequency FR1 cell within MAX (800 ms, [25] x TSMTC, i) and an unknown inter-frequency FR2 cell within MAX (1000 ms, (K5 x [25]) x TSMTC, i)) for RRC re-establishment to that cell.

It is straightforward to follow the same derivation for other default SSB periodicities. One issue RAN4 needs to discuss is the definition of TSMTC. In our view, it should be the periodicity with which NW transmits SSB, instead of the periodicity UE assumes for initial access. The reason is that even UE assumes 160ms SSB periodicity for blind search, it should be able to find a suitable cell when it searches for the first 40ms in an attempt, if NW transmits SSB with 40ms periodicity. According to the following UE behaviour in 38304 for cell selection, UE is only required to select to a suitable cell but is not required to find the best cell. Therefore, once UE finds a suitable cell based on blind search for the first 40ms, it can start the RRC re-establishment procedure towards that cell.

### 5.2.3 Cell Selection process #### 5.2.3.1 Description Cell selection is performed by one of the following two procedures: a) Initial cell selection (no prior knowledge of which RF channels are NR frequencies): 1. The UE shall scan all RF channels in the NR bands according to its capabilities to find a suitable cell. 2. On each frequency, the UE need only search for the strongest cell, except for operation with shared spectrum channel access where the UE may search for the next strongest cell(s). 3. Once a suitable cell is found, this cell shall be selected.

Proposal 4: Update the delay requirements for RRC re-establishment for the case of Serving cell SSB Ês/Iot < -8dB as

- Tidentify_intra_NR: MAX (800 ms, 22 x TSMTC)

- Tidentify_inter_NR,i: MAX (800 ms, 25 x TSMTC,i)

where TSMTC is the periodicity with which NW transmits SSB.

In last meeting, some companies suggested to discuss the cell reselection requirements. The concern is that “Observe that for enhanced measurements, when the DRX Cycle is equal to 0.32 s, only two SSB periods (of 160 ms) can fit one DRX Cycle, and the UE shall perform measurements across both of these opportunities necessarily in order to achieve the requirements for Tmeasure,NR_xx_enh/2”. We do not see this is an issue because 1) SMTC of 160ms is already possible based on existing spec and 2) the samples for filtering does not need to be within same DRX cycle, e.g. UE should be allowed to take one sample per DRX cycle.

Some companies also suggested to update the paging interruption requirements. In our understanding, the existing interruption requirements, which are based on configured SMTC periodicity, should apply for all configurable SMTC periodicities including 160ms. We can see the point that the paging interruption time becomes longer with larger SMTC periodicity, but this is the consequence from the trade-off for better coverage, and we do not think RAN4 should tighten the requirements for this trade-off.

Proposal 5: Do not update the requirements for cell reselection or paging interruption.

Beam hopping

RAN2#128 reached the following agreements related to beam hopping including cell DTX/DRX.

Agreements:

1.	As part of the work on DL-CE, RAN2 investigates related UE power consumption impact (including legacy UEs)

2.	RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells

Agreements:

1.	For cell level DTX for NTN DL coverage enhancement, RAN2 currently understands that there will be only two states: "cell DTX On period" and "Cell DTX Off period" (FFS if the UE behaviour will be different from the NES states or the same). FFS on Cell DRX states.

It can be seen that possible enhancements related to beam hopping are still under discussion in RAN2. Given this status, RAN4 should continue to wait before discussing possible RRM impacts.

Proposal 6: RAN4 to wait more progress from RAN1/2 before discussing possible RRM impacts related to beam hopping including cell DTX/DRX.

Conclusions

In this paper we provided provide our views on RRM requirements for DL coverage enhancement in NTN.

Proposal 1: Do not update the default SMTC periodicity assumption in HO requirements.

Proposal 2: Update the default SMTC periodicity assumption in RRC re-establishment requirements.

Proposal 3: Do not update the default SMTC periodicity assumption in RRC release with re-direction requirements.

Proposal 4: Update the delay requirements for RRC re-establishment for the case of Serving cell SSB Ês/Iot < -8dB as

- Tidentify_intra_NR: MAX (800 ms, 22 x TSMTC)

- Tidentify_inter_NR,i: MAX (800 ms, 25 x TSMTC,i)

where TSMTC is the periodicity with which NW transmits SSB.

Proposal 5: Do not update the requirements for cell reselection or paging interruption.

Proposal 6: RAN4 to wait more progress from RAN1/2 before discussing possible RRM impacts related to beam hopping including cell DTX/DRX.

Reference

- R4-2420105, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)

- R4-1810777, Analysis of RRC Re-establishment without Serving Cell Timing, Ericsson