3GPP TSG-RAN WG4 Meeting #114	R4-2501284

Athens, Greece, 17th – 21st, February 2025

Source: 	Ericsson

Title:  	Discussion on RRM core requirements for beam management

Agenda Item:	7.19.3

Document for:	Discussion

# Introduction

In last meeting 3GPP TSG RAN WG4 Meeting #113, RAN4 discussed RRM requirements for AI/ML based beam management and the discussion summary is listed in [1]. Following the discussions, we herein in this contribution present our views and proposals on the remaining open issues relevant to core requirements.

# Discussion

## Core requirements

Issue 3-1-1: Absolute RSRP accuracy

Agreement in RAN4#112: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

We understand the concern raised by some companies in the last meeting is which one shall be referred as the benchmark, the top-K beams w.r.t. the predicted RSRP or the top-K beams w.r.t. the ground truth RSRP.

In our opinion, the beams that shall be taken care are the predicted beams. In other words, the prediction (accuracy) shall focus on the beams that the UE reports but not the beams supplied by the genie way.

If there are offsets between the two approaches, it’s questionable to adopt the top-K beams w.r.t. the ground truth RSRP. Two instances that reflect our concerns are provided below:

- If the requirement is only relevant to the top-K beams with respect to the ground truth RSRP, does the beam's RSRP have no requirements if it is present in the top-K predicted beams but not in the genie beams?

- Conversely, why should we need the RSRP accuracy of a beam that the UE doesn't even report if it is present in the top-K genie beams but not in the predicted beams?

Proposal 1: For the absolute RSRP, the index i shall be any beam index in top-K beams based on predicted L1-RSRP as per UE reporting.

Issue 3-1-2: Relative RSRP Accuracy

Regarding the relative RSRP accuracy, the proposals in the last meeting are captured in the below.

- Proposals - Option 1: The definition of relative RSRP accuracy shall be: The relative RSRP accuracy for AI/ML-based beam prediction = the ground truth of L1-RSRP of the predicted beam index i - the ground truth of the L1-RSRP of the Top-1 ideal beam index j - Option 2: The definition of relative RSRP accuracy shall be: predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest RSRP among the predicted beams. - Option 3: The definition of relative RSRP accuracy shall be: the RSRP difference between the predicted L1-RSRP of different beams. - Option 4: others - Recommended WF - Option 1

Firstly, recap the legacy relative RSRP accuracy definition in Clause 10.1.20.1.2 of TS38.133, which is copied as follows:

﻿The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the serving cell.

The intention of the relative accuracy requirement is to guarantee the accuracy of the relative L1-RSRP, which allows the network to compare the received signal power from different beams (or cells) more accurately by comparing the relative differences among them. It consequently aids the UE and the network in determining the optimal beam or cell. Hence, for serving the purpose, the reference L1-RSRP of relative accuracy shall be assessed.

In legacy, there is no doubt that the reference L1-RSRP is the largest measured value of L1-RSRP. Differently, in AI/ML based beam management, even we understand there may be certain underlying advantage by adopting measured L1-RSRP as reference, however a simple way is to define the reference L1-RSRP is the largest predicted value of L1-RSRP, as a peer-to-peer comparison.

Furthermore, in our opinion, Option 1 is somewhat closer to a beam label (beam ID) prediction than an RSRP prediction.

Proposal 2: For the relative RSRP, support Option 2. Relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest RSRP among the predicted beams.

Observation 1: Option 1 of the relative RSRP accuracy is somewhat closer to a beam label (beam ID) prediction than an RSRP prediction.

Issue 3-2: UE Rx beam knowledge

Agreement: It is common understanding that the UE would have knowledge of the Rx beams used for the measurements which are input to the for inference, however, the prediction model used by the UE may only predict the gNB Tx beam, not the UE Rx beam to be paired with the predicted Tx beam. Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS RAN4 does not need to discuss whether the UE knows the Rx beam paired with the predicted Tx beam or not.

Issue 3-3: TCI State knowledge

- Proposals - Option 1: Reuse the legacy conditions with the following additions (marked in red) The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state or AI/ML based inference to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state, or the RS resources for AI/ML based inference is the RS in SetB or in target TCI state or QCLed to the target TCI state. - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement or inference - The UE has sent at least 1 L1-RSRP report for the target TCI state or completes AI/ML based inference before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3dB Otherwise, the TCI state is unknown. - Option 2: TCI state QCL to an RS that is not in Set B is unknown even if this RS was included in the L1-RSRP report by UE within 1280ms before TCI state activation. - Option 3: The TCI state which is QCL to an RS in Set B is known: - if the corresponding predicted beam is reported in 1280ms before the TCI state switch command and SNR of the RS is above -3dB for spatial-domain beam prediction. - if the last observation occasion is within 1280ms before the TCI state switch command and SNR of the RS is above -3dB for temporal beam prediction. - Option 4: The known conditions of the predicted TCI state shall take below conditions into account: - The TCI state switch command is received during valid inference phase of AI/ML mode. - The predicted beam in Set A is the RS in target TCI state. - The predicted beam in Set A is detectable during training and inference phase, which implies the beam has been trained successfully and available in inference phase. - The UE knows the Rx beam for the predicted beam. - Option 5: RAN4 should consider relaxing known conditions for TCI states - TCI is known if measured within X+1280 ms. Further analysis is required to define the exact value of X. - Option 6: others - Recommended WF - To be discussed

To our understanding, the UE is able to acquire its Rx beam, where the Rx beam is speculated through the process of Rx beam sweeping in order to detect and receive reference signals/beams, at the least in measurement phase. In the consequent inference phase, the UE may keep or discard the Rx beam information depending on UE capability, since the AI/ML model doesn’t necessitate the Rx beam as input/output. Even not mandating the Rx beam information, it appears that retaining the Rx beam information may gain the prediction performance enhancement from the association between the Tx beam and Rx beam if the UE is willing to do so, which is applicable for both two output types of the AI/ML model.

- For the label type, the AI/ML model type is classifier network.

- For label & RSRP type, the AI/ML model type is CNN or MLP network.

Taking AI/ML model into account, if the known condition of the TCI state switch still relies on the legacy way, e.g., depending on the criteria whether the RS has been measured or not, no gain is obtained from the AI/ML model application.

Such that, we suppose that switching to a TCI state, to which the RS referred is predicted in absence of measurement, is a crucial improvement on beam management speeding up the TCI state switch procedure delay. In this manner, the criteria for known conditions regarding the predicted RS instead of the measured RS must be reconstructed. Where, knowing the Rx beams is a crucial factor in TCI state switch enhancement with respect to prediction.

Proposal 3:  RAN4 shall study the new known condition of the TCI state switch, to which the RS referred is which is ‘known’ by the prediction without any measurement.

Proposal 4: Knowing the Rx beams is a crucial factor in TCI state switch enhancement in terms of RS prediction, it may be supported by the UE as UE capability.

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3dB

Proposal 5: The ‘predicted known’ conditions of the prediction-based TCI state switch shall take below conditions into account:

- The TCI state switch command is received during valid inference phase of AI/ML mode.

- The predicted beam in Set A is the RS in target TCI state.

- The predicted beam in Set A is detectable during training and inference phase, which implies the beam has been trained successfully and available in inference phase.

- The UE knows the Rx beam for the predicted beam.

Proposal 6: The ‘predicted known’ conditions doesn’t apply to the RS which has been measured in prediction procedure, including measurement phase and prediction phase, either in Set A or Set B.

Issue 3-4: Metrics/KPIs for beam prediction

Beam prediction KPIs Different metrics/KPIs have been discussed and were captured in the TR: For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy -Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" -Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" -Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, -Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

Proposals - Option 1: Use Option 1 - Option 2: Use Option 2 - Option 3: Use combined Option 2 and 3: - Option-New: The successful rate for the correct prediction which is considered as "the Top-1 predicted beam is one of the Top-K strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB” - FFS: K = 2 and x = 3 as baseline - Option 4: Add new metric: - Top-K/N (%): the percentage of "the Top-N genie-aided beam is one of the Top-K predicted beams". FFS how N is derived. - Option 5: Add a requirement based on relative difference between predictions under different conditions/inputs (for example varying signal levels by N dBs but maintain same relative difference). - consider defining multiple tests under different conditions but with the same outcome(same predicted beam) - Option 6: both RSRP accuracy and beam prediction accuracy are used as KPIs for beam prediction - RSRP absolute accuracy applies to all predicted beams - Recommended WF - To be discussed Companies suggesting to use different metrics(new) should come up with a concrete proposal

We consider the metrics/KPIs including the options as follow:

- In the case of the AI/ML model with label (beam ID) output type, the metrics/KPIs for beam management requirements/tests are derived by the beam prediction accuracy, i.e., the UE shall only fulfil beam prediction accuracy, i.e., Option 2 + Option 3.

- In the case of the AI/ML model with label (beam ID) + RSRP output type, the Metrics/KPIs for beam management requirements/tests shall take both of the beam prediction accuracy and RSRP accuracy into account, i.e., the UE shall fulfil beam prediction accuracy and RSRP accuracy simultaneously, i.e., Option 1 + Option 2 + Option 3.

Proposal 7: Metrics/KPIs for beam management requirements/tests are outlined by the below options:

- In case of the AI/ML model with label (beam ID) output type

  - Beam prediction accuracy, i.e., Option 2 + Option 3 is adopted.

- In case of the AI/ML model with label (beam ID) + RSRP output type

  - Beam prediction accuracy and RSRP accuracy, i.e., Option1, 2 an 3, are adopted simultaneously.

Proposal 8: In Option2, RAN4 only considers the below option.

- Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Issue 3-6:	RSRP Error modeling in simulations

Initial simulation assumptions for assessing the impact that the measurement error has on prediction accuracy/ performance were agreed in the previous RAN4 meeting. The RSRP measurement error modeling should be further discussed/refined. - Proposals - Option 1: model RF error as same within an RF module but varies across modules, FFS the distribution - Option 2: Gaussian distribution with +/- 2.5dB for BB errors and total of +/-6.5 for BB+RF - Option 3: model RF error as - An offset value in the original dataset; - Truncated Uniform distribution; - Truncated Gaussian distribution. - Option 4: Model error as RF error + BB error - model RF error as independent for each Tx beams. FFS on the distribution and error margin - BB error with Gaussian distribution. FFS on mean and sigma. - Recommended WF - Model error as RF error + BB error - RF error as a fixed bias for each UE module, FFS which distribution - BB error with Gaussian distribution with 0 mean and sigma=0.6dB Should the BB error distribution be changed based on SNR?

In this section, we first simulated the LLS based on the simulation assumption in R4-2420480 and obtained the absolute values for L1-RSRP differences between the ideal and practical channel estimation, as listed in Table 1.

Table 1 The L1-RSRP differences of SSB

| SNR (dB) | L1-RSRP differences (dB) |
| --- | --- |
| -15 | 2.58 |
| -12 | 1.48 |
| -9 | 0.87 |
| -6 | 0.55 |
| -3 | 0.38 |
| 0 | 0.26 |
| 3 | 0.18 |
| 6 | 0.13 |
| 9 | 0.09 |

From the table we can see that even though the RSRP differences can be up to 2.5 dB with a worse channel condition, such as SNR = -15 dB, the L1-RSRP differences is around 0.38 dB with the suggested SNR value -3dB, which could have minor impact on the performance metric compared to the +/- 4.5dB RF error on prediction accuracy.

Observation 2: The L1-RSRP differences of SSB with the SNR value -3 is only a few tenths of dBs so its impact would be minor as the baseband (BB) error.

We then generated the BB error at SNR 0 dB with TDL-C channel assumption. Results show that the BB error distribution follows the Gaussian distribution as the below figure shows, with zero mean.

Figure 1 BB error distribution of SSB L1-RSRP with 0 dB SNR

Therefore, we support to model the BB error with Gaussian distribution if the BB error is applied into the measurement error. The sigma value can be further discussed after the BB error distribution is confirmed.

Proposal 9: The distribution of baseband error can be fitted with zero mean Gaussian distributions.

Issue 3-7: Measurement error impact evaluation

In the previous RAN4 meeting it was agreed to conduct a study on the impact of measurement errors. Based on the submitted simulation results it should be discussed what to do next. - Proposals - Option 1: Check on simulation results - Option 2: simulation assumptions change? What to do next? - Option 3: Whether training/inference dataset contains error or not - is there a need to model errors for both training data or only for inference? - Option 4: SLS results should be considered first and then LLS simulations can be performed if needed. - Option 5: others - Recommended WF - To be discussed

The impact of the measurement errors on the Top-K/1 prediction accuracy is shown in Table 2 when K is 1,3,5. In addition to the parameter settings mentioned in the R4-2420480, 1 dB margin is considered in the beam prediction accuracy. Same RF errors are applied for each Tx.

Table 2 The impact of RF errors on AI/ML prediction accuracy for Top-K/1

|  | Prediction accuracy no measurement error | Prediction accuracy with RF errors +/-4.5dB (Gaussian) | Prediction accuracy with RF errors +/-4.5dB (Uniform) |
| --- | --- | --- | --- |
| Top-1/1 | 94.3 | 85.7 | 79.4 |
| Top-3/1 | 98.3 | 98.2 | 96.7 |
| Top-5/1 | 99.3 | 99 | 98.7 |

From the table, by considering ±4.5 dB RF error with Gaussian distribution and Uniform distribution, the beam prediction accuracy degrades ~10% (from 94.3  85.7) and ~18% (from 94.3  79.4) respectively, in terms of Top-1 beam prediction accuracy when comparing to the one without measurement error. While its impact on Top-3 is relatively small and can even be neglectable for Top-5 case.

Observation 3: Both ±4.5 dB RF error with Gaussian distribution and uniform distribution brings large performance degradation on prediction accuracy especially for Top-1 beam, where the impact of uniform distributed RF error is even worse.

The 90%-tile L1-RSRP difference between the maximum RSRP of the Top-1 predicted beam(s) and the RSRP of the genie aided strongest beam is also evaluated. Results show that ~5.18 dB degradation can be caused by ±4.5 dB RF error with Gaussian distribution. Therefore, RAN4 should further discuss whether the degradation is acceptable or not.

It might be worth to mention that, the 90%-tile L1-RSRP difference mentioned in the R4-240480 is defined as “90%-tile L1-RSRP difference = max(abs(95%-tile L1-RSRP), abs(5%-tile L1-RSRP)) ”. In our understanding, the “95%-tile L1-RSRP” and “5%-tile L1-RSRP” should be the “95%-tile L1-RSRP difference” and “5%-tile L1-RSRP difference”. It is suggest to clarify this metric during the meeting.

Proposal 10: The definition of “90%-tile L1-RSRP difference” should be clarified, such as:

90%-tile L1-RSRP difference = max(abs(95%-tile L1-RSRP difference), abs(5%-tile L1-RSRP difference))

Proposal 11: The performance degradation from ±4.5 dB RF error is not neglectable, further discussion is needed for whether accept this performance degradation or not.

Issue: Requirements for beam predictions

Agreement: - Requirements to be defined: - Measurement period for prediction (how long will set B be measured before making a prediction) - FFS on whether the legacy measurement period can be reused or not. - Prediction accuracy - FFS how/what to define depending on the chosen KPI (e.g. predicted RSRP, beam ID, Top-K/1 or Top 1/K, etc) - TCI state known/unknown - FFS on whether the other requirements are needed

In this issue, we share our views on the first issue ‘measurement period for prediction (how long will set B be measured before making a prediction)’.

To our understanding, the question shall be separated for BM-case 1 and BM-case 2. Based on the understanding of RAN1 studies, for BM-case1, one shot L1-RSRP measurement result on Set B is enough for the UE to predict Set A, for BM-case 2, a UE may measure Set B for multiple L1-RSRP measurement periods/occasions, and the exact number of the occasions may be configurable, depending on the RAN1 study results.

Proposal 12: Regarding measurement period for prediction (how long will set B be measured before making a prediction),

- For BM-case1, one shot L1-RSRP measurement result on Set B is enough for the UE to predict Set A.

- For BM-case 2, the UE may measure Set B for multiple L1-RSRP measurement periods/occasions before prediction. The exact number of the periods/occasions may be configurable which is to be studied and defined in RAN1. RAN4 is only aware of measurement accuracy for each L1-RSRP measurement.

Issue 3-5: Prediction delay requirements

- Proposals - Option 1: Prediction delay requirements to include: - Measurement for prediction - Inference delay - Other components are FFS - Option 2: Do not define prediction delay, the measurement delay is enough - Option 3: others - Recommended WF - Option 1 It was also proposed to include the reporting delay in the requirement but so far reporting delays are not included in the RRM requirements

First of all, the term prediction delay is an ambiguous term regarding the online discussion results in the last meeting, which shall be clarified.

Let’s start from the existing requirements, in the legacy way, when configured by the network, a UE shall be able to execute the L1-RSRP measurements of configured CSI-RS, SSB or CSI-RS and SSB resources for L1-RSRP for reporting. To the end, there are two requirements are defined for the purpose.

- Measurement Reporting Requirements are defined in Clause 9.5.3 of TS38.133, which actually only captures some requirements with respect to CSI-measconfig, no delay is defined.

- Measurement delay normally refers to Clause 9.5.4 of TS38.133, ‘When groupBasedBeamReporting-r17 is not configured, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB.’. Given that, the requirement derives the minimal delay in which the UE shall be able to complete an entire L1-RSRP measurement.

However, those requirements may be different in the predication case. Before digging into the details, we list all delay variants concerning measurement and inference phases in prediction in Figure 2.

Note: those variants in the figure are not agreed in 3GPP, just for interpretation demonstration.

Figure 2 Timeline of measurement and inference phase in prediction

It’s noted that BM case 1 and BM case 2 may have different set A and set B configurations. Also, the length of measurement/inference for BM case 1 and BM case 2 are typically different, e.g.,

- m, n for BM case 1 typically equal to 1, i.e., one-shot measurement and one-shot inference.

- On the contrary,  m, n for BM case 2 typically are larger than 1, for tracking beam changes.

- Wherein, m, n is the number of reference signal measurement occasions or predication occasions respectively. Normally, the exact number of m, n shall be configured by the NW or up to UE implementation/capability or predefined in specification.

It’s noted that, as illustrated in Figure 1, after measurements, the beam report, which could be an extension of CSI-ReportConfig, is for BM-case 2, indicates the prediction results in the future time instants.

It’s noted that no beam report for BM-case 1 is explicitly shown in Figure 1, the UE reporting position may rely on configurations on CSI-ReportConfig, e.g., periodic, semi-periodic, aperiodic and so on.

By the way, we suppose the timeline has the generalization to cover both BM cases.

It is assumed that the purpose of the measurement delay to be addressed in the issue is getting L1-RSRP measurement for prediction, where the prediction may be for beam (ID) prediction or L1-RSRP prediction.

Moreover, referring to Figure 3, the starting point of the overall measurement and inference actually is from Step 6, which represents UE activating AL/ML model upon reception of the inference signaling in Step 5.

Figure 3 Signaling procedure for functionality applicability reporting

Regarding measurement delay, which may be defined by one of the below options:

- T1: delay from receiving prediction configurations, e.g., set A/B configuration, to the completion of all measurement occasions, e.g., completion of the ‘m’th measurement occasions.

- T2: delay of single/one shot measurement occasion. Note: it is identical to measurement delay in legacy.

- T3: delay of all measurement occasions for prediction, including m measurement occasions.

Firstly, as T1, the delay from reception of prediction configurations in legacy way similar to measurement configurations until the first measurement occasion or the last measurement occasion, is defined in RAN1 not RAN4. Apparently, we can preclude it.

Secondly, in our opinion, the more than one measurement occasion for prediction concerned by T1 and T3, typically is not for obtaining an accumulated measurement accuracy, but for sort of beam characteristics analysis which is proprietary process by AI/ML model vendors, e.g., UE’s movements for BM-case 2.

Regarding measurement accuracy, a certain accuracy requirement shall be met for each L1-RSRP measurement delay either for BM-case 1 and BM-case 2. In this way, we don’t need to further restrict the accumulated measurement accuracy averaged on multiple measurement occasions provided the accuracy of each measurement occasion is guaranteed. That’s also the reason why m=1 is assumed for BM-case1.

RRM requirements only takes care of the minimal delay of the UE being capable to complete an entire L1-RSRP measurement, it is T2 which is identical to the legacy one.

Maybe a potential requirement for BM-case 2 is evaluating the capability of a UE tracking changes of L1-RSRP, but it is concerned to capture such requirement in RRM specification.

The below may be regarded as options of reporting delay (for BM-case2):

- T4: delay from receiving prediction configurations to sending prediction report.

- T5: delay from the first measurement to sending prediction report.

- T6: delay from the last measurement to sending prediction report.

As legacy, we don’t take reporting time into the delay definition, but we can define measurement reporting requirements, similar to the legacy ones, if needed.

Regarding inference delay, which may be defined by one of the below options:

- T7: delay from sending prediction report to the first predicted reference signal occasion.

- T8: delay from the last measurement to the first predicted reference signal occasion.

The reporting time is subject to be changed according to NW configuration, especially it is unable to be defined for BM-case1. Instead, RRM requirements shall be aware of the minimal delay from UE’s capability perspective. It appears that T8 may be a proper choice since it evaluates a UE processing capability based on the measured reference signal samples.

- For BM-case 1, since m=1, the inference delay is started from the completion of the one-shot measurement occasion.

- For BM-case 2, the inference delay is started from the completion of all measurement occasions provided m > 1.

It’s noted that a possible prediction implementation is that the (partial) prediction procedure may start from the completion of the first measurement occasion in an iteration way. In this case, inference delay may be started from the first measurement occasion not the last measurement occasion.

Meanwhile, it’s noted that RAN1 #118bis meeting achieved some agreements on the reference time of the earliest time instance as follow.

Agreement For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following alternatives for potential down-selection: - Option 1: Based on the uplink slot for the report - Option 2: Based on the CSI reference resource corresponding to the report - Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource

In our opinion, the earliest time instance defined by RAN1 is a time offset in terms of the report, which doesn’t impact RAN4 requirements on the time delay from measurement to inference.

Observation 4: The RAN2 defined ‘the reference time of the earliest time instance for the predicted results’ for BM-Case 2 of UE-side model, which is from the prediction report, isn’t relevant to RAN4 requirements on inference delay.

Proposal 13: Measurement delay is defined as the delay of single/one shot measurement occasion, which is identical to measurement delay in legacy, it is same to BM-case 1 and BM-case 2.

Proposal 14: RAN4 to study if any requirement is needed for evaluating the capability of a UE tracking changes of L1-RSRP by measurements in BM-case 2.

Proposal 15: Don’t take reporting time into the delay definition, but we can define measurement reporting requirements similar to the legacy ones, if needed.

Proposal 16: Inference delay is defined as the processing delay from the completion of the last (or first) measurement occasion (for BM-case 1, only one-shot measurement occasion) until the start of the first predicted reference signal occasion.

Observation 5: For beam (ID) prediction and L1-RSRP prediction, the inference delay may be different.

Issue: Performance monitoring

Based on RAN1’s agreements, we may focus on Type 1, Option 2 of UE-side model monitoring.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

In RAN WG1 #118, the below agreements were achieved.

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

In RAN WG1 #118bis, the below agreements were achieved.

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring: - Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference - FFS: the resource set(s) for monitoring - UE measures the resource set(s) for monitoring. - FFS how/when to report the monitoring results. - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The dedicated report configuration used for monitoring links to an inference report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - UE measures the resource set(s) for monitoring. - FFS when to report the monitoring results.

In RAN WG1 #119, the below agreements were achieved.

Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

Our understanding is, for BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), UE shall be able to compare the difference between the prediction results and the measurement results on a resource set/resources for monitoring.

Based on RAN1 agreements up to today, we don’t see the essential difference between measurement/inference delay for prediction and measurement/inference delay for monitoring, since the metric is same, e.g., Top 1 or Top K beam prediction accuracy and no proposal or agreement indicates the difference. But the final decision of metrics could be different considering their respective purposes.

Proposal 17: RAN4 shall study measurement/inference delay for monitoring.

Observation 6: The metric/report type of prediction results, prediction accuracy or L1-RSRP offset, for prediction and the metric for monitoring may be different since they serve different purposes.

## Testability and interoperability

Issue 3-9: AoA/AoD:

- Proposals - Option 1: exclude test setup with single AoA(single probe) - Option 2: further study single AoA setup: - Companies should bring further analysis on relevance of such a test to beam prediction and how it would relate to field deployments - Option 3: other - Recommended WF - Option 1 If option 2 is preferred, it should be clarified what will be further studied and how

To our understanding, spatial characters, e.g., TX beams in spatial distribution, in CDL channel is an unneglectable factor in AI/ML model.

However, as per the recommendation by moderator in the last meeting, R4-2418941(R&S) indicates the concern that the emulation of multiple beams in TE needs tremendous probes, like 8 beams <->100 probes and when the beam number is 64, the probe number grows up to thousand level accordingly, it’s unrealistic and even simplification on CDL channel can’t help.

On the contrary, we may set a number of probes with a proper designed distribution, e.g., uniformly spaced on Theta and Phi axis. As demonstrated in Figure 4, which is quoted from R4-1706669, the probes are installed in the form of a sector of angles with approximately equal distance R from UE and with certain angular spacing.

Figure 4 Probes with a proper designed distribution

The purpose is to limit the directions of TX beams and AOA/AOD, only probe direction can be utilized for their injections. By this, the simplified CDL channel may work.

Observation 7: If the mapping between beam number and probe number, like 8 beams <->100 probes, is valid, it’s difficult to do multiple beam (up to 64) emulation by CDL channel.

Proposal 18: We may set a number of probes (e.g., in the order of hundreds or dozens, if it is available) with a proper designed distribution, and TX beams are only injected to those probes, e.g., with granularized AOA/AOD, the CDL channel may be simplified (depending on the probe number) and but still can provide enough spatial characters for AI/ML model conformance test.

Proposal 19: For evaluating CDL channel or simplified CDL channel, RAN4 to study how many probes can be implemented in TE in a realistic way.

Back to single AOA issue, if we arrange multiple Tx beams after channel model emulator to feed into the single probe, the flow is as what Figure 1 demonstrates.

- For TDL channel, no RX beam gain issue exists, after channel modelling for each Tx beam, the outputs are combined to get the combined signal.

- For CDL channel, due to Rx beam gain, after channel modelling for each Tx beam and Rx beam gain, the outputs are combined to get the combined signal.

In case of CDL channel, the final signals entering AI/ML model are the combination of RSRPs, which have gone through artificial spatial filtering as represented in Figure 5. However, from the standpoint of the AI/ML model input, the Rx beam gain cannot represent the UE capability and does not significantly differ from the TDL channel because it’s not the actual one implemented by the UE. The only effect of artificial spatial filtering is that the RSRPs of different Tx beams are different in the combined RSRP signals. Such that it is a compromising solution which can’t assess the prediction's total competence.

Figure 5 Single AOA flow

Proposal 20: For single AOA/probe, adding CDL channel and Rx gain function in TE before combined and fed into the single probe may be a compromise approach, but the effect of testing prediction performance may not be expected.

Issue 3-10: Beam prediction test segregation

- Proposals - Option 1: Proposal 1: Segregate CDL testing for AI/ML BM in a first single-AoA test with CDL emulation with known UE Rx Beam/Antenna pattern (referenced / provided / measured) and necessary number of beams, and in a second multi-AoA test for UE Rx Beam sweeping (channel TBD). - Option 2: Do not further discuss such proposal as it is not feasible: - TE does not know the UE beam pattern, measuring it will be very time consuming anyway - Artificial segregation will not work with a UE model which takes into account the UE Rx spherical coverage - Option 3: further study - Should be clarified what should be further studied - Recommended WF - Option 2 If further study is needed, it should be discussed what analysis is needed

We have some reservations about the goal of a second multi-AoA test for UE Rx beam sweeping after reading the relevant proposals.

From high level perspective, the target of the second test is to identify the proper Tx beam by UE Rx beam sweeping. And the output is the ‘highest’ detected RSRP which shall be adopted by the AI/ML model test. However, it’s unclear how to map the relationship between the RSRP detection performance and the prior AI/ML model performance?

Proposal 21: We don’t oppose the segregate test, but before proceeding with the directive, we must respond to at least the following questions:

- What’s the target of the multi-AoA test for UE Rx Beam sweeping?

- What’s the test metric of the multi-AoA test for UE Rx Beam sweeping?

- How to map the test metric/result of the multi-AoA test for UE Rx Beam sweeping and the test metric/result of single-AoA test for AI/ML model?

Issue 3-11: Channel model for beam prediction

- Proposals - Option 1: multi-AoA TDL - Option 2: simplified CDL - Option 3: further study both multi-AoA TDL and simplified CDL - Option 4: CDL without simplification - Option 5: others - Recommended WF - To be discussed whether the channel model can be downscoped to Option 1 or Option 2. If Option 3 is preferred, it should be clarified what needs to be analyzed further. Aspects to be analysed: - How to simplify the CDL channels. Is it necessary to stick to currently used CDL models(CDL-A,.., CDL-D) or can the channels be further simplified to just a few clusters - How many spatial beams can be generated with multi-AoA TDL(can the TE emulate beams reaching the UE between the AoDs) and what is the relationship between angular distance between the probes and emulated beams/codebook. - Others?

The definition of multi-AoA TDL is vague, at the least it’s not described in detail. In our understanding, it’s a multi-AOA set up by Enhanced IFF TE. As demonstrated in Figure 6, one Tx beam after TDL is fed into and reflected from each AOA, however it results in that no spatial coherence exists among AOAs, in other words, a UE receives a single and separated Tx beam from each AOA by Rx beam sweeping.

Figure 6 Enhanced IFF OTA test system

Observation 8: Multi-AoA TDL, may be regarded as a particular oversimplified CDL channel, but only one cluster for each beam.

Observation 9: The maximum number of Tx beams in multi-AoA TDL is unclear.

The simplified CDL channel may adapt the TE implementation, especially the maximum allowed probe number. As aforementioned in the above content, the clusters in simplified CDL channel are granulated with respect to the probe distribution.

Proposal 22: CDL channel may be simplified by granulating the clusters in channel with respect to the probe distribution in TE.

The simplified CDL channel essentially can be applied for training and inference. However, what we shall specify with common test configurations is inference only, since training isn’t in the scope of the conformance test, such that we don’t expect to compare the simplified CDL channel and the reference CDL channel in training, in other words, the training data may be done by applying the reference CDL channel.

Proposal 23: The simplified CDL channel only is considered to be applied in inference, no need to be applied in training.

Proposal 24: The metric to estimate the “goodness” of a simplified CDL channel model may be represented by the below steps:

Step 1: Test (inference) with the reference CDL channel.

Step 2: Test (inference) with the simplified CDL channel.

Step 3: The prediction difference between the results of the two tests in the above steps is less than a threshold with [95] percentile.

Issue 3-12: Test setup requirements

- Proposals - Option 1: - set B Tx beams: baseline 8; set A Tx beams: baseline 64 - Does the TE need to broadcast all Tx beams in the same test? - UE rotation during the test - multiple TRP (or reflections from another direction) - AoA – to be discussed later - AoD – to be discussed later - Option 2: - other parameters - Recommended WF - To be discussed

We’d like to draw attention to the ratio between the numbers of Set A and B, instead of directly discussing the achievable maximal beam numbers from implementation or test perspective.

First of all, we’d like to raise the below questions:

- What AI/ML model performance or characteristics shall be verified by the AI/ML model test?

- What impact to the prediction performance may be caused by the beam number varieties (the ratio between the numbers of Set A and B) in the model?

Since we have not exploited the unclearness, we don’t think it’s ready to define the exact numbers of Set A and B for the conformance test.

In our opinion, the test shall at least verify the following concerns on the numbers of Set A and B,

- Prediction scaling, i.e., the capability of using fewer inputs to predict same outputs, or the capability of using same inputs to predict more outputs.

- Prediction model computational power/complexity, i.e., the capability of handling more inputs or outputs.

Observation 10: Different ratios between the number of Set A and Set B demonstrates various AI/ML model characteristics.

Since the two concerns take care of different AI/ML model characteristics, in order to get full evaluation results, eventually more than one set of number of Set A/Set B shall be defined in the conformance test in perspective.

Proposal 25: For BM-case1, at least two sets of numbers of Set A and Set B are provided as test configurations. Below are two examples:

- For verifying the prediction model computational power/complexity of AI/ML model, the number of set A = X and the number of set B = X/4.

- For verifying the prediction scaling capability of AI/ML model, the number of set A = X and the number of set B = X/16

Note 1: Different accuracy requirements may be applied for different sets of number of set A and set B.

Note 2:  X may be the maximum number of Set A that a UE supports mandatorily.

Issue 3-13: Baseline test setup

There are 2 competing proposal for baseline test setup, these need further discussions - Proposals - Option 1: Define eIFF system, including potential upgrades, as the baseline system for AI/ML BM testing, since it is feasible and supports single-AoA CDL and multi-AoA TDL, as well as all RF / Demod / RRM legacy tests (one system fits all, side condition fulfilled), allows black box testing for beam sweeping (big quiet zone) and precise UE measurements (low MUs). - Option 2: Consider an 8-probe OTA system with 2 additional probes when compared to the FR2 MIMO OTA system with a range of AoAs/ZoAs of 110°/25° as the starting point for AI/ML Beam management testing for 1 TRP and/or 2 TRPs with close angular proximity. - Option 3: single AoA test setup - Option 4: others - Recommended WF - To be discussed

We quote the content supporting option 2 from R4-2419391 in the below.

a relatively simple and straightforward extension of an FR2 MIMO OTA system to 8 probes with a sample system visualized in Figure 1, is suitable to meet the following assumptions for a single TRP or two TRPs with close angular proximity: Emulation of Tx-beam powers and RX-PAS for 4 Tx-beams (dual polarization) [6] or 8 Tx-beams (single polarization) for CDL-A, B, C, D and E UMi/UMa/InO scenarios. - The emulation of Tx-beams with accurate RX-PAS AoA profiles to take into account the strongest beams at the center of the test zone. - Additionally, to emulate set B Tx beams beyond 4 [up to 16] while exercising the AI/ML algorithms, power offsets can be applied to the Tx-beams with the closest match of the 4 available RX-PAS profiles and exact power offset based on the 3D beam and CDL model characteristics - More importantly, to emulate set A Tx beams [up to 128], power offsets can be applied to the Tx-beams with the closest match of the 4 available RX-PAS profiles and exact power offset based on the 3D beam and CDL model characteristics

We understand that the proposals try to prove that 4 Tx beams through CDL channel can be realized by 8 probes. And for N Tx beams, in an example, N/4 beams are fed into same probe but with power offsets among the N/4 beams.

Proposal 26: RAN4 shall study the mapping between the Tx beam number and the probe number, if the ratio 1:2 is feasible?

Observation 11: Option 2, i.e., 8-probe OTA system, could oversimplify the test channel. For supporting such approach, we lead towards that more probes can be implemented in TE.

Issue: Beam prediction testing process

- Proposals - Option 1: Test needs to have at least 2 parts: - 1. inference part of the test (UE has to make the actual prediction) - 2. predicted beam measurement (UE has to measure and report “the ground truth”) - Option 2: others - Recommended WF - Take Option 1 as baseline, discuss further details of tests such as under what condition should the ground truth be measured, how to ensure the UE does not cheat, etc.

Below are some preliminary analyses on the condition of ground truth, e.g., the SNR condition.

The purpose of defining the SNR condition is to get an ideal RSRP measurement accuracy by minimizing (most of) measurement errors caused by low SNR. Hence, determining the SNR condition may rely on the configurations of reference signal, e.g., periodicity, density, bandwidth, subcarrier spacing and channel model in order to obtain a SNR level which is applicable for most typical field scenarios.

Recap the procedure deriving the accuracy requirement on the measured L1-RSRP in Rel-15, the achievable accuracy at certain SNR level is derived by simulations. In a same way, it is rational to extend those simulations to a higher SNR level where the L1-RSRP estimation error can be less than a threshold with X percentile.

Generating such SNR level in the test environment is another issue to be addressed, a concern is that the realization of SNR level is not only about generating a proper signal level but also the noise floor level generated as a side effect needs to be considered. To realize high SNR, the probes in test environment shall be able to generate signals at high output power level over the air. An underlying problem is that the wanted signal level can sometimes become too high to be created in the OTA environment. In that case, the noise floor becomes dominant which may deteriorate the test performance. Test equipment vendors are encouraged to comment on the SNR level from a test environment perspective.

Proposal 27: RAN4 to evaluate the SNR condition for the ground truth through simulation.

If the ideal measurement of RSRP is acquirable, then the test procedure shall at least comprise the below three steps:

- Acquire the predicted RSRP under the required corresponding SNR condition

- Acquire the ideal measurement of RSRP under high SNR condition

- Compare the two RSRPs acquired in the above steps

Still, arranging the details on the sequence and test configuration may need further investigation. In essence, to our understanding, the UE is deemed to execute the RSRP measurement and prediction with same test configurations except for different SNR conditions.

Proposal 28: Test procedure shall at least comprise the below three steps:

- Acquire the predicted RSRP under the required corresponding SNR condition

- Acquire the ideal measurement of RSRP under high SNR condition

- Compare the two RSRPs acquired in the above steps

Note 1: Test configurations are kept same in the steps except for different SNR conditions.

Note 2: Arranging the details on the sequence and test configuration may need further investigation in performance part.

Yet another issue is, at least one company in the last meeting commented that since the ideal measurement of RSRP and the predicted RSRP both are provided by the UE under test, the UE may deliberately modify the two values to be better aligned in the report. On the contrary, the test environment hasn’t any available approach to evaluate the authenticity of the measured and predicted values. We agree on the observation and the issue shall be solved.

A simple way for the test environment is adding a procedure manually changing the beam index between the prediction duration and measurement duration. By this way, the UE reports the results from the corresponding prediction and measurement with the corresponding beam indexes, but only the test environment has knowledge of the beam index mapping between each other.

By this way, the UE reports the results from the prediction and measurement with the corresponding different beam indexes for the same beam, while the UE doesn’t know the relationship between the beam indexes, only the test environment has the knowledge of beam index mapping between each other.

Proposal 29: How to avoid cheating in the reports on RSRP measurement and prediction is a valid issue to be studied. A simple way for the test environment is adding a procedure to manually switch the beam indexes between the prediction duration and the measurement duration.

Issue: OTA Test setup for beam management

In this issue, we discuss the setup of channels to be applied in measurement and inference.

For BM-case1, which predicts the best beams in Set A (different from Set B) with respect to the measurements on Set B, at the same time domain with respect to spatial correlation between Set B and Set A. Since the time delay of measurement delay and inference delay is in dozens or hundreds of milli-seconds order, there’s no chance to rearrange random distributions and variations between measurement and prediction, the channel for measurement and prediction have to apply a same channel.

For BM-case2, which harnesses the historical predictable non-random variation of Set B to anticipate Set A (same as Set B) for one or more future time instances with respect to temporal correlation between Set B and Set A, from test perspective, it needs some trackable changes, e.g., UE monotonous movements which reflect the temporal correlation as prerequisite of the temporal prediction.

A question is relevant to measurement window and the inference window, which are configured by NW, may have respective minimal length with respect to a certain prediction performance. Such that a set of consequent changes of channel parameters shall be designed during measurement window and the inference window. At the least, establishing a set of channel instants from channel #a+1 to channel #a+m for measurement window and channel #b+1 to channel #b+n for inference window.

These channels contain a set of parameters which update sequentially and gladfully starting from measurement until end of inference. The parameters wherein to be updated may comprise one or more than one of the below:

- Departure angles (AOD, ZOD)

- Arrival angles (AOA, ZOA)

- UE position and/or rotation

- Channel coefficient

In an illuminative example, for BM-case 2, the updates of parameters, in order to mimic the UE’s movement, may include the below:

- AOD in channel #1 is 10 degrees,

- AOD in channel #2 is 20 degrees,

- …,

- AOD in channel #k is (10*k) degree

Proposal 30: Same channel is applied for measurement and inference in BM-case1.

Proposal 31: For BM-case2, plural channel instants with a series of changes of the channel parameters are necessary to reflect temporal correlation between channels. At least, it includes:

- Generating a set of channel instants, from channel #a+1 to channel #a+m, for measurement, and

- Generating a set of channel instants, from channel #b+1 to channel #b+n for inference

- Where,

  - The channels instants comprise sequent and grade updates of a set of parameters.

  - The numbers of m and n depend on the measurement window and prediction window of BM-case2.

  - The channel parameters may comprise one or more than one of

    - Departure angles (AOD, ZOD)

    - Arrival angles (AOA, ZOA)

    - UE position and/or rotation

    - Channel coefficient

Issue 2-6:	Datasets for training/testing

Agreement: Vendors may take into account the test environment/conditions defined by RAN4 when training the UE - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training FFS on proposals to augment training data to avoid overfitting of UE models to the test environment RAN4 to strive to make the test conditions similar to field deployment conditions

The rationale behind the agreements on the issue is that the coverage of datasets for training/testing shall be broad adequately to meet or be close to the practical field deployment scenario as much as possible. In this manner, we suggest considering defining the test environment/conditions which can demonstrate the robustness of the AI/ML model.

Proposal 32: In order to avoid overfitting UE models to the test environment only, the test (including data sets) shall be able to demonstrate the robustness of the AI/ML model and match the practical field deployment conditions.

Since the training data are uniquely affected by the specific UE hardware and digital processing units, the error of training data could be left for implementation from each UE vendor, the effect to the predict accuracy may be covered by the overall measurement error.

Observation 12: the measurement error in datasets for training/testing may be covered by the overall measurement error.

# Conclusion

Observation 1: Option 1 of the relative RSRP accuracy is somewhat closer to a beam label (beam ID) prediction than an RSRP prediction.

Observation 2: The L1-RSRP differences of SSB with the SNR value -3 is only a few tenths of dBs so its impact would be minor as the baseband (BB) error.

Observation 3: Both ±4.5 dB RF error with Gaussian distribution and uniform distribution brings large performance degradation on prediction accuracy especially for Top-1 beam, where the impact of uniform distributed RF error is even worse.

Observation 4: The RAN2 defined ‘the reference time of the earliest time instance for the predicted results’ for BM-Case 2 of UE-side model, which is from the prediction report, isn’t relevant to RAN4 requirements on inference delay.

Observation 5: For beam (ID) prediction and L1-RSRP prediction, the inference delay may be different.

Observation 6: The metric/report type of prediction results, prediction accuracy or L1-RSRP offset, for prediction and the metric for monitoring may be different since they serve different purposes.

Observation 7: If the mapping between beam number and probe number, like 8 beams <->100 probes, is valid, it’s difficult to do multiple beam (up to 64) emulation by CDL channel.

Observation 8: Multi-AoA TDL, may be regarded as a particular oversimplified CDL channel, but only one cluster for each beam.

Observation 9: The maximum number of Tx beams in multi-AoA TDL is unclear.

Observation 10: Different ratios between the number of Set A and Set B demonstrates various AI/ML model characteristics.

Observation 11: Option 2, i.e., 8-probe OTA system, could oversimplify the test channel. For supporting such approach, we lead towards that more probes can be implemented in TE.

Observation 12: the measurement error in datasets for training/testing may be covered by the overall measurement error.

Proposal 1: For the absolute RSRP, the index i shall be any beam index in top-K beams based on predicted L1-RSRP as per UE reporting.

Proposal 2: For the relative RSRP, support Option 2. Relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest RSRP among the predicted beams.

Proposal 3:  RAN4 shall study the new known condition of the TCI state switch, to which the RS referred is which is ‘known’ by the prediction without any measurement.

Proposal 4: Knowing the Rx beams is a crucial factor in TCI state switch enhancement in terms of RS prediction, it may be supported by the UE as UE capability.

Proposal 5: The ‘predicted known’ conditions of the prediction-based TCI state switch shall take below conditions into account:

- The TCI state switch command is received during valid inference phase of AI/ML mode.

- The predicted beam in Set A is the RS in target TCI state.

- The predicted beam in Set A is detectable during training and inference phase, which implies the beam has been trained successfully and available in inference phase.

- The UE knows the Rx beam for the predicted beam.

Proposal 6: The ‘predicted known’ conditions doesn’t apply to the RS which has been measured in prediction procedure, including measurement phase and prediction phase, either in Set A or Set B.

Proposal 7: Metrics/KPIs for beam management requirements/tests are outlined by the below options:

- In case of the AI/ML model with label (beam ID) output type

  - Beam prediction accuracy, i.e., Option 2 + Option 3 is adopted.

- In case of the AI/ML model with label (beam ID) + RSRP output type

  - Beam prediction accuracy and RSRP accuracy, i.e., Option1, 2 an 3, are adopted simultaneously.

Proposal 8: In Option2, RAN4 only considers the below option.

- Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Proposal 9: The distribution of baseband error can be fitted with zero mean Gaussian distributions.

Proposal 10: The definition of “90%-tile L1-RSRP difference” should be clarified, such as:

90%-tile L1-RSRP difference = max(abs(95%-tile L1-RSRP difference), abs(5%-tile L1-RSRP difference))

Proposal 11: The performance degradation from ±4.5 dB RF error is not neglectable, further discussion is needed for whether accept this performance degradation or not.

Proposal 12: Regarding measurement period for prediction (how long will set B be measured before making a prediction),

- For BM-case1, one shot L1-RSRP measurement result on Set B is enough for the UE to predict Set A.

- For BM-case 2, the UE may measure Set B for multiple L1-RSRP measurement periods/occasions before prediction. The exact number of the periods/occasions may be configurable which is to be studied and defined in RAN1. RAN4 is only aware of measurement accuracy for each L1-RSRP measurement.

Proposal 13: Measurement delay is defined as the delay of single/one shot measurement occasion, which is identical to measurement delay in legacy, it is same to BM-case 1 and BM-case 2.

Proposal 14: RAN4 to study if any requirement is needed for evaluating the capability of a UE tracking changes of L1-RSRP by measurements in BM-case 2.

Proposal 15: Don’t take reporting time into the delay definition, but we can define measurement reporting requirements similar to the legacy ones, if needed.

Proposal 16: Inference delay is defined as the processing delay from the completion of the last (or first) measurement occasion (for BM-case 1, only one-shot measurement occasion) until the start of the first predicted reference signal occasion.

Proposal 17: RAN4 shall study measurement/inference delay for monitoring.

Proposal 18: We may set a number of probes (e.g., in the order of hundreds or dozens, if it is available) with a proper designed distribution, and TX beams are only injected to those probes, e.g., with granularized AOA/AOD, the CDL channel may be simplified (depending on the probe number) and but still can provide enough spatial characters for AI/ML model conformance test.

Proposal 19: For evaluating CDL channel or simplified CDL channel, RAN4 to study how many probes can be implemented in TE in a realistic way.

Proposal 20: For single AOA/probe, adding CDL channel and Rx gain function in TE before combined and fed into the single probe may be a compromise approach, but the effect of testing prediction performance may not be expected.

Proposal 21: We don’t oppose the segregate test, but before proceeding with the directive, we must respond to at least the following questions:

- What’s the target of the multi-AoA test for UE Rx Beam sweeping?

- What’s the test metric of the multi-AoA test for UE Rx Beam sweeping?

- How to map the test metric/result of the multi-AoA test for UE Rx Beam sweeping and the test metric/result of single-AoA test for AI/ML model?

Proposal 22: CDL channel may be simplified by granulating the clusters in channel with respect to the probe distribution in TE.

Proposal 23: The simplified CDL channel only is considered to be applied in inference, no need to be applied in training.

Proposal 24: The metric to estimate the “goodness” of a simplified CDL channel model may be represented by the below steps:

Step 1: Test (inference) with the reference CDL channel.

Step 2: Test (inference) with the simplified CDL channel.

Step 3: The prediction difference between the results of the two tests in the above steps is less than a threshold with [95] percentile.

Proposal 25: For BM-case1, at least two sets of numbers of Set A and Set B are provided as test configurations. Below are two examples:

- For verifying the prediction model computational power/complexity of AI/ML model, the number of set A = X and the number of set B = X/4.

- For verifying the prediction scaling capability of AI/ML model, the number of set A = X and the number of set B = X/16

Note 1: Different accuracy requirements may be applied for different sets of number of set A and set B.

Note 2:  X may be the maximum number of Set A that a UE supports mandatorily.

Proposal 26: RAN4 shall study the mapping between the Tx beam number and the probe number, if the ratio 1:2 is feasible?

Proposal 27: RAN4 to evaluate the SNR condition for the ground truth through simulation.

Proposal 28: Test procedure shall at least comprise the below three steps:

- Acquire the predicted RSRP under the required corresponding SNR condition

- Acquire the ideal measurement of RSRP under high SNR condition

- Compare the two RSRPs acquired in the above steps

Note 1: Test configurations are kept same in the steps except for different SNR conditions.

Note 2: Arranging the details on the sequence and test configuration may need further investigation in performance part.

Proposal 29: How to avoid cheating in the reports on RSRP measurement and prediction is a valid issue to be studied. A simple way for the test environment is adding a procedure to manually switch the beam indexes between the prediction duration and the measurement duration.

Proposal 30: Same channel is applied for measurement and inference in BM-case1.

Proposal 31: For BM-case2, plural channel instants with a series of changes of the channel parameters are necessary to reflect temporal correlation between channels. At least, it includes:

- Generating a set of channel instants, from channel #a+1 to channel #a+m, for measurement, and

- Generating a set of channel instants, from channel #b+1 to channel #b+n for inference

- Where,

  - The channels instants comprise sequent and grade updates of a set of parameters.

  - The numbers of m and n depend on the measurement window and prediction window of BM-case2.

  - The channel parameters may comprise one or more than one of

    - Departure angles (AOD, ZOD)

    - Arrival angles (AOA, ZOA)

    - UE position and/or rotation

    - Channel coefficient

Proposal 32: In order to avoid overfitting UE models to the test environment only, the test (including data sets) shall be able to demonstrate the robustness of the AI/ML model and match the practical field deployment conditions.

# References

- R4-2414323 WF on AI/ML, Qualcomm