3GPP TSG-RAN WG4 Meeting #114	R4-2501293

Athens, Greece, 17th – 21st, February 2025

Source:	Ericsson

Title:	Discussion on reply LS to RAN2 on soft satellite switch with re-sync

Agenda item:	5.21.3

Document for:	Discussion

# 1	Introduction

In RAN2 #128 meeting, a LS was sent to RAN4 and RAN1 [1]:

… # 1. Overall Description: For soft satellite switching with re-synchronization, to ensure non-colliding SSB with same PCI at the UE side, RAN2 has discussed one possible solution to introduce a temporarily shifted SSB pattern during [t-ServiceStart, t-Service]. To be specific, the target satellite transmits SSB during [t-ServiceStart, t-Service] which is not overlapping with SSB from the source satellite. At t-Service, the new serving satellite SSB pattern (e.g., SSB offset) is restored to the same as the previous source satellite. RAN2 wonders whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service. # 2. Actions: To RAN4, RAN1 ACTION: RAN2 kindly asks RAN4 and RAN1 whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service. …

In this paper we analyze the LS.

# 2	Discussion

The concern of the LS is whether a UE can re-obtain DL sync. on one consecutive shifted-back SSB occasion after one temporarily shifted SSB occasion, which timeline is demonstrated in Figure 1 in the below.

Figure 1: Shift back occurring around t-Service

In our opinion, the UE keeping the information of the shifted-back SSB location, e.g., ssb-TimeOffset, can capture the first shifted-back SSB occasion after T-Service for DL sync.

- As per T-Service definition in TS38.331, the time granularity of ﻿T-Service ﻿is 10 ms, and the ﻿exact stop time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field.

- ﻿As per ssb-TimeOffset in TS38.331, shifted SSB occasion is derived by the parameter that indicates the time offset between the SSB from source and target satellite at the uplink time synchronization reference point. It is given in number of subframes.

The two consecutive SSB occasions, the shifted one is before T-service, the shifted-back one is after T-service, shall at the least have several symbols distance in worst case (in one example, there exist more than one slot for the case of SCS 15KHz and 5ms SSB periodicity) if strictly following the field description, but ssb-TimeOffset actually will be configured with 5 or multiples of 5 subframe granularity to meet SSB pattern specifications.

From an implementation standpoint, a UE does not have to wait until after T-service to prepare DL sync following the SSB event. Since we assume a UE can manage it using a PDP-based method with a timing discrepancy of less than 50% of an OFDM symbol, the distance between the T-service and the SSB occasion after T-service is not considered an issue for time tracking.

Moreover, from synchronization state perspective, the UE is in a DL sync. state already before T-service, even the time sync. reference signal resources are changed, it doesn’t affect the DL sync. state, since the timing/frequency remain unchanged.

Observation 1: UE can handle timing tracking for the (first) shifted-back SSB occasion after T-Service, it doesn’t impact the DL sync. state that the UE is keeping.

In addition to checking the timing tracking capability, we shall consider the underlying impacts to RAN4 requirements. We observe the below RRM requirements may need to be updated.

##### 6.1C.3.2.3 Satellite switch delay for soft satellite switch with re-sync The Satellite switch delay is from t-serviceStart to the time instance when UE is ready to receive any DL channel/signal and transmit any UL channel/signal from/to the target satellite. When intra-frequency soft switch to NR SAN cell is commanded, the satellite switch time shall be less than Tsoft_switch Tsoft_switch = max(t-service-t-seviceStart, Tsearch + T∆ + Tmargin) + Tprocessing ms Where: - Tsearch is the time required to search the target NR SAN cell assuming the target cell is not already known when the handover command is received by the UE. If the target cell Es/Iot ≥ -2 dB, then Tsearch = Tfirst_SSB ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. - T∆ is same as the one defined in section 6.1C.2.2.2.1. - Tprocessing is time for UE processing. Tprocessing can be up to 10 ms. - Tmargin is same as the one defined in section 6.1C.2.2.2.1. - Tfirst_SSB is is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from [SSB-TimeOffset reference point as defined in 38.331 [2]] to UE. During the time period from t-seviceStart to t-service, scheduling restriction as defined in clause 9.2C.5.3 is allowed, with the exception that the locations of SSB symbols of target satellite where scheduling restriction applies are determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from the [SSB-TimeOffset reference point as defined in 38.331 [2]] to UE. UE is allowed to skip measurements for other cells and satellites than the target satellite and source satellite from t-seviceStart to the satellite switch completion. The requirement in this clause applies and UE is required to keep the connection (DL and UL) with the source NGSO satellite, under the following conditions: - SSBs from the two satellites are spaced apart from each other at least by 1 OFDM symbol in the time domain at UE Rx side.

Firstly, there is an issue to RAN4 about cell detection: for the target cell with shifted-back SSB occasions after T-Service, shall the UE assume the the target cell is already known or unknown after T-service if measurement during Tsearch has been done with temporarily shifted SSB before T-service?

In our opinion, if the UE has acquired the information of the shifted-back SSB location and the UE has measured the temporarily shifted SSB before T-service, the UE should presume the target cell is already known., since the only difference is the SSB location and there’s no signal quality/level change.

Observation 2: In satellite switch delay, for the target cell with shifted-back SSB occasions after T-Service, UE assumes the target cell is known provided the target cell with temporarily shifted SSB occasions is already known. Its reflection in specification is that Tsearch can be based on temporally shifted SSB prior to T-service or based on shifted-back SSB later than T-service.

Secondly, another issue is: shall satellite switch delay always comprise the first complete (shifted-back) SSB burst of target satellite for time/frequency tracking after T-Service?

We understand it’s doable but not mandatory. As was previously said, since the UE is in a DL sync. state already before T-service, even the time sync. reference signal resources are changed, which has no effect on the DL sync. state since the timing/frequency isn’t changed. As a result, timing tracking is operated by the UE, but satellite switch delay isn’t impacted. In other words, T∆  shall not be mandatorily added after T-service if timing tracking has been done with temporally shifted SSB before T-service.

Observation 3: In satellite switch delay, for the target cell with shifted-back SSB occasions after T-Service, T∆  can be based on temporally shifted SSB prior to T-service or based on shifted-back SSB later than T-service.

According to the above analysis, the requirements of satellite switch delay for soft satellite switch with re-sync aren’t impacted by the LS.

Proposal 1: The requirements of satellite switch delay for soft satellite switch with re-sync aren’t impacted by the LS.

# 3 	Summary

Observation 1: UE can handle timing tracking for the (first) shifted-back SSB occasion after T-Service, it doesn’t impact the DL sync. state that the UE is keeping.

Observation 2: In satellite switch delay, for the target cell with shifted-back SSB occasions after T-Service, UE assumes the target cell is known provided the target cell with temporarily shifted SSB occasions is already known. Its reflection in specification is that Tsearch can be based on temporally shifted SSB prior to T-service or based on shifted-back SSB later than T-service.

Observation 3: In satellite switch delay, for the target cell with shifted-back SSB occasions after T-Service, T∆  can be based on temporally shifted SSB prior to T-service or based on shifted-back SSB later than T-service.

Proposal 1: The requirements of satellite switch delay for soft satellite switch with re-sync aren’t impacted by the LS.

# References

[1] R2-2410978, LS on soft satellite switch with re-sync, Huawei

# Appendix draft LS reply

3GPP TSG-RAN WG4 Meeting #114		R4-xxxx

Athens, Greece, 17th – 21st, February 2025

Title:	Reply LS to RAN2 on soft satellite switch with re-sync

Response to:	R2-2410978

Release:	Release 18

Work Item:	NR_NTN_enh-Core

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact person:

Name:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1 Overall description

RAN4 would like to thank RAN2 for their LS R2-2405769.

From RAN4 perspective, since the UE experiencing soft satellite switch with re-sync is in a DL sync. state to the target satellite already before T-service, even the time sync. reference signal resources are changed (from the shifted SSB to the shifted-back SSB), it actually doesn’t affect the DL sync. state since the timing/frequency isn’t changed.

Given that, regarding RAN2’s question, the DL synchronization started between [t-ServiceStart, t-Service] is maintained even the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

# 2 Actions

To RAN2:

ACTION: 	RAN4 respectfully requests RAN2 to take the above feedback into consideration.

# 3 Dates of next TSG-RAN WG4 meetings

RAN4#114

RAN4#115