# 3GPP TSG-RAN WG4 Meeting #114	R4-2501299

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	5.12.1

Source: 	Ericsson

Title:	Remaining issues on MG further enhancement

Document for:	Discussion

# UE behaviour in NCSG and MG

In last meeting, companies proposed that the UE behaviour between deactivated SCell and after SCell activated are still unclear if NW configures both NCSG and MG.

### Sub-topic 1-1: Rel-18 UE behavior for deactivated SCell measurements with NCSG - Background: < Agreement >: Agreement from RAN4 meeting#111: - For UE configured with one NCSG and one Type 1/2 MG: All deactivated SCells are measured within NCSG, regardless of the reported UE capabilities [and gap association]. - Further details on the processing delay between NCSG and Type 1/2 MG can be further discussed. - For UE configured with 2 NCSG, deactivated SCells are measured with NCSG - If the association is provided, deactivated SCells are measured with NCSG according to gap association. - If the association is not provided, UE is not expected to cause interruption outside the VIL due to measurement on any of the deactivated SCells, and the existing measurement delay requirement does not apply to this case. Agreement from RAN4 meeting#112: - When UE switches measurement between NCSG and Type 2 MG, no additional processing delay is expected.. Issue 1-1-1: When the UE is configured with Concurrent gaps with NCSG, whether all deactivated SCells are measured within NCSG, regardless the gap association? - Further discuss in the next meeting. Issue 1-1-2: When the measurement of the measurement object is related to a deactivated SCell, and after this SCell is activated, and when UE is configured with the concurrent measurement gap and NCSG, what is the expected UE behaviour? - Further discuss in the next meeting

In Rel-17, deactivated SCell is agreed to be measured within NCSG instead of outside gap if NW configures NCSG. In RAN4 #111 meeting, it was already agreed all deactivated SCells are measured within NCSG, regardless of the reported UE capabilities and gap association with a bracket. To follow the similar approach in Rel-17 NCSG, deactivated SCells shall be measured within NCSG. Thus, we propose to confirm the agreement and remove the bracket.

However, after SCell activation, the related SSB may not be measured if the SSB is outside active BWP when UE reports ‘gap’ for the band. Thus, NW will configure the gap association to UE to indicate the related MO to be measured within gap other than not measured if following deactivated SCell rule.  In general, whether the MO may will be measured within gap or NCSG or outside gap depends on UE’s band reporting and BWP status.

- If UE reports ‘ncsg’ for the band, and

  - if the SSB is within the active BWP, the MO will be measured outside MG and NCSG;

  - if the SSB is outside the active BWP, the MO will be measured within NCSG.

  - in this case, the MO can be associated within the NCSG.

- If UE reports ‘gap’ for the band, and

  - if the SSB is within the active BWP, the MO will be measured outside MG and NCSG;

  - if the SSB is outside the active BWP, the MO will be measured within MG.

  - in this case, the MO can be associated within the Type-2 MG.

Figure 1. The transition from NCSG to MG for a MO with deactivated SCell

Observation 1: In Rel-17, when UE supports NCSG, deactivated SCell measurement will be performed within NCSG provided that NCSG is configured by the NW.

Proposal 1: When the SCell is deactivated, and NW configures NCSG+MG,

- the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped regardless of the reported UE capabilities and gap association.

- When the SCell is activated, UE shall follow the gap association rule to perform measurement.

- NeedForGaps

In last meeting, companies discussed the distance between SMTC and DRX to avoid the interruption within DRX ON duration.

Issue 2-1-1: Misalignment between DRX-on duration and SMTC for NFG measurements? - Background: - Agreement from RAN4#112 meeting: - Interruptions are not allowed during DRX ON duration - Exclude the time extended due to drx-inactivityTimer - if there is no SMTC occasion within a time period starting [4ms] before the starting point of the DRX ON duration and ending [4ms] after the ending point of the DRX ON duration. - Proposal: - Option 1: - Revise the value of the time period before and after the DRX-ON cycle for which the presence of SMTC allows interruptions during the DRX-ON period to 1 ms.

On the one hand, as mentioned from NW side, it will limit to scenarios to avoid the interruption if the distance is defined too large between the SMTC and the DRX ON duration. On the other hand, we understand that UE needs post process time for measurement. In Rel-17 concurrent gaps, the distance between two gaps is defined as 4ms. However, current situation is different. UE doesn’t need to handle consecutive measurement between two gaps. After the measurement, UE will perform data reception in DRX ON duration. The post processing is similar as SMTC post processing time in other procedures, such as SCell activation, Handover in which 2ms is normally used. Thus, to consider a compromise between NW and UE side, we propose to use 2ms as the distance between the SMTC and the DRX ON duration.

Proposal 2:  Interruption is not allowed during DRX ON duration if the distance between SMTC and DRX ON duration is larger than 2ms.

- Summary

In this contribution, we discussed the mandatory gap capability mismatch. Based on the discussions, we have made following proposals and observations:

Proposal 1: When the SCell is deactivated, and NW configures NCSG+MG,

- the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped regardless of the reported UE capabilities and gap association.

- When the SCell is activated, UE shall follow the gap association rule to perform measurement.

Proposal 2:  Interruption is not allowed during DRX ON duration if the distance between SMTC and DRX ON duration is larger than 2ms.