3GPP TSG-RAN WG4 Meeting #114	R4-2501319

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia

Title:	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

Agenda item:	7.18.2

Document for:	Discussion

Introduction

In RAN4#113 the topic of intercell and intracell inter user interference for 8Rx configuration were further discussed with focus on reducing the current agreed simulations for PDSCH as well as agree on the initial simulation setup for CQI requirement definition.

In the following we will discuss the remaining open issues and provide our view with observations and proposals to enable further progress on the topic

Discussion

General

Definition of MMSE-IRC receiver capability for 8Rx UE

In RAN4#113 the definition of the MMSE-IRC receiver capability for 8Rx UEs were discussed (see [2]):

Definition of MMSE-IRC receiver capability for 8Rx UE - Proposals - Option 1: Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE supporting SU-MIMO 8Rx receiver. - Option 2: Update the existing Rel-18 UE capability definition as a maintenance part of the R18 FR1 RF enhancement WI. - Option 3: Introduce new Rel-19 UE capabilities for MU-MIMO 8Rx receiver. And clearly state that the UE supporting ‘MU-MIMO 8Rx receiver’ are also required to support ‘SU-MIMO 8Rx receiver’ capability. - Option 4: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following: - Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

As we highlighted in RAN4#113, the current definition of baseline and simplified receiver in 38.101-4 [3] cannot be used directly for intra-cell inter-user interference configuration as the definitions are written for SU-MIMO configuration only:

Baseline SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. Simplified SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

Several options were proposed in RAN4#113 and captured in the WF and can be condensed to the following:

- Do not introduce new UE capabilities but instead state in applicability rules that also intra-cell inter-user interference is applicable.

- Introduce new UE capabilities for intra-cell inter-user interference.

- Update existing definition in 38.101-4 [3] from Rel-18.

We do not any reason why the definition has to contain the specific focus on MIMO type (i.e. SU-MIMO). Even for existing Rel-18 8Rx requirements removing the SU-MIMO restriction from the definitions will not make a difference as the requirements in Rel-18 are anyway focusing on SU-MIMO only. Hence, we conclude that there is no problem in simply removing SU-MIMO from the existing definitions.

We therefor propose that in Rel-19 the definition shall be changed to be more generic to also cover the interference cases discussed in this WID. In addition, we do not see any issue with back-porting the change to Rel-18, which will streamline the definition across the two releases.

The existing definition of baseline and simplified receivers in 38.101-4 is focused on SU-MIMO configuration. This needs to be changed to also cover the discussed interference cases for Rel-19.

We have identified any issue with using a more generic definition also in Rel-18, hence, to streamline the definition across Rel-18 and Rel-19 the new definition can most likely be back-ported to Rel-18.

Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following:
- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

If the agreed receiver definitions are also acceptable for Rel-18 8Rx, introduce the change in Rel-18 maintenance [NR_ENDC_RF_FR1_enh2-Perf].

8Rx UE MMSE-IRC receiver coverage

In RAN4#113 the receiver coverage was discussed (see [2]):

8Rx UE MMSE-IRC receiver coverage - Proposals - Option 1: RAN4 to define two sets of requirements (baseline 8Rx receiver and simplified 8Rx receiver). - Option 2: Set the criteria for defining two sets of requirements as: If the span after removing a reasonable number or outliers (at least 5 results should be left) is >2.5dB, define two sets of requirements. The evaluation of the span shall be done for each configuration agreed for requirement definition. - Option 3: Postpone defining criteria for decision of one or two sets of requirements until all test cases are finalized.

The focus in the discussion of receiver coverage is if there is a need to introduce requirements for both simplified and base line receivers. We would prefer to only introduce one set of requirements; however, we also see the issue that if two receiver types exist, defining only one set of requirements is likely to be biased towards the simplified receiver. In addition, it is not likely to achieve <2.5dB span across the results if companies are to provide only one result with free selection off receiver type used.

From the PDSCH inter-cell simulation results provided by companies in RAN4#113 and our updated results for RAN4#114 (see [6]) there is a difference of 1.3dB or more between the two receiver types when outliers are removed. In most cases the difference is even higher. We would have liked to see even more difference; however, we understood from the discussion in RAN4#113, that most companies see 1dB as the minimum margin for introducing two set of requirements.

Based on the available simulation results for PDSCH inter-cell interference from RAN4#113 as well as our simulation results provided in RAN4#114 the difference between simplified and baseline receiver is large enough to introduce requirements for both simplified and baseline receiver when defining requirements for PDSCH with inter-cell interference.

RAN4 to define two sets of requirements (baseline 8Rx receiver and simplified 8Rx receiver) when defining requirements for PDSCH with inter cell interference.

Further discuss the need to introduce two sets of requirements for PDSCH with intra-cell inter-user interference and CQI with inter-cell interference when more simulation results are available.

PDSCH requirements with inter cell interference

Rank and MCS

In RAN4#113 it was agreed which configurations should be used for further evaluation. The question of the final requirements was kept open pending simulation results (see [2]):

Rank and MCS Agreement: - Use the following for evaluation. TBD on requirements: - Rank1: MCS17 - Rank2: MCS17 - Rank4: MCS13 - 2nd priority simulations can be removed for next meeting for both baseline and simplified receiver.

Our updated simulations results provided in [6] together with simulation results from RAN4#113 show good alignment between companies (<2.5dB span when outliers are removed). We see the alignment to be good enough for all evaluated configurations for defining requirements, hence we see no reason to exclude any of the configurations and thereby cover rank 1,2 and 4.

Based on the simulation results from RAN4#114 and Nokia’s updated results, the span is <2.5dB with outliers removed for all agreed simulated cases, hence requirements can be defined for all agreed simulation cases.

Introduce PDSCH requirements with inter cell interference for the following configurations:
- Rank1: MCS17
- Rank2: MCS17
- Rank4: MCS13

Number of interference cells

In RAN4#113 it was discussed if the agreement to cover HomNet and Hetnet scenarios also would mean that requirements are to be defined with both 1 and 2 interference cells [2]:

Number of interference cells - Background: - We have an existing agreement from RAN#112bis, to reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx, with TBD for applicability rules to reduce UE efforts [R4-2416555] - Proposals: - Option 1: Both 1 interference cell and 2 interference cells

We see it valid to continue with the same test configurations as was used in Rel-17, where requirements were defined with both 1 and 2 interference cells. In Rel-17 to counter the increased test effort by introducing 1 and 2 interference cells the test effort was reduced by applicability rules, hence the test effort was not increased even with requirements defined for both 1 and 2 interference cells. Also, in the case of 8Rx we do not see an issue introducing applicability rules for 8Rx requirements similar to Rel-17 to reduce the test effort if requirements are introduced for both 1 and 2 interference cells.

Introduce requirements with both 1 and 2 interference cells.

Further discuss applicability rules. Use applicability rules from Rel-17 as a starting point.

PDSCH requirements with intra-cell inter-user interference

Rank and MCS

In RAN4#113 it was agreed to introduce requirements for Rank 2+2 however it was kept open if MCS17 or MCS19 were to be used. In addition, it was kept TBD if requirements for rank 4+4 are to be introduced.

Agreement: - Define requirements for: - Rank 2+2 - FFS: MCS17/19 - TBD: Rank 4+4: MCS13

For Rank 2+2, down selection can be done from the options agreed in RAN4#113. Both MCS17 and MCS19 are 64QAM and both seems from existing simulation results to be feasible for requirement definition. We do not see a specific reason to select either of the options from a technical perspective, however since in Rel-17 MCS17 has already been used in Rel-17, using MCS17 also in Rel-18 make it more likely that applicability rules can be agreed to possible reduce the test effort.

Selecting MCS17 or MCS19 for rank2+2 is not expected to make a difference seen from test coverage perspective as both are 64QAM. Since existing requirements for Rel-17 have been defined using MCS17, we see it potentially easier to agree on applicability rules if MCS17 is used for rank 2+2.

Use MCS17 for requirement definition with Rank 2+2

We see rank 4+4 configuration is possible and a likely use case in deployment and the most exhaustive test of the implementation of 8Rx, hence we see the need for introducing requirements for Rank 4+4 to be important for securing full UE performance test coverage.

We expect Rank 4+4 to be possible and a likely use case seen in deployment, hence we see it important to introduce requirements for Rank 4+4 with MCS13.

Define PDSCH requirements with intra-cell inter-user interference for rank 4+4 with MCS13.

Wideband CQI reporting requirements with inter-cell interference

Propagation condition

Propagation condition Agreement: - For initial simulation use - TDLA30-5 for the serving cell - AWGN for the interference cell

At this point we do not see a reason to change the agreed initial simulation. The final decision of propagation condition can be made once simulation results are available.

CQI calculation algorithm

In RAN4#113 it was decided to cover both baseline and simplified receiver for initial analysis and decide in this meeting if one or two sets of requirements will be defined (see [2])

CQI calculation algorithm Agreement: - Cover both Baseline 8Rx UE MMSE-IRC receiver and simplified 8Rx UE MMSE-IRC Receiver for initial analysis. - - Decide latest next meeting, if one or two sets of requirements are to be defined.

For AWGN we expect no difference between simplified and baseline performance with the discussed rank 1 and rank 2.

For fading conditions, we expect the simplified receiver will perform worse than baseline receiver, however for the discussed rank 1 and rank 2 the difference is likely not much.

If the difference in gamma is below 0.1 between simplified and baseline receiver we expect only one set of requirements are needed. Based on the difference in gamma for the existing requirements for 2Rx and 4Rx in Rel-17, we expect this will be the case.

If the difference in gamma between baseline and simplified receiver is 0.1 or lower only one set of requirements shall be defined. Further discuss when more simulation results are available.

Rank for serving cell

In RAN4#114 the rank for the serving cell was discussed. It was agreed for companies to provide results for Rank 1 and interested companies can provide results also for Rank 2 (see [2]).

Rank for serving cell Agreement: - For initial study use: - Rank 1 - Interested companies can also provide results with Rank 2.

Overall rank2 is minimum to not trivialize the CQI selection (i.e. we need two different streams to be captured in one value).

We see Rank2 as minimum to not trivialize the CQI selection to only capture one stream.

As a minimum, define CQI with inter-cell interference requirements for Rank 2.

CSI-IM and NZP CSI-RS configurations

In RAN4#113 it was agreed to reuse the agreement from Rel-17 for CSI-IM and NZP CSI-RS configuration. The actual configuration to be used are still to be discussed (see [2]):

CSI-IM and NZP CSI-RS configurations Agreement: - For CSI-IM and NZP CSI-RS assumption, reuse agreement from R17 2/4Rx inter-cell interference suppressing requirement for simulation, which are: - CSI-IM on serving cell overlapping with PDSCH from interference cell - NZP CSI-RS on serving cell overlapping with NZP CSI-RS from interference cell - TBD: Actual configuration to be used. - Proposals: - Option 1: FDD: First subcarrier index in the PRB used for CSI-RS (k0) - Row 5,(4) for the serving cell - Row 5,(6) for the interference cell - Other options not precluded

In RAN4#113 one company highlighted an issue in the existing Rel-17 configuration for the already defined configuration related to the requirements for wideband CQI reporting with inter-cell interference. The issue is that the agreed overlap of CSI-IM on serving cell with PDSCH from interference cell was not correctly configured. Option 1 from the WF introduces a correction to the configuration used in Rel-17, which secures the correct overlap.

The proposed configuration will correct the issue in Rel-17 configuration where CSI-IM does not overlap with PDSCH as was agreed it should.

To ensure correct overlap of CSI-IM on serving cell with PDSCH on interference cell, use the configuration from option 1:
FDD:
First subcarrier index in the PRB used for CSI-RS (k0)
- Row 5,(4) for the serving cell
- Row 5,(6) for the interference cell

Applicability rule and release independency

Test applicability for 8Rx UEs

In RAN4#113 the test applicability for 8Rx UEs were discussed, and it was agreed that new requirements for 8Rx MMSE-IRC receivers shall be mandatory requirements for all 8Rx UEs. It is still open if additional test applicability rules are to be defined [2].

Test applicability for 8Rx UEs Agreement: - New requirements for 8Rx MMSE-IRC receiver to be mandatory requirements for all 8Rx UEs. - TBD: additional test applicability rule.

At this point it is not possible to determine if future 8Rx requirements will fully cover the same UE functionality as existing 2Rx/4Rx requirements, hence we do not see it reasonable to discuss test applicability until it is clear which requirements are to be defined.

The discussion on test applicability can be postponed until it is clear, which requirements are to be defined.

Postpone the discussion on possible introduce additional test applicability rules until the requirements are defined.

Test applicability to skip 2/4Rx requirements

In RAN4#113 it was discussed if 2Rx and/or 4Rx requirements could be skipped in case 8Rx requirements are passed (see [2]):

Test applicability to skip 2/4Rx requirements - Proposals - Option 1: Defining applicability rules to skip 2/4Rx requirements if 8Rx tests are passed. - Option 1a: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not. - Option 1b: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

In case 8Rx requirements corresponds to Rel-17 2/4Rx requirements, we are open to discuss introducing applicability rules to skip 2Rx and/or 4Rx requirements if UE passes the corresponding 8Rx requirements. This should however only be applicable to configurations where the 8Rx requirements fully match the configuration for the requirements skipped.

If an introduced 8Rx requirement match the configuration of a 2Rx or 4Rx requirement and the 8Rx requirement is passed, we expect the UE would also be able to pass the equivalent 2Rx/4Rx requirement.

Consider to introducing test applicability under inter-cell interference and under intra-cell inter user interference to skip 2/4Rx requirements if the following applies for each considered 8Rx test:
- UE support both 8Rx and 2/4Rx
- The UE passes an 8Rx requirement equivalent to a 2/4Rx requirement.

Test applicability for FDD/TDD

In RAN4#113 it was discussed if applicability rules should be introduced to reduce UE test effort in case requirements for both 1 and 2 interference cells are introduced (see [2]):

Test applicability for FDD/TDD - Proposals - Option 1: For 8Rx inter-cell interference scenario, for demodulation performance tests, introduce similar applicability rules defined in section 5.1.1.10 for UE supporting both duplex mode TDD and FDD as legacy 2Rx/4Rx, i.e., if UE is supporting both TDD and FDD with 8Rx, UE will pass test case under HomNet scenario with FDD mode, and pass test case under HetNet scenario with TDD mode. - Option 2: Do not reduce UE effort to only comply to either 1 or 2 interference cells.

This issue depends on the conclusion about introducing requirements for only 1 or 1 and 2 interference cells for inter-cell interference scenarios. In case it is agreed to introduce requirements for 1 and 2 interference cells we see no issue with introducing similar applicability rules are defined in Rel-17 for 2/4Rx hence reducing the test load for the UE.

In case agreed to introduce requirements for 1 and 2 interference cells similar applicability rules as used in the Rel-17 can be discussed to reduce test load on the UE.

Introduce similar applicability rules as in Rel-17 2Rx/4Rx so if a UE is supporting both TDD and FDD with 8Rx, the UE will pass test case under HomNet (2 interference cells) scenario with FDD mode and pass test case under HetNet (1 interference cell) scenario with TDD mode (Option 1).

Release independent

In RAN4#113 the question to introduce release independency was brought up, but not further discussed, hence the options provided by companies were captured in the WF [2]:

Release independent - Proposals - Option 1: Discuss release independence requirements at a later stage in the WI. - Option 1a: RAN4 start discussing the issue of release independent when RAN4 agree on baseline assumptions, test parameters and test metrics, i.e., after RAN4#113. - Option 2: The new requirements for 8Rx should be release independent from Rel-18. - Option 2a: The requirement should be released independent from Rel-18 if no new signalling or UE capability introduced. - Option 3: The performance requirements for 8Rx MMSE-IRC should be release independent from Rel-17, but should be mandatory for Rel-18 and optional for Rel-17.

There are already 8Rx requirements defined in Rel-18 and it is expected that a Rel-19 UE will perform equal in interference scenarios to a Rel-18 UE, hence we see no issue for new requirements for 8Rx to also be applicable from Rel-18. Still, we would like to postpone the discussion on release independency until agreements are made on baseline assumptions, test parameters and test metrics.

8Rx requirements are already defined in Rel-18 and it is expected that Rel-19 UE will perform equal in interference scenarios to a Rel-18 UE, hence new requirements for 8Rx can be applicable from Rel-18. It cannot however be fully known, until Rel-19 requirements are agreed.

Postpone the discussion on release independency until agreements are made on baseline assumptions, test parameters and test metrics.

Conclusion

In this paper we have provided our view on the topic of requirement definition for 8Rx inter-cell and intra-cell inter-user requirements.

The following Observations and Proposals were made:

General

Definition of MMSE-IRC receiver capability for 8Rx UE

- The existing definition of baseline and simplified receivers in 38.101-4 is focused on SU-MIMO configuration. This needs to be changed to also cover the discussed interference cases for Rel-19.

We have identified any issue with using a more generic definition also in Rel-18, hence, to streamline the definition across Rel-18 and Rel-19 the new definition can most likely be back-ported to Rel-18.

- Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following:
- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

If the agreed receiver definitions are also acceptable for Rel-18 8Rx, introduce the change in Rel-18 maintenance [NR_ENDC_RF_FR1_enh2-Perf].

8Rx UE MMSE-IRC receiver coverage

Based on the available simulation results for PDSCH inter-cell interference from RAN4#113 as well as our simulation results provided in RAN4#114 the difference between simplified and baseline receiver is large enough to introduce requirements for both simplified and baseline receiver when defining requirements for PDSCH with inter-cell interference.

RAN4 to define two sets of requirements (baseline 8Rx receiver and simplified 8Rx receiver) when defining requirements for PDSCH with inter cell interference.

Further discuss the need to introduce two sets of requirements for PDSCH with intra-cell inter-user interference and CQI with inter-cell interference when more simulation results are available.

PDSCH requirements with inter cell interference

Rank and MCS

Based on the simulation results from RAN4#114 and Nokia’s updated results, the span is <2.5dB with outliers removed for all agreed simulated cases, hence requirements can be defined for all agreed simulation cases.

Introduce PDSCH requirements with inter cell interference for the following configurations:
- Rank1: MCS17
- Rank2: MCS17
- Rank4: MCS13

Number of interference cells

Introduce requirements with both 1 and 2 interference cells.

Further discuss applicability rules. Use applicability rules from Rel-17 as a starting point.

PDSCH requirements with intra cell inter user interference

Rank and MCS

Selecting MCS17 or MCS19 for rank2+2 is not expected to make a difference seen from test coverage perspective as both are 64QAM. Since existing requirements for Rel-17 have been defined using MCS17, we see it potentially easier to agree on applicability rules if MCS17 is used for rank 2+2.

Use MCS17 for requirement definition with Rank 2+2

We expect Rank 4+4 to be possible and a likely use case seen in deployment, hence we see it important to introduce requirements for Rank 4+4 with MCS13.

Define PDSCH requirements with intra-cell inter-user interference for rank 4+4 with MCS13.

Wideband CQI reporting requirements with inter-cell interference

CQI calculation algorithm

If the difference in gamma between baseline and simplified receiver is 0.1 or lower only one set of requirements shall be defined. Further discuss when more simulation results are available.

Rank for serving cell

We see Rank2 as minimum to not trivialize the CQI selection to only capture one stream.

As a minimum, define CQI with inter-cell interference requirements for Rank 2.

CSI-IM and NZP CSI-RS configurations

The proposed configuration will correct the issue in Rel-17 configuration where CSI-IM does not overlap with PDSCH as was agreed it should.

To ensure correct overlap of CSI-IM on serving cell with PDSCH on interference cell, use the configuration from option 1:
FDD:
First subcarrier index in the PRB used for CSI-RS (k0)
- Row 5,(4) for the serving cell
- Row 5,(6) for the interference cell

Applicability rule and release independency

Test applicability for 8Rx UEs

Postpone the discussion on possible introduce additional test applicability rules until the requirements are defined.

Test applicability to skip 2/4Rx requirements

If an introduced 8Rx requirement match the configuration of a 2Rx or 4Rx requirement and the 8Rx requirement is passed, we expect the UE would also be able to pass the equivalent 2Rx/4Rx requirement.

Consider to introducing test applicability under inter-cell interference and under intra-cell inter user interference to skip 2/4Rx requirements if the following applies for each considered 8Rx test:
- UE support both 8Rx and 2/4Rx
- The UE passes an 8Rx requirement equivalent to a 2/4Rx requirement.

Test applicability for FDD/TDD

In case agreed to introduce requirements for 1 and 2 interference cells similar applicability rules as used in the Rel-17 can be used to reduce test load on the UE.

Introduce similar applicability rules as in Rel-17 2Rx/4Rx so if a UE is supporting both TDD and FDD with 8Rx, the UE will pass test case under HomNet (2 interference cells) scenario with FDD mode and pass test case under HetNet (1 interference cell) scenario with TDD mode (Option 1).

Release independent

8Rx requirements are already defined in Rel-18 and it is expected that Rel-19 UE will perform equal in interference scenarios to a Rel-18 UE, hence new requirements for 8Rx can be applicable from Rel-18. It cannot however be fully known, until Rel-19 requirements are agreed.

Postpone the discussion on release independency until agreements are made on baseline assumptions, test parameters and test metrics.

- References

- R4-2416555 - WF for [112bis][320] NR_demod_Ph5_Part2_UE, Nokia

- R4-2419784 - WF for [113][322] NR_demod_Ph5_Part2_UE, Nokia

- TS38.212 - Multiplexing and channel coding (Release 18)

- TS38.101-4 - User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 18)

- RP-241297, “NR demodulation performance: Phase 5”, RAN#104, June 2024

- R4-2501320 - Simulations for UE demodulation performance req. for 8Rx with MMSE-IRC