3GPP TSG RAN WG4 #114	R4-2501324

Athens, Greece, Feb 17 – 21, 2025

Agenda item:		7.23.2.3

Source:	Nokia

Title:	On existing BS RF RX requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#113 [1] regarding evolution of NR duplex operation. There were some agreements made that have impact on the existing BS RF requirements for SBFD operation. Also, the open items are listed in the WF. The requirement with the biggest impact is the RX in-band blocking. This contribution displays Nokia’s view on the existing BS RF RX requirements for SBFD operation and emphasizes the importance of setting correctly the RX in-band blocking requirement to enable functionality in real-life deployment.

# Discussion

## In-band blocking simulations

RAN4#113 reached the following agreement and way forward related to the in-band blocking requirements:

Agreement: - For FR2 local area SBFD BS, re-use existing FR2 local area BS in-band blocking power requirements. - For FR1 WA BS, FFS following options. - Option 1: a unified blocking requirement to consider both scenarios - Option 2: separate requirements are defined - One set of requirements for coexisting with legacy TDD system in adjacent channel - One set of requirements for coexisting with new SBFD system in adjacent channel - Option 3: In-band blocking requirement is defined by manufacture declared level of interference in the specified range, which comes from the system-level evaluation (from 100% grid-shifting to 10% grid-shifting). - For FR1 WA BS co-existence study, FFS what grid-shift value(s) should be considered. - Option 1: 10% - Option 2: 20% - Option 3: 50% - Option 4: 100% - Option 5: 5% - Option 6: 20% ~ 50% - For FR2-1 WA SBFD BS, FFS following two options. - Option 1: The current FR2-1 WA BS in-band blocking requirements can be re-used - Option 2: Define in-band blocking interferer signal power to -23 dBm

### FR1 WA BS in-band blocking requirement

#### Requirement definition

After RAN4#113, 3 options are listed as candidates for how to define the FR1 WA BS in-band blocking requirements. Option 1 proposes a unified in-band blocking requirement that covers both scenarios where SBFD coexists with a TDD deployment in the adjacent channel and scenarios where SBFD coexist with another SBFD deployment in the adjacent channel. Option 2 proposes to have different requirements for the 2 scenarios mentioned above. Lastly, Option 3 proposes that the requirements are defined by manufacturer declaration.

Our preference is Option 1. Regarding Option 2, there is no need to specify additional/tighter requirements for the scenario of SBFD coexisting with other SBFD network and, if needed, those enhanced BS capabilities could be met on a customer demands basis. Moreover, this would require additional discussions on the feasibility of the mechanisms to achieve such enhanced performance, e.g. the implementation of sub-band RF filtering in the transmitter side. Therefore, we don’t support having tighter BS requirements and, thus, we should not expect any differences in the in-band blocking requirements depending on whether the SBFD BSs coexists with TDD BSs or SBFD BSs. Option 3 is not our preferred way-forward either, since it gives full freedom to the network vendors to design/decide its own in-band blocking requirements. This same approach could in principle be taken for any RF requirement, which we think it is not suitable. Furthermore, if different vendors declare different blocking requirements, it wouldn’t be possible to deploy a multi-vendor network. Instead, we think that 3GPP specifications should provide a common ground to all network vendors on which should be the minimum RF requirements. Given this, our preference is defining a unified in-band blocking requirement that covers all possible deployments cases.

RAN4 to define a unified in-band blocking requirement to cover both the cases where SBFD BS coexists in the adjacent channel with TDD and SBFD base stations (Option 1)

#### Grid shift assumptions

Another open FFS topic is the grid shift assumptions. The list of grid shift options captured in RAN4#113 spans 5% to 100%. In our RAN4#112-bis contribution [3], we included a comparison of the in-band blocking power at the SBFD BS for different grid shift assumptions, particularly 10%, 50% and 100%. It can be noted that the grid shift assumption is quite important for the derivation of the in-band blocking requirements. It can also be observed that, among the considered grid-shift values, 10% grid-shift results in the worst-case scenario from an in-band blocking perspective since it results in the highest adjacent channel interference. For instance, the relative in-band blocking power difference between 10% grid shift and 100% grid shift exceeds 10 dB. Thus, a SBFD BS that fulfils the in-band blocking requirement based on the 10% grid-shift would be automatically compliant for any of the other considered grid-shift values such as 20%, 50% or 100%. However, if the in-band blocking requirement is derived from, e.g., a grid shift of 50%, the SBFD operation for 10% grid-shift would be compromised since the interfering signal power would be higher than the used to determine the in-band blocking requirement. Note that site-sharing is commonly used when deploying base stations from different operators, and 10% grid shift is what closest resembles the site-sharing scenario.

Results show that the smaller is the grid-shift, the larger is the impact of the adjacent channel interference to the in-band blocking power.

Only consider 10% grid-shift to derive the interfering signal power for the in-band blocking requirement.

#### Analysis of the in-band blocking simulation results

A summary of the results shown in [3] can be found in the following tables. Table 1 shows the 99th percentile of the received in-band blocking power at the FR1 WA BS. Note that after our analysis of grid shift above, we only focus on scenarios with grid shift of 10% to derive our observations and proposals.

Table 1. In-band blocking power [dBm] for FR1 WA BS scenario

|  | Interfering signal power at 99th percentile | Current in-band blocking power requirement in TS 38.104 |
| --- | --- | --- |
| Scenario 1 (Urban macro FR1) | -2.5 dBm | FR1 WA BS: -43 dBm |

Based on our results, we can observe that the current in-band blocking power requirements for FR1 WA BS defined in TS 38.104 can’t be re-used for FR1 WA SBFD BS. Thus, we propose to define a new requirement according to the observed in-band blocking power:

For FR1 WA SBFD BS, the in-band blocking power at the 99th percentile is -2.5 dBm, which is approximately 40 dB higher than the current in-band blocking requirement.

Table 2 collects the in-band blocking simulation results from all companies that submitted results to RAN4#112-bis for the FR1 WA BS. A total of 7 companies provided results, which we believe is enough to draw solid observations. The table additionally includes the average across the different companies. It can be noted that the resulting average in-band blocking requirement is much higher than the existing FR1 WA BS in-band blocking requirements (-43 dBm). Therefore, we propose to introduce a new requirement for FR1 WA SBFD BS. This new in-band blocking requirement should be around -2 dBm.

Table 2. Collection of in-band blocking results for FR1 WA BS at 10% grid shift

| Company | Interfering signal power at 99th percentile (10% grid shift, FR1 WA BS) |
| --- | --- |
| CATT | 0.3 dBm |
| Ericsson | -4.7 dBm |
| Huawei | -5.1 dBm |
| Samsung | -4.39 dBm |
| Tejas Network | -0.78 dBm |
| ZTE | -1.5067 dBm |
| Nokia | -2.5 dBm |
| Average | -2.67 dBm |

Based on companies’ simulation results, the existing FR1 WA BS in-band blocking requirement defined in TS 38.104 can’t be re-used for FR1 WA SBFD BS.

Introduce a new in-band blocking requirement for FR1 WA SBFD BS of -2 dBm.

### FR1 MR BS in-band blocking requirement

Similar discussion as for FR1 WA BS is also applicable to the FR1 MR BS. Thus, we propose also to define a unified in-band blocking requirements and derive based on the simulation results with 10% grid shift.

#### Analysis of the in-band blocking simulation results

A summary of the results shown in [3] can be found in the following tables. Table 1 shows the 99th percentile of the received in-band blocking power at the FR1 MR BS. Note that after our analysis of grid shift above, we only focus on scenarios with grid shift of 10%.

Table 1. In-band blocking power [dBm] for FR1 MR BS scenario

|  | Interfering signal power at 99th percentile | Current in-band blocking power requirement in TS 38.104 |
| --- | --- | --- |
| Scenario 3 (Urban micro FR1) | -4.3 dBm | FR1 MR BS: -38 dBm |

Based on our results, we can observe that the current in-band blocking power requirements for FR1 MR BS defined in TS 38.104 can’t be re-used for FR1 MR SBFD BS. Thus, we propose to define a new requirement according to the observed in-band blocking power:

For FR1 MR SBFD BS, the in-band blocking power at the 99th percentile is -4.3 dBm, which is approximately 34 dB higher than the current in-band blocking requirement.

Introduce a new in-band blocking requirement for FR1 MR SBFD BS of -4 dBm.

### FR2-1 WA BS in-band blocking requirement

#### Requirement definition

As captured in the RAN4#113 WF, companies should agree on whether the current in-band blocking requirement can be re-used or whether a new in-band blocking interfering signal power should be defined. As shown in the results below in Table 3, our conclusion is that we can re-use the existing in-band blocking requirements. We rely on the ratio of in-band blocking power versus wanted signal to reach such conclusion. From Table 10.5.2.3.-1 from TS 38.104, we note that wanted and interfering signal mean powers are defined relative to the EIS_REFSENS and EIS_REFSENS_50M. Given the relation between EIS_REFSENS and EIS_REFSENS_50M, we can define the in-band blocking requirements as the ratio between both signal components. For instance, the current FR2-1 in-band blocking requirement assumes that the OTA interfering signal power is 27 dB higher than the OTA wanted signal power.

[TS 38.104, Section 10.5.2.3]

Use the ratio of interfering signal power and wanted signal power for the FR2-1 WA BS in-band blocking requirement derivation.

#### Analysis of the in-band blocking requirements

Table 3 includes absolute in-band blocking power and wanted signal power as well as the ratio of in-band blocking power versus wanted signal power at 99th percentile for the FR2-1 WA SBFD BS at 10% grid shift.

Table 3. Ratio of in-band blocking power and wanted signal [dBm] for FR2-1 scenarios

|  | Interfering signal power at 99th percentile | Wanted signal power at 99th percentile | Ratio of interfering signal power versus wanted signal at 99th percentile | Current ratio of interfering signal power versus wanted signal in TS 38.104 |
| --- | --- | --- | --- | --- |
| Scenario 6 (Urban macro FR2) | -37.4 dBm | -66.1 dBm | 28.7 dB | FR2-1 WA BS: 27 dB |

Based on the results above the following is observed:

For FR2-1 WA SBFD BS, the ratio of in-band blocking power versus wanted signal at the 99th percentile is 30 dB, which is within the limits defined by the current FR2-1 WA BS requirements in TS 38.104.

We think that there is no need to define new in-band blocking requirements for FR2-1 WA SBFD BS given that the obtained ratio of in-band blocking power versus wanted signal is within the current FR2-1 WA BS requirements.

The current FR2-1 WA BS in-band blocking requirements can be re-used for FR2-1 WA SBFD BS.

### Clarification on the in-band blocking requirement derivation

In the previous RAN4#112 meeting, companies agreed to not account for the SBFD specific co-channel interferences to derive the in-band blocking requirement. In other words, BS self-interference and inter-sector interferences are not considered. However, these interferences also impact the receiver performance since they are also received by the BS LNA. This is an important assumption that should be clearly stated as part of the in-band blocking requirement definition for SBFD-capable BS in order to avoid possible misunderstandings.

RAN4 specifications should include a clear statement indicating that the in-band blocking requirements for SBFD BS are derived without accounting for BS self-interference and inter-sector interference.

### ACLR and ACS effects

One important aspect for the in-band blocking power derivation is whether effects such as ACLR and ACS effects should be included. As described in TR 38.858, Clause E.2.3.1 the DL adjacent channel interference for the in-band blocking requirement should be measured following the definition of IACI, and it is explicitly mentioned that it does not include the “frequency isolation” aspects. i.e., the ACLR and ACS effects.

RAN4 to agree/clarify that the derivation of the in-band blocking requirement is aligned with the definition of IACI in TR 38.858, Clause E.2.3.1 which does not include ACLR and ACS effects.

## Dynamic range requirement

The following was agreed in RAN4#113:

Agreement: - FFS on the followings: - The dynamic range requirements of FR1 can reuse the existing IoT level. - The dynamic range requirements of FR2-1 can reuse existing IoT level.

For SBFD, the BS IoT level will be influenced by the RSI capability of the base station as well level of interference from other sectors and neighbouring base stations. Nevertheless, based on our analysis we the dynamic range requirement of FR1 can reuse the existing IoT level.

For receiver dynamic range requirement of FR1, reuse the existing IoT level.

## ACS Requirement

The following was agreed in RAN4#113:

Agreement: - FFS on the following: - The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD.

Adjacent channel selectivity is an important measure of the receiver's ability to receive a wanted signal with interfering signal in the adjacent channel. In our view, the definition of ACS can be reused for SBFD. The general in-band blocking interference level defined for SBFD with interference level of 9dB offset should be used.

RAN4 to agree that definition of ACS is reused for SBFD and the interference level keeps 9dB offset to the in-band blocking interference level.

## RX intermodulation

### Necessity of New RX intermodulation requirement with 1 interfering signal

The following was agreed in RAN4#113:

Agreement: - FFS on the following options: - Option 1: IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement. - Option 2: RAN4 to define new RX intermodulation scenario with self-interference and 1 interfering signal.

The proposed new RX intermodulation scenario with 1 interfering signal might be a possible scenario, and it should be discussed further. Caution needs to be applied when defining RF requirements and especially reusing existing ones. The differences with legacy TDD and SBFD systems are imminent and might end up causing disastrous effects in the field in the worst case.

Careful consideration is needed when defining requirements for SBFD.

As the power of the own TX sub-band signal (self-interference) could be significantly higher than what is currently assumed in the general RX IMD requirement (-52dBm for WA BS), this could lead to situation where the current RX IMD requirement is inadequate and doesn’t guarantee the linearity of RX is enough. If one would take the values companies calculated in the study item phase for the self-interference signal in gNB TX subband, measured at the input of LNA, the range varied from -23dBm to -56dBm. Additional robustness to RX would be highly welcomed to ensure functionality in this situation. The SBFD RX blocking requirement might not capture this implicitly, unless the own TX is on during the test. Therefore, we think additional RX IMD requirement with self-interference (TX on) and one interfering signal might be needed. The frequencies of the interfering signals need to be adjusted to cause IMD in the UL subband. If self-interference can be cancelled to reasonable level, this should not be an issue for SBFD-capable BS.

The power level of own TX signal might be significantly higher than what is currently defined for interfering signals.

RAN4 to consider the new RX IMD scenario with self-interference and 1 interfering signal.

It also must be noted that in the current FR1 WA BS RX IMD requirement, the mean power of interfering signals is defined as the RX in-band blocking requirement (-43dBm) -9dB offset = -52dBm. If, for SBFD, the RX in-band blocking interfering signal mean power is agreed and more stringent than the expected self-interference, there might not be need for the new RX IMD requirement with self-interference and 1 interfering signal. But in this case, there is then a need to update the SBFD RX IMD requirement to reflect RX in-band blocking interfering signal mean power with the 9dB offset. Frequencies of the interfering signals to cause IMD to UL subband need to be adjusted if needed.

RAN4 to consider adjusting current RX IMD requirement with interfering signals mean power levels and frequencies.

### Impact on existing RX intermodulation requirement with 2 interfering signals

The following was agreed in RAN4#113:

Agreement: - FFS on the following options: - Option 1: - for the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal. - for receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver. - Option 2: new IMD requirement with 2 interfering signal scenario is not needed. - Option 3: No need to define new RX intermodulation requirement with 2 interfering signals if new requirement with self-interference and 1 interfering signal is defined. - Option 4: The scenario for 2 input signals can be regarded as the worst case when setting requirement on receiver linearity. For RX IMD, the mean power of both CW and modulated interfering signals is [the final agreed in-band blocking level] – 9 dB.

The new RX IMD requirement with 2 interfering signals could also be a possible scenario and should be considered further. The mean power of the interfering signals should reflect the RX in-band blocking with the 9dB offset.

RAN4 to consider new RX intermodulation requirement with 2 interfering signals.

After RAN4 has considered new possible RX IMD requirements with 1 or 2 interfering signals and the adjustment of current RX IMD requirement, the most challenging one should be chosen to cover the others and save effort on the conformance testing.

RAN4 to consider the three possibilities for RX IMD and choose the most challenging one to cover others.

# Conclusion

In this contribution, our views on the SBFD BS RF requirements are presented. There were observations and proposals as listed below.

- Results show that the smaller is the grid-shift, the larger is the impact of the adjacent channel interference to the in-band blocking power.

- For FR1 WA SBFD BS, the in-band blocking power at the 99th percentile is -2.5 dBm, which is approximately 40 dB higher than the current in-band blocking requirement.

- For FR1 MR SBFD BS, the in-band blocking power at the 99th percentile is -4.3 dBm, which is approximately 34 dB higher than the current in-band blocking requirement.

- For FR2-1 WA SBFD BS, the ratio of in-band blocking power versus wanted signal at the 99th percentile is 30 dB, which is within the limits defined by the current FR2-1 WA BS requirements in TS 38.104.

- Careful consideration is needed when defining requirements for SBFD.

- The power level of own TX signal might be significantly higher than what is currently defined for interfering signals.

- RAN4 to define a unified in-band blocking requirement to cover both the cases where SBFD BS coexists in the adjacent channel with TDD and SBFD base stations (Option 1).

Only consider 10% grid-shift to derive the interfering signal power for the in-band blocking requirement.

Based on companies’ simulation results, the existing FR1 WA BS in-band blocking requirement defined in TS 38.104 can’t be re-used for FR1 WA SBFD BS.

Introduce a new in-band blocking requirement for FR1 WA SBFD BS of -2 dBm.

Introduce a new in-band blocking requirement for FR1 MR SBFD BS of -4 dBm.

Use the ratio of interfering signal power and wanted signal power for the FR2-1 WA BS in-band blocking requirement derivation.

The current FR2-1 WA BS in-band blocking requirements can be re-used for FR2-1 WA SBFD BS.

RAN4 specifications should include a clear statement indicating that the in-band blocking requirements for SBFD BS are derived without accounting for BS self-interference and inter-sector interference.

RAN4 to agree/clarify that the derivation of the in-band blocking requirement is aligned with the definition of IACI in TR 38.858, Clause E.2.3.1 which does not include ACLR and ACS effects.

For receiver dynamic range requirement of FR1, reuse the existing IoT level.

RAN4 to agree that definition of ACS is reused for SBFD and the interference level keeps 9dB offset to the in-band blocking interference level.

RAN4 to consider the new RX IMD scenario with self-interference and 1 interfering signal.

RAN4 to consider adjusting current RX IMD requirement with interfering signals mean power levels and frequencies.

RAN4 to consider new RX intermodulation requirement with 2 interfering signals.

RAN4 to consider the three possibilities for RX IMD and choose the most challenging one to cover others.

# References

- R4-2419908 “Way Forward for [113][307] NR_duplex_evo_BSRF”, RAN4#113, Orlando, Florida, USA, 18th-22nd November, 2024

- R4-2418022 “On existing BS RF TX requirements for SBFD operation” RAN4#113

- R4-2418021, “SBFD RX in-band blocking simulation results”, Nokia, RAN4#113