3GPP TSG RAN WG4 #114	R4-2501325

Athens, Greece, Feb 17 – 21, 2025

Agenda item:		7.23.2.2

Source:	Nokia

Title:	On existing BS RF TX requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#112bis [1], and updates were made in RAN4#113 [2] regarding evolution of NR duplex operation. There were some agreements made that have impact on the existing BS RF requirements for SBFD operation. Also, the open items are listed in the WF. This contribution displays Nokia’s view on the existing BS RF TX requirements for SBFD operation.

# Discussion

## Unwanted emissions

The following was agreed in RAN4#112bis:

WF: - FFS on the followings: - Whether to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB firstly? - Option 1: Yes, since legacy ACLR and OBUE requirement might not be sufficient to ensure feasible coexistence. - Option 2: No, since specific co-existence requirements can be covered by regional requirements or customer specific proprietary requirement. - How to define the additional co-existence requirements: - Option 1: based on CL assumption for BS2BS CLI interference and BS DESENS level. - [1dB DESENS in the victim SBFD receiver] - [typical CL values based on typical SBFD-capable gNB implementation] - [frequency offset for the additional OBUE requirement is declared] - Option 2: by improving ACLR1 and ACLR2 requirements - Option 3: others

Also, the following was agreed in RAN4#113:

Agreement: - FFS whether/how to introduce the additional ACLR and/or OBUE requirements for FR1 WA BS.

We do not see the necessity of additional ACLR and/or OBUE would be needed from 3GPP to define. The described scenario where two operators SBFD capable gNB’s would coexist, is a new scenario and a rare one. It has not been discussed in the study item phase and should not be included here. Thus, we believe that there is no need to define additional 3GPP requirements for this special case. Also, one thing to notice here is that if two SBFD networks would coexist, this would mean that TDD networks would not be possible to deploy. For FR1 frequencies, only the current n104 band would then be suitable. This in effect would mean reserving n104 only to SBFD use. In our view the customer specific proprietary requirement can be used in such case where two SBFD systems are co-existing and reasonable co-location is needed. If we look on the proposal on how to define the additional co-existence requirements, there are many things that would make it almost impossible to agree on. Typical CL values according to different companies are ranging excessively and there is no consensus what a typical SBFD-capable gNB implementation would be. Therefore, it would be practically impossible to define the needed improvements to related requirements.

No need for RAN4 to specify additional ACLR and/or OBUE requirements.

## Joint measurement for normal DL symbols/slots and SBFD symbols/slots

The following was agreed in RAN4#113:

Agreement: - FFS on the Proposal 3 from R4-2419221.

R4-2419221 is a contribution from ZTE Corporation, and it states the following:

Calculation of averaged EVM is defined in Annex B.7 for FR1 in TS 38.104 [5]:

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of Ndl_symbol symbols where Ndl_symbol  is the number of dl symbols in a 10 ms measurement interval.

For TDD, let Ndl_symbolsTDD be the number of downlink SBFD symbols or non-SBFD symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from  Ndl_symbolsTDD symbols of different 10 ms measurement intervals and should have a minimum of Ndl_symbol symbols averaging length where Ndl_symbol is the number of symbols in a 10 ms measurement interval.

-	EVMframe is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS).

EVMframe=1i=1Ndl_symbolTDDNii=1Ndl_symbolTDDj=1NiEVMi,j2

-	Where Ni is the number of resource blocks with the considered modulation scheme in symbol i.

-	The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆th (l and h, low and high; where low is the timing ∆c-W/2 and and high is the timing ∆c+W/2).

EVMframe=maxEVMframe,l,EVMframe,h

-	In order to unite at least Ndl_symbol slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by.

Nframe=10×Nsymbol Ndl_symbolsTDD

and Nsymbol=14 for 15 kHz SCS, Nsymbol=28 for 30 kHz SCS and Nsymbol=56 for 60 kHz SCS normal CP.

-	Unite by RMS.

EVM=1Nframek=1NframeEVMframe,k2

Proposal 3 proposes to update the EVM calculation in Annex B.7 as above.

In our view separate EVM measurements for downlink SBFD symbols or non-SBFD symbols are needed to verify performance reliably. Via joint measurement realistic performance results might not be available leading to non-guaranteed performance within those conditions. In addition, joint measurement leads to complex test models and complicated verification. In our view, the above proposal for average EVM calculation from ZTE can be approved and performed both non-SBFD and SBFD symbols.

EVM calculation should be performed for both SBFD symbols and non-SBFD symbols separately.

EVM calculation in Annex B.7 can be updated as proposed.

## Text Proposals for implementing SBFD RF requirements

The following was agreed in RAN4#113:

Agreement: - The BS specification structure conclusion captured in General thread shall be applied here. - FFS on how to implement corresponding SBFD RF requirements by using the following proposals as baseline. - Text Proposal in R4-2418099 on BS output power requirement (taking the new clause 6.2B, titled as “Base station output power for SBFD” as example): =================== Start of Text Proposal =================== 6.2B Base station output power for SBFD 6.2.1B General The BS conducted output power requirement for SBFD operation is specified over SBFD symbols, in which the rated carrier output power for SBFD could be different from the rated carrier output powers for normal symbols as specified in clause 6.2.1. 6.2.2B Minimum requirement for BS type 1-C The same requirement as clause 6.2.2 is applicable to BS output power requirement for SBFD operation. 6.2.3B Minimum requirement for BS type 1-H The same requirement as clause 6.2.3 is applicable to BS output power requirement for SBFD operation. =================== End of Text Proposal =================== - Text Proposal in R4-2418099 on BS output power dynamics requirement (taking the new clause 6.3.3B, titled as “Total power dynamic range for SBFD” as example): =================== Start of Text Proposal =================== 6.3.3B Total power dynamic range for SBFD 6.3.3.1B General The BS total power dynamic range for SBFD is the difference between the maximum and the minimum transmit power of an SBFD symbol for a specified reference condition. For BS type 1-C this requirement shall apply at the antenna connector supporting transmission in the operating band. For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band. NOTE 1: The upper limit of the total power dynamic range for SBFD is the OFDM symbol power for a BS when transmitting on all RBs in transmission bandwidth configuration for SBFD DL subband(s) at maximum output power. The lower limit of the total power dynamic range for SBFD is the average power for single RB transmission. The OFDM symbol shall carry PDSCH and not contain RS or SSB. NOTE 2: The requirement does not apply to operation with shared spectrum channel access. 6.3.3.2 Minimum requirement for BS type 1-C and BS type 1-H The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level calculated as 10log(NRB,SBFD,DL) if one SBFD DL subband is configured, and as 10log(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured. =================== End of Text Proposal =================== - Text Proposal in R4-2418099 on transmitter intermodulation requirement (taking the new clause 6.7B, titled as “Transmitter intermodulation for SBFD” as example): =================== Start of Text Proposal =================== 6.7B Transmitter intermodulation for SBFD The transmitter intermodulation requirement for SBFD is specified over SBFD symbols. For the transmitter intermodulation requirement for SBFD, the same requirement as specified in clause 6.7 shall be applied. =================== End of Text Proposal ===================.

Also, the following was agreed in RAN4#113:

Agreement: - Draft CR’s endorsed against 38.104 does not imply that a new specification could not be agreed.

As there is an agreement that a new specification could still be agreed even though endorsing draft CR’s to TS38.104, the discussion on the TP’s is beneficial to have as the text can also be utilized on the new specification, if agreed upon.

The following was also agreed in RAN4#113:

Agreement: - For SBFD, BS type 1-C is precluded.

Based on the above agreement, there is no need to mention BS type 1-C as it is precluded for SBFD.

Remove all mentions in TP’s of BS type 1-C as it is precluded for SBFD.

# Conclusion

In this contribution, our views on the existing SBFD BS RF Tx requirements are presented. Our proposals are listed below.

- No need for RAN4 to specify additional ACLR and/or OBUE requirements.

EVM calculation should be performed for both SBFD symbols and non-SBFD symbols separately.

EVM calculation in Annex B.7 can be updated as proposed.

Remove all mentions in TP’s of BS type 1-C as it is precluded for SBFD.

# References

- R4-2419908 “Way Forward for [113][307] NR_duplex_evo_BSRF”, RAN4#113, Orlando, Florida, USA, 18th-22nd November, 2024