# 3GPP TSG-RAN WG4 Meeting #114	R4-2501333

# Athens, GR, 17th – 21st February, 2025

Agenda item:	7.23.3.1

Source:	China Telecom

Title:	Discussion on RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation

Document for:	Discussion

- Introduction

In RAN4 #113 meeting, RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation was discussed and the WF was approved [1].

In this paper, we present our views on RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation issues captured in the WF [1] and summary [2].

- Discussion

Status in the WF [1]:

#### Issue 1-1-2: Measurement methods - Agreements (from Thu AH session) - RAN4 shall specify L1 CLI measurement requirements at least for below methods: - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Additional methods can be considered based on further RAN1 agreements. - Differentiation on requirements based on different methods should be considered, based on further RAN1 conclusions.

In RAN1 #119 meeting, for L1 UE-to-UE CLI measurement and reporting, it was agreed that Method#3 is supported, Method #3 is that UE measures RSSI within UL subband, a UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig. Therefore, RAN4 shall also specify L1 CLI measurement requirements for Method#3.

Proposal 1: RAN4 shall also specify L1 CLI measurement requirements for Method#3.

Status in the summary [2]:

- Issue 1-1-4: Measurement resources - Background Agreement (RAN1#118) For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported - Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband - Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands - FFS: Number of resources that can be configured - FFS: UE behavior for measurement Agreement (RAN1#118) For L1 UE-to-UE CLI measurement and reporting, the following are supported - Wideband CLI-RSRP reporting - Wideband CLI-RSSI reporting - FFS: Subband CLI-RSSI reporting Agreement (RAN1#118-bis) For Method#1 (RSSI measurement within DL subband), the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands) is derived by excluding the frequency resources outside DL usable PRBs. - A single wideband RSSI measurement report is supported - FFS: two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband.. - Proposals - Proposal 1 (CMCC, ZTE, MTK, Samsung): - RAN4 firstly define requirements based on wideband reporting, and further discuss sub-band reporting pending on RAN1 progress. - Proposal 2 (Nokia): - RAN4 should define new requirements for the sub-band CLI-RSSI reporting supported by RAN1. Sub-band CLI-RSSI reporting is applicable for both measurement resource type #1 and type #2. - Proposal 3 (CATT, MTK): - RAN4 to evaluate whether the legacy CLI-RSSI measurement accuracy requirements are applicable for L1 CLI-RSSI measurements, when one CLI-RSSI resource occupies two discontinuous DL subbands (measurement resource type#2) - Proposal 4 (vivo): - For the wide-band reporting when measurement results from different discontinuous DL subbands need be combined (measurement resource type#2), how to combine the measurement results could be up to UE implementation. - Proposal 5 (LGE): - RAN4 need to wait for RAN1 conclusion on measurement resources to discuss whether is an impact on the requirements. - Proposal 6 (HW): - RAN4 to confirm that measurement delay and accuracy requirements for L1-CLI-RSSI are agnostic to measurement resource or reporting configuration. - Proposal 7 (vivo): - RAN4 needs to discuss the applicability of requirements when the number of RBs used for measurement is less than 48. - Proposal 8 (MTK): - For measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband, existing requirements are applicable provided that the minimum number of resources are satisfied. - Proposal 9 (Samsung): - For measurement resource type, RAN4 shall cover semi-static, aperiodic and periodic resource types following the configuration in CSI-ResourceConfig. - Recommended WF - RAN4 to define requirements for L1-CLI-RSRI and L1-SRS-RSRP measurement for semi-static, aperiodic and periodic resource types. - RAN4 to discuss whether and how L1-CLI-RSSI measurement delay or accuracy requirements would be impacted by measurement resource configuration (e.g. resource type#1 or type#2) or measurement reporting configuration (e.g. wideband or sub-band reporting), as well as whether an applicability condition on number of PRBs is needed for accuracy requirements.

In RAN1 #118bis meeting, it was agreed that for Method#1 (RSSI measurement within DL subband), the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands) is derived by excluding the frequency resources outside DL usable PRBs, and a single wideband RSSI measurement report is supported, and FFS for two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband. In RAN1 #119 meeting, for Method#1 with the frequency resources of CLI-RSSI measurement resource type#2, two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband is not supported. Therefore, RAN4 to define requirements based on a single wideband RSSI measurement reporting.

Proposal 2: RAN4 to define requirements based on a single wideband RSSI measurement reporting.

Status in the WF [1]:

#### Issue 1-1-6: Measurement period - Agreements (from Wed AH session) - For L1-SRS-RSRP, further discuss the following options based on simulation results: - Option 1: requirement specified with [1] sample measurement assumption - Option 2: requirement specified based on [1] and [3] samples according to NW configuration, e.g. timeRestrictionForChannelMeasurement - For L1-CLI-RSSI, - Confirm measurement period requirements are defined based on 1 sample, and - Clarify in the requirements that the measurement is based on DL timing.

For L1-SRS-RSRP measurement period requirement, considering the legacy L1-RSRP measurement period is defined based on higher layer parameter timeRestrictionForChannelMeasurement and both 1 and 3 samples are defined, and legacy L3 SRS-RSRP measurement period is defined based on 3 samples, in order to guarantee system performance and adapt more scenarios, we prefer to follow legacy L1-RSRP measurement period requirement to specify L1-SRS-RSRP measurement period requirement based on 1 and 3 samples according to NW configuration, e.g. timeRestrictionForChannelMeasurement.

Proposal 3: For L1-SRS-RSRP, requirement specified based on 1 and 3 samples according to NW configuration, e.g. timeRestrictionForChannelMeasurement.

Status in the summary [2]:

- Issue 1-1-7: Measurement reporting - Proposals - Proposal 1 (CATT, Nokia, CTC, vivo, HW, ZTE, E///, MTK): - At least L1-RSRP measurement reporting requirement for aperiodic reporting can be reused for the L1 based CLI measurement reporting. - Proposal 2 (LGE, CMCC, vivo, HW, ZTE, MTK, Samsung): - Focus on aperiodic reporting mode for Rel-19 CLI measurement, for other reporting modes hold on the discussion pending on further progress in RAN1. - Recommended WF - RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting. FFS on reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement. - For aperiodic reporting, L1-RSRP measurement reporting requirement for aperiodic reporting can be re-used as baseline.

L1 based UE-to-UE CLI measurement and reporting are based on existing CSI framework, measurement resource can be periodic, semi-persistent, or aperiodic measurement resource, and measurement reporting is at least aperiodic reporting. Rel-19 L1 CLI measurement reporting is different from Rel-16 CLI measurement reporting including periodic, event-triggered periodic, event triggered reporting, therefore, Rel-19 L1 CLI measurement reporting requirements need to be specified. RAN4 shall define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting, for aperiodic reporting, L1-RSRP measurement reporting requirement for aperiodic reporting can be re-used as baseline.

Proposal 4: RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting, for aperiodic reporting, L1-RSRP measurement reporting requirement for aperiodic reporting can be re-used as baseline.

Status in the summary [2]:

- Issue 1-1-8: Measurement accuracy - Proposals for L1-SRS-RSRP - Proposal 1 (LGE, CTC, ZTE, MTK): - RAN4 need to define the measurement accuracy requirements for L1-SRS-RSRP based on the simulation results at agreed side condition and measurement sample. - RAN4 not to reuse legacy UE-to-UE CLI accuracy requirements directly. - Proposal 2 (vivo): - The accuracy of L1-SRS-RSRP should be kept and the corresponding side conditions of L1-SRS-RSRP could be relaxed if necessary. - Proposal 3 (Samsung): - With residual timing error assumption case 2, single shot measurement can be specified with unified L1-SRS-RSRP accuracy for all SCSs i.e., 3dB with RF margin. - Recommended WF - Further discuss L1-SRS-RSRP accuracy based on agreed side condition(s) and measurement sample(s).

For L1 UE-to-UE CLI measurement accuracy requirements, it’s proposed to define measurement accuracy requirements for L1-SRS-RSRP and L1-CLI-RSSI measurements. Since Rel-16 CLI measurement requirements are based on L3 framework while Rel-19 CLI measurement requirements are based on L1 framework, it’s proposed not to reuse legacy UE-to-UE CLI accuracy requirements directly. The measurement accuracy requirements for L1-SRS-RSRP can be based on the simulation results and agreed side condition and measurement sample.

Proposal 5: The measurement accuracy requirements for L1-SRS-RSRP can be based on the simulation results and agreed side condition and measurement sample.

- Conclusion

The contribution presents our views on RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation, with the following proposals:

Proposal 1: RAN4 shall also specify L1 CLI measurement requirements for Method#3.

Proposal 2: RAN4 to define requirements based on a single wideband RSSI measurement reporting.

Proposal 3: For L1-SRS-RSRP, requirement specified based on 1 and 3 samples according to NW configuration, e.g. timeRestrictionForChannelMeasurement.

Proposal 4: RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting, for aperiodic reporting, L1-RSRP measurement reporting requirement for aperiodic reporting can be re-used as baseline.

Proposal 5: The measurement accuracy requirements for L1-SRS-RSRP can be based on the simulation results and agreed side condition and measurement sample.

- References

- R4-2420097, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #113, 18 – 22 November, 2024.

- R4-2418281, Topic summary for [113][223] NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #113, 18 – 22 November, 2024.