3GPP TSG RAN WG4 Meeting #114	R4-2501343

Athens, GR, 17th – 21st February, 2025

Agenda item:			7.29.4.2

Source:	vivo

Title:	Discussion on RRM impacts on DL coverage for R19 NR NTN Phase 3

Document for:	Discussion

- Introduction

In the last meeting, RAN4 further analyzed the potential RRM impacts on the objectives of downlink coverage enhancements, support of regenerative payload. The considerations from companies and the latest progress were summarized in WF [1]. Based on the progress, the issues to be settled mainly focus on the aspect of DL coverage. In this contribution, we would like to provide further analysis of the potential RRM impacts on DL coverage based on the latest progress in RAN1 and RAN plenary.

- 2.	Discussion

  - Downlink coverage enhancement

In the last meeting, RAN4 identified some potential RRM impacts if SSB periodicity enhancement is introduced. In particular, for the following case in which the SSB periodicity is larger than 160ms, the case in which the SSB periodicity is 320ms has been excluded according to the latest plenary conclusions and revised WID [2].

- Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms

According to the following conclusion reached by RAN4 last time, we believe that the corresponding RRM requirement for SSB periodicity larger than 160ms does not need to be defined.

Agreement (online): For SSB periodicity larger than 160ms if supported by other WGs: - Whether RAN4 will define the RRM requirements for SSB periodicity larger than 160ms is depending on whether it is supported by WGs.

In more detail, the potential RRM impacts to be further discussed are shown below. Next, we will provide our analysis on each bullet for the case that SSB periodicity is less than 320ms.

### Issue 1-1: SSB periodicity enhancement Agreement (online): For SSB periodicity up to 160ms: - No further discussion on the impact on initial access from the perspective of RRM requirement - Discuss whether to Modify the default values for SSB and SMTC periodicities for the following if the UE is not provided with SMTC configuration or measurement object on the corresponding frequency by NW - RRC Re-establishment: TSMTC,i assumption - RRC connection release with redirection to NR: Trs assumption - Handover, Conditional handover, RACH-less HO and satellite switch with re-sync: Trs assumption - Further discuss the details on how to update the requirement. - Further discuss the following for SSB periodicity > 20ms: - Cell Reselection: for the SSB-periodicity of 160ms, whether the values for Tmeasure_NR_xxx_enh need to be updated for DRX Cycles equal to 0.32 s and other DRX cycles - Paging interruption in RRC Idle/Inactive: whether/how to adjust the requirements when SSB periods =160 ms in NGSO scenarios - RRC re-establishment: RRC re-establishment requirements for Serving cell SSB Ês/Iot < -8dB - Handover, Conditional handover, RACH-less HO and satellite switch with re-sync - Further discuss whether and how to update the requirement

    - Whether to Modify the default values for SSB and SMTC periodicities

- RRC Re-establishment: TSMTC,i assumption

- RRC connection release with redirection to NR: Trs assumption

- Handover, Conditional handover, RACH-less HO and satellite switch with re-sync: Trs assumption

The impact of all three requirements is on the same issue: whether to update the Trs default because of SSB periodicity extension (when SMTC configuration is not provided). Now, we will take the existing requirement on RRC connection release with redirection to NR as an example, the specific Trs assumption is as follows. According to the description as highlighted, the default value (20ms) is only referenced when the SMTC is not configured. The SSB configured to 160ms can be seen as a case where the network provides the corresponding SMTC configuration. Therefore, it is not necessary to update the default values specifically for adapting the enhancement of SSB periodicity extension.

- Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the redirection command, otherwise Trs is the SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing configured for the RRC connection release with redirection. If the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection then: - the requirement in this clause is applied with Trs = 20 ms if the SSB transmission periodicity is not larger than 20 ms; otherwise, - there is no requirement if the SSB transmission periodicity is larger than 20ms.

Proposal 1: No need to update the default values for SSB and SMTC periodicities in the following requirements:

- RRC Re-establishment: TSMTC,i assumption

- RRC connection release with redirection to NR: Trs assumption

- Handover, Conditional handover, RACH-less HO and satellite switch with re-sync: Trs assumption

    - For the requirements that include SSB periodicity > 20ms

- Cell Reselection: for the SSB-periodicity of 160ms, whether the values for Tmeasure_NR_xxx_enh need to be updated for DRX Cycles equal to 0.32 s and other DRX cycles

- For enhanced requirement for cell reselection measurement, the number of samples is reduced to 1 compared to the normal requirement as shown below. Due to the enhancement of SSB periodicity, there is concern that UE may not be able to successfully decode the limited SSB opportunities within a DRX Cycle.

- Regarding this concern, we believe it has been taken into account in Note1 highlighted below, where the statement is: when SMTC > 40 ms, M2 = 2, M3 = M4 = 2.5. When the DRX Cycles are equal to 0.32s, the corresponding latency has been extended by multiple of M2, M3, and M4 respectively, and the UE only needs to decode the SSB correctly in the corresponding latency.

- Table 4.2C.2.3-2: Tdetect,NR_Intra_enh, Tmeasure,NR_Intra_enh and Tevaluate,NR_Intra_enh

| DRX cycle length [s] | Tdetect,NR_Intra_enh [s] (number of DRX cycles) | Tmeasure,NR_Intra_enh [s] (number of DRX cycles) | Tevaluate,NR_Intra_enh [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 2.56 x M2 (8 x M2)Note 1 | 0.32 x M3 (1 x M3) Note 1 | 0.96 x M4 (3 x M4) Note 1 |
| 0.64 | 5.12 (8) | 0.64 (1) | 1.92 (3) |
| 1.28 | 8.96 (7) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 2, M3 = M4 = 2.5 |  |  |  |

- Proposal 2: For Cell Reselection requirements, in the case of the SSB-periodicity of 160ms, no need to update the values for Tmeasure_NR_xxx_enh for DRX Cycles equal to 0.32 s and other DRX cycles

- Paging interruption in RRC Idle/Inactive: whether/how to adjust the requirements when SSB periods =160 ms in NGSO scenarios

#### 4.2C.2.5 Maximum interruption in paging reception UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + K*Ttarget_cell_SMTC_period ms. Where, If the target cell belongs to the same satellite as the current one, and if the target cell is known, then K = 2. If the target cell belongs to a different satellite than the current one and the target cell’s satellite is GEO, and if the target cell is known, then K = 2. If the target cell belongs to a different satellite than the current one and the target cell’s satellite is non-GEO, then K = 5 if the target cell is known.

- For Maximum interruption in paging reception, some companies propose that when the SSB cycle adopts to 160ms, this will result in a long interruption time (may exceed 1 second for LEO scenario), for which we think:

- Before discussing the enhancement of supporting extended default SSB periodicity to 160ms during initial access, we would like to point out that with the following configuration, the configurable values of SSB periodicity have already included 160ms (not in case initial access).

ssb-PeriodicityServingCell ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160},

- Correspondingly, for interruption in paging reception, the existing requirements do not exclude the case where the SSB cycle is configured to 160ms. Therefore, as we understand it, the issue already exists for legacy requirements, not as a new scenario adaptation issue. From this perspective, our preference is to keep the original requirement unchanged.

- Proposal 3: For paging interruption in RRC Idle/Inactive, no adjustments will be conducted for the requirements when SSB periods =160 ms in NGSO scenarios

- RRC re-establishment: RRC re-establishment requirements for Serving cell SSB Ês/Iot < -8dB

Table 6.2C.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell

| Serving cell SSB Ês/Iot (dB) | FR of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 6 x TSMTC, i) | K_satellite * MAX (800 ms, 13 x TSMTC, i) |
| < -8 | FR1 | N/A | k * 800 ms Note1, 3 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: K_satellite is defined in clause 9.3C.4. Note 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbor cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt. |  |  |  |

First, we would like to recall the identification of Tidentify_inter_NR, i in the R15 discussion requirement, where:

The 800ms for unknown cell case came from the current NR intra-frequency measurement requirements the minimum cell search times for FR1 is 600 ms + 200 ms (800 ms). And 13 is the minimum number of measurement samples to be obtained (including 5 SMTC periods for PSS/SSS acquisition, 5 SMTC periods for SS-RSRP measurement, and 3 SMTC periods for AGC settings). For the case specified in note 1 when TSMTC,i > 20 ms and serving cell SSB Es /Iot < -8 dB, this is because when SNR is < -8, UE may be almost losing track of the serving cell timing. In this case, it is infeasible to search all frequency layers, especially with SMTC periodicity larger than 20ms.

As some company raised in the last meeting, it can be seen that for the case with Serving cell SSB Ês/Iot (dB) < -8, the delay requirements are not based on configured SMTC periodicity but a default SMTC periodicity of 20ms. When the default SSB periodicity is extended beyond 20ms, the assumption of default SMTC periodicity of 20ms is not reasonable.

In response to this issue, the delay requirements, we think that possible changes to the current delay requirements based on extended default SMTC periodicity can be:

- Option 1: For the case with Serving cell SSB Es /Iot (dB) < -8, RAN4 to change the [when TSMTC,i >20ms] to [when TSSB >20ms] in note 1. That is, cases in which the SSB periodicity is configured to be greater than 20ms will not be defined

- Option 2: To further divide the case with Serving cell SSB Es /Iot (dB) < -8 into SMTC≤20m and SMTC>20ms, and a new definition of SMTC>20ms is given: The value of i for Tidentify_inter_NR corresponding to this scenario (SMTC>20ms) is k* N* TSMTC, i. The value of N can be further discussed

Proposal 4: For RRC re-establishment, to adopt the SSB periodicity extension, RAN4 to consider the following options for redefining the requirements

- Option 1: For the case with Serving cell SSB Es /Iot (dB) < -8, RAN4 to change the [when TSMTC,i >20ms] to [when TSSB >20ms] in note 1

- Option 2: To further divide the case with Serving cell SSB Es /Iot (dB) < -8 into SMTC≤20m and SMTC>20ms

  - When TSMTC, i ≤20ms, Tidentify_inter_NR, i for unknown cell = k * 800 ms

  - When TSMTC, i > 20ms, Tidentify_inter_NR, i for unknown cell = k *[ 800+X] ms, the value of X can be further determined

- Summary

Proposal 1: No need to update the default values for SSB and SMTC periodicities in the following requirements:

- RRC Re-establishment: TSMTC,i assumption

- RRC connection release with redirection to NR: Trs assumption

- Handover, Conditional handover, RACH-less HO and satellite switch with re-sync: Trs assumption

- Proposal 2: For Cell Reselection requirements, in the case of the SSB-periodicity of 160ms, no need to update the values for Tmeasure_NR_xxx_enh for DRX Cycles equal to 0.32 s and other DRX cycles

- Proposal 3: For paging interruption in RRC Idle/Inactive, no adjustments will be conducted for the requirements when SSB periods =160 ms in NGSO scenarios

Proposal 4: For RRC re-establishment, to adopt the SSB periodicity extension, RAN4 to consider the following options for redefining the requirements

- Option 1: For the case with Serving cell SSB Es /Iot (dB) < -8, RAN4 to change the [when TSMTC,i >20ms] to [when TSSB >20ms] in note 1

- Option 2: To further divide the case with Serving cell SSB Es /Iot (dB) < -8 into SMTC≤20m and SMTC>20ms

  - When TSMTC, i ≤20ms, Tidentify_inter_NR, i for unknown cell = k * 800 ms

  - When TSMTC, i > 20ms, Tidentify_inter_NR, i for unknown cell = k *[ 800+X] ms, the value of X can be further determined

- References

- R4-2420105, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm Incorporated

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT