# 3GPP TSG-RAN WG4 Meeting #114	R4-2501356

Athens, Greece, 17th – 21st February, 2025

Source:			Ericsson

Title:			NTN Ku bands –WI update impacts on NTN VSAT RF

Agenda item:		7.11.4

Document for:		Approval

# Introduction

In last RAN#106, the NTN KU-band WI was revised ([1]) and captured the agreement that NTN Ku-bands shall be defined for both FR1-NTN and FR2-NTN.

Based on our other contribution analyzing the WI update impacts [2], this contribution is studying the relevant NTN VSAT RF parameters for the NTN Ku-bands considering previous RAN4 agreements as well.

# Discussion

## WI updates

### FCC

### FR1-NTN and FR2-NTN

According to the WI [1],  only VSAT type of devices should be supported in the NTN Ku-bands, and not handheld devices. Also, as all NTN Ku-bands shall be defined in both FR1-NTN and FR2-NTN, NTN VSAT shall be supported in both FR1-NTN and FR2-NTN.

Therefore, to prepare TS 38.101-5 to support NTN VSAT device in FR1-NTN, the following changes shall be done:

- Update of NTN VSAT definition.

- Update of the requirement set applicability (Table 4.2-1).

- In subclause 9.3.3, replace reference to “FR2-NTN” with “NTN VSAT”.

- In subclause 9.5.2.1, replace “emissions for FR2-NTN” with “radiated emissions”

- In subclause 9.5.2.2, replace “FR2-NTN” with “VSAT NTN in the Table 9.5.2.2.1-1’s title.

- In sub-clause 9.5.2.3, replace reference to “FR2-NTN” with “NTN VSAT”.

- In subclause 9.7.1, remove “FR2-NTN”.

Our companion draft CR ([5]) captures all the proposed changes listed above.

Proposal1: Endorse draft CR R4-2501358 to prepare TS 38.101-5 to support NTN VSAT in FR1-NTN.

### Additional channel bandwidths

As highlighted in our other contribution evaluating the last update of the WI and its impacts ([2]), 5 new channel bandwidths shall be specified in FR1-NTN (25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz).

As discussed in 2.1.2, the NTN devices operating in the NTN Ku-bands are only NTN VSAT type. This means that those new channel bandwidths shall only be added to the radiated requirements.

Proposal2: Only radiated requirements shall be updated with the new channel bandwidths to be supported.

In the following table, we are analyzing the related impacts on FR1-NTN requirements.

Proposal3: Update TS 38.101-5 according to Table 1 when adding the new channel bandwidths (30kHz SCS only) mentioned in the NTN Ku-bands WI.

Table 1: TS 38.101-5 impacts when adding the new NTN Ku-bands channel bandwidths

| Requirements | Any impact? | Comments |
| --- | --- | --- |
| 9. Radiated transmitter characteristics |  |  |
| 9.1 General | No |  |
| 9.2 Transmitter power |  |  |
| 9.3 Output power dynamics |  |  |
| 9.4 Transmitter signal quality |  |  |
| 9.5 Output RF spectrum emissions |  |  |
| 9.5.1 Occupied bandwidth | Yes | Requirements for the new channel BWs shall be added |
| 9.5.2 Out of Band Emissions |  |  |
| 9.5.2.1 General | No |  |
| 9.5.2.2 SEM | No |  |
| 9.5.2.3 ACLR | Yes | Requirements for the new channel BWs shall be added based on the outcome of the coexistence study |
| 9.5.3 Spurious emissions |  |  |
| 9.5.3.1 General | Yes | Requirements for the new channel BWs shall be added |
| 9.5.3.2 On-axis spurious requirement | No |  |
| 9.5.3.3 Off-axis spurious requirement | No |  |
| 9.6 Antenna pointing accuracy and performance | No |  |
| 9.7 Additional regional requirements indicated by NS | No | No NS for the NTN Ku-bands has been proposed so far |
| 10. Radiated receiver characteristics |  |  |
| 10.1 General | No |  |
| 10.2 Polarization characteristics | No |  |
| 10.3 OTA reference sensitivity level | Yes | Requirements for the new channel BWs shall be added |
| 10.4 Maximum input level | Yes | Requirements for the new channel BWs shall be added for the Mobile VSAT only (10.4.2) |
| 10.5 Adjacent channel selectivity | Yes | Requirements for the new channel BWs shall be added for the Mobile VSAT only (10.5.1) based on the conclusion of the coexistence study |
| 10.6 Blocking characteristics | Yes | Requirements for the new channel BWs shall be added for the Mobile VSAT only (10.6.2) |
| 10.7 Spurious emissions | NA |  |
| 10.8 Receiver antenna off-axis performance | No |  |

### NTN VSAT types – new scope

#### Scope reduction

As mentioned in our other contribution on WI update ([2]), NTN VSAT types 1, 2 and 3 have been removed from the scope in the last WI. RAN4 should then only focus on specifying requirements for mobile VSAT operating with GSO and LEO satellites.

Proposal4: Aligned with the revised WI, Fixed VSAT (NTN VSAT types 1, 2 and 3) operating in the NTN Ku-bands will not be supported by TS 38.101-5.

#### Mobile VSAT types

The following NTN VSAT types have been specified in the context of the NTN Ka-band:

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 52 | Mobile VSAT communicating with GSO with electronic steering antenna. |

It should be noted that, according to the current definition, the mobile VSAT types 4 and 5 doesn’t support operation with LEO satellites.

It’s not an option to just add LEO operation to types 4 and 5. By doing so, the mobile VSAT operating in the Ka-bands with LEO satellite will automatically be covered by TS 38.101-5, which would not be correct as RAN4 didn’t study the corresponding regulations when the NTN Ka-band was introduced. This could be done later of course but not in the scope of this WI.

To solve this, there are 2 options:

- Option 1: Introduce 2 new mobile VSAT types (6 and 7) for communication with GSO and LEO (like types 1 ad 2).

- Option 2: Add communication with LEO in the types 4 and 5 definition, highlighting that communication with LEO is only for mobile VSAT operating in the NTN Ku-bands.

Introducing new mobile VSAT types (option 1) might not be desirable, it would increase the specifications complexity. Moreover, in the future, RAN4 might want to support as well mobile VSAT operating in the NTN Ka-band and communicating with both GSO and LEO, introducing then the missing requirements. Option 2 seems so preferable.

Proposal5: Update the definition of NTN VSAT types 4 and 5 according to the following table:

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. Mobile VSAT communicating with LEO with mechanical steering antenna and operating in [NTN Ku-bands]. |
|  | 52 | Mobile VSAT communicating with GSO with electronic steering antenna. Mobile VSAT communicating with LEO with electronic steering antenna and operating in [NTN Ku-bands]. |

#### Regulations reference

In our contributions to RAN4#113 ([6]) and RAN4#112bis ([7]), we shared a list of ECC Decisions and ETSI Harmonized Standards to be further studied. With the update of the WI scope, adding mobile VSAT communicating with LEO and removing fixed VSAT, the list of regulations and Harmonized Standards to be studied should be updated accordingly.

The following Table 2 and Table 3 list the relevant ECC Decisions and ETSI Harmonized Standards to be studied in the updated scope of the NTN Ku-bands WI.

Table 2: Relevant ECC Decisions to be considered for the NTN KU-bands WI

| ECC Decision | Frequency Range | Title |
| --- | --- | --- |
| ECC Decision(18)04 | 10.7 - 12.75 GHz (S-to-E) 14.0 – 14.5 GHz (E-to-S) | The harmonised use, exemption from individual licensing and free circulation and use of land based Earth Stations In-Motion (ESIM) operating with GSO FSS satellite systems in the frequency bands 10.7-12.75 GHz and 14.0- 14.5 GHz |
| ECC Decision(18)05 | 10.7 - 12.75 GHz (S-to-E) 14.0 – 14.5 GHz (E-to-S) | The harmonised use, exemption from individual licensing and free circulation and use of Earth Stations In-Motion (ESIM) operating with NGSO FSS satellite systems in the frequency bands 10.7-12.75 GHz and 14.0-14.5 GHz |
| ECC Decision(19)04 | 10.7 - 12.75 GHz (S-to-E) 12.75 – 13.25 GHz (E-to-S) | The harmonised use of spectrum, free circulation and use of earth stations on-board aircraft operating with GSO FSS networks and NGSO FSS systems in the frequency bands 12.75-13.25 GHz (Earth-to-space) and 10.7-12.75 GHz (space-to-Earth) |
| ECC Decision(05)10 | 10.7 – 11.7 GHz and 12.5 – 12.75 GHz (S-to-E) 14.0 - 14.5 GHz (E-to-S) | The free circulation and use of Earth Stations on board Vessels operating in fixed satellite service networks in the frequency bands 14-14.5 GHz |

Table 3: Relevant ETSI Harmonized Standards to be considered for the NTN KU-bands WI

| ETSI EN | Frequency Range | Title |
| --- | --- | --- |
| EN 302 340 | 10.7 - 12.75 GHz (S-to-E) 14.0 – 14.5 GHz (E-to-S) | Harmonised Standard for satellite Earth Stations on board Vessels (ESVs) operating in the 11/12/14 GHz frequency bands allocated to the Fixed Satellite Service (FSS) |
| EN 302 448 | 10.7 - 12.75 GHz (S-to-E) 14.0 – 14.25 GHz (E-to-S) | Harmonised Standard for tracking Earth Stations on Trains (ESTs) operating in the 14/12 GHz frequency bands |
| EN 302 977 | 10.7 - 12.75 GHz (S-to-E) 14.0 – 14.5 GHz (E-to-S) | Harmonised Standard for Vehicle-Mounted Earth Stations (VMES) operating in the 14/12 GHz frequency bands |
| EN 303 980 | 10.7 - 12.75 GHz (S-to-E) 14.0 – 14.5 GHz (E-to-S) | Fixed and in-motion Earth Stations communicating with non-geostationary satellite systems (NEST) in the 11 GHz to 14 GHz frequency bands; |
| EN 303 981 | 10.7 - 12.75 GHz (S-to-E) 14.0 – 14.5 GHz (E-to-S) | Fixed and in-motion Wide Band Earth stations communicating with non-geostationary satellite systems (WBES) |
| EN 302 186 | 10.7 - 12.75 GHz (S-to-E) 12.75 – 13.25 GHz (E-to-S) 14.0 – 14.5 GHz (E-to-S) | Satellite mobile Aircraft Earth Stations (AESs) operating in the 11/12/14 GHz frequency bands; |

## Regulatory study

### FCC

In last RAN4 meeting, the agreed Way Forward ([8]) listed (issue 1-2-7) the FCC regulatory requirements to be captured in TS 38.101-5.

### CEPT

In last RAN4 meeting, the agreed Way Forward ([8]) listed (issue 1-2-3) the maximum EIRP CEPT regulatory requirements to be captured in TS 38.101-5.

Different values have been specified in the ECC Decisions listed above but, as RAN4 is now focusing on Mobile VSAT, we would propose to consider the 54.5 dBW limit for the 14.0-14.5GHz frequency range from ECC Decision(18)04 (Land ESIM operation).

Even if this limit is higher than the 50dBW one specified in ECC Decision(03)04 for the 14.24-14.5 GHz frequency range, ECC Decision(18)04 considers ECC Decision(03)04 and ECC Report 272 concluded that the 54.5 max EIRP limit will be stringent enough to coexist with high intensity radiated field (HIRF) levels established by the European Aviation Safety Agency (EASA) to protect aircraft safety systems.

It should be noted that, in addition to this limit, according to ECC Decision(18)04, the Land ESIM shall implement the proposed methodologies to define protection zones towards Fixed Service stations and Radio Astronomy Service stations.

Proposal6: Consider the 54.5 dBW maximum EIRP limit for Mobile VSAT operating in NTN CEPT Ku-band(s).

For the NTN Ka-band, RAN4 only studied Land ESIM as being in the scope, Maritime ESIM and Aeronautical-ESIM were not considered. It seems the same consideration could be taken for the NTN Ku-band, i.e. only considering Land ESIM then.

Such clarification would be helpful to confirm the current scope of the coexistence study where only Land ESIM is assumed, and also to save time when studying the corresponding regulation (this was our assumption so far). Of course, this doesn’t prevent to introduce support for M-ESIM ad A-ESIM in the future.

Proposal7: RAN4 should clarify that only Land ESIM is considered for the NTN Ku-band.

Based on this proposal, looking only at Land ESIM regulatory requirements, ECC Decision(18)05 specifies also PFD limits for Land ESIM.

RAN4 didn’t capture PFD limits for the SAN RF specifications as those limits are closely related to the satellite deployment. But for the NTN Mobile VSAT, those limits are relevant and should also be captured in TS 38.101-5.

The relevant PFD limits specified in ECC Decision(18)05:

Proposal8: For NTN CEPT Ku-bands, capture that Mobile VSAT shall not exceed a PFD threshold of -169 dBW/m2/150kHz (more than 2% of the time), following ITU-R recommendation RA.769

Proposal9: For NTN CEPT Ku-bands, capture the PFD limit of -116 dBW/m2/MHz for Mobile VSAT operating in the 14.25-14.5 GHz frequency range when Fixed Service are deployed in this frequency range.

### ETSI Harmonized Standards

When studying the ETSI Harmonized standards listed above (2.1.4.3), we noticed some discrepancies which would require further discussion.

In the following, assuming RAN4 will only look at Land ESIM, we will only focus on EN 302 977, EN 303 980 and  EN 3030 981.

#### Off-axis EIRP requirements

First, EN 303 980 and EN 303 981 don’t specify any Off-axis EIRP requirements, while EN 302 977 does. It would be good to understand the motivation for this difference. Nevertheless, in last RAN4#113 meeting, it was already agreed to specify Off-axis EIRP requirement ([8]). Aligned with the previous agreement and to be compliant with all ENs, we would then make the following proposal:

Proposal10: Confirm the specification of the Off-axis EIRP requirement based on EN 302 977.

#### On-axis spurious requirements

All ENs specified the On-axis spurious emission requirements but again, with some differences between them.

First, for the “emissions disabled” and “carrier-off” states, the limit is identical, but the measurement bandwidth is different: 1MHz for EN 303 980/EN 303 981 and 100kHz for EN 302 977. As it was agreed for the NTN Ka-band, considering the 3GPP specification shall be compliant with all ENs, the 100 kHz measurement bandwidth should be preferred.

Proposal11: Specify the On-axis spurious requirements for the “emissions disabled” and “carrier-off” states according to the following table:

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 14.0 – 14.5 | 9 | 100 |

For the “carrier-on” state, on top of the difference for the measurement bandwidth, EN 303 980 and EN 303 981 specify only one requirement outside the “nominated” bandwidth while EN 302 977 specifies the requirements (inside/outside 5 times the occupied bandwidth) similarly to the NTN Ka-band ones (as captured in TS 38.101-5). It should also be noted that the definition of the “nominated” bandwidth would require some clarification:

Following the assumption that TS 38.101-5 shall be compliant with the related ETSI ENs and, to save time, we would then make the following proposal:

Proposal12: Specify the On-axis spurious requirements for the “carrier-on” state according to the following tables:

- Outside a bandwidth of 5 times the occupied bandwidth:

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 14.0 – 14.5 | 34 - K (K=10log10(N) with N the number of terminals) | 100 |

- Inside a bandwidth of 5 times the occupied bandwidth and outside the transmission bandwidth:

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 14.0 – 14.5 | 48 - K (K=10log10(N) with N the number of terminals) | 100 |

#### Off-axis spurious requirements

First, the general requirement is the same for all 3 ENs.

Proposal13: Specify the Off-axis spurious general requirement at 10 meters distance according to the following table:

| Frequency range (MHz) | EIRP Limit (dBμV/m) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 30 – 230 | 30 | 120 |
| 230 – 1000 | 37 | 120 |

For the “Emissions disabled” state and “Carrier-on” and “Carrier-off” states, there is again a difference between EN 303 980/EN 303 981 and EN 302 977/TS 38.101-5 for the NTN Ka-band, on the considered measurement bandwidth. Also, in EN 303 980/EN 303 981, the requirements have been specified up to 60.0 GHz, while it’s only up to 40.0 GHz in EN 302 977. Further discussion would be needed to converge on those requirements.

EN 303 980 / EN 303 981					EN 302 977

#### Antenna related requirements

For the antenna pointing accuracy and performance requirements and for the receiver antenna off-axis performance, EN 303 980 and EN 302 977 have also specified different requirements, which are also different from TS 38.101-5 for EN 301 980/EN303 981.

Further study and discussion would be needed to finalize those requirements in RAN4.

Proposal14: RAN4 should further discuss how to define the following requirements:  Off-axis spurious requirements (“Emissions disabled” state and “Carrier-on” and “Carrier-off” states), antenna pointing accuracy and performance requirements, and receiver antenna off-axis performance requirements.

## Summary

The following table gives an overview of all TS 38.101-5 impacts when introducing NTN KU-bands, highlighting in blue the subclauses to be updated.

# Conclusion

In this contribution, we analyzed the NTN UE RF specification TS 38.101-5 for the new NTN Ku-bands considering the last update of the WID. We made the following observations and proposals:

Proposal1: Endorse draft CR R4-2501358 to prepare TS 38.101-5 to support NTN VSAT in FR1-NTN.

Proposal2: Only radiated requirements shall be updated with the new channel bandwidths to be supported.

Proposal3: Update TS 38.101-5 according to Table 1 when adding the new channel bandwidths (30kHz SCS only) mentioned in the NTN Ku-bands WI.

Proposal4: Aligned with the revised WI, Fixed VSAT (NTN VSAT types 1, 2 and 3) operating in the NTN Ku-bands will not be supported by TS 38.101-5.

Proposal5: Update the definition of NTN VSAT types 4 and 5 according to the following table:

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. Mobile VSAT communicating with LEO with mechanical steering antenna and operating in [NTN Ku-bands]. |
|  | 52 | Mobile VSAT communicating with GSO with electronic steering antenna. Mobile VSAT communicating with LEO with electronic steering antenna and operating in [NTN Ku-bands]. |

Proposal6: Consider the 54.5 dBW maximum EIRP limit for Mobile VSAT operating in NTN CEPT Ku-band(s).

Proposal7: RAN4 should clarify that only Land ESIM is considered for the NTN Ku-band.

Proposal8: For NTN CEPT Ku-bands, capture that Mobile VSAT shall not exceed a PFD threshold of -169 dBW/m2/150kHz (more than 2% of the time), following ITU-R recommendation RA.769

Proposal9: For NTN CEPT Ku-bands, capture the PFD limit of -116 dBW/m2/MHz for Mobile VSAT operating in the 14.25-14.5 GHz frequency range when Fixed Service are deployed in this frequency range.

Proposal10: Confirm the specification of the Off-axis EIRP requirement based on EN 302 977.

Proposal11: Specify the On-axis spurious requirements for the “emissions disabled” and “carrier-off” states according to the following table:

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 14.0 – 14.5 | 9 | 100 |

Proposal12: Specify the On-axis spurious requirements for the “carrier-on” state according to the following tables:

- Outside a bandwidth of 5 times the occupied bandwidth:

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 14.0 – 14.5 | 34 - K (K=10log10(N) with N the number of terminals) | 100 |

- Inside a bandwidth of 5 times the occupied bandwidth and outside the transmission bandwidth:

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 14.0 – 14.5 | 48 - K (K=10log10(N) with N the number of terminals) | 100 |

Proposal13: Specify the Off-axis spurious general requirement at 10 meters distance according to the following table:

| Frequency range (MHz) | EIRP Limit (dBμV/m) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 30 – 230 | 30 | 120 |
| 230 – 1000 | 37 | 120 |

Proposal14: RAN4 should further discuss how to define the following requirements:  Off-axis spurious requirements (“Emissions disabled” state and “Carrier-on” and “Carrier-off” states), antenna pointing accuracy and performance requirements, and receiver antenna off-axis performance requirements.

# References

- RP-243314, Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT

- R4-2501354, NTN Ku bands - WI update impacts, Ericsson

- R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General, Eutelsat

- R4-2416593, Way Forward for [112bis][313] NR_NTN_Ku_Band_General, Eutelsat

- R4-2501358, Draft CR to TS 38.101-5 – Needed updates to support NTN VSAT in FR1-NTN, Ericsson

- R4-2416143, NTN Ku-band – VSAT RF, Ericsson

- R4-2418912, NTN Ku-band – VSAT RF, Ericsson

- R4-2419790, Way Forward for [112bis][314] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL